3GPP TSG-RAN WG2 Meeting #127b	R2-2408308

Hefei, China, Oct 14th – 18th, 2024

Agenda item:		8.7.5

Source:		Lenovo

Title:		AM RLC enhancement

Document for:		Discussion and Decision

- Introduction

In this contribution we discuss RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

- Discussion

The Rel-19 Work Item on XR Phase 3 was agreed in RP-240791.

The Rel-19 XR Phase 3 objectives are as follows: Specify Enhancements for Scheduling, as follows: - Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

In RAN2#126, the following discussion on RLC related enhancements for Rel-19 took place with the following agreements:

- For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives: - Rx initiated approach - Tx initiated approach - RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer. - For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs - Rx side updates state variables according to the information from Tx side - For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - FFS how Rx side determines that an SDU should be abandoned - To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism - Impact on capacity should be considered - RAN2 focuses on the enhancements for UL traffic

In RAN2#127, the following discussion on RLC related enhancements for Rel-19 took place with the following agreements:

- Any solution should ensure that windows at Tx side and Rx side are not out of sync. As a baseline, we assume Rx window advances before Tx window advances. FFS if for Tx approach window sync needs to be achieved in another way, e.g. advancing Tx window first. - In the RX-initiated approach for avoiding unnecessary retransmissions, RLC receiver abandons missing SDUs like already done by PDCP, i.e. based on a timer. - In addition to Tx and Rx approaches, RAN2 will consider a combined Rx and Tx approach, where: Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach. Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach.

2.1 Avoiding Unnecessary Retransmissions

In the previous meetings, it has been agreed that for avoiding unnecessary retransmissions, two approaches - Tx-based Approach and Rx-based Approach - will be considered.

The Tx-based approach consists of the RLC transmitter informing its peer entity of the obsolete SDUs and then stops the retransmission of such obsolete SDUs. There are different flavors of this approach, as follows:

- A: Discard based on RLC SN gap report from Tx RLC entity

In this option, the PDCP transmitting entity provides information to the RLC transmitter of the PDUs whose discardTimer or discardTimerforLowImportance has expired. The RLC transmitter then provides this information of the obsolete SDUs within a new control PDU to the RLC receiver (e.g., Outdated SN Report). In response to such a report, the RLC receiving entity transmits a status report containing an ACK for those SDUs indicated as outdated/ obsolete and updates its reception window, the state variables, timers etc. as per legacy procedure. The RLC transmitter upon reception of the status report, discards the obsolete SDUs as per legacy procedure.

- B: Proactive discard in RLC Tx:

Here, the PDCP transmitting entity provides information to the RLC transmitter of the PDUs whose discardTimer or discardTimerforLowImportance has expired. The RLC transmitter provides this information of the SDUs for discard within a new control PDU to the RLC receiver (e.g., Discard SN Report). The RLC transmitter then proactively discards these SDUs and updates its transmission window. The RLC receiver upon reception of the discard SN report, updates its reception window, state variables, timers etc.

- C: Discard based on PDCP SN gap report from Tx PDCP entity

With this option, the PDCP transmitter informs the PDCP receiverof the discarded packets, e. g., PDCP SN gap report to the receiving PDCP entity. The receiving PDCP entity may further notify the RLC receiving entity to update its receiving window respectively, i.e., update the corresponding state variables, reassemble and deliver RLC SDUs to upper layer and start/stop t-Reassembly as needed based on the provided information on discarded packets. The RLC receiver can additionally transmit a status report to its peer entity based on which the RLC transmitter can update its window.

The Rx-based approach consists of the RLC receiver indicating to its peer entity the SDUs that are abandoned, i.e., those RLC SDUs whose discardTimer or discardTimerforLowImportance has expired or those SDUs that are no longer anticipated for by PDCP.

- D: Discard based on a new timer in Rx RLC entity

The RLC receiver can maintain an additional timer that dictates how long the RLC AM receiver needs to try to receive/ recover a packet, such that upon expiry of the timer, the RLC receiver transmits a status report to its peer entity containing an ACK for those SDUs that are abandoned. The timer can be started when the RLC delivers a gap (i.e., a RLC SDU (corresponding to a PDCP SDU) out of order) to PDCP. The intention is to maintain the synchronization between the start of the new RLC timer and the PDCP reordering timer. The RLC receiver then updates its reception window, state variables and timers as per legacy procedure. The RLC transmitter, upon reception of the status report, discards the SDUs indicated as ACKed (i.e., the abandoned SDUs) and updates its transmission window as per legacy procedure.

Additionally in the last meeting, some companies have proposed a combined Rx and Tx approach wherein the RLC transmission window is updated by means of discard indication from the PDCP transmitter. Separately, the RLC receiver maintains a new timer as in the Rx-initiated approach to give up missing RLC PDUs. This combined approach no longer requires the RLC transmitter to send a RLC SN gap report (from RLC transmitter to receiver) or a status report containing dummy ACKs (from RLC receiver to transmitter) since the RLC windows are independently updated. In our view, this approach is another flavour of the Rx-initiated approach and may actually cause the RLC transmission window and receiver windows to be out of sync. PDCP Control PDUs are not assigned a PDCP SN, so when the PDCP transmitter indicates discard information to the RLC transmitter, the PDCP Control PDUs are not included. That is, the RLC transmitter only updates its window to the first RLC PDU corresponding to a PDCP Control PDU for which an ACK was not received. While, the RLC receiver has no information of the RLC PDUs associated with a PDCP Control PDU, so the receiver window will be updated regardless of such PDUs according to the new timer. Without a status report, the RLC transmission window and receiver windows will now be mismatched. Hence, the combined approach is not supported.

The Tx-initiated as well as Rx-initiated approaches both work to resolve the issue of retransmission of obsolete RLC PDUs. With the Tx-initiated approach, there have been some comparison to the MRW procedure in UMTS where the Tx-initiated approach may be more complex if specified similar to the MRW procedure (due to the need for reliably transmitting the MRW SUFI and receiving the MRW_ACK SUFI). In our view, the Tx-initiated approach may not be so comparable to MRW as RLC no longer concatenates packets, and hence the new RLC ‘Discard SN report’ may be more easily specified (e.g., similar to PDCP SN gap report). Here, additional timers or thresholds may be configured to ensure that the discard SN report is reliably received by the RLC receiver without the need to explicitly specify an additional discard SN report ACKs (corresponding to the MRW_ACK SUFI).

The Rx-initiated approach poses a risk of abandonment of RLC SDUs corresponding to PDCP Control PDUs. Hence, some additional mechanism is required to prevent/ combat the loss of PDCP Control PDUs. Additionally, there may be some discrepancy between the start of the new timer in RLC receiver and the start of t-Reordering which causes some mismatch between the RLC receiving window and PDCP reordering window. Furthermore, if t-StatusProhibit is running, the update of transmission window at RLC may be stalled after the RLC receiver has already received the obsolete PDU indication from PDCP, leading to additional delays on the user-plane. Hence, the Tx-initiated approach is preferred.

Here is analysis for above solutions:

| Description | Potential Specification Impact | Analysis. |
| --- | --- | --- |
| A: Discard based on RLC SN gap report from Tx RLC entity | 1.Tx RLC entity provides information of the obsolete SDUs in a new control PDU to the RLC receiver based on discard indication from PDCP Tx. 2.Rx RLC entity updates variable, timers etc. based on the new RLC control PDU and transmits a status report to Tx RLC entity. 3. Tx RLC entity updates the Tx window based on the status report. | Tx RLC entity can provide accurate information on discarded RLC SDUs. |
| B: Proactive Discard in RLC Tx | 1.Tx RLC entity provides information of the obsolete SDUs in a new control PDU to the RLC receiver based on discard indication from PDCP Tx, and updates the RLC Tx window. 2.Rx RLC entity updates variable, timers etc. based on the new RLC control PDU. | Tx RLC entity can provide accurate information on discarded RLC SDUs. RLC Tx window gets updated before the RLC Rx window which may cause discrepancies if the RLC SN gap report is not received quick enough. |
| C: Discard based on PDCP SN gap report from Tx PDCP entity | 1.Rx PDCP entity provides information of the obsolete SDUs to RLC receiver based on discard SN gap from PDCP Tx. 2. Rx RLC entity updates variables, timers etc. based on the received information and transmits a report to Tx RLC entity. 3. Tx RLC entity updates its transmission window based on the received status report. | Unified and accurate discard information is used for discarding in both Rx PDCP entity and Rx RLC entity, and synchronous discard processing is achieved. |
| D: Discard based on a new timer in Rx RLC entity | 1. RLC receiver starts an additional timer if an out of order PDCP SDU is delivered to PDCP Rx entity. 2. Upon expiry of the timer, Rx RLC entity updates its receiving window accordingly and transmits a status report to Tx RLC entity. | It is beneficial in case the discard information cannot be timely received from Tx RLC entity. The Rx-initiated approach poses a risk of abandonment of RLC SDUs corresponding to PDCP Control PDUs. |

Observation 1. It is benefical to enable the discard of RLC PDUs associated with an SN whose discardTimer has expired in order to prevent the unnecessary retransmission of such PDUs.

Proposal 1. RAN2 to consider the following options for Tx-Initiated Approach:

-	Discard based on RLC SN gap report from Tx RLC entity

-  Proactive Discard in RLC Tx

-	Discard based on PDCP SN gap report from Tx PDCP entity

Proposal 2. RAN2 to consider to discard based on a new timer in Rx RLC entity for Rx-Initiated Approach. The new timer is started upon delivery of an out of order packet to PDCP.

2.2 Timely Retransmissions

RLC-AM feedback based on current polling parameters are not well adapted for short packet delay budgets applicable to XR traffic. The data volume of PDU set of XR traffic is variable, i.e., the number of PDUs or the size of the PDUs of a PDU set is variable. One possibility, the polling parameter and t-PollRetransmit is set to small value for triggering more frequent polling to the RLC receiver, and gNB frequently transmits status report, which can cause higher overhead.

In legacy, a polling can be triggered if both the transmission buffer and the retransmission buffer become empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU, or if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling). However, if there are multiple PDU sets in transmission or retransmission buffer, the corresponding polling may not be triggered after the transmission of a PDU set.

And the legacy receiving side of an AM RLC entity shall trigger a STATUS report when t-Reassembly expires upon

detection of reception failure of an AMD PDU. However, if the SN of lost RLC PDUs is continuous, the receiver RLC cannot detect the data loss till the correct reception of a RLC PDU with higher RLC SN, therefore the status report may not be triggered timely in this case.

Hence, the RLC retransmission based on current RLC-AM feedback mechanism may not be timely triggered, which causes the packets to arrive too late at the receiver, thus impacting the user experience.

Observation 2. It is difficlut to set the polling related parameters and status prohibit timer for a timely status report, to adapt for XR traffic with short packet delay budgets and variable data volume of PDU set, without significantly increasing the overhead.

Observation 3. If there are multiple PDU sets in transmission or retransmission buffer, the polling may not be triggered after the transmission of a PDU set.

Observation 4. If the SN of lost RLC PDUs is continuous, the status report may not be triggered in a timely manner. If gNB frequently transmits status reports, it causes higher overhead.

To avoid the late RLC retransmission, RLC transmitter may trigger early RLC retransmission for data based on the delay status, provided the transmitting entity did not receive a status report, assuming polling has been transmitted to the peer RLC entity. For example, the PDCP entity delivers a delay-critical indication to the corresponding AM RLC entity, and the delay critical data may be determined based on a separately configured threshold, different than R18 value.

Additionally, another condition that can be considered to enable autonomous retransmissions is the PSI value. For example, only those packets with PSI value greater than a preconfigured threshold are eligible for autonomous retransmissions. Furthermore, the cancellation of an autonomous retransmission can be considered when such a retransmission was triggered and an ACK (e.g., HARQ ACK or a RLC status-report containing an ACK for the corresponding PDU) is received before it is submitted to lower layers. Lastly, RAN2 can also look into scheduling enhancements (e.g., LCP enhancements) for packets that are autonomously retransmitted to avoid any delays in transmission of such data before the discardTimer expires.

In addition, the enhancement of polling mechanism may be considered for the early RLC retransmission. For example, the transmitter RLC entity can trigger a new polling based on the end of PDU set, and if the remaining discard time of the data in the PDU set is below a threshold. Another condition could be that there is more than one PDU set in transmission buffer after transmission of a PDU set, in which case, the AM RLC transmitter entity may poll its peer AM RLC receiver entity in order to trigger STATUS reporting at the peer AM RLC entity. And if gNB receives the polling and a t-StatusProhibit is running, it may ignore the running timer and send a status report to the peer AM RLC receiver entity at least when the status report is triggered by this new polling condition, and if there is a NACK status report to transmit.

Autonomous retransmissions can congest the link if configured poorly, due to excessive transmissions of redundant RLC PDUs. Hence, autonomous retransmission can be network controlled to provide additional control when this function is used, e.g., via RRC configuration for each RLC entity. We think both enhancements can be used together without it conflicting the other, each may use a different threshold. The new polling trigger may avoid/reduce the number of early RLC retransmission.

Proposal 3. To achieve timely retransmissions on RLC layer for XR traffic, RAN2 to consider the following:

- RLC transmitter triggers a new polling based on PDU set for data with remaining delay below a threhold.

- RLC transmitter triggers autonomous RLC retransmission for data with remaining delay below a threhold.

- Additional conditions such as PSI-based threshold for triggering autonomous RLC retransmissions.

Proposal 4. The enhanced option(s) to achieve timely retransmissions on RLC layer is configured per RLC entiy by RRC.

- Conclusion

In this contribution, we discuss the RLC AM enhancement for XR traffic with small delay budget. We have the following observation and proposals:

Observation 1. It is benefical to enable the discard of RLC PDUs associated with an SN whose discardTimer has expired in order to prevent the unnecessary retransmission of such PDUs.

Observation 2. It is difficlut to set the polling related parameters and status prohibit timer for a timely status report, to adapt for XR traffic with short packet delay budgets and variable data volume of PDU set, without significantly increasing the overhead.

Observation 3. If there are multiple PDU sets in transmission or retransmission buffer, the polling may not be triggered after the transmission of a PDU set.

Observation 4. If the SN of lost RLC PDUs is continuous, the status report may not be triggered in a timely manner. If gNB frequently transmits status report, it causes higher overhead.

Proposal 1. RAN2 to consider the following options for Tx-Initiated Approach:

-	Discard based on RLC SN gap report from Tx RLC entity

-  Proactive Discard in RLC Tx

-	Discard based on PDCP SN gap report from Tx PDCP entity

Proposal 2. RAN2 to consider to discard based on a new timer in Rx RLC entity for Rx-Initiated Approach. The new timer is started upon delivery of an out of order packet to PDCP.

Proposal 3. To achieve timely retransmissions on RLC layer for XR traffic, RAN2 to consider the following:

- RLC transmitter triggers a new polling based on PDU set for data with remaining delay below a threhold.

- RLC transmitter triggers autonomous RLC retransmission for data with remaining delay below a threhold.

- Additional conditions such as PSI-based threshold for triggering autonomous RLC retransmissions.

Proposal 4. The enhanced option(s) to achieve timely retransmissions on RLC layer is configured per RLC entiy by RRC.

- References

TS 38.322 v 18.0.0

R2-2402734, AM RLC enhancement, Lenovo