3GPP TSG-RAN WG2 #127bis	R2-2408313

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	   8.1.2.3

Source:		Lenovo

Title:	LCM for AIML based positioning with UE-sided model

Document for:		Discussion and Decision

# Introduction

In this paper we discuss the 1st priority use case for LCM for AIML based positioning with UE-sided model, i.e., case 1.

- (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

# Discussion

## 2.1	General

In the previous RAN2 meeting, for the LCM for beam management use case, companies have reached high level understanding as captured in the figure below.

For AI based positioning use case, in general, we believe the similar logic can be applied with three main phases

- Phase 1: Training data collection

- Phase 2: AI based positioning capability report

- Phase 3: Applicability information report based on necessary configuration

- Phase 4: Activation of AI based positioning based on necessary configuration

Similar as the beam management use case, the LCM for AI based positioning with UE-sided model includes the following three phases:

  - Phase 1: Training data collection

  - Phase 2: AI based positioning capability report

  - Phase 3: Applicability information report based on necessary configuration

  - Phase 4: Activation of AI based positioning based on necessary inference configuration

In the following sections, we will discuss the aspects that are positioning specific and distinguished from the beam management use case.

## 2.1	Training data collection

RAN1 Agreement For training data collection of Case 1 and 2a, in terms of DL PRS configuration for collecting training data, RAN1 study the following options on assistance data, using legacy mechanisms as a starting point: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB

In the previous RAN1 discussion, RAN1 has considered using the following two options to provide necessary DL PRS configuration. And in our view, both can be supported based on legacy LPP Assistance Data Transfer procedure (RequestAssistanceData/ProvideAssistanceData) or LPP Assistance Data Delivery Procedure (ProvideAssistanceData)

Option A. 	(UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE.

For Option A, UE can send a RequestAssistanceData message to LMF requesting on-demand DL PRS configuration, including relevant information, e.g., time duration, PRS characteristics. Then LMF will provide the demanded DL PRS configuration to UE via ProvideAssistanceData message.

Option B. 	(LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

For Option B, LMF could provide the DL PRS configuration to UE via ProvideAssistanceData message directly.

The legacy LPP Assistance Data Transfer procedure (via RequestAssistanceData/ ProvideAssistanceData messages) is taken as the baseline for UE to request DL PRS configuration from LMF for training data collection.

The legacy LPP Assistance Data Delivery procedure (via ProvideAssistanceData messages) is taken as the baseline for LMF to proactively provide DL PRS configuration to UE for training data collection.

## 2.2	AIML based positioning capability and applicable functionalities

RAN2#125bis

Agreements

1.	Which AI/ML-enabled Features/FGs and functionalities are supported should be standardized. The details wait for RAN1’s progress.   “supported” means that the UE is capable of supporting the functionality and doesn’t mean neccesarily that the UE has the model available.  FFS what functionality refers to.

2.	Supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability.

Agreements for positioning and beam management

- Support proactive reporting of UE-sided applicable functionality, e.g., the UE reports its applicable AI/ML functionalities via UAI message/LPP message.

- Support reactive reporting of UE-sided applicable functionality.  The NW configures AI/ML functionalities via RRC/LPP message.  FFS what the configuration contains. FFS how to report applicable functionality and what is applicable functionality

3	FFS how the two approaches will be specified and whether we can combine them into one procedure.    FFS how to report applicable functionality, what is applicable functionality, how the UE determines which function is applicable or not (if it is needed)

In the previous RAN2 meeting, it was agreed to report supported functionalities (which is considered static) in UE capability, and report applicable functionalities (which is considered semi-static) via UAI/LPP message. When it comes to AIML based positioning with UE-sided model, we believe the legacy LPP procedure related to capability transfer can be taken as a baseline.

- LPP capability transfer procedure: UE transmits LPP ProvideCapabilities message to LMF in response to a LPP RequestCapabilities message received from LMF. Such procedure can be enhanced to transfer the static UE capabilities related to supported AIML based POS functionalities, or reactively report of applicable AIML based POS functionalities.

- LPP capability indication procedure: UE can provide unsolicited capabilities to the LMF without being requested by LMF. Such procedure can be enhanced to transfer the semi-static applicable functionalities.

5.1 Procedures related to capability transfer The purpose of the procedures that are grouped together in this clause is to enable the transfer of capabilities from the target device to the server. Capabilities in this context refer to positioning and protocol capabilities related to LPP and the positioning methods supported by LPP. These procedures instantiate the Capability Transfer transaction from TS 36.305 [2] and TS 38.305 [40]. ### 5.1.1 Capability Transfer procedure The Capability Transfer procedure is shown in Figure 5.1.1-1. Figure 5.1.1-1: LPP Capability Transfer procedure 1. The server sends a RequestCapabilities message to the target. The server may indicate the types of capability needed. 2. The target responds with a ProvideCapabilities message to the server. The capabilities shall correspond to any capability types specified in step 1. This message shall include the endTransaction IE set to TRUE. ### 5.1.2 Capability Indication procedure The Capability Indication procedure allows the target to provide unsolicited capabilities to the server and is shown in Figure 5.1.2-1. Figure 5.1.2-1: LPP Capability Indication procedure 1. The target sends a ProvideCapabilities message to the server. This message shall include the endTransaction IE set to TRUE.

The legacy LPP capability transfer procedure (i.e., via LPP RequestCapabilities/ ProvideCapabilities messages) is taken as the baseline for

  - Transfer of supported AIML based POS functionalities, and

  - Reactive transfer of applicable AIML based POS functionalities.

The legacy LPP capability indication procedure (i.e., via LPP ProvideCapabilities message) is taken as the baseline for proactive transfer of applicable AIML based POS functionalities from UE to LMF.

## 2.3	Configuration and report of AIML inference output

For AIML based positioning with UE-sided model, the inference output will be either estimated UE location using the Direct AIML positioning method (case 1) or enhanced intermediate measurements like LOS/NLOS indicator (case 2a). The inference output will be sent from UE to LMF according to the configuration/request by LMF. To support the configuration and report of inference output from UE to LMF, we believe the legacy LPP location information transfer procedure can be taken as a baseline, which has been used to support UE based positioning or UE assisted LMF based positioning in the legacy.

TS 37.355 5.3 Procedures related to Location Information Transfer The purpose of the procedures in this clause is to enable the server to request location measurement data and/or a location estimate from the target, and to enable the target to transfer location measurement data and/or a location estimate to a server in the absence of a request. These procedures instantiate the Location Information Transfer transaction in TS 36.305 [2] and TS 38.305 [40]. NOTE: The service layer (e.g. NAS or OMA SUPL ULP) would be used to transfer information associated with a location request from a target to a server (MO-LR). 5.3.1 Location Information Transfer procedure The Location Information Transfer procedure is shown in Figure 5.3.1-1. Figure 5.3.1-1: LPP Location Information transfer procedure 1. The server sends a RequestLocationInformation message to the target to request location information, indicating the type of location information needed and potentially the associated QoS. 2. The target sends a ProvideLocationInformation message to the server to transfer location information. The location information transferred should match or be a subset of the location information requested in step 1 unless the server explicitly allows additional location information. If step 3 does not occur, this message shall set the endTransaction IE to TRUE. 3. If requested in step 1, the target sends additional ProvideLocationInformation messages to the server to transfer location information. The location information transferred should match or be a subset of the location information requested in step 1 unless the server explicitly allows additional location information. The last message shall include the endTransaction IE set to TRUE.

The legacy LPP location information transfer procedure (i.e., RequestLocationInformation / ProvideLocationInformation messages) is taken as the baseline to support the inference configuration and inference output report of AIML based POS with UE-sided model (case 1).

As an example of direct AI/ML positioning, RF Fingerprinting is a well-known positioning technique, which leverages RSS measurements collected offline which are then mapped to ground truth locations. Any subsequent measurements received by the UE during the online phase are then mapped to these ground truth locations, in order to determine a UE’s location. Since legacy positioning discussion and information exchange are grouped in a per positioning method manner (e.g., DL-TDOA, A-GNSS, Enhanced Cell ID etc.) and any direct AI/ML positioning method (e.g., fingerprinting) can be regarded as a new positioning method after all since new potential channel measurements may be developed by RAN1 (pending RAN1 progress)’, the signalling design can follow legacy principle, e.g., for each direct AI/ML positioning method, introducing new IE in existing LPP message carrying all relevant capability/assistance information.

#### – RequestLocationInformation The RequestLocationInformation message body in a LPP message is used by the location server to request positioning measurements or a position estimate from the target device. -- ASN1START RequestLocationInformation ::= SEQUENCE { criticalExtensions CHOICE { c1 CHOICE { requestLocationInformation-r9 RequestLocationInformation-r9-IEs, spare3 NULL, spare2 NULL, spare1 NULL }, criticalExtensionsFuture SEQUENCE {} } } RequestLocationInformation-r9-Ies ::= SEQUENCE { commonIEsRequestLocationInformation CommonIEsRequestLocationInformation OPTIONAL, -- Need ON a-gnss-RequestLocationInformation A-GNSS-RequestLocationInformation OPTIONAL, -- Need ON otdoa-RequestLocationInformation OTDOA-RequestLocationInformation OPTIONAL, -- Need ON ecid-RequestLocationInformation ECID-RequestLocationInformation OPTIONAL, -- Need ON epdu-RequestLocationInformation EPDU-Sequence OPTIONAL, -- Need ON ..., [[ sensor-RequestLocationInformation-r13 Sensor-RequestLocationInformation-r13 OPTIONAL, -- Need ON tbs-RequestLocationInformation-r13 TBS-RequestLocationInformation-r13 OPTIONAL, -- Need ON wlan-RequestLocationInformation-r13 WLAN-RequestLocationInformation-r13 OPTIONAL, -- Need ON bt-RequestLocationInformation-r13 BT-RequestLocationInformation-r13 OPTIONAL -- Need ON ]], [[ nr-ECID-RequestLocationInformation-r16 NR-ECID-RequestLocationInformation-r16 OPTIONAL, -- Need ON nr-Multi-RTT-RequestLocationInformation-r16 NR-Multi-RTT-RequestLocationInformation-r16 OPTIONAL, -- Need ON nr-DL-AoD-RequestLocationInformation-r16 NR-DL-AoD-RequestLocationInformation-r16 OPTIONAL, -- Need ON nr-DL-TDOA-RequestLocationInformation-r16 NR-DL-TDOA-RequestLocationInformation-r16 OPTIONAL -- Need ON ]] } -- ASN1STOP

Legacy LPP protocol is designed to serve different positioning methods, and information carried in LPP messages is also grouped in a per positioning method way.

RAN2 considers direct AI/ML positioning method (e.g., case 1) as a new separate positioning method.

## 2.4	Activating a non-applicable AIML functionality

In case 1, when LMF requests the UE to estimate its location using AIML based method that was reported to be applicable, it is possible that the AIML based positioning functionality becomes non-applicable. In this case, from UE point of view, there are two possible options:

- Option 1: UE cannot perform the AIML based positioning, and reply an LPP Error message to LMF indicating the AIML functionality is not applicable now, or

- Option 2: UE can be configured with a default non-AIML based positioning method, and UE will fallback to the default non-AIML based positioning and still provide the estimated UE location.

Option 1 will be similar as the handling in the legacy when some or all of the requested information in RequestLocationInformation is not supported. Option 2 will allow UE to still perform location estimation as requested by LMF, while a default non-AIML method needs to be configured in advance.

If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, RAN2 discusses the following two options:

  - Option 1: UE cannot perform the AIML based positioning, and reply an LPP Error message to LMF indicating the AIML functionality is currently not applicable, or

  - Option 2: UE can be configured with a default non-AIML based positioning method, and UE will fallback to the default non-AIML based positioning and still provide the estimated UE location.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	Legacy LPP protocol is designed to serve different positioning methods, and information carried in LPP messages is also grouped in a per positioning method way.

Based on the discussion above, we propose:

Proposal 1	Similar as the beam management use case, the LCM for AI based positioning with UE-sided model includes the following three phases:

a.	Phase 1: Training data collection

b.	Phase 2: AI based positioning capability report

c.	Phase 3: Applicability information report based on necessary configuration

d.	Phase 4: Activation of AI based positioning based on necessary inference configuration

Proposal 2	The legacy LPP Assistance Data Transfer procedure (via RequestAssistanceData/ ProvideAssistanceData messages) is taken as the baseline for UE to request DL PRS configuration from LMF for training data collection.

Proposal 3	The legacy LPP Assistance Data Delivery procedure (via ProvideAssistanceData messages) is taken as the baseline for LMF to proactively provide DL PRS configuration to UE for training data collection.

Proposal 4	The legacy LPP capability transfer procedure (i.e., via LPP RequestCapabilities/ ProvideCapabilities messages) is taken as the baseline for

a.	Transfer of supported AIML based POS functionalities, and

b.	Reactive transfer of applicable AIML based POS functionalities.

Proposal 5	The legacy LPP capability indication procedure (i.e., via LPP ProvideCapabilities message) is taken as the baseline for proactive transfer of applicable AIML based POS functionalities from UE to LMF.

Proposal 6	The legacy LPP location information transfer procedure (i.e., RequestLocationInformation / ProvideLocationInformation messages) is taken as the baseline to support the inference configuration and inference output report of AIML based POS with UE-sided model (case 1).

Proposal 7	RAN2 considers direct AI/ML positioning method (e.g., case 1) as a new separate positioning method.

Proposal 8	If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, RAN2 discusses the following two options:

a.	Option 1: UE cannot perform the AIML based positioning, and reply an LPP Error message to LMF indicating the AIML functionality is currently not applicable, or

b.	Option 2: UE can be configured with a default non-AIML based positioning method, and UE will fallback to the default non-AIML based positioning and still provide the estimated UE location.

# Reference