3GPP TSG-RAN WG2 Meeting #127bis	R2-2408326

Hefei, China, Oct 14th – 18th, 2024

Source:	NTT DOCOMO, INC.

Title:	Discussions and evaluations on RRM measurement prediction

Agenda Item:	8.3.2

Document for: 	Discussion and Decision

- Introduction

At RAN#102 meeting, the SID on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”[1] was approved. The SID describes the study scope of the AI/ML-aided mobility enhancements, as below,

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects:

- AI/ML based RRM measurement and event prediction,

  - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

    - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

  - HO failure/RLF prediction (UE sided model) [RAN2]

  - Measurement events prediction (UE sided model) [RAN2]

- Study the need/benefits of any other UE assistance information for the network side model [RAN2]

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

  - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

  - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID

- Potential specification impacts of AI/ML aided mobility [RAN2]

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

At RAN2 #125bis meetings, it was agreed to start the simulations for the following items regarding the RRM measurement prediction on both FR1 and FR2 and three cases will be considered for the cell-level measurement predictions [2].

Agreements to start evaluations

- FR1-to-FR1

  - Focus on intra-frequency in time domain prediction for the purpose of measurement reduction

  - Study inter-frequency scenario in terms of which scenarios can be studied without requiring new channel model and also resolving any simulation assumptions (if possible).

- FR2-to-FR2

  - Focus on intra-frequency

  - Perform evaluation both in time and spatial domain

Agreements

- For cell level measurement prediction model, at least consider the following cases:

  - Case 1: To predict beam level results, then generate cell level results based on the predicted beam results;

  - Case 2: To directly predict cell level results based on cell level results.

  - Case 3: To directly predict cell level results based on beam level results.

At RAN2 #126 meetings, six cases and their priorities were agreed, and the detailed simulation assumptions were listed [4].

The following is agreed

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In this paper, we submit our evaluation results about the 3 high-priority cases and discuss the candidate scope of the following studies.

- AI/ML-based predictors for each scenario

  - Case 2, FR1-to-FR1 intra-frequency temporal domain case B

The study goal is to reduce the measurement overhead. It is expected that the UE can lessen the actual measurement to 1/n of that without AI/ML-based predictions. The AI/ML-based predictor can output the ones not measured by the UE.

Figure 1 shows the diagrams of the AI/ML models for all 3 sub-cases with different inputs and outputs. For all cases evaluated, we consider a Transformer-based AI/ML model and cluster-based approaches, where the inputs to the AI/ML model consist of the measurements from multiple cells, and the outputs are the predictions of the measurements for corresponding cells. Based on the UE capabilities, we consider the cluster with 7 cells.

The significant differences between Sub-Case 1 and Sub-Case 2 are the position of the beam consolidation/selection and Layer 3 filter. In contrast, for Sub-Case 3, the behavior of the beam consolidation/selection and Layer 3 filter are implicitly included in the AI/ML models.

Figure 1 Diagram of AI/ML models for 3 sub-cases of Case 2.

Figure 2 Sequence of the measurements and predictions for Case 2.

Figure 2 demonstrates the sequence of the measurements and predictions for Case 2. After the new measurement is obtained, UE will use it and the historical ones to predict a few future results, which will be omitted to save the overhead. In our schemes, the predictions are not looped back to the input of the AI/ML. All the inputs to the model are actually measured by the UE.

  - Case 3, FR1-to-FR1 inter-frequency (frequency domain)

The study goal of this case is also to reduce the measurement overhead, especially the measurement gap needed for the measurements on other frequency bands. Based on the measurements on the band of the serving cell, the AI/ML-based predictor outputs the predicted measurements on the other bands.

Figure 3 shows the diagrams of the AI/ML models for all 3 sub-cases of this scenario. We also use the Transformer-based model and follow the cluster-based approaches with a 7-cell cluster. The inputs to the model are the measurements of 7 cells on frequency band #1, and the outputs from the model are the predicted measurements of 7 co-located cells on frequency band #2 as the input ones.

Figure 3 Diagram of AI/ML models for 3 sub-cases of Case 3.

Figure 4 Sequence of the measurements and predictions for Case 3.

In our implementation, the frequency domain predictor predicts the results on Band #2 based on the UE measurements on Band #1. The sample time is aligned, as shown in Figure 4.

  - Case 4, FR2-to-FR2 intra-frequency domain case A

Differing from the previous cases, the study goal of this one is to improve the handover performance with AI/ML-based temporal predictions. With predictions on the temporal domain, the network or UE can report measurement events, select the target cell more intelligently, or predict the happening of some unintended events, such as the radio link failure.

The diagram of schemes is shown in Figure 5, which also follows the Transformer-based backbone and the cluster-based approach with 7 cells. The inputs to the AI/ML models are the historical measurements, and the outputs of the AI/ML model are the predictions for future slots.

Figure 5 Diagram of AI/ML models for 3 sub-cases of Case 4.

Figure 6 Sequence of the measurements and predictions for Case 4.

Figure 6 shows the sequence of Case 4. The UE uses the measured results to predict future ones. The AI/ML can have multiple input and output results. For Case 4, the prediction window can be long to foresee the happening of mobility-related events.

- Evaluation results

  - Common simulation assumptions

Following the agreements at RAN2 #126 and RAN2 #127, the common simulation assumptions are listed below in Table 1.

Table  1 Common simulation assumptions

| Parameter | FR1 Band #1 (common) | FR1 Band #2 (inter-frequency) | FR2 |
| --- | --- | --- | --- |
| Carrier frequency | 4 GHz (TDD) | 2 GHz (FDD) | 30 GHz |
| SCS | 30 kHz | 15 kHz | 120 kHz |
| Deployment | 2-tier with wrap around (7 sites, 3 sectors per site) |  | 2-tier with wrap around (7 sites, 3 sectors per site) |
| ISD | 500 m |  | 200 m |
| Channel model | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. Inter-frequency correlation model: Section 7.6.5 in TR38.901. |  | UMi w/ distance-dependent LoS probability function defined in Table 7.4.2-1 of TR38.901. |
| System bandwidth | 20 MHz |  | 80 MHz |
| Sample period | 40 ms |  | 80 ms |
| UE speed | 90 kmph |  | 60, 90, 120 kmph |
| UE distribution/dropping | 100% outdoor |  | 100% outdoor |
| BS antenna configuration | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ Number of BS beams: 4 |  | (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Number of BS beams: 32 |
| BS antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi |  | TR 38.802 Table A.2.1-6 |
| UE antenna configuration | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |  | (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) Number of UE beams: 4 |
| UE antenna radiation pattern | Omni-direction |  | TR 38.802 Table A.2.1-8 |
| BS Tx power | 44 dBm for 20 MHz bandwidth |  | 40 dBm |
| Maximum UE Tx power | 23 dBm |  | 23 dBm |
| BS receiver noise figure | 5 dB |  | 7 dB |
| UE receiver noise figure | 9 dB |  | 10 dB |
| BS antenna height | 25 m |  | 10 m |
| UE antenna height | 1.5 m |  | 1.5 m |
| Spatial consistency | Procedure A in TR38.901 |  | Procedure A in TR38.901 |
| UE trajectory model | Option 3 |  | Option 3 |
| UE boundary model | Option 3 |  | Option 3 |

The RRC parameters are configured following the post-RAN2 #126 email agreements in the following tables.

Table  2 RRC parameters for the Layer 3 filtering

| L3 filtering parameter | value |
| --- | --- |
| FR1 Filter Coefficient | 4 |
| FR2 Filter Coefficient | 4 |

Table  3 RRC parameters for the measurement period

| Measurement period | value |
| --- | --- |
| FR1 to FR1 intra-frequency w.o. gap | 200 ms |
| FR1 to FR1 inter-frequency with gap | 200 ms |
| FR2 to FR2 intra-frequency w.o. gap | 400 ms |

Table  4 RRC parameters for beam consolidation/selection

| Consolidation parameter | value |
| --- | --- |
| nrofSS-BlocksToAverage for FR1 | 1 |
| nrofSS-BlocksToAverage for FR2 | 3 |
| absThreshSS-BlocksConsolidation for FR1 | -110 dBm |
| absThreshSS-BlocksConsolidation for FR2 | -110 dBm |

We report our evaluation results for all 3 high-priority cases in the following sub-sections. All the reported values regarding the model complexity are about the models that can handle the inputs of a cluster of 7 cells and output the predictions for the corresponding cluster of 7 cells.

  - Case 2, FR1-to-FR1 intra-frequency temporal domain case B

We evaluated this case with MRRT = 1/2, 3/4, 7/8. When more than 1 measurement is skipped to reduce the overhead, the average RSRP gap of each time position is shown in the table and figure below.

Table  5 Evaluation results for Case 2

| Report parameters |  | Sub-Case 1 | Sub-Case 2 | Sub-Case 3 |
| --- | --- | --- | --- | --- |
| Setting | MRRT | 0.5 / 0.75 / 0.875 |  |  |
|  | Observation window | 2000 ms |  |  |
|  | Prediction window | 200 / 600 / 1400 ms |  |  |
|  | UE speed | 90 kmph |  |  |
|  | Number of Tx beams | 4 |  |  |
|  | Number of Rx beams | 1 |  |  |
| AI model | Training size | 212 k (UE trajectories) |  |  |
|  | Testing size | 53 k (UE trajectories) |  |  |
|  | Model input | L1-RSRP on Point A1, 7 cells (incl. serving) measured | L3-RSRP on Point C 7 cells (incl. serving) measured | L1-RSRP on Point A1, 7 cells (incl. serving) measured |
|  | Model output | Predicted L1-RSRP, same cells and beams as the input | Predicted L3-RSRP, same cells as the input | Predicted L3-RSRP, same cells as the input |
|  | Model type | Transformer |  |  |
|  | Model complexity in a number of parameters(M) | 7.91 | 7.89 | 7.90 |
|  | Model complexity in model size (e.g. Mbyte) | 36.3 | 36.3 | 36.3 |
|  | Computational complexity [FLOPs] | 79.19 M | 79.08 M | 79.14 M |
| Metric | Average L3 cell level RSRP difference (dB) MRRT = 0.5 | 0.53 | 0.45 | 0.43 |
|  | Average L3 cell level RSRP difference (dB) MRRT = 0.75 | 0.76 (200ms) 1.30 (400ms) 1.74 (600ms) | 0.71 (200ms) 1.22 (400ms) 1.64 (600ms) | 0.63 (200ms) 1.07 (400ms) 1.42 (600ms) |
|  | Average L3 cell level RSRP difference (dB) MRRT = 0.875 | 0.80 (200ms) 1.30 (400ms) 1.71 (600ms) 2.05 (800ms) 2.31 (1.0s) 2.51 (1.2s) 2.66 (1.4s) | 0.75 (200ms) 1.22 (400ms) 1.62 (600ms) 1.90 (800ms) 2.13 (1.0s) 2.30 (1.2s) 2.46 (1.4s) | 0.72 (200ms) 1.15 (400ms) 1.52 (600ms) 1.80 (800ms) 2.00 (1.0s) 2.16 (1.2s) 2.27 (1.4s) |

The results are also plotted in Figure 7 to show the increasing tendency of the RSRP gap with the prediction distance.

Figure 7 Evaluation results for Case 2.

For Case 2, we have the following observations,

Observation 1

- For Case 2 (FR1-to-FR1 intra-frequency temporal domain case B)

  - Of the three cases, Sub-Case 3 (L1 beam-level RSRP to L3 cell-level RSRP) achieves the best performance.

  - The average L3 cell-level RSRP of the AI/ML-based predictions is 0.43-0.53 dB when the MRRT is 50%. The AI/ML-based predictors can predict the measurements omitted by the 50% overhead reduction.

  - The average RSRP gap of Sub-Case 3 is below 1.5 dB when the MRRT is 75%. When more measurements are reduced, the average RSRP gap is more significant than 1.5 dB for the predictions of longer distances.

  - Case 3, FR1-to-FR1 inter-frequency (frequency domain)

As discussed during RAN2 #127 meeting, we evaluate the performance of Case 3 considering both the prediction for 4 GHz based on the measurements on 2 GHz band and the prediction for 2 GHz based on the measurements on 4 GHz band.

Table  6 Evaluation results for Scenario 3

| Report parameters |  | Case 1 | Case 2 | Case 3 |
| --- | --- | --- | --- | --- |
| Setting | UE speed | 90 kmph |  |  |
|  | Number of TX beams | 4 |  |  |
|  | Number of RX beams | 1 |  |  |
| AI/ML model | Training/validity | 1615 k (UE trajectories) |  |  |
|  | Testing | 403 k (UE trajectories) |  |  |
|  | Model input | L1-RSRP on Point A1, 7 cells (incl. serving) measured | L3-RSRP on Point C 7 cells (incl. serving) measured | L1-RSRP on Point A1, 7 cells (incl. serving) measured |
|  | Model output | Predicted L1-RSRP, co-located cells and beams as the input | Predicted L3-RSRP, co-located cells as the input | Predicted L3-RSRP, co-located cells as the input |
|  | Model type | Transformer |  |  |
|  | Model complexity in a number of parameters(M) | 7.89 | 7.89 | 7.89 |
|  | Model complexity in model size (e.g. Mbyte) | 30.2 | 30.2 | 30.2 |
|  | Computational complexity [FLOPs] | 446.68 M | 110.75 M | 446.68 M |
| Metrics | Average L3 cell level RSRP difference (dB) (2 GHz -> 4 GHz) | 0.97 | 0.93 | 0.57 |
|  | Average L3 cell level RSRP difference (dB) (4 GHz -> 2 GHz) | 0.66 | 0.79 | 0.54 |

For Case 3, we have the following observations,

Observation 2

- For Case 3 (FR1-to-FR1 inter-frequency)

  - Among 3 sub-cases, sub-case 3 (L1 beam-level RSRP to L3 cell-level RSRP) performs best.

  - For Case 3, the average L3 cell-level RSRP of the AI/ML-based predictions is 0.54 - 0.97 dB. The AI/ML-based predictors can predict the RSRP of co-located cells on Band #2 based on the actual measurements of cells on Band #1.

  - The performances of using 4GHz measurements to predict 2GHz ones and 2GHz measurements to predict 4GHz ones are comparable. The performance gap is minor, i.e., 0.03 dB (Sub-case 3) - 0.31 dB (Sub-case 1).

  - Case 4, FR2-to-FR2 intra-frequency temporal domain case A

Table  7 Evaluation results for Scenario 4

| Report parameters |  | Case 1 | Case 2 | Case 3 |
| --- | --- | --- | --- | --- |
| Setting | UE speed | 60 / 90 / 120 kmph |  |  |
|  | Number of TX beams | 32 |  |  |
|  | Number of RX beams | 8 |  |  |
| AI/ML model | Training/validity | 162 k (UE trajectories) |  |  |
|  | Testing | 40 k (UE trajectories) |  |  |
|  | Model input | L1-RSRP on Point A1, 7 cells (incl. serving) measured | L3-RSRP on Point C 7 cells (incl. serving) measured | L1-RSRP on Point A1, 7 cells (incl. serving) measured |
|  | Model output | Predicted L1-RSRP, same cells and beams as the input | Predicted L3-RSRP, same cells as the input | Predicted L3-RSRP, same cells as the input |
|  | Model type | Transformer |  |  |
|  | Model complexity in a number of parameters(M) | 9.59 | 9.48 | 9.53 |
|  | Model complexity in model size (e.g. Mbyte) | 36.7 | 36.2 | 36.5 |
|  | Computational complexity [FLOPs] | 96.75 M | 95.08 M | 95.64 M |
| Metrics | Average L3 cell level RSRP difference (dB) (UE speed = 60 kmph) | 1.61 (400 ms) 2.18 (800 ms) 2.73 (1200 ms) 3.16 (1600 ms) 3.45 (2000 ms) | 1.28 (400 ms) 1.90 (800 ms) 2.32 (1200 ms) 2.63 (1600 ms) 2.85 (2000 ms) | 0.94 (400 ms) 1.07 (800 ms) 1.14 (1200 ms) 1.22 (1600 ms) 1.32 (2000 ms) |
|  | Average L3 cell level RSRP difference (dB) (UE speed = 90 kmph) | 1.62 (400 ms) 2.22 (800 ms) 2.76 (1200 ms) 3.18 (1600 ms) 3.48 (2000 ms) | 1.35 (400 ms) 1.94 (800 ms) 2.32 (1200 ms) 2.62 (1600 ms) 2.85 (2000 ms) | 0.95 (400 ms) 1.13 (800 ms) 1.25 (1200 ms) 1.39 (1600 ms) 1.55 (2000 ms) |
|  | Average L3 cell level RSRP difference (dB) (UE speed = 120 kmph) | 1.69 (400 ms) 2.31 (800 ms) 2.84 (1200 ms) 3.26 (1600 ms) 3.56 (2000 ms) | 1.11 (400 ms) 1.68 (800 ms) 2.06 (1200 ms) 2.37 (1600 ms) 2.63 (2000 ms) | 1.14 (400 ms) 1.34 (800 ms) 1.50 (1200 ms) 1.65 (1600 ms) 1.85 (2000 ms) |

Figure 8 Evaluation results for Case 4.

For Case 4, 5 measurement samples with 400 ms intervals are predicted. We report the average L3 cell-level RSRP for these 5 predictions in Table 7 and plot them in Figure 8. From the results, we have the following observations,

Observation 3

- For Case 4 (FR2-to-FR2 intra-frequency temporal domain case A)

  - Among 3 sub-cases, sub-case 3 (L1 beam-level RSRP to L3 cell-level RSRP) performs best.

  - The average RSRP gap of sub-case 3 is lower than 1 dB when the UE speed is 60-90 kmph and the prediction distance is 400ms.

  - The average RSRP gap increases with the prediction distance and UE speed. It increases to about 1.5 dB for 2 sec prediction distance when the UE speed is 60-90 kmph, and to about 1.8 dB when the UE speed is 120 kmph.

  - Summary of the evaluations

Our observations for all 3 high-priority scenarios are summarized as the following,

Observation 4

- Regarding 3 sub-cases with different inputs and outputs, Sub-Case 3 outperforms the others for all 3 cases.

  - The Layer 1 measurement results are more informative as the inputs to the AI/ML models, while the Layer 3 filtering may filter some information out from the data.

  - It reduces the complexity of the learning by letting the AI/ML model learn the final Layer 3 outputs instead of the more complex intermediate Layer 1 results.

- For sub-case 3, the average RSRP difference for Case 2 is less than 1.5 dB when MRRT ≤ 0.75, less than 1 dB for Case 3, and less than 2 dB for Case 4 when the prediction window is 2 seconds.

Based on our evaluation of these three scenarios, we observed that the AI/ML-based predictors generate predictions with acceptable accuracy to motivate further studies, which will be discussed in the following section. We have the following proposals,

Proposal 1

- Capture the evaluation results shown in Table 5, Table 6, and Table 7.

- Conclude that the AI/ML-based RRM measurement predictions for three high-priority cases are acceptable for the next-stage studies.

  - Consider Case 3 (L1 beam-level RSRP to L3 cell-level RSRP) for future studies based on the evaluations.

- Further study on the impacts of RRM measurement predictions on the handover performance

From the performance evaluations of the AI/ML-based predictions, the RSRP differences are acceptable for motivating further study. However, the RSRP difference cannot directly reflect the final mobility performance since it has complex impacts on the multiple stages of the handover procedures. RAN2 should identify whether and how much the predicted measurements can improve the mobility performance for further studies. For this purpose, we discuss the potential usage of the RRM measurement predictions case-by-case and propose proper evaluation methodologies for different cases. Finally, some modelling issues are discussed to align the simulation.

  - Potential usage of the measurement predictions and evaluation methodologies

RAN2 has agreed on two study goals for the RRM measurement predictions. The 1st study goal is overhead reduction, and the 2nd is HO performance enhancement. Different evaluation methodologies can be adopted to examine if the predictions satisfy the goals. Hence, we discuss them separately.

- For the 1st study goal (overhead reduction), where Case 2 and Case 3 are involved,

  - For the NW-sided model, the major task is to recover the omitted RRM measurements by the UE based on the UE reports of reduced measurements. The recovery can be on the temporal domain (Case 2) or frequency domain (Case 3).

  - For the UE-sided model, UE can recover the reduced measurements directly. The recovered measurements can further be used for measurement reports.

- For the 2nd study goal (HO performance), where Case 4 is involved,

  - For the NW-sided model, the major task is to use the predicted RRM measurements to avoid unintended events or select the target cell smartly and proactively. The predictions significantly impact the handover procedures.

  - For the UE-sided model, the predictions can be used for measurement event predictions and reports (under the scope of Session 8.3.3) or RLF predictions (under the scope of Session 8.3.4). How UE utilizes these predictions also introduces significant impacts on the handover procedures.

According to the discussions, for the 1st study goal, the major task of the AI/ML model is to recover the omitted measurements through predictions. These predictions, either generated by the network or reported from the UE, will be used for HO and target cell decision-making. If the predicted measurements have a minor impact on the HO decision, then the overhead reduction is achieved.

Observation 5

- For the 1st study goal with either NW-sided or UE-sided model, the measurements recovered by AI/ML are eventually used for HO decision and target cell selection.

  - It is essential to verify whether the measurements recovered by AI/ML have an ignorable impact on system-level performance.

Proposal 2

- For the first study goal, further study is needed to determine whether predicted measurements after the measurement overhead reductions will impact the HO decision and cell selection.

For the 2nd study goal, which directly enhances mobility performance, system-level simulations (SLS) with a mobility model should be involved due to the complex procedures of the handover. Otherwise, we will not know whether and how much the predictions can improve the performance. For the NW-sided model, the predictions can be used for the target cell selection. For the UE-sided model, the predictions can also be used for the measurement event predictions or RLF predictions, which fall into the scope of Session 8.3.3 and 8.3.4, respectively. Therefore, we have the following proposals for the 2nd study goal,

Observation 6

- For the 2nd study goal (HO performance), the handover procedures are significantly impacted, which performance should be verified directly by system-level simulations.

  - The potential gain in HO performance provided by the predicted measurements should be verified to conclude the study.

Proposal 3

- For the 2nd study goal, further study the impacts of the RRM measurement predictions on the mobility performance by the SLS.

  - For the UE-sided model, further study can be conducted in Sessions 8.3.3 and 8.3.4 when the predictions are used for the measurement event or RLF predictions.

  - KPIs

To evaluate the 1st study goal, it is essential to verify if the predicted measurement results significantly impact mobility decisions. Therefore, we propose the corresponding KPIs for this purpose.

Proposal 4

- Define KPIs to verify whether and how much the overhead reduction and AI/ML predictions impact the HO decision and target cell selection, which are

  - Ratio of the same/different HO decisions.

    - Same HO decision: The HO decisions with and without overhead reductions happened within a time gap, and the same target cell is selected.

    - Different HO decision: The HO decisions with and without overhead reduction are different, or different target cells are selected.

  - Average RSRP differences for the difference HO decisions.

For the evaluation of the 2nd study goal, we suggest evaluating the typical handover KPIs, such as HOF rate, ping-pong rate, and time-of-stay, from the SLS. Considering that there are still many detailed implementations up to the companies, we feel that it is hard to align the benchmark performance from companies. Therefore, we suggest companies report the benchmark performance and the relative gains of AI/ML to their benchmark performance. The relative gains will be used to conclude the studies.

Proposal 5

- Study the HOF rate, Ping-pong rate, and CDF of the time-of-stay from system-level simulations.

- Companies to implement the HO model and a rule-based HO procedure based on the models in TR36.839 and possible RAN2 agreements as a benchmark.

- Companies to report the benchmark performance and relative gain/loss of AI/ML to their benchmark for the result collections.

  - EVM for SLS evaluations

For the SLS regarding the handover performance, there is a 3-state HO model introduced in TR36.839 [5], which includes the following detailed models,

- Definition of the 3 states based on the A3 entering, HO command reception, and HO complete transmission.

- RLF modelling and RLF states. The Qin and Qout for RLF are generated based on L1 measurement samples with a given sample rate and L1 filtering.

- Handover/PDCCH failure modelling based on the T310 state, RLF, and the filtered average wideband CQI with a given sample rate and L1 filtering.

- Ping-pong and time-of-stay (ToS) modelling.

With these detailed models, the HOF caused by several seasons can be modelled, which includes the T310 expiry in the source cell (HO too late), HO command failure (HO too late), and HO complete failure (HO too early/wrong). The HOF caused by the RA failure (HO too early/wrong) and the T310 expiry in the target cell (HO too early/wrong) are not modelled. The state of the art of model in [5] is summarized in Table 8.

Table  8 HO Models in TR36.839

| HOF type | Main reason | Sub-reason | Model availability |
| --- | --- | --- | --- |
| Too late HO | Failure in Source | T310 running/expiry in source | Yes |
|  |  | HO command failure | Yes |
| Too early/Wrong HO | Failure in Target | RA failure | No |
|  |  | HO complete failure | Yes |
|  |  | T310 running/expiry in target | No |

Considering that TR36.839 provides a simplified simulation model, we suggest using it as much as possible and discussing whether to enhance it for the issues not addressed. Our proposals are then,

Proposal 6

- Reuse the TR36.839 model for the HOF causes modeled for the SLS regarding 2nd study goal as much as possible.

  - FFS whether and how to enhance the model for unaddressed issues, e.g., RA failure in the target cell, T310 running/expiry in the target cell.

Another issue regarding the models in TR36.839 is that they are designed for the HO evaluations for LTE networks with the HetNet deployments. Some detailed parameters should be revisited to align with our current studies, and some unmentioned parameters can be further defined to obtain comparable results from different companies. Therefore, we propose the following details about the model,

Proposal 7

- Consider the following for the evaluations with SLS,

  - Reuse the definition of the 3 HO states in TR36.839 for the HOF and Ping-pong modelling.

  - Align the L1 sample period and L3 filter with the simulation assumptions of this study.

  - Align the threshold of wideband CQI for handover/PDCCH failure modelling, and the corresponding sample period and filter.

  - Align the parameters for Qin and Qout threshold, T310, N310, and N311.

  - Align the parameters for Event A3 entering/leaving.

  - Other implementations, such as HO command/complete transmission, and HO decision/target cell selection, are up to the companies.

- Conclusion

In this contribution, we have the following observations and proposals,

Observation 1

- For Case 2 (FR1-to-FR1 intra-frequency temporal domain case B)

  - Sub-Case 3 (L1 beam-level RSRP to L3 cell-level RSRP) achieves the best performance of the three sub-cases.

  - The average L3 cell-level RSRP of the AI/ML-based predictions is 0.43-0.53 dB when the MRRT is 50%. The AI/ML-based predictors can predict the measurements omitted by the 50% overhead reduction.

  - The average RSRP gap of Sub-Case 3 is below 1.5 dB when the MRRT is 75%. When more measurements are reduced, the average RSRP gap is more significant than 1.5 dB for the predictions of longer distances.

Observation 2

- For Case 3 (FR1-to-FR1 inter-frequency)

  - Among 3 sub-cases, sub-case 3 (L1 beam-level RSRP to L3 cell-level RSRP) performs best.

  - The average L3 cell-level RSRP of the AI/ML-based predictions is 0.54 - 0.97 dB. The AI/ML-based predictors can predict the RSRP of co-located cells on Band #2 based on the actual measurements of cells on Band #1.

  - The performances of using 4GHz measurements to predict 2GHz ones and 2GHz measurements to predict 4GHz ones are comparable. The performance gap is minor, i.e., 0.03 dB (Sub-case 3) - 0.31 dB (Sub-case 1).

Observation 3

- For Case 4 (FR2-to-FR2 intra-frequency temporal domain case A)

  - Among 3 sub-cases, sub-case 3 (L1 beam-level RSRP to L3 cell-level RSRP) performs best.

  - The average RSRP gap of sub-case 3 is lower than 1 dB when the UE speed is 60-90 kmph and the prediction distance is 400ms.

  - The average RSRP gap increases with the prediction distance and UE speed. It increases to about 1.5 dB for 2 sec prediction distance when the UE speed is 60-90 kmph, and to about 1.8 dB when the UE speed is 120 kmph.

Observation 4

- Regarding 3 sub-cases with different inputs and outputs, Sub-Case 3 outperforms the others for all 3 cases.

  - The Layer 1 measurement results are more informative as the inputs to the AI/ML models, while the Layer 3 filtering may filter some information out from the data.

  - It reduces the complexity of the learning by letting the AI/ML model learn the final Layer 3 outputs instead of the more complex intermediate Layer 1 results.

- For sub-case 3, the average RSRP difference for Case 2 is less than 1.5 dB when MRRT ≤ 0.75, less than 1 dB for Case 3, and less than 2 dB for Case 4 when the prediction window is 2 seconds.

Observation 5

- For the 1st study goal with either NW-sided or UE-sided model, the measurements recovered by AI/ML are eventually used for HO decision and target cell selection.

  - It is essential to verify whether the measurements recovered by AI/ML have an ignorable impact on system-level performance.

Proposal 1

- Capture the evaluation results shown in Table 5, Table 6, and Table 7.

- Conclude that the AI/ML-based RRM measurement predictions for three high-priority cases are acceptable for the next-stage studies.

  - Consider Case 3 (L1 beam-level RSRP to L3 cell-level RSRP) for future studies based on the evaluations.

Proposal 2

- For the first study goal, further study is needed to determine whether predicted measurements after the measurement overhead reductions will impact the HO decision and cell selection.

Proposal 3

- For the 2nd study goal, further study the impacts of the RRM measurement predictions on the mobility performance by the SLS.

  - For the UE-sided model, further study can be conducted in Sessions 8.3.3 and 8.3.4 when the predictions are used for the measurement event or RLF predictions.

Proposal 4

- Define KPIs to verify whether and how much the overhead reduction and AI/ML predictions impact the HO decision and target cell selection, which are

  - Ratio of the same/different HO decisions.

    - Same HO decision: The HO decisions with and without overhead reductions happened within a time gap, and the same target cell is selected.

    - Different HO decision: The HO decisions with and without overhead reduction are different, or different target cells are selected.

  - Average RSRP differences for the difference HO decisions.

Proposal 5

- Study the HOF rate, Ping-pong rate, and CDF of the time-of-stay from system-level simulations.

- Companies to implement the HO model and a rule-based HO procedure based on the models in TR36.839 and possible RAN2 agreements as a benchmark.

- Companies to report the benchmark performance and relative gain/loss of AI/ML to their benchmark for the result collections.

Proposal 6

- Reuse the TR36.839 model for the HOF causes modeled for the SLS regarding 2nd study goal as much as possible.

  - FFS whether and how to enhance the model for unaddressed issues, e.g., RA failure in the target cell, T310 running/expiry in the target cell.

Proposal 7

- Consider the following for the evaluations with SLS,

  - Reuse the definition of the 3 HO states in TR36.839 for the HOF and Ping-pong modelling.

  - Align the L1 sample period and L3 filter with the simulation assumptions of this study.

  - Align the threshold of wideband CQI for handover/PDCCH failure modelling, and the corresponding sample period and filter.

  - Align the parameters for Qin and Qout threshold, T310, N310, and N311.

  - Align the parameters for Event A3 entering/leaving.

  - Other implementations, such as HO command/complete transmission, and HO decision/target cell selection, are up to the companies.

References

- Moderator (CMCC), RP-234055, “Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR”, Dec. 2023.

- RAN2 Chair (InterDigital), “RAN2 #125bis Chair Notes”, Apr., 2024.

- 3GPP TR 38.901 v17.0.0, “Study on channel model for frequencies from 0.5 to 100 GHz”, Mar., 2022.

- RAN2 Chair (InterDigital), “RAN2 #126 Chair Notes”, May, 2024.

- RAN2 Chair (InterDigital), “RAN2 #127 Chair Notes”, Aug., 2024.