3GPP TSG-RAN-WG2 Meeting #127bis	R2-2408328

Hefei, China, 14th– 18th Oct, 2024

Agenda item	:		8.6.2

Source	: 	Sharp

Title	: 	Discussion on issues for supporting inter-CU LTM

Document for	:	Discussion and Decision

Introduction

WID [1] for enhancement mobility was approved and RAN2 started discussion on the WI. In RAN2#127, RAN2 made agreements as follows [2];

Agreements on inter-CU LTM

Reference configuration:

- Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

CSI resource and report configuration:

- The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration.

- The source CU is responsible to generate the common CSI resource configuration.

Early DL sync:

- For inter-CU LTM, the R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused.

Inter-CU LTM switch:

- For inter-CU LTM cell switch, it’s the source DU that triggers the MAC CE and informs the source CU about the target LTM cell.

RLC and PDCP re-establishment in inter-CU LTM:

- If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

Handling of candidate configuration after inter-CU LTM cell switch:

- As in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after at least a inter-CU LTM cell switch procedure where the UE is not configured with DC, unless these are explicitly released by the network.

DRB/PDU session mismatch in subsequent inter-CU LTM:

- RAN2 understand NW implementation can handle the concern raised in P6, R2-2407421.

LTM and L3 HO:

- L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM):

- When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

- The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- For the execution order between CHO and LTM, Rel-18 principle is applied.

Inter-CU SCG LTM:

- Inter-CU SCG LTM preparation can be initiated by source SN.

- The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message.

- For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN.

- RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works.

- Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

Inter-CU MCG LTM:

- SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

- Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied).

- For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

In this paper, we discuss inter-CU LTM related potential issues.

Discussion

Inter-CU SCG LTM

RAN2 agreed that source SN initiates inter-CU SCG LTM preparation, source SN generates SCG part configuration and MN includes it into its MN RRC configuration message. So, candidate configurations are sent via MN RRC Reconfiguration message. RAN2 also agreed that the LTM cell switch command MAC CE is sent by source SN for inter-CU SN LTM. Here, in legacy LTM, LTM configuration sent via MCG RRC message is maintained in ltm-Config associated with MCG. It is the same with CHO/CPAC mechanism, i.e. entries of conditional reconfiguration sent via SRB1 is stored in VarConditionalReconfig associated with MCG and entries of conditional reconfiguration sent via SRB3 (or embedded RRCReconfiguration sent via SRB1) is stored in VarConditionalReconfig associated with SCG. Based on this premise, in case of inter-CU SCG LTM, UE needs to apply candidate configuration associated with MCG RRC based on the LTM CSC MAC CE which SCG MAC received. This behaviour is not considered in Rel-18. Therefore, RAN2 needs to discuss how to handle candidate configuration for inter-SCG LTM. For example, UE stores candidate configuration received via MN RRC message in ltm-Config associated with SCG RRC.

Observation 1. Candidate configuration for inter-SCG LTM is provided by MN RRC message and applied by LTM CSC MAC CE which is sent from source SN.

Proposal 1. RAN2 discuss how to handle candidate configuration for inter-SCG LTM.

And, there can be multiple LTM candidate cells in single target candidate CU. In that case, the multiple LTM candidate cells are intra-CU LTM candidates from the target candidate CU’s perspective. It means that it is managed in ltm-Config associated with SCG in Rel-18 LTM. However, from the perspective of source CU, it should be included in MN RRC message.

Proposal 2. RAN2 assumes that target CU’s intra-CU LTM candidate configuration can be mixed in MN RRC message.

Early UL/DL synchronization

RAN2 agreed to support early DL/UL synchronization for inter-CU LTM. In Rel-18, there are two options for early synchronization, i.e. PDCCH ordered early RACH and UE-based TA measurement. RAN2 already agreed former option. Following steps, as shown in Figure 1, can be considered as baseline of RACH-based early TA acquisition in case of inter-CU LTM.

Figure 1 Steps for RACH-based early TA acquisition in case of inter-CU LTM

In this procedure, a UE performs DL synchronization and RA to target gNB. Then, target DU can calculate the TA value for the UE. And then, TA value should be sent to the source gNB since TA value should be included in LTM CSC MAC CE. It is reasonable for the source NB to manage the TA since source DU can send a PDCCH for early RACH when the source DU predicts that the TA will no longer be valid.

Proposal 3. Target DU calculates TA value.

Proposal 4. Target DU send the TA value to source DU via CUs and source DU maintains the TA value.

Proposal 5. As with legacy, UE will apply the TA value provided in CSC MAC CE sent from source DU.

Subsequent LTM

The intention of Rel-18 subsequent LTM is to avoid RRC configuration so as to reduce the handover interruption. Thus, subsequent intra-CU LTM was supported in Rel-18. Based on the definition of subsequent LTM [3], LTM preparation phase including RRC reconfiguration (LTM candidate configuration) is neglected in subsequent LTM. In other words, the UE keeps the RRC reconfiguration (LTM candidate configuration) in subsequent LTM.

In RAN2#125bis, it was agreed that Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch. In RAN2#127, it was agreed that as in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after at least an inter-CU LTM cell switch procedure where the UE is not configured with DC, unless the LTM candidate configurations are explicitly released by the network. However, in case that an inter-CU LTM occurs, the gNB-CU is switched from the source gNB-CU to the target gNB-CU, where the candidate CU of the source gNB-CU may not be applicable as the candidate CU of the target gNB-CU. Moreover, the number of LTM candidate configurations is limited at most 8,so it is possible that when subsequent LTM is applied in Rel-19, the RRC configuration transmitted by the source gNB-CU may not be applicable for the target gNB-CU after an inter-CU cell switch in the subsequent LTM.

As a result, it is desire to ensure the accuracy of the RRC configuration content applicable to the target gNB-CU and to avoid RRC configuration overhead during the mixture of subsequent inter-CU LTM and subsequent intra-CU LTM. Hence, it is proposed that the validity of the subsequent LTM should be monitored. For example, the number of subsequent cell switches should be limited. The duration of subsequent cell switches to be allowed should be limited.

Proposal 6. The validity of the subsequent LTM should be monitored.

Proposal 7. The approaches for monitoring the validity of the subsequent LTM can consider the limited number of subsequent cell switches and the limited duration allowed for subsequent cell switches.

LTM and L3 HO

In RAN2#127, it was agreed that L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE while inter-CU LTM is being configured (w/o DC). It was also agreed that when performing L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations. That is, the UE simultaneously keeps the RRC configuration of L3 mobility (HO and CHO) and inter-CU LTM. Depending on the triggering timing, the UE performs either L3 mobility or inter-CU LTM. However, it is the MAC entity of the UE that receives the LTM CSC MAC CE from the source DU to trigger the inter-CU LTM, while the RRC layer of the UE receives the L3 HO command from the source CU to trigger L3 mobility. Based on Rel-18 intra-CU LTM, when the intra-CU LTM is triggered, the MAC entity of the UE subsequently notifies the RRC layer of the UE. Therefore, we propose that when the inter-CU LTM is triggered at the MAC entity of the UE, the MAC entity of the UE further notifies the RRC layer of the UE, by following the rationale in Rel-18. Afterwards, RAN2 further discuss the cases when the RRC layer of the UE receives the L3 HO command and the MAC entity of the UE receives the LTM CSC MAC CE to execute the inter-CU LTM, and the execution order of L3 HO and inter-CU LTM.

Proposal 8. By following the rationale in Rel-18, when the inter-CU LTM is triggered at the MAC entity of the UE, the MAC entity of the UE notifies the RRC layer of the UE.

Proposal 9. RAN2 further discuss

- The case when the RRC layer of the UE receives the L3 HO command and the MAC entity of the UE receives the LTM CSC MAC CE to execute the inter-CU LTM, and

- The execution order of L3 HO and inter-CU LTM.

Conclusion

In this paper, we made the following observations and proposals;

Observation 1. Candidate configuration for inter-SCG LTM is provided by MN RRC message and applied by LTM CSC MAC CE provided by SCG DU.

Proposal 1. RAN2 discuss how to handle candidate configuration for inter-SCG LTM.

Proposal 2. RAN2 assumes that target CU’s intra-CU LTM candidate configuration can be mixed in MN RRC message.

Proposal 3. Target DU calculates TA value.

Proposal 4. Target DU send the TA value to source DU via CUs and source DU maintains the TA value.

Proposal 5. As with legacy, UE will apply the TA value provided in CSC MAC CE sent from source DU.

Proposal 6. The validity of the subsequent LTM should be monitored.

Proposal 7. The approaches for monitoring the validity of the subsequent LTM can consider the limited number of subsequent cell switches and the limited duration allowed for subsequent cell switches.

Proposal 8. By following the rationale in Rel-18, when the inter-CU LTM is triggered at the MAC entity of the UE, the MAC entity of the UE notifies the RRC layer of the UE.

Proposal 9. RAN2 further discuss

- The case when the RRC layer of the UE receives the L3 HO command and the MAC entity of the UE receives the LTM CSC MAC CE to execute the inter-CU LTM, and

- The execution order of L3 HO and inter-CU LTM.

Reference

- RP-240299, Revised Work Item: NR mobility enhancements Phase 4

- R2-24xxxx, Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan

- TS 38.300 V18.1.0 (2024-05), 5G; NR; NR and NG-RAN Overall description; Stage-2