3GPP TSG-RAN WG2 Meeting #127bis　　　　　　                                    R2-2408329

Hefei, CN, 14th – 18th Oct, 2024

Agenda item:	8.6.3

Source: 	Sharp

Title: 	L1 event triggered measurement reporting

Document for:	Discussion and Decision

Introduction

WID for mobility enhancement was approved and RAN2 started discussion on the WI. Also, RAN2 started discussion and reached some agreements follows in RAN2#127 [1].

In this paper, we discussed measurement event evaluation related potential issues.

Discussion

measurement event triggering

In L3 measurement reporting, reportOnLeave is supported. The reportOnLeave indicates whether or not the UE shall initiate the measurement reporting procedure when leaving condition is met. This function allows to gNB know that leaving condition is met and to perform more appropriate mobility control operation. Similarly, for the L1 event triggered measurement reporting for LTM, reportOnLeave allows to gNB know that leaving condition is met and to perform more appropriate mobility control operation for LTM (i.e. select the candidate beam/cell to trigger early synchronization and select the target beam/cell and trigger LTM cell switch procedure). Therefore, we propose to support reportOnLeave for L1 event triggered measurement reporting.

Proposal 1. reportOnLeave is supported for L1 event triggered measurement reporting.

In L3 measurement reporting, periodical reporting after an event criterion have been fulfilled is supported. This function allows to gNB to have timely information for more appropriate mobility control behavior. Similarly, for the L1 event triggered measurement for LTM, event triggered periodic report allows to gNB to have timely information for more appropriate mobility control behavior for LTM. Therefore, we proposed to support event triggered periodic report.

Proposal 2. Event triggered periodic report is supported for L1 event triggered measurement reporting.

What contents are included in MR MAC CE?

In the last meeting, it is agreed that MAC CE is used as container of measurement reporting. Based on this agreement, it is needed to discuss what contents are included in MR MAC CE.

In legacy LTM procedure, MR UCI includes SSBRI and L1-RSRP (measurement result). The candidate beam and candidate cell information are implicitly indicated by the SSBRI. Likewise, in Rel-19 event triggered measurement report, MR MAC CE should include these information (i.e. measurement result per beam and beam information) as base line. Instead of SSBRI, candidate cell ID and RS id can be included in MR MAC CE to indicate beam information explicitly.

Proposal 3. MR MAC CE includes measurement result per beam (e.g. RSRP or SINR).

Proposal 4. MR MAC CE includes beam information (explicitly or implicitly).

In addition, report config ID is needed to allow the gNB knows what type of event triggered the measurement report which gNB received, what type of quantity (i.e. RSRP or SINR), and any other information.

Proposal 5. MR MAC CE includes report config ID.

In addition, NW may use measurement result (e.g. RSRP) per cell and current beam RSRP to select the candidate beam/cell to trigger early synchronization and select the target beam/cell and trigger LTM cell switch procedure. This information is not absolutely necessary, but may be helpful for proper LTM execution and early sync. Therefore, we think these information can be optionally included in MR MAC CE.

Proposal 6. MR MAC CE may include RSRP per cell and current beam RSRP.

In addition, it is needed to discuss how UE choose candidate cells and beams to include related measurement result in MAC CE. It is needed to discuss whether the MAC CE can include the measurement result of a candidate beam which did not trigger the event if MAC CE can include more than one measurement reports of each candidate beam.

Proposal 7. RAN2 discuss whether MR MAC CE can include the information of candidate beam did not trigger the event or not.

If measurement report can include the measurement result of the beam which did not trigger the event, it is needed to discuss which beams should be reported.

Proposal 8. If it is agreed that measurement report can include the information of candidate beam did not trigger the event, RAN2 discuss which beams should be reported.

Conclusion

Proposal 1. reportOnLeave is supported for L1 event triggered measurement reporting.

Proposal 2. Event triggered periodic report is supported for L1 event triggered measurement reporting.

Proposal 3. MR MAC CE includes measurement result per beam.

Proposal 4. MR MAC CE includes beam information (explicitly or implicitly).

Proposal 5. MR MAC CE includes report config ID.

Proposal 6. MR MAC CE may include RSRP per cell and current beam RSRP.

Proposal 7. RAN2 discuss whether measurement report can include the information of candidate beam did not trigger the event or not.

Proposal 8. If it is agreed that measurement report can include the information of candidate beam did not trigger the event, RAN2 discuss which beams should be reported.

Reference

[1] R2_127_R19 MOB_Session Notes (Kyeongin Jeong)_EoM.docx