3GPP TSG-RAN-WG2 Meeting #127bis	R2-2408330

Hefei, China, 14th– 18th Oct, 2024

Agenda item	:		8.6.4

Source	: 	Sharp

Title	: 	Discussion on Conditional LTM

Document for	:	Discussion and Decision

Introduction

Revised WID [1] for enhancement mobility was approved and RAN2 started discussion on the WI. For conditional LTM, RAN2 starts discussion from this meeting;

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In this paper, we discuss inter-CU LTM related potential issues.

Discussion

Key features for (Subsequent) Conditional LTM

Conditional LTM(C-LTM) will be similar with CHO/CPC, and subsequent C-LTM (SC-LTM) is similar with SCPAC from the mobility scenario’s point of view. According to the TS38.300[2], LTM has following features;

- UE is provided RRC configuration to be applied upon LTM cell switch in advance.

- UE is DL synchronized with the candidate cell before LTM cell switching.

- UE is UL synchronized with the candidate cell before LTM cell switching.

- gNB decides to switch UE’s serving cell at least based on L1 measurement report.

- Random access procedure isn’t needed for cell switch if the UE is DL/UL synchronized with the target cell.

If the UE needs to perform random access procedure for conditional LTM cell switch, an interruption during cell switching is the same as with legacy mobility mechanism, i.e. CHO [2]. Such a mechanism would only complicate UE/NW implementation without providing much benefit. Not only that, it may have a higher risk of ping-pong switching due to cell switching decisions based on instantaneous L1 measurements results compared to CHO. There is also a view that C-LTM is used as insurance, which coincides with the main benefit of CHO. However, if there’s only RACH-based C-LTM, it’s same with Rel-18 LTM i.e. UE applies LTM candidate configuration, performs RA and switches to target cell when the UE fails in a serving cell and finds a suitable candidate cell. In that sense, the introduction of only RACH-based C-LTM does not have much meaning. Therefore, RACH-less LTM cell switch should be supported for Rel-19 conditional LTM.

- Observation 1. RACH-based conditional LTM has only one advantage (early DL sync) over CHO.

Proposal 1. RACH-less conditional LTM cell switch should be supported.

Proposal 2. Early DL/UL synchronization should be supported for conditional LTM.

Early DL/UL synchronization

In Rel-18, NW configures and activate TCI states of candidate cells so as UE to be DL synchronized with candidate cell before cell switching. In Rel-19 conditional LTM, UE should be configured with activated TCI states for DL synchronization (if RACH-less C-LTM cell switch is supported). Since gNB can configure UE to perform measurement report to determine which TCI states to activate, even if it is conditional LTM, TCI state activation/deactivation MAC CE should be used as with Rel-18 LTM. On the other hand, since UE will perform cell switch by itself according to the measurement result and the configured conditions, UE can perform early DL synchronization without waiting for the TCI activation/indication MAC CE. Thus, UE can automatically determine/activate a TCI state for the target cell based on the measurement results.

- Observation 2. For DL synchronization, TCI state (de)activation is needed.

- Proposal 3. TCI state activation/deactivation MAC CE in Rel-18 LTM can be reused for conditional LTM .

Proposal 4. RAN2 considers autonomous TCI activation/determination performed by UE for conditional LTM.

For UL synchronization, in Rel-18, early TA acquisition procedure is triggered by PDCCH order or realized through UE-based TA measurement. Here, we discuss whether the PDCCH order, RRC based and/or UE-based TA acquisition can be applied to the conditional LTM.

For the PDCCH order-based RA procedure, even if it is conditional LTM, gNB can configure UE to perform (event triggered) L1 measurement report. Then, gNB will trigger early TA acquisition by sending the PDCCH order based on the L1 measurement report. Therefore, if PDCCH ordered based RA procedure is applied, early TA acquisition mechanism triggered by PDCCH order in Rel-18 LTM will be reused for conditional LTM. In addition, source gNB doesn’t need to send PDCCH order to UE if UE perform RA procedure for early synchronization based on the LTM configuration itself (e.g., UE selects preamble index/SSB index for candidate cells based on RRC configuration), i.e. RRC-based RA procedure for early synchronization. The main difference between RRC-based RA procedure and PDCCH order-based RA procedure is whether UE needs to receive PDCCH order or not. RRC-based RA procedure can benefit from low latency because the UE does not need to wait for the PDCCH order and performs early UL synchronization according to the LTM candidate configuration. Therefore, RRC-based RA procedure for early synchronization should be considered. However, for both of PDCCH order-based and RRC-based RA procedure for early synchronization, how to receive TA information for conditional LTM should be further discussed since TA value is provided in LTM CSC MAC CE in Rel-18 LTM, and we assume that LTM CSC MAC CE is not used for conditional LTM cell switch. Therefore, UE needs to calculate TA value by itself or to be provided TA value by gNB before cell switching. In Rel-18, RAN2 has discussed RAR-based TA acquisition option for a few meetings. Hence, we think RAR-based TA acquisition can be reconsidered for conditional LTM.

For UE-based TA acquisition, UE can calculate TA value by itself without gNB’s involvement, and it is suitable for conditional LTM as UE can trigger early synchronization and cell switch during the whole process. Thus, we support using UE-based TA acquisition for conditional LTM.

Observation 3. For PDCCH order, (event triggered) L1 measurement report can be used.

Proposal 5. For conditional LTM cell switch, LTM CSC MAC CE in Rel-18 LTM is not used.

Proposal 6. Early TA acquisition mechanism triggered by PDCCH order can be reused for conditional LTM.

Proposal 7. RRC-based early TA acquisition mechanism for early synchronization can be considered for conditional LTM.

Proposal 8. Support of RAR-based early TA acquisition for conditional LTM.

Proposal 9. Support of UE-based TA acquisition mechanism for conditional LTM.

If UE has valid TA, which is provided in RAR or calculated by UE, UE can perform RACH-less LTM cell switch. However, if there is no valid TA for the target upon execution of conditional LTM cell switch, UE needs to perform random access to the target cell. The drawback is long interruption time for cell switch process, and thus we may consider this as a fall-back behaviour.

Proposal 10. If there is no valid TA for the target upon execution of conditional LTM cell switch, UE needs to perform random access to the target cell after cell switching, so we can consider it as a fall-back behaviour.

Configuration/Evaluation

In Rel-18LTM, UE starts L1 measurement and reporting when L1 measurement is configured in RRC reconfiguration. And gNB determines whether to trigger LTM cell switch for the UE based on the L1 measurement reports. In case of CHO, when UE is configured with CHO configuration, UE starts evaluating event condition of conditional reconfiguration. Therefore, in the same way, UE should start conditional LTM event evaluation upon UE is configured with C-LTM configuration.

Proposal 11. UE starts conditional LTM event evaluation once it is configured with C-LTM configuration.

Furthermore, legacy LTM configuration is not maintained as UE variables. On the other hand, configurations for conditional reconfiguration are managed as UE variable. If RAN2 follows CHO method, RAN2 needs to introduce a new UE variable for conditional LTM. If RAN2 introduce a new UE variable, UE/gNB needs to manage the entries for conditional LTM separately from the entries for the normal LTM, the interactions between the entries for conditional LTM and the entries for the normal LTM will be complicated and CSI measurement configuration will become more complicated. Therefore, the entries for conditional LTM is managed in ltm-Config which maintains Rel-18 LTM configuration.

Proposal 12. The entries for conditional LTM is managed in ltm-Config which maintains Rel-18 LTM configuration.

And, just as the event evaluation mechanism for CHO should be aligned with event triggered L3 measurement report, the event evaluation mechanism for C-LTM should be aligned with event triggered L1 measurement report. This helps reduce the workload and keeps the specifications simple.

Proposal 13. The event evaluation mechanism for C-LTM should be same with the event evaluation mechanism for event-triggered L1 measurement reporting.

Condition for C-LTM

Regarding to the condition of executing conditional LTM cell switch, it should be linked to the discussion in event triggered L1 measurement report. In that discussion, LTM 2/3/4/5 was agreed;

- Event LTM2: Beam of serving cell becomes worse than absolute threshold;

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM4: Beam of candidate cell becomes better than absolute threshold;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

In case of C-LTM, UE needs to move to a cell which is different from the serving cell and can provide good radio link quality. Therefore, LTM2 is not needed. And using LTM3/4/5 for event of C-LTM is aligned with CHO mechanism (A3/4/5 is used for event triggering CHO execution). So, event LTM3/4/5 should be considered as condition for conditional LTM.

Observation 4. Event A3/4/5 can be used for CHO

Proposal 14. Event LTM3/4/5 should be considered as condition for conditional intra-CU MN LTM.

Cell switch execution

Upon LTM cell switch execution, UE can switch the serving cell with/without random access procedure. For RACH-based LTM cell switch, there are two type of RACH procedure, i.e. CFRA and CBRA. In Rel-18LTM, source DU determines to trigger LTM cell switch and notifies it to target DU if source DU and target DU are not the same DUs. Hence, target gNB (target DU) can prepare dedicated resource for the UE when CFRA is needed. Using CFRA is useful for reducing communication interruption period. On the other hand, if CFRA is used for C-LTM, all of candidate cells need to reserve CFRA resources for the UE for the entire duration that C-LTM is configured. However, in case of CHO, both of CBRA and CFRA are supported. It means that the network can tolerate some waste of CFRA resources. So, in the same way, both CBRA and CFRA should be supported for conditional LTM.

Observation 5. In case of CHO, both of CBRA and CFRA are supported.

Proposal 15. Both CBRA and CFRA should be supported for conditional LTM.

For RACH-less cell switch, we assume that legacy mechanism will be used. It means that UE firstly sends RRCReconfigurationComplete message after the switch as with Rel-18LTM when UE performs RACH-less LTM cell switch.

Proposal 16. When the UE executes a RACH-less LTM cell switch, it first transmits the RRCReconfigurationComplete message following the switch, as is customary in a Rel-18 LTM procedure.

Furthermore, there is an issue how to handle UL grant to complete LTM cell switch procedure. In Rel-18 LTM, an UL grant is provided by configured grant or dynamic grant. In case of CG-based RACH-less cell switch, UE is configured with configured grant to be used for target cell by RRC signalling, and the UE sends complete message by using a beam which is indicated in the LTM CSC MAC CE. Target cell will prepare UL resource after source DU indicates the UE’s switch to the target. In case of C-LTM, if this mechanism is applied to UE, whose cell switch timing is unknown, the candidate cells must constantly reserve UL resources for the UE. In case of DG-based RACH-less cell switch, UE monitors PDCCH on the target cell and sends complete message upon receiving PDCCH for DG. Then, the target cell will prepare UL resource after source DU indicates the UE’s switch to the target. Similarly, in the case of DG-based, target cell will prepare UL resource and send PDCCH to the UE after source DU indicates the UE’s switch to the target. In case of C-LTM, if this mechanism is applied to UE, whose cell switch timing is unknown, the candidate cells must constantly reserve UL resources for the UE. In both cases configured grant and dynamic grant, it is not reasonable to consistently reserve UL grant for the UE, whose cell switch timing is unknown.

Observation 6. To perform RACH-less LTM cell switch, gNB needs to provide UL grant so as UE to send an RRCReconfigurationComplete message.

Observation 7. It is not reasonable to continue providing Dynamic/Configured grant for the UE, whose cell switch timing is unknown.

The key point is that target cell (DU) knows when UE switch the serving cell. If target cell (DU) knows when UE switch the serving cell, the target can reserve the UL grant depending on when the UE switches the serving cell (as mentioned above). Here, we propose two solutions. One of them is UE notifies execution of conditional LTM cell switch to source cell after deciding to execute the switch and before executing the switch. Another one is the target provides UL grant in early TA acquisition procedure (triggered by PDCCH order).

Figure 1. Notification of C-LTM cell switch (Alt1)

Figure 2. RAR with UL grant (Alt2)

For Alt1, when the event conditions are met, the UE can perform RACH-less LTM cell switch if the UE is DL/UL synchronized with target cell. UE needs UL grant towards the source for the notification, but it seems no problem based on the consideration of LTM use case i.e. LTM is the mobility mechanism of the UE where communication occurs continuously and it is desirable to have little interruption.

For Alt2, it needs RA preamble transmission and RAR reception after the event condition is met. It means that the UE performs early sync, cell switch and notification to the target simultaneously after UE receives PDCCH order. This scenario is feasible but it is not desirable because RACH is required after the event condition is met.

Proposal 17. To support RACH-less LTM cell switch, RAN2 discusses which procedure should be considered as baseline of conditional LTM;(target DU may or may not be source DU)

Alternative 1. UE notifies that UE will perform LTM cell switch to the source DU just before the LTM cell switch, source DU notifies it to target DU, and target DU ensures UE has UL grant.

Alternative 2. UE performs event triggered measurement report to source DU for conditional LTM, source DU triggers early synchronization by PDCCH order, and target DU ensures UE has UL grant.

Completion

For C-LTM completion, we don’t see any issues to apply legacy mechanism.

Proposal 18. UE considers LTM cell switch has been successfully completed upon RACH completion for the case RACH-based cell switch as with normal LTM.

Proposal 19. UE considers LTM cell switch has been successfully completed upon sending RRCReconfigurationComplete for the case RACH-less cell switch as with normal LTM.

Subsequent C-LTM

The intention of Rel-18 subsequent LTM is to avoid frequent RRC configuration upon cell switch. The definition of subsequent LTM in Rel-18 is “Subsequent LTM is done by repeating the early synchronization, LTM cell switch execution, and LTM cell switch completion steps without releasing other LTM candidate configurations after each LTM cell switch completion” in [2]. Since Rel-18 only supports the subsequent intra-CU LTM and now Rel-19 subsequent C-LTM focuses only on intra-CU cases, we propose that the definition concept of Rel-18 subsequent LTM can be reused for Rel-19 subsequent intra-CU C-LTM with the revision as “Subsequent intra-CU C-LTM is done by repeating the early synchronization, LTM cell switch execution, and LTM cell switch completion steps without releasing other C-LTM candidate configurations after each C-LTM cell switch completion”.

Observation 8. The definition concept of Rel-18 subsequent LTM can be reused for Rel-19 subsequent intra-CU C-LTM since Rel-18 LTM focuses on intra-CU cases.

Proposal 20. The definition of Rel-19 subsequent intra-CU C-LTM is “Subsequent intra-CU C-LTM is done by repeating the early synchronization, C-LTM cell switch execution, and C-LTM cell switch completion steps without releasing other C-LTM candidate configurations after each C-LTM cell switch completion.”

Conclusion

In this paper, we made the following observations and proposals;

Observation 1. RACH-based conditional LTM has only one advantage (early DL sync) over CHO.

Proposal 1. RACH-less conditional LTM cell switch should be supported.

Proposal 2. Early DL/UL synchronization should be supported for conditional LTM.

Observation 2. For DL synchronization, TCI state (de)activation is needed.

Proposal 3. TCI state activation/deactivation MAC CE in Rel-18 LTM is can be reused for conditional LTM.

Proposal 4. RAN2 considers autonomous TCI activation/determination performed by UE for conditional LTM.

Observation 3. For PDCCH order, (event triggered) L1 measurement report can be used.

Proposal 5. For conditional LTM cell switch, LTM CSC MAC CE in Rel-18 LTM is not used.

Proposal 6. Early TA acquisition mechanism triggered by PDCCH order can be reused for conditional LTM.

Proposal 7. RRC-based early TA acquisition mechanism for early synchronization can be considered for conditional LTM.

Proposal 8. Support of RAR-based early TA acquisition for conditional LTM.

Proposal 9. Support of UE-based TA acquisition mechanism for conditional LTM.

Proposal 10. If there is no valid TA for the target upon execution of conditional LTM cell switch, UE needs to perform random access to the target cell after cell switching, so we can consider it as a fallback behaviour.

Proposal 11. UE starts conditional LTM event evaluation once it is configured with C-LTM configuration.

Proposal 12. The entries for conditional LTM is managed in UE’s ltm-Config which maintains Rel-18 LTM configuration.

Proposal 13. The event evaluation mechanism for C-LTM should be same with the event evaluation mechanism for event-triggered L1 measurement reporting.

Observation 4. Event A3/4/5 can be used for CHO

Proposal 14. Event LTM3/4/5 should be considered as condition for conditional intra-CU MN LTM.

Observation 5. In case of CHO, both of CBRA and CFRA are supported.

Proposal 15. Both CBRA and CFRA should be supported for conditional LTM.

Proposal 16. When the UE executes a RACH-less LTM cell switch, it first transmits the RRCReconfigurationComplete message following the switch, as is customary in a Rel-18 LTM procedure.

Observation 6. gNB needs to provide UL grant so as UE to send an RRCReconfigurationComplete message.

Observation 7. It is not reasonable to continue providing Dynamic/Configured grant for the UE, whose cell switch timing is unknown.

Proposal 17. To support RACH-less LTM cell switch, RAN2 discusses which procedure should be considered as baseline of conditional LTM;(target DU may or may not be source DU)

Alternative 1. UE notifies that UE will perform LTM cell switch to the source DU just before the LTM cell switch, source DU notifies it to target DU, and target DU ensures UE has UL grant.

Alternative 2. UE performs event triggered measurement report to source DU for conditional LTM, source DU triggers early synchronization by PDCCH order, and target DU ensures UE has UL grant.

Proposal 18. UE considers LTM cell switch has been successfully completed upon RACH completion for the case RACH-based cell switch as with normal LTM.

Proposal 19. UE considers LTM cell switch has been successfully completed upon sending RRCReconfigurationComplete for the case RACH-less cell switch as with normal LTM.

Observation 8. The definition concept of Rel-18 subsequent LTM can be reused for Rel-19 subsequent intra-CU C-LTM since Rel-18 LTM focuses on intra-CU cases.

Proposal 20. The definition of Rel-19 subsequent intra-CU C-LTM is “Subsequent intra-CU C-LTM is done by repeating the early synchronization, C-LTM cell switch execution, and C-LTM cell switch completion steps without releasing other C-LTM candidate configurations after each C-LTM cell switch completion.”

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- TS 38.300 V18.1.0 (2024-05), 5G; NR; NR and NG-RAN Overall description; Stage-2