3GPP TSG-RAN WG2 Meeting #127bis	R2-2408333

Hefei, China, 14th – 18th October, 2024

Agenda Item:	8.9.2

Source:	ZTE Corporation, Sanechips

Title:	Further consideration on S&F operation in IoT NTN

Document for:	Discussion and Decision

Introduction

In RAN#102, a new WID for further enhancements on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) has been agreed and in RAN#103, the WID is further refined in [1]. The main objectives related to support of Store&Forward (S&F) satellite operation are as following:

The aim of this WI is enhancement of IoT-NTN with the following objectives: - Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In RAN2#125bis meeting, RAN2 has had some high level discussion on this topic and made the following assumptions:

RAN2 assumptions: - S&F implies that at least the full eNB will be onboard - An IoT NTN network shall be able to inform UE(s) whether S&F Satellite operation is applied, either via NAS or AS (wait for SA2 progress on this) - The S&F satellite operation is common for NB-IoT and eMTC. - The S&F satellite operation is applied to both CP solution and UP solution (for the UP solution pending on SA2 conclusions on the architecture)

In RAN2#126 meeting, RAN2 has made more agreements for this objective as following:

1. For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite): - The UE sends uplink data signalling to eNB when service link is available and the eNB stores it. - When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN. - The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available). - The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again 2. S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed 3. RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

In RAN2#127 meeting, based on the latest conclusions in SA2, RAN2 focus the discussion on RAN2 impacts of full CN and spit-MME payload architecture options and achieved the following agreements:

Agreements: - RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options) - RAN2 will consider both single satellite pass and multiple satellite pass scenarios - RAN2 will consider both MO and MT data within scope - UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature) - RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected. - RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

In this paper, we will further discuss the remaining issues from last RAN2 meeting and give our proposals.

Discussion

  - Whether to introduce new barring bit

In last RAN2 meeting, RAN2 has agreed to find a way to bar legacy UEs and still allow R19 S&F capable UEs. The exact solution is still FFS. In order to bar legacy UEs while allow new UEs to access, a straightforward way is to introduce a new barring bit. The new R19 UEs supporting S&F operation will ignore the old barring bit and only check the new one. Then three cases can be achieved:

- When network wants to bar both the old UEs and the new UEs (e.g., when network is overloaded), it needs to set both old bit and new bit to “barred”.

- When network wants to bar ONLY the old UEs (e.g., when the feeder link is not available), the network would set the old barring bit to “barred” while set the new barring bit to “notBarred”.

- When network sets the new barring bit to “barred” and set the old barring bit to “notBarred”, it means network wants to bar only new UEs. This would be allowed from specification perspective, but it may not be a practical case.

- It may be also possible that network doesn’t want to bar any UEs (e.g., when feeder link is available and network is not overloaded), then the network needs to set both old bit and new bit to “notBarred”.

Please note, for a R19 UE that doesn’t support S&F operation, we understand it cannot identify the new barring bit and still follows the old barring bit, that is, such R19 UE acts same as an old UE.

We use a simple table, Table 1, to show the possible combinations of these barring bits and also the expected UE behaviors:

Table 1

| For R19 and early release UEs | old barring bit = “barred” | old barring bit =“notBarred” |
| --- | --- | --- |
| new barring bit = “barred” | Both old UEs and new UEs are barred | New UEs are barred while old UEs are allowed to access (it’s possible but may not be a practical case) |
| new barring bit = “notBarred” | Old UEs are barred while new UEs are allowed to access | Both old UEs and new UEs are allowed to access |

If such new barring bit is introduced, it generally should be mandatory and generally included in SIB1. Since it’s mandatory, the presence of this indication can also notify new R19 UEs supporting S&F operation that the camping network also supports S&F operation.

In previous meeting, RAN2 also agreed that an IoT NTN network shall be able to inform UE(s) whether S&F Satellite operation is applied and such S&F indication can be provided by SIB. Furthermore, in last meeting, there were some views in companies’ contributions that the new barring bit can be represented by such S&F indication, or vice versa. The following Alt1 is an example which has been mentioned by several companies:

- Alt1: The S&F indication is present only when both legacy cellBarred and cellBarredNTN are set to “barred”. In other word, when feeder link is not available, the S&F mode supporting cell should include the S&F mode indication and meantime, set legacy cellBarred and cellBarredNTN to “barred”. In UE side, if a R19 UE supporting S&F operation finds that S&F mode indication is present in SIB, it will ignore the legacy cell barring bits, regardless of whether they are “barred” or “notBarred”. We also use a simple table, Table 2, to show the possible combinations of these bits and also the expected UE behaviors:

Table 2

| For R19 and early release UEs | old barring bit = “barred” | old barring bit = “notBarred” |
| --- | --- | --- |
| new S&F indication is absent (It may be a legacy R18 cell, or a R19 cell capable of S&F and feeder link is available) | New UEs still follow old barring bit. Both old UEs and new UEs are barred | New UEs still follow old barring bit. Both old UEs and new UEs are allowed to access |
| new S&F indication is present (It’s a R19 cell capable of S&F, and feeder link is unavailable) | New UEs ignore the old barring bit. Old UEs are barred while new UEs are allowed to access | New UEs ignore the old barring bit. Both old UEs and new UEs are allowed to access |

For Alt1, based on the comparison between Table 1 and Table 2, we have the following observations:

- Alt1 is feasible to fulfill the requirement that legacy UEs are barred (using legacy cellBarred and/or cellBarred-NTN bit) while R19 S&F capable UEs are still allowed to access.

- Compared with Table 1, the only scenario that cannot be represented in Table 2 is “New UEs are barred while old UEs are allowed to access”. Given that this may not be a practical scenario, we think it is acceptable that this cannot be implemented by Alt1. Overall, the scenarios that Alt1 can achieve are generally the same as those achieved by introducing a new barring bit.

- For Alt1, it’s no need to introduce a new barring bits but only need to implement the new S&F indication, so it has benefit of reducing signaling overhead.

Furthermore, the proponent companies think such S&F indication is present ONLY when both legacy barring bits are set to “barred”. Based on the above Table2, it means that the combination in the lower-right corner of the table (marked with blue text) should not exist. But we think such restriction may be not so needed, and this can be left to eNB’s implementation. We can allow the configuration flexibility in the spec, whether further restriction is needed can be left to Stage 3 discussion.

Proposal 1: A new S&F indication can be introduced to a R19 cell supporting S&F operation:

- If such S&F indication is present, the R19 UEs supporting S&F should ignore the legacy barring bits and assume the access to network is allowed.

- If such S&F indication is absent, R19 UEs supporting S&F should still follow the legacy barring bits.

- Whether legacy barring bits must be set to “barred” when providing the new S&F indication can be discussed in Stage-3.

  - Discussion on details of new S&F indication

According to previous RAN2 agreements, the new S&F indication should be used to inform UE(s) whether S&F Satellite operation is applied.

Such indication can be with meaning of “S&F operation is applied” or “feeder link becomes unavailable”. Per our understanding, “S&F operation is applied” generally means the network node onboard the satellite needs to store the received UL data when feeder link is unavailable. So minimum 1 bit can be considered to define this indication and its value can be defined as “ENUMERATED {true}”.

In previous meeting, there was thought that “S&F operation is applied” and “feeder link becomes unavailable” may need to be independently defined. But we see no any sense that a R19 network supporting S&F operation doesn’t apply S&F operation when feeder link is unavailable. So we think it’s no need to treat “S&F Satellite operation is applied” and “feeder link becomes unavailable” as two separate different indications. In our thinking, they are equivalent.

Moreover, SA2 has agreed to only support full CN onboard or a Split MME architectures. No matter which architecture is deployed, their air interface procedure and NAS procedure are almost as legacy. So even when feeder link is unavailable,  the UE still can access the network and send uplink data to core network as usual, and also get ACK/NACK in AS/NAS layers for the uplink data timely, e.g., without loss in performance. In other word, with either full CN onboard architecture or a Split MME architecture, the UE generally doesn’t need to care about the availability of the feeder link and the UE can always transmit data. So we think the intention of introducing such S&F indication is not so clear, or it may be not so useful to only introduce a simple indication as “S&F operation is applied” or “feeder link becomes unavailable”

One scenario we can assume is that, if the characteristics of UE’s traffic is primarily downlink data traffic, when UE receives such indication “feeder link becomes unavailable”, even it’s feasible for UE to access the network, the UE still can choose not to trigger the connection establishment as it might be a waste to resources and UE power for monitoring the DL channel without receiving the expected DL data for a period of time. Furthermore, if UE chooses to suspend the connection establishment, it would be better to inform the UE when the feeder link can become available, so that the UE can proactively re-initiate the connection establishment at that time.

Proposal 2a: For the new S&F indication, it can be used to inform UE with the information of “S&F operation is applied” or “feeder link becomes unavailable”.

Proposal 2b: Besides to indicating “S&F operation is applied” or “feeder link becomes unavailable”, the new S&F indication can also be used to inform UE when the feeder link can become available.

Conclusion

Based on the above discussion, the following proposals are given:

Proposal 1: A new S&F indication can be introduced to a R19 cell supporting S&F operation:

- If such S&F indication is present, the R19 UEs supporting S&F should ignore the legacy barring bits and assume the access to network is allowed.

- If such S&F indication is absent, R19 UEs supporting S&F should still follow the legacy barring bits.

- Whether legacy barring bits must be set to “barred” when providing the new S&F indication can be discussed in Stage-3.

Proposal 2a: For the new S&F indication, it can be used to inform UE with the information of “S&F operation is applied” or “feeder link becomes unavailable”.

Proposal 2b: Besides to indicating “S&F operation is applied” or “feeder link becomes unavailable”, the new S&F indication can also be used to inform UE when the feeder link can become available.

Reference

[1] RP-240776 Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things MediaTek Inc., RAN#103, March, 2024

[2] 3GPP TR 23.700-29 V19.0.0, Study on integration of satellite components in the 5G architecture; Phase 3 (Release 19), 2024-06