3GPP TSG-RAN WG2 Meeting #127bis                                                                   R2-2408334

Hefei, China, Oct 14th – 18th, 2024

Source:	ZTE Corporation, Sanechips

Title:	Comparison of solutions for UL capacity enhancements in IoT NTN

Agenda Item:	8.9.3

Document for:	Discussion and Decision

Introduction

In RAN#102, a new WID for further enhancements on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) has been agreed and in RAN#105, the WID is further refined in [1]. The main objectives related to support of capacity enhancements for uplink are as following:

The aim of this WI is enhancement of IoT-NTN with the following objectives: <<skip the unrelated parts>> - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In previous RAN2 meetings, RAN2 has discussed some general aspects for this objective and the feasibility of contention-based Msg3 transmission, and also gave some initial comparison among different anti-collision schemes. The related RAN2 agreements are as below:

RAN2#125bis agreements: - Both NB-IoT and eMTC are within scope of uplink capacity enhancements - Both C-plane and U-plane solutions are within scope of uplink capacity enhancements. - Only CIoT EPS is within scope of uplink capacity enhancements RAN2#126 agreements: - RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). - RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this) - If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3. RAN2#127 agreements: - RAN2 will continue working on a CB-msg3 EDT-like mechanism. - RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts - In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1. - For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition

As reference, in previous RAN1 meetings, for this objective, RAN1 has focused on the OCC schemes and has reached the following agreements:

RAN1#116: Agreement At least the following NPRACH OCC schemes are considered by RAN1 for study: - Intra-symbol group OCC - Inter-symbol group(s) OCC - Inter-repetition OCC Agreement OCC multiplexing is not supported between a UE using NPUSCH format 1 with 3.75kHz SCS and another UE using NPUSCH format 1 with 15kHz SCS. Agreement For OCC of NPUSCH format 1, RAN1 will not consider multiplexing more than 4 UEs. RAN1#117: Agreement For 3.75kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued. For 15kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued. Agreement Inter-repetition OCC for NPRACH is not studied further in RAN1. Agreement - For the time-domain DMRS pattern (including blanked DMRS, if any): - For 15kHz single-tone, RAN1 strives to reuse the Rel-17 DMRS pattern. - For 3.75kHz single-tone - RAN1 studies - Rel-17 DMRS pattern - A new DMRS pattern - The DMRS overhead (including blanked DMRS, if any) for OCC is the same as for Rel-17. Agreement The Rel-17 guard period locations and length for NB-IoT 3.75kHz UL slot are preserved when OCC is applied to NPUSCH format 1. RAN1#118: Agreement RAN1 studies whether the following types of UL transmission gap will impact the design of OCC for IoT-NTN when considering e.g. phase continuity - UL gaps for synchronization (from Rel-13) - Gaps around NPRACH occasions - UL timing adjustment gaps and segmentation for IoT-NTN (from Rel-17) - TDM DMRS that are muted - Guard periods for 3.75kHz UL transmissions Agreement The following combinations are considered for further simulation in RAN1 for 3.75kHz SCS OCC for NPUSCH format 1: - Option 1: OCC2, Symbol-level, TDM DMRS - Option 2: OCC2, Symbol-level, CDM DMRS with new pattern - Option 3: OCC2, Slot-level, TDM DMRS - Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern - Option 6: OCC4, Symbol-level, CDM DMRS with new pattern The following combinations are considered for further simulation in RAN1 for 15kHz SCS OCC for NPUSCH format 1: - Option 1: OCC2, Symbol-level, TDM DMRS - Option 3: OCC2, Slot-level, TDM DMRS - Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern - Option 5: OCC4, Symbol-level, TDM DMRS - Option 7: OCC4, Slot -level, TDM DMRS - Option 8: OCC4, Slot-level, CDM DMRS with legacy pattern Note 1: For TDM, the legacy DMRS pattern, with DMRS symbols appropriately muted/blanked is used. Companies to report their assumption on whether spreading is applied to the legacy DMRS pattern for 15 kHz SCS. Note 2: Companies to report DMRS sequence applied. Agreement For 3.75kHz SCS, NPUSCH format 1 simulations are performed using an appropriate MCS with SNR at least in the range of -8dB to 0dB.

In this document, we will firstly give our comparison on SA, DSA, CRDSA, OCC-based SA schemes, and further give analysis on other issues of Msg3 and Msg4 transmission. Based on the analysis, we give our proposals.

Discussion

Comparison on SA, DSA, CRDSA, OCC-based SA schemes

Simulation assumption

Generally, the data transmission based on shared time slots resources can be seen as a kind of Slotted ALOHA scheme which will be taken as baseline scheme for supported contention-based Msg3 transmission.

In previous RAN2 meetings, some enhanced Slotted ALOHA schemes, e.g., DSA, CRDSA, were proposed for contention-based Msg3 transmission without Msg1/Msg2, with intention to reduce collisions and further improve uplink capacity. In RAN2#127 meeting, some simulations have been brought up [2][3] to explain the performance of different anti-collision schemes for contention-based small data transmission.

Besides the anti-collision solutions which have been mentioned in RAN2 contributions, based on the WID study scope and also RAN1 progress, OCC (orthogonal cover codes) sequence can be further applied to NPUSCH (Msg3) transmission to avoid the collision caused by using shared time/frequency domain resources and facilitate more UEs multiplexing on a same set time/frequency domain resources. Therefore, we think OCC sequence can also be combined with Slotted ALOHA schemes with expectation to further improve uplink capacity.

In this contribution, we will further evaluate and compare the following schemes:

- SA: Msg3 transmission with existing contention-based scheme in 3GPP spec that can be likened to a Slotted ALOHA scheme.

- DSA: Diversity Slotted ALOHA in which at least two replicas are generated for each user packet and transmitted on randomly selected resources. This means multiple transmission opportunities for a Msg3 transmission, and even if the first replica collides, subsequent replicas may be successfully decoded, thereby increasing the probability of successful reception of the Msg3 transmission.

- CRDSA: Contention Resolution Diversity Slotted ALOHA in which at least two replicas are generated for each Msg3 transmission and transmitted on randomly selected resources. Furthermore, interference cancelation techniques are used at the receiver side to eliminate the interference of a decoded packet on other packets, thereby further improving the probability of successful reception of other Msg3 transmissions.

- OCC-based SA: Msg3 transmission for at most 4 users are multiplexed with additional OCC (orthogonal cover codes) sequence on the shared time/frequency domain resources and with existing contention-based scheme.

- OCC-based DSA: Msg3 transmission for at most 4 users are multiplexed with additional OCC (orthogonal cover codes) sequence on the shared time/frequency domain resources. At least two replicas are generated for each Msg3 transmission and transmitted on randomly selected time/frequency resources and also OCC sequence.

- OCC-based CRDSA: Msg3 transmission for at most 4 users are multiplexed with additional OCC (orthogonal cover codes) sequence on the shared time/frequency domain resources. At least two replicas are generated for each Msg3 transmission and transmitted on randomly selected time/frequency resources and also with OCC sequence. Furthermore, interference cancelation techniques are used at the receiver side.

In [3], company used a short Monte Carlo simulation to simulate UL transmission, the collision situation and the final performance for SA and CRDSA. In our analysis, almost the similar simulation assumptions are used and explained in detail as below:

- Within a certain number of users, e.g., 300, some users are selected to transmit service packets based on the arrival of packets.

- The arrival of service packets satisfies Poisson distributed. As the lambda increases, the number of arrived packets increases, finally resulting in a high load. The formula of Poisson distributed is as below:

P[k users access] = (lambda)^k*exp(-lambda)/k!                                                          (1)

- Each user with service packets arrival triggers the UL data transmission and selects resource(s) from the acertain number of available slots, e.g., 100:

  - For SA scheme: At the transmitter side, a slot is randomly selected from the 100 slots to transmit a packet. At the receiver side, if the transmission of one packet doesn’t collide with others (e.g., these packets select different slots), the decoding for this packet could be deemed successful. If the transmission of one packet collides with the other packet (e.g., these packets select same slot), only one of these two packets could be deemed successful, while the other would be deemed unsuccessful. If the transmission of one packet collides with two or more other packets, the decoding for all involved packets is deemed unsuccessful.

  - For DSA scheme: Only two replicas case is considered. At the transmitter side, two slots are randomly selected from the 100 slots to transmit two replicas for a packet. At the receiver side, as long as one replica of a packet is decoded successfully, the reception of this packet could be deemed successful. How to determine a replica decoding success is the same as the method described in above SA scheme to determine the success of packet decoding.

  - For CRDSA scheme: Only two replicas case is considered. At the transmitter side, two slots are randomly selected from the 100 slots to transmit two replicas for a packet. At the receiver side, if one replica of a packet is decoded successfully, the reception of this packet could be deemed successful. Furthermore, the interference generated by other replica of this packet on other slot would be canceled. At the other slot, after canceling a replica, if only one replica is remained, the packet to which the remained replica belongs is also deemed successful

  - For OCC-based SA scheme: At the transmitter side, a slot is randomly selected from the 100 slots and an OCC index is randomly selected from 4 available OCC indexes to transmit a packet. At the receiver side, if the transmission time resource of one packet collides with that of other packets, but the selected OCC index is different, the decoding of this packet could be deemed successful. If the transmission resource of one packet collides with another packet, and the selected OCC index also collides with that of the other packet, the decoding for all involved packets is deemed unsuccessful.

  - For OCC-based DSA scheme: At the transmitter side, two slots are randomly selected from the 100 slots to transmit two replicas for a packet. And an OCC index is randomly selected from 4 available OCC indexes to transmit a packet. That means two replicas of this packet share a same OCC index. At the receiver side, if the transmission resource of one packet collides with that of other packets, but the selected OCC index is different, the decoding of this packet could be deemed successful. If the transmission resource of one packet collides with another packet, and the selected OCC index also collides with that of the other packet, the decoding for all involved packets is deemed unsuccessful.

  - For OCC-based CRDSA scheme: At the transmitter side, two slots are randomly selected from the 100 slots to transmit two replicas for a packet. And an OCC index is randomly selected from 4 available OCC indexes to transmit a packet. At the receiver side, if the transmission resource of one packet collides with that of other packets, but the selected OCC index is different, the decoding of this packet could be deemed successful. Furthermore, the interference generated by other replica of this packet on other slot would be canceled. At the other slot, after canceling a replica, if only one replica is remained, the packet to which the remained replica belongs is also deemed successful.

- In order to compare the transmission performance of different schemes, packet loss ratio is calculated corresponding to an increased lambda value. For DSA and CRDSA, if one or both replicas for a packet is received successfully, the counted number of successfully received packets are increased by 1. In order to eliminate the fluctuations caused by random numbers, the step 3 could be performed many times to achieve an averaged packet loss ratio. The packet loss ratio is calculated as below:

Packet Loss Ratio =1-(number of successfully received packets) /number of all packets                          (2)

- In order to compare the radio resource usage of different schemes, throughput is calculated corresponding to an increased lambda value. Please note, for DSA and CRDSA, if one replica is received successfully, the counted number of successfully received packets is increased by 1. If both replicas are received successfully, the counted number of successfully received packets is increased by 2. Also in order to eliminate the fluctuations caused by random numbers, the step 3 could be performed many times to achieve the averaged throughput. The throughput is calculated as:.

Throughput = counted number of successfully received packets /total number of slots                              (3)

- Transmission with repetitions is a key technology for IoT to enhance the coverage. As the repetition transmission requires more transmission resources and will cause more collisions, it may have non-negligible impacts on small data mechanism. Therefore, the simulation also evaluate the anti-collision schemes integrated with repetition transmission. For each transmission, repetition needs to occupy the continuous slots. In the simulation, only a kind of small repetition number, e.g., 8 is assumed. At the receiver side, when all the repetitions of a packet or replicas are decoded successfully, the reception of the packet or replicas could be deemed successful. Moreover, for packet loss ratio calculation, only when all the repetitions of one or both replicas (or a packet) are received successfully, the counted number of successfully received packets is increased by 1. For throughput calculation, each time a repetition of one or both replicas (or a packet) is received successfully, the counted number of successfully received packets is increased by 1.

Evaluation and comparison

#Ev-1: The packet loss ratio against the load (lambda), assuming no repetition transmission

Figure 1. The packet loss ratio against the load (without repetition)

In Figure 1, the packet loss ratio is plotted against the load (lambda) for each anti-collision scheme, assuming no repetition transmission. It can be seen that, under the low load condition which corresponds to fewer selected users and more idle time slots, the solutions of DSA and CRDSA, as well as their combinations with OCC can make better use of the idle slots to reduce the probability of collisions. Furthermore, the interference mitigation inherent in CRDSA's design significantly enhances the probability of successful packet decoding. Therefore, CRDSA and its combination with OCC solutions show the best performance at low load condition. However, we think the low load case isn’t the typical scenario for UL capacity enhancement in IoT NTN and therefore can't be emphasized to prove the advantage of an anti-collision scheme. On the contrary, as the load increases, the replica strategy in DSA or CRDSA related solutions can paradoxically escalate the resources occupation and finally result in additional collisions and system loads.

It can be further seen that, under the medium or high load conditions, OCC-based SA scheme requires less transmission resources and cause less collision. Therefore, OCC-based SA scheme achieves the lowest packet loss ratio.

It can also be noticed that OCC-based CRDSA, with the dual benefits of code division and interference cancellation technologies, offers the best performance, even in the medium or high load conditions.

The legacy SA scheme performs the worst under the low load condition. The reason may be that in this scheme, not only it lacks the higher probability of successful reception won by multiple replicas sending, but also the conflict cannot be effectively resolved due to the lack of optimized contention resolution or multiplexing mechanism. However, under high load condition, legacy SA scheme is still slightly better than DSA and CRDSA.

#Ev-2: The packet loss ratio against the load (lambda), assuming with repetition transmission

(a) Raw Figure                                                                                  (b) Zoomed Figure

Figure 2. The packet loss ratio against the load (with repetition)

In Figure 2, the packet loss ratio is plotted against the load (lambda) for each anti-collision scheme, assuming with repetition transmission. The repetition transmission is observed to significantly increase the probability of collisions. Under the high load condition, all solutions of DSA and CRDSA, as well as their combinations with OCC face exacerbated collisions and quickly degraded performance. Meanwhile, OCC-based SA scheme shows the lowest packet loss ratio and best performance under all the load conditions.

Observation 1a: Under the medium or high load conditions and with repetition transmission, OCC-based SA scheme can achieve the lowest packet loss ratio.

#Ev-3: The throughput against the load (lambda), assuming no repetition transmission

Figure 3. The throughput against the load (without repetition)

In Figure 3, the throughput is plotted against the load (lambda), assuming no repetition transmission. As the load increases, DSA and CRDSA related solutions would consume more resources and cause more collisions, thereby the probability of successful decoding is quickly decreased. In contrast, for OCC-based SA scheme, the introduction of the code division multiplexing mechanism can approximately increase the system capacity several times. Therefore, OCC-based SA scheme shows the better throughput.

#Ev-4: The throughput against the load (lambda), assuming with repetition transmission

(a) Raw Figure                                                                                  (b) Zoomed Figure

Figure 4. The throughput against the load (with repetition)

In Figure 4, the throughput is plotted against the load (lambda), assuming with repetition transmission. With similar reasons as that for packet loss ratio in the scenario with repetition, under high collision condition, although OCC-based SA scheme also starts to show the decreased throughput, it still shows better performance than other schemes.

Observation 1b: With repetition transmission, OCC-based SA scheme shows unparalleled advantages on throughput performance.

In summary, OCC-based SA scheme has substantially less packet loss ratio and higher throughput than legacy SA, DSA, CRDSA schemes. Furthermore, considering that transmission with repetition is one of the most commonly used techniques in IoT, although both OCC-based DSA and OCC-based CRDSA solutions can perform well in scenarios without repetition, they generally perform worse than OCC-based SA in scenarios with repetitions. Again, according to current RAN1 situation, it’s almost impossible to allocate additional TU to study interference cancellation related to CRDSA scheme. Therefore, we do not recommend DSA. CRDSA and their combination with OCC. We suggest to support only OCC-based SA scheme for CB-msg3 EDT-like mechanism.

Proposal 1a: It’s suggested to support only OCC-based SA scheme for CB-mg3 EDT-like mechanism.

Proposal 1b: Due to that DSA, CRDSA and their combination with OCC generally perform worse than OCC-based SA solution in scenario with repetitions, and also due to that RAN1 has no additional time to study interference cancellation related to CRDSA schemes, it’s also suggested not to further study the CRDSA-based solutions.

Contention-based Msg3 transmission

In this section, we will give our analysis on the RAN2 spec impacts for supporting contention-based Msg3 transmission.

Conditions for using contention-based Msg3 transmission

Taking legacy PUR as reference, in legacy PUR, UE needs to firstly fulfil the Timing alignment validation condition (including both PUR TA timer-related criteria and RSRP-related criteria) for triggering PUR as below, then UE can skip the steps of preamble transmission and RAR reception and directly to send Msg3 on the dedicated resources.

<<skip the unrelated parts>> 5.3.3.19 Timing alignment validation for transmission using PUR The UE shall consider the timing alignment value for transmission using PUR to be valid when the following conditions are fulfilled: 1> either pur-TimeAlignmentTimer is not configured or pur-TimeAlignmentTimer is running as confirmed by lower layers; and 1> either pur-RSRP-ChangeThreshold (pur-NRSRP-ChangeThreshold in NB-IoT) is not configured or the following conditions are fulfilled: 2> compared to the stored serving cell reference (N)RSRP value, the serving cell (N)RSRP has not increased by more than increaseThresh; and 2> compared to the stored serving cell reference (N)RSRP value, the serving cell (N)RSRP has not decreased by more than decreaseThresh;

Also in general, UE specific PUR transmission can be performed only when the following conditions are met:

- Cond#1: The TA of the UE in IDLE state keeps valid;

- Cond#2: The UE camps on the cell where pur-Config was provided (if the UE is establishing or resuming an RRC connection in a cell that is different from the cell where pur-Config was provided, pur-Config is released).

In the early stage discussion for R17 IoT NTN, due to the following reasons, the legacy PUR which is based on dedicated resources may not be always applicable to IoT NTN, especially for the IoT over LEO case:

- Firstly, due to the movement of satellite, even for stationary UEs, the TA of UE in IDLE state may change frequently and the stored TA may not be able to always keep valid during the relative long pur-TimeAlignmentTimer period.

- Secondly, for moving cell of IoT over LEO and fixed cell of IoT over LEO with different PCI cases, satellite change would cause frequent cell change. Then it would be very easy or frequent to occur that the cell where UE camps is different from the cell where pur-Config was provided. As mentioned above, if UE initiates RACH on a new cell, the UE should release the PUR resources. That means, in such case cell changes frequently, the allocated PUR resources may hardly have chance to be used.

However, with the new objective in R19 IoT NTN, e.g., to support contention-based or shared resources-based Msg3 transmission, the restriction in legacy PUR that PUR can only be used in the last cell would be no longer needed. This will further make the contention-based Msg3 transmission applicable to not only the case of fixed cell of IoT over LEO with same PCI but also the cases of moving cell of IoT over LEO and fixed cell of IoT over LEO with different PCI.

Observation 2: To support contention-based Msg3 transmission may make the restriction that legacy PUR can only be used in the last cell no longer needed. This will further make the contention-based Msg3 transmission function applicable to not only the case of fixed cell of IoT over LEO with same PCI but also the cases of moving cell of IoT over LEO and fixed cell of IoT over LEO with different PCI.

Based on the Observation 2, the following proposal is given:

Proposal 2a: If contention-based Msg3 transmission is supported, when the camping cell of the UE changes, the UE doesn’t need to stop using this function.

Furthermore, for IoT NTN, TA compensation has been supported, it may make it easier for IoT NTN UE in IDLE state to maintain TA validation than IoT UE in TN network. In previous meeting, RAN2 has agreed to focus the study on contention-based Msg3 transmission and UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3. RAN2 has sent LS [4] to RAN1 and RAN4 to trigger them to discuss the feasibility and requirements for verifying uplink synchronization for sending msg3 without Msg1 and Msg2.

In [5], RAN1 gave the following response:

1. Overall Description: RAN1 would like to thank RAN2 for the LS in R1-2405786/R2-2405769 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2. Regarding RAN2’s questions, following is the RAN1’s response: Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2? RAN1’s response: From RAN1 perspective, assuming Note 1 and Note 2 are satisfied, an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy requirement for Msg3 transmission/reception without Msg1/Msg2 at least in the following cases: - NB-IoT with 3.75kHz SCS. - eMTC CE mode A. Note 1: RAN1 assumes NTA=0 for Msg3 transmission without Msg1/Msg2 Note 2: RAN1 assumes that the RAN4 UL synchronization requirements specified in Table 7.20A.2-1 and Table 7.24A.2-1 of TS 36.133 apply to the Msg3 transmission without Msg1/Msg2 in NB-IoT over NTN and eMTC over NTN, respectively. Note 3: RAN1 may further discuss the cases of NB-IoT with 15kHz SCS and eMTC CE mode B 2. Actions: To RAN2: ACTION: RAN1 respectfully asks RAN2 to take the response into account. To RAN4: ACTION: RAN1 respectfully asks RAN4 to confirm whether RAN1’s assumptions in Note 1 and Note 2 are valid.

In [6], RAN4 gave the following response:

# 1. Overall Description: RAN4 would like to thank RAN2 for the LS R2-2405769 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2. RAN2 asked for RAN4 following questions: Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2? Q2: If the answer for Q1 is no, from RAN4 and RAN1 perspective, how the required timing accuracy for Msg3 transmission can be satisfied in this case? RAN4’s feedback: For Msg3 transmission without Msg1/Msg2, UE can meet UE UL transmit timing requirements defined for IoT NTN in TS 36.133[attached in Annex] from UE RRM requirement definition point of view. Note 1: RAN4 assumed that NTA_Ref is not configured by the network for MSG3 transmission without Msg1/Msg2. RAN4 will follow the conclusion of the other working groups in the future, if any. Note 2: For NB-IoT with 15kHz SCS, the timing error may be larger than half CP. # 2. Actions: RAN4 respectfully asks RAN2 to take the above information into account.

Per our understanding, RAN1 and RAN4 generally confirm that RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2. And the timing accuracy requirements for PUR transmission in IoT NTN can be applied. There may be some performance issue for the case of NB-IoT with 15kHz SCS which can be further discussed by RAN1 and RAN4.

Observation 3: RAN1 and RAN4 gave response LS and could confirm that RRC Idle UE with a pre-compensated TA can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2.

Therefore, the timing alignment validation condition (including both PUR TA timer-related criteria and RSRP-related criteria) in legacy PUR would not be needed for contention-based Msg3 transmission in R19 IoT NTN.

Proposal 2b: The timing alignment validation condition in legacy PUR (including both PUR TA timer-related criteria and RSRP-related criteria) would not be needed for contention-based Msg3 transmission in R19 IoT NTN.

The resources configuration for contention-based Msg3 transmission

For legacy PUR, NW needs to provide UE with dedicated resources configuration via RRC connection release message. For reference, the resources configuration is copied as below (taking NB-IoT as example):

– PUR-Config-NB The IE PUR-Config-NB is used to specify PUR configuration. PUR-Config-NB information element -- ASN1START PUR-Config-NB-r16 ::= SEQUENCE { pur-ConfigID-r16 PUR-ConfigID-NB-r16 OPTIONAL, --Need OR pur-TimeAlignmentTimer-r16 INTEGER (1..8) OPTIONAL, --Need OR pur-NRSRP-ChangeThreshold-r16 SetupRelease {PUR-NRSRP-ChangeThreshold-NB-r16} OPTIONAL, --Need ON pur-ImplicitReleaseAfter-r16 ENUMERATED {n2, n4, n8, spare} OPTIONAL, --Need OR pur-RNTI-r16 C-RNTI OPTIONAL, --Need ON pur-ResponseWindowTimer-r16 ENUMERATED {pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64} OPTIONAL, --Need ON pur-StartTimeParameters-r16 SEQUENCE { periodicityAndOffset-r16 PUR-PeriodicityAndOffset-NB-r16, startSFN-r16 INTEGER (0..1023), startSubframe-r16 INTEGER (0..9), hsfn-LSB-Info-r16 BIT STRING (SIZE(1)) } OPTIONAL, --Need ON pur-NumOccasions-r16 ENUMERATED {one, infinite}, pur-PhysicalConfig-r16 SEQUENCE { carrierConfig-r16 CarrierConfigDedicated-NB-r13, npusch-NumRUsIndex-r16 INTEGER (0..7), npusch-NumRepetitionsIndex-r16 INTEGER (0..7), npusch-SubCarrierSetIndex-r16 CHOICE { khz15 INTEGER (0..18), khz3dot75 INTEGER (0..47) }, npusch-MCS-r16 CHOICE { singleTone INTEGER (0..10), multiTone INTEGER (0..13) }, p0-UE-NPUSCH-r16 INTEGER (-8..7), alpha-r16 ENUMERATED {al0, al04, al05, al06, al07, al08, al09, al1}, npusch-CyclicShift-r16 ENUMERATED {n0, n6}, npdcch-Config-r16 NPDCCH-ConfigDedicated-NB-r13 } OPTIONAL, -- Need ON ..., [[ pur-PhysicalConfig-v1650 SEQUENCE { ack-NACK-NumRepetitions-r16 ACK-NACK-NumRepetitions-NB-r13 } OPTIONAL --Need ON ]], [[ pur-PhysicalConfig-v1700 SEQUENCE { pur-UL-16QAM-Config-r17 SetupRelease {PUR-UL-16QAM-Config-NB-r17} OPTIONAL, -- Need ON pur-DL-16QAM-Config-r17 SetupRelease {NPDSCH-16QAM-Config-NB-r17} OPTIONAL -- Need ON } OPTIONAL -- Need ON ]] } PUR-NRSRP-ChangeThreshold-NB-r16 ::= SEQUENCE { increaseThresh-r16 NRSRP-ChangeThresh-NB-r16, decreaseThresh-r16 NRSRP-ChangeThresh-NB-r16 OPTIONAL --Need OP } PUR-UL-16QAM-Config-NB-r17 ::= SEQUENCE { uplinkPowerControlDedicated-r17 UplinkPowerControlDedicated-NB-v1700 } NRSRP-ChangeThresh-NB-r16 ::= ENUMERATED {dB4, dB6, dB8, dB10, dB14, dB18, dB22, dB26, dB30, dB34, spare6, spare5, spare4, spare3, spare2, spare1} -- ASN1STOP ------

With reference to above PUR-Config IE, from RAN2 perspective, we understand at least the following resources types need to be configured for contention-based Msg3 transmission:

- Time domain resources, e.g., Periodicity, Offset, start time (H-SFN, frame, SFN etc.) of NPUSCH occasions

- Frequency domain resources, e.g., Carrier, SubCarrier

- TBS

- NPDCCH search space window

It can be expected that, although some of the resources for contention-based Msg3 transmission may be similar as the ones in legacy pur-Config, it’s still possible that the detailed configuration contents will be somewhat different from the legacy PUR-Config. For example, the resources for contention-based Msg3 may also have a “start time” element which is similar as the corresponding one in PUR-Config. For dedicated PUR, the start time of PUR resources is relative to the last subframe of the first transmission of RRCConnectionRelease message containing pur-Config, but it’s highly possible this would not be the case for the definition of start time in the resources for contention-based Msg3.

Observation 4: Even shared resources configuration for contention-based Msg3 transmission may have some resources IEs which are similar as the ones in legacy PUR-Config, it’s still possible that the detailed configuration contents in the resources configuration for contention-based Msg3 transmission will be somewhat different from the one in legacy PUR resources configuration.

According to our analysis and comparison in the section 2.1, it can be seen that OCC-based SA scheme can achieve the best performance. Therefore, RAN2 can make general assumption that OCC (orthogonal cover codes) sequence can be further applied to NPUSCH (Msg3) transmission to reduce the collision caused by using shared time/frequency domain resources and facilitate more UEs multiplexing on a same set time/frequency domain resources. From RAN2 perspective, the OCC sequences will be a new resource type which needs to be introduced in the NPUSCH resources configuration for contention-based Msg3 transmission.

Proposal 3a: With reference to legacy PUR configuration, RAN2 can take the following resources types as start point for the discussion on resources configuration for contention-based Msg3 transmission:

- Time domain resources, e.g., Periodicity, Offset, start time (H-SFN, frame, SFN etc.) of NPUSCH occasions

- Frequency domain resources, e.g., Carrier, SubCarrier

- TBS

- NPDCCH search space window

- OCC resources

As for the configuration way, a straightforward way for providing resources for contention-based Msg3 transmission is to provide a common resource configuration via SIB (Alt1) that can be shared by all the UEs. This way may have highest collision probability.

Taking into account the potential contents mentioned above, we think it’s also possible to provide resources for contention-based Msg3 transmission via dedicated signaling, e.g., via RRC connection release message (Alt2). By this way, NW is allowed to configure some different resources for different UEs (e.g., different OCC sequence/index) while the other resources still can be same for all the UEs.

Another usually used configuration way is to provide multiple sets of resources for contention-based Msg3 transmission in the SIB instead of only one set, and then NW can indicate index of the resource set for a specific UE via the dedicated signaling (Alt3). By this way, the UEs seem as being divided into several groups. The UEs within a group share same set of resource while the UEs in different groups using different resource set.

Both Alt2 and Alt3 are better ways for reducing the collision, e.g., better than Alt1, but may need more resources provision. And another disadvantage of Alt2 and Alt3 is that they may still need the restriction that the resources can only be used in the last cell.

It suggests RAN2 to discuss/evaluate the following three alternatives:

Proposal 3b: RAN2 is suggested to discuss/evaluate the following alternatives for configuring resources for contention-based Msg3 transmission:

- Alt1: To provide a totally shared resource via SIB.

- Alt2: To provide (partially) shared or dedicated resources via dedicated signaling. It can be allowed to configure some different resources for different UEs while the other resources may be same for all the UEs.

- Alt3: To provide multiple sets of shared resources via the SIB and NW can indicate index of the resource set for a specific UE via the dedicated signaling.

RNTI and contention resolution

RNTI is another aspect that needs to be discussed. It’s mainly related to contention resolution and scheduling of Msg4.

In legacy PUR, PUR-RNTI is used which is allocated by eNB and provided to UE within dedicated pur-Config. At least in UE side, it can be seen as a UE-specific RNTI. The DL transmission using PUR resources are addressed by this dedicated PUR-RNTI and no other contention resolution scheme is needed.

For contention-based Msg3 transmission, it’s easy to understand such RNTI is used not only for address the DL transmission, but also maybe with the intention to resolve at least part of the collisions.

The following alternatives can be considered for the RNTI design (now still call it as PUR-RNTI for discussion convenience, but exact naming can be late decided) for contention-based Msg3 transmission:

- Common PUR-RNTI (Alt1): That means only one PUR-RNTI may be configured in the shared Msg3 transmission resources configuration. All the UEs using shared Msg3 transmission resources can use this common PUR-RNTI to address DL transmission. Such common PUR-RNTI may be similar as P-RNTI or SI-RNTI and just for identifying the contention-based Msg3/Msg4 transmission. It seems that no benefit of collision resolution can be expected.

- A UE-specific PUR-RNTI allocated by NW (Alt2): Even the shared Msg3 transmission resources can be configured via SIB, it still can be possible to let NW separately configure a use-specific PUR-RNTI via dedicated signalling. This UE-specific PUR-RNTI seems similar as that in dedicated PUR, but the difference is that it’s used in a shared resources space.

- A resource-specific PUR-RNTI (Alt3): Such RNTI may be similar as RA-RNTI. UE and eNB can respectively calculate the PUR-RNTI according to the resources used/selected by the UE for Msg3 transmission. The eNB use the calculated RA-RNTI to address the DL transmission (e.g., PDCCH/Msg4). The UE only need to handle the DL transmission addressed by its resource-specific PUR-RNTI. However, as it’s still possible that some UEs use same shared resources for Msg3 transmission and will expect same resource-specific PUR-RNTI, the other contention resolution may still be needed.

As this RNTI design is also applicable to the scheduling of Msg4, the proposal will be given under the discussion for Msg4.

Efficient delivery of Msg4

In this section, we will discuss RAN2 aspects for supporting efficient delivery of Msg4 which corresponds to the contention-based Msg3 transmission.

Msg4 types

In legacy PUR, the following types of “Msg4” are possibly received by UE in the normal process (the abnormal cases, e.g., the PUR fallback or PUR failure indicated from lower layer are not mentioned):

- Physical layer L1 ACK or Timing Advance Command MAC CE: if no DL data needs to be transmitted to the UE, NW may send the L1 ACK to acknowledge the successful Msg3 transmission. It’s also possible for NW to only send a TAC MAC CE to update the TA for the UE in idle. Upon reception one of these two indications, UE can assume the transmission using PUR was successful

- RRCEarlyDataComplete or RRCConnectionRelease with data: if only one DL data needs to be sent to UE, NW can send RRCEarlyDataComplete or RRCConnectionRelease message containing the DL data. Upon reception of such signalling, UE also can think the transmission using PUR was successful.

- RRCConnectionSetup or RRCConnectionResume for fallback: if more data need to be transmitted, NW can send RRCConnectionSetup or RRCConnectionResume messages to fall back UE to connected mode. A newUE-Identity (C-RNTI) may be provided for UE to use after moving to RRC_CONNECTED mode.

Generally, on one hand, we think the above Msg4 types are already enough to cover the main transmission scenarios, so at least now no need can be seen to introduce additional Msg4 signaling. On the other hand, all above types of Msg4 need to be supported for contention-based Msg3 transmission scheme to handle different scenarios.

Moreover, the very simple Physical layer L1 ACK has already been introduced in legacy PUR and it’s still applicable to contention-based Msg3 transmission scheme to achieve the most efficient Msg4 transmission.

Proposal 4: The existing types of “Msg4” for normal PUR process, e.g., Physical layer L1 ACK, Timing Advance Command MAC CE, and RRCEarlyDataComplete/RRCConnectionRelease/RRCConnectionSetup/ RRCConnectionResume, need to be supported for contention-based Msg3 transmission scheme. The very simple Physical layer L1 ACK can be used in suitable cases to achieve the most efficient Msg4 transmission.

The scheduling of Msg4

In legacy PUR, UE in IDLE mode monitors Msg4 addressed by PUR-RNTI within a search space which is specifically configured for UE. But for contention-based Msg3 transmission scheme, we assume only common search space can be used. RAN1 can give further information on this aspect.

Observation 5: For contention-based Msg3 transmission scheme, we assume only common search space can be used. This can be further discussed based on RAN1 progress.

Based on the discussion in section 2.2.3, RAN2 can discuss the following alternatives for scheduling of Msg4.

Proposal 5: RAN is suggested to discuss the following alternatives for scheduling of Msg4:

- Alt1: Common PUR-RNTI configured in the shared resources configuration.

- Alt2: UE-specific PUR-RNTI allocated by NW that is similar as the one for dedicated PUR.

- Alt3: Resource-specific PUR-RNTI which can be respectively calculated by UE and eNB according to the resources used/selected by the UE for Msg3 transmission.

Contention resolution identity

As mentioned above, it’s highly possible that the collision caused by using shared resources for Msg3 transmission cannot be completely resolved by RNTI design. The additional contention resolution scheme is still needed.

The straightforward way is to reuse the contention resolution scheme in legacy random access procedure. That is, after recpetion of Msg3, eNB can send a DL MAC PDU in which a Contention Resolution Identity is included. For UE side, it needs to check whether the UE Contention Resolution Identity included in the DL MAC PDU matches the 48 first bits of the CCCH SDU transmitted in Msg3.

Proposal 6: The contention resolution scheme in legacy random access procedure can be reused for contention-based Msg3 transmission, e.g., a UE Contention Resolution Identity can be included in the DL MAC PDU and UE checks whether the UE Contention Resolution Identity matches the 48 first bits of the CCCH SDU transmitted in Msg3.

Other issues

For now, the following other issues are identified for contention-based Msg3 transmission:

- Backoff scheme when Msg3 collision occurs

When the collision or congestion is detected by eNB, the eNB may provide kind of backoff indication via the Msg4 (or DL MAC CE before Msg4) to alleviate the situation. That generally means the UE needs to delay a backoff time and then can choose another time domain resources within the shared resources for Msg3 transmission. RAN2 can firstly discuss the need for backoff scheme after Msg3 and then further discuss the detailed scheme.

For the OCC-based SA scheme which is preferred in section 2.1, with combined with Backoff scheme, it may have similar benefit as DSA that is brought by more transmission opportunities. But OCC-based SA+Backoff may be better as multiple replicas are always needed in DSA while for OCC-based SA+Backoff, the additional transmission only occurs (may be seldom) when the previous transmission fails. So OCC-based SA scheme can achieve a better balance between more transmission opportunities and reduced resource occupation.

Observation 6: OCC-based SA can be combined with Backoff scheme, it may have similar benefit as DSA or may be better than it. OCC+DSA even can be seen as a sort of lower bound of the OCC-based SA + Backoff performance.

Therefore, it’s suggested to further study OCC-based SA+Backoff scheme.

- Coexistence of dedicated PUR and contention-based Msg3 transmission

It may be possible for eNB to configure both dedicated PUR for specific UEs and broadcast shared resources for contention-based Msg3 transmission. In such case, we assume UE will prioritize the use of dedicated PUR. Only when the trigger condition is not fulfilled or the dedicated PUR becomes invalid, UE can consider to use shared resources for contention-based Msg3 transmission.

Proposal 7: RAN2 is suggested to discuss the following other issues:

- Issue#1: Whether and how to introduce a backoff scheme when Msg3 transmission collision occurs

- Issue#2: How to handle the coexistence of dedicated PUR and contention-based Msg3 transmission

Conclusion

Based on the above discussion, the following agreements are given:

Proposal 1a: It’s suggested to support only OCC-based SA scheme for CB-mg3 EDT-like mechanism.

Proposal 1b: Due to that DSA, CRDSA and their combination with OCC generally perform worse than OCC-based SA solution in scenario with repetitions, and also due to that RAN1 has no additional time to study interference cancellation related to CRDSA schemes, it’s also suggested not to further study the CRDSA-based solutions.

Proposal 2a: If contention-based Msg3 transmission is supported, when the camping cell of the UE changes, the UE doesn’t need to stop using this function.

Proposal 2b: The timing alignment validation condition in legacy PUR (including both PUR TA timer-related criteria and RSRP-related criteria) would not be needed for contention-based Msg3 transmission in R19 IoT NTN.

Proposal 3a: With reference to legacy PUR configuration, RAN2 can take the following resources types as start point for the discussion on resources configuration for contention-based Msg3 transmission:

- Time domain resources, e.g., Periodicity, Offset, start time (H-SFN, frame, SFN etc.) of NPUSCH occasions

- Frequency domain resources, e.g., Carrier, SubCarrier

- TBS

- NPDCCH search space window

- OCC resources

Proposal 3b: RAN2 is suggested to discuss/evaluate the following alternatives for configuring resources for contention-based Msg3 transmission:

- Alt1: To provide a totally shared resource via SIB.

- Alt2: To provide (partially) shared or dedicated resources via dedicated signaling. It can be allowed to configure some different resources for different UEs while the other resources may be same for all the UEs.

- Alt3: To provide multiple sets of shared resources via the SIB and NW can indicate index of the resource set for a specific UE via the dedicated signaling.

Proposal 4: The existing types of “Msg4” for normal PUR process, e.g., Physical layer L1 ACK, Timing Advance Command MAC CE, and RRCEarlyDataComplete/RRCConnectionRelease/RRCConnectionSetup/ RRCConnectionResume, need to be supported for contention-based Msg3 transmission scheme. The very simple Physical layer L1 ACK can be used in suitable cases to achieve the most efficient Msg4 transmission.

Proposal 5: RAN is suggested to discuss the following alternatives for scheduling of Msg4:

- Alt1: Common PUR-RNTI configured in the shared resources configuration.

- Alt2: UE-specific PUR-RNTI allocated by NW that is similar as the one for dedicated PUR.

- Alt3: Resource-specific PUR-RNTI which can be respectively calculated by UE and eNB according to the resources used/selected by the UE for Msg3 transmission.

Proposal 6: The contention resolution scheme in legacy random access procedure can be reused for contention-based Msg3 transmission, e.g., a UE Contention Resolution Identity can be included in the DL MAC PDU and UE checks whether the UE Contention Resolution Identity matches the 48 first bits of the CCCH SDU transmitted in Msg3.

Proposal 7: RAN2 is suggested to discuss the following other issues:

- Issue#1: Whether and how to introduce a backoff scheme when Msg3 transmission collision occurs

- Issue#2: How to handle the coexistence of dedicated PUR and contention-based Msg3 transmission

Reference

[1] RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc., RAN#105, September, 2024

[2] R2-2407502, Discussion on DSA and CRDSA Performance, ESA, Eutelsat Group, Viasat, Inmarsat, Novamint, Echostar, Sateliot, RAN2#127, August, 2024

[3] R2-2407555, UL capacity enhancements objectives for IoT NTN, Ericsson, RAN2#127, August, 2024

[4] R2-2405769, LS to RAN4 and RAN1 on TA for Msg3, ZTE Corporation, Sanechips, RAN2#126, May, 2021

[5] R1-2407548, Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1Msg2 (contact: ZTE), RAN1, To: RAN2, RAN4, RAN1#118, August, 2024

[6] R4-2414114, Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1Msg2 (contact: ZTE), RAN4, To: RAN2, RAN1, RAN1#118, August, 2024