3GPP TSG-RAN WG2 Meeting #127bis	R2-2408335

Hefei, China, 14th – 18th October, 2024

Source:	ZTE Corporation, Sanechips

Title:	PWS support for NB-IoT over NTN

Agenda Item:	8.9.4

Document for:	Discussion and Decision

Introduction

In RAN#102, a new WID for further enhancements on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) has been agreed and in RAN#105, the WID is further refined in [1]. A new objective has been added as below:

The aim of this WI is enhancement of IoT-NTN with the following objectives: <<skip the unrelated parts>> - Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: <skip the unrelated part> - Support of Capacity enhancements for uplink <skip the unrelated part> - Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In this document, we will give our initial analysis on the issues for broadcasting PWS message for NB-IoT by re-using the LTE mechanisms. Based on the analysis, we also give our proposals.

Discussion

  - PWS acquisition for UE in idle mode

To support PWS (Public Warning System) for NB-IoT over NTN is mainly to support CMAS (Commercial Mobile Alert Service) and ETWS (Earthquake and Tsunami Warning System). The purpose of ETWS is to provide emergency alerts to mobile devices within an affected area to warn users about potential dangers such as earthquakes and tsunamis. CMAS is a public safety alert system used in the United States to disseminate emergency messages through mobile devices.

Legacy LTE and eMTC can support providing PWS to UEs via SIB10~SIB12 which can be found as below for reference:

– SystemInformationBlockType10 The IE SystemInformationBlockType10 contains an ETWS primary notification. SystemInformationBlockType10 information element -- ASN1START SystemInformationBlockType10 ::= SEQUENCE { messageIdentifier BIT STRING (SIZE (16)), serialNumber BIT STRING (SIZE (16)), warningType OCTET STRING (SIZE (2)), dummy OCTET STRING (SIZE (50)) OPTIONAL, -- Need OP ..., lateNonCriticalExtension OCTET STRING OPTIONAL } -- ASN1STOP SystemInformationBlockType10 field descriptions. messageIdentifier Identifies the source and type of ETWS notification. The leading bit (which is equivalent to the leading bit of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.44) contains bit 7 of the first octet of the equivalent IE, defined in and encoded according to TS 23.041 [37], clause 9.4.3.2.1, while the trailing bit contains bit 0 of the second octet of the same equivalent IE.. serialNumber Identifies variations of an ETWS notification. The leading bit (which is equivalent to the leading bit of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.45), contains bit 7 of the first octet of the equivalent IE, defined in and encoded according to TS 23.041 [37], clause 9.4.3.2.2, while the trailing bit contains bit 0 of the second octet of the same equivalent IE.. dummy This field is not used in the specification. If received it shall be ignored by the UE.. warningType Identifies the warning type of the ETWS primary notification and provides information on emergency user alert and UE popup. The first octet (which is equivalent to the first octet of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.50) contains the first octet of the equivalent IE defined in and encoded according to TS 23.041 [37], clause 9.3.24, and so on.. – SystemInformationBlockType11 The IE SystemInformationBlockType11 contains an ETWS secondary notification. SystemInformationBlockType11 information element -- ASN1START SystemInformationBlockType11 ::= SEQUENCE { messageIdentifier BIT STRING (SIZE (16)), serialNumber BIT STRING (SIZE (16)), warningMessageSegmentType ENUMERATED {notLastSegment, lastSegment}, warningMessageSegmentNumber INTEGER (0..63), warningMessageSegment OCTET STRING, dataCodingScheme OCTET STRING (SIZE (1)) OPTIONAL, -- Cond Segment1 ..., lateNonCriticalExtension OCTET STRING OPTIONAL } -- ASN1STOP SystemInformationBlockType11 field descriptions. dataCodingScheme Identifies the alphabet/coding and the language applied variations of an ETWS notification. The octet (which is equivalent to the octet of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.52), contains the octet of the equivalent IE defined in TS 23.041 [37], clause 9.4.3.2.3, and encoded according to TS 23.038 [38].. messageIdentifier Identifies the source and type of ETWS notification. The leading bit (which is equivalent to the leading bit of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.44), contains bit 7 of the first octet of the equivalent IE, defined in and encoded according to TS 23.041 [37], clause 9.4.3.2.1, while the trailing bit contains bit 0 of second octet of the same equivalent IE.. serialNumber Identifies variations of an ETWS notification. The leading bit (which is equivalent to the leading bit of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.45) contains bit 7 of the first octet of the equivalent IE, defined in and encoded according to TS 23.041 [37], clause 9.4.3.2.2, while the trailing bit contains bit 0 of second octet of the same equivalent IE.. warningMessageSegment Carries a segment of the Warning Message Contents IE defined in TS 36.413 [39], clause 9.2.1.53. The first octet of the Warning Message Contents IE is equivalent to the first octet of the CB data IE defined in and encoded according to TS 23.041 [37], clause 9.4.2.2.5, and so on.. warningMessageSegmentNumber Segment number of the ETWS warning message segment contained in the SIB. A segment number of zero corresponds to the first segment, one corresponds to the second segment, and so on.. warningMessageSegmentType Indicates whether the included ETWS warning message segment is the last segment or not.. Conditional presence; Explanation. Segment1; The field is mandatory present in the first segment of SIB11, otherwise it is not present.. – SystemInformationBlockType12 The IE SystemInformationBlockType12 contains a CMAS notification. SystemInformationBlockType12 information element -- ASN1START SystemInformationBlockType12-r9 ::= SEQUENCE { messageIdentifier-r9 BIT STRING (SIZE (16)), serialNumber-r9 BIT STRING (SIZE (16)), warningMessageSegmentType-r9 ENUMERATED {notLastSegment, lastSegment}, warningMessageSegmentNumber-r9 INTEGER (0..63), warningMessageSegment-r9 OCTET STRING, dataCodingScheme-r9 OCTET STRING (SIZE (1)) OPTIONAL, -- Cond Segment1 lateNonCriticalExtension OCTET STRING OPTIONAL, ..., [[ warningAreaCoordinatesSegment-r15 OCTET STRING OPTIONAL -- Need OR ]] } -- ASN1STOP SystemInformationBlockType12 field descriptions. dataCodingScheme Identifies the alphabet/coding and the language applied variations of a CMAS notification. The octet (which is equivalent to the octet of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.52), contains the octet of the equivalent IE defined in TS 23.041 [37], clause 9.4.3.2.3, and encoded according to TS 23.038 [38].. messageIdentifier Identifies the source and type of CMAS notification. The leading bit (which is equivalent to the leading bit of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.44) contains bit 7 of the first octet of the equivalent IE, defined in and encoded according to TS 23.041 [37], clause 9.4.3.2.1, while the trailing bit contains bit 0 of second octet of the same equivalent IE.. serialNumber Identifies variations of a CMAS notification. The leading bit (which is equivalent to the leading bit of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.45), contains bit 7 of the first octet of the equivalent IE, defined in and encoded according to TS 23.041 [37], clause 9.4.3.2.2, while the trailing bit contains bit 0 of second octet of the same equivalent IE.. warningAreaCoordinatesSegment If present, carries a segment, with one or more octets, of the geographical area where the CMAS warning message is valid as defined in [98]. The first octet of the first warningAreaCoordinatesSegment is equivalent to the first octet of Warning Area Coordinates IE defined in and encoded according to TS 23.041 [37] and so on.. warningMessageSegment Carries a segment, with one or more octets, of the Warning Message Contents IE defined in TS 36.413 [39]. The first octet of the Warning Message Contents IE is equivalent to the first octet of the CB data IE defined in and encoded according to TS 23.041 [37], clause 9.4.2.2.5, and so on.. warningMessageSegmentNumber Segment number of the CMAS warning message segment contained in the SIB. A segment number of zero corresponds to the first segment, one corresponds to the second segment, and so on. If warning area coordinates are provided for the warning message, then this field applies to both warning message segment and warning area coordinates segment.. warningMessageSegmentType Indicates whether the included CMAS warning message segment is the last segment or not. If warning area coordinates are provided for the warning message, then this field applies to both warning message segment and warning area coordinates segment.. Conditional presence; Explanation. Segment1; The field is mandatory present in the first segment of SIB12, otherwise it is not present.. ------

Proposal 1: With reference to the structure of SIB10~SIB12, to introduce SIB10-NB~SIB12-NB for providing ETWS primary notification, ETWS secondary notification and CMAS notification to NB-IoT which can be applicable to both NB-IoT and NB-IoT over NTN.

ETWS primary notification and/ or ETWS secondary notification can occur at any point in time. As mentioned in TS 36.331, for LTE and eMTC, The Paging message and Direct Indication information can be used to inform ETWS capable UEs in RRC_IDLE about presence of an ETWS primary notification and/ or ETWS secondary notification. If the UE receives a Paging message or control channels associated with the shared data channel including the etws-Indication, it shall start receiving the ETWS primary notification and/ or ETWS secondary notification according to schedulingInfoList contained in SystemInformationBlockType1.

Similarly, CMAS notification can also occur at any point in time. The Paging message and Direct Indication information can be used to inform CMAS capable UEs in RRC_IDLE about presence of one or more CMAS notifications. If the UE receives a Paging message or Direct Indication information including the cmas-Indication, it shall start receiving the CMAS notifications according to schedulingInfoList contained in SystemInformationBlockType1.

With reference to above procedures, the following proposal is given:

Proposal 2: To introduce etws-Indication and cmas-Indication in Paging message and Direct Indication information to inform ETWS capable or CMAS capable NB-IoT UEs in RRC_IDLE about presence of ETWS primary notification and/or ETWS secondary notification, and/or one or more CMAS notifications.

  - PWS acquisition for UE in connected mode

In Rel-16 MTC enhancements, one of the objectives is to support that non-BL UEs operating in CE mode receive public warning messages such as ETWS and CMAS in connected mode as below:

Extreme coverage for non-BL UEs: - Specify CE mode A and B improvements for non-BL UEs from among the following list [RAN1, RAN2, RAN4] - ETWS/CMAS in connected mode

There were long discussion for this objective. The related agreements achieved step-by-step are copied in below to give reference to the possible discussion on support of PWS in NB-IoT over NTN:

| Meeting | Agreements |
| --- | --- |
| RAN2 #103bis | - RAN2 intends to support CMAS/ETWS for non-BL UEs in CE mode in connected mode |
| RAN2 #105 | - SIBs for ETWS and CMAS are not provided via dedicated signaling. |
| RAN2 #105bis | - Working assumption: In connected mode non-BL UEs in CE monitor MPDCCH to receive ETWS indication and/or CMAS indication using CSS in the same narrowband where unicast transmission can be received. LS to RAN1R2-1905278 is agreed to confirm something with RAN1, e.g., whether Type-0 CSS uses the same narrowband with USS |
| RAN2#107 | - In connected mode, non-BL UEs in CE monitor MPDCCH to receive ETWS and/or CMAS notification using Type 0 CSS in the same narrowband where unicast transmission can be received. - UE stays in RRC_CONNECTED after receiving the indication for ETWS/CMAS notification. - It is up to UE implementation how ETWS/CMAS notification is acquired. - RAN2 does not intend to introduce any signalling optimization regarding MIB, SIB1-BR and SIB10/11/12 acquisition. - RAN2 understands that UEs in CE Mode B may not be able to acquire ETWS notification within 4 seconds requirement. |
| RAN2#107bis | - Non-BL UE supporting ETWS/CMAS in CE mode is required to monitor the same subframes as in legacy, i.e. non-BL UEs in CE, to receive ETWS/CMAS notification when in connected mode. - Introduce capability for support of ETWS/CMAS indication reception in RRC CONNECTED by UE supporting CE Mode A. - Introduce capability for support of ETWS/CMAS indication reception in RRC CONNECTED by UE supporting CE Mode B. - Introduce in dedicated signalling an indication whether ETWS/CMAS notification is enabled for UE in connected mode. |
| RAN2#108 | - Use SI-RNTI for ETWS/CMAS notification. - UE is enabled to monitor for ETWS/CMAS notification via dedicated signalling. - The reception types for SI-RNTI for ETWS/CMAS notification is captured in TS 36.302. - For feedback based on CSI-RS, capability bit is introduced only for CE Mode A. |

Furthermore, some solutions have been mentioned during R16 discussion but finally not adopted. In order to give reference to the discussion and also to avoid duplicated discussion for NB-IoT, some main points are list as below:

- To provide ETWS and CMAS via dedicated signaling: In R15, non-BL UEs in CE mode cannot receive paging in connected mode. Therefore, non-BL UEs in CE mode are not able to receive the notifications for ETWS/CMAS and the corresponding SI message for ETWS/CMAS. So in R16, There once was view that the indication can be provided via dedicated message. However, more companies think dedicated signaling is not efficient from network resource standpoint. And it would be hard for the network to timely schedule all UEs in connected that support the features in case notification is needed. Even such scheduling is possible, the delay may be large which would finally result in that ETWS recpetion cannot meet the requirement, e.g., within 4 secs.

- Whether the UE can transits to RRC_IDLE to acquire ETWS/CMAS information upon reception of ETWS/CMAS notification: A bit more companies does not prefer UEs to go to idle. One issue may be that, If those UEs go to idle mode, they may trigger access to the network alsmost at the same time later, creating other signalling storm problem. Also some companies think it should be possible for these UEs to acquire the notification still in connected mode, considering that it is posssible for those UEs to receive using multiple narrownband. Another option that UE can switch to wideband has been mentioned but some companies think this can be left to UE implementation. So finlaly, RAN2 agreed that UE stays in RRC_CONNECTED after receiving the indication for ETWS/CMAS notification and it’s up to UE implementation about how ETWS/CMAS notification is acquired.

With reference to above information, the following proposals are given for NB-IoT:

Proposal 3a: RAN2 discuss whether to support ETWS/CMAS acquisition in connected mode for NB-IoT.

Proposal 3b: If RAN2 agrees to support ETWS/CMAS acquisition in connected mode for NB-IoT, RAN2 check with RAN1 whether it’s feasible for NB-IoT UE in connected mode to monitor NPDCCH CSS to receive etws-indication and/or cmas-indication.

Conclusion

Based on the above analysis, the following proposals are given:

Proposal 1: With reference to the structure of SIB10~SIB12, to introduce SIB10-NB~SIB12-NB for providing ETWS primary notification, ETWS secondary notification and CMAS notification to NB-IoT which can be applicable to both NB-IoT and NB-IoT over NTN.

Proposal 2: To introduce etws-Indication and cmas-Indication in Paging message and Direct Indication information to inform ETWS capable or CMAS capable NB-IoT UEs in RRC_IDLE about presence of ETWS primary notification and/or ETWS secondary notification, and/or one or more CMAS notifications.

Proposal 3a: RAN2 discuss whether to support ETWS/CMAS acquisition in connected mode for NB-IoT.

Proposal 3b: If RAN2 agrees to support ETWS/CMAS acquisition in connected mode for NB-IoT, RAN2 check with RAN1 whether it’s feasible for NB-IoT UE in connected mode to monitor NPDCCH CSS to receive etws-indication and/or cmas-indication.

Reference

[1] RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc., RAN#105, September, 2024