3GPP TSG-RAN WG2 Meeting #127bis	R2-2408338

Hefei, China, 14 - 18 Oct, 2024

Source: 	Huawei, HiSilicon, Turkcell

Title: 	Discussion on MBS broadcast over NTN

Agenda Item:	8.8.4

Document for:	Discussion and decision

Introduction

One of the objectives for Rel-19 NTN WI is the support of broadcast service over NTN:

Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signaling between CN and NG-RAN. [RAN3]

During RAN2 #127 meeting, the following agreements were made:

Agreements:

1.	The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

2.	RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

3.	For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2)

4.	For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

In this contribution, we provide our views on the remaining issues.

Discussion

RAN2 narrowed down the methods for providing the intended area and consider these following possibilities: SIB20, SIB21, or MBSBroadcastConfiguration. Each candidate for provision of intended service area has different implications, since SIB20, SIB21 and MBSBroadcastConfiguration take effect at different stages of the attempt to receive MBS service:

- SIB20 is used to convey the MCCH/MTCH configuration information (the frequency resource for MCCH and MTCH, the time resource for MCCH). If the serving cell sends SIB20, UE will receive the MCCH of this cell.

- SIB21 provides the information related to service continuity of MBS broadcast, i.e., the mapping of MBS FSAI and the current/ neighbor frequencies. If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider the MBS frequency to be the highest priority during the cell selection procedure.

- MBSBroadcastConfiguration message (provided via MCCH) is used to convey the control information for MBS (including the MRB configuration, PDSCH configuration for MTCH per MBS session, and neighbor cells which are also broadcasting this MBS session). UE will apply the corresponding configuration of an MBS session, e.g., the scheduling information and the MRB configuration per MBS session. Besides, the UE also determine whether the MBS session of interest is provided in any neighbor cell or not.

Provision of intended service area by SIB20 and MBSBroadcastConfiguration are used to direct the UE in the current serving cell to further acquire/apply the necessary parameter for the MBS session based on whether the UE is located in the intended service area or not. However, there are differences between the two kind of provisions.

If SIB20 provides the intended service area information, within which the cell provides MBS broadcast session (the detailed MBS session ID information is not in SIB20), the UE can determine not to monitor MCCH if it is not located in any of the service areas. This is also in line with the agreement “RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH.”

Proposal 1: Provide the intended service area info via SIB20 for the power saving purpose.

Furthermore, we observe that it is desirable to associate the intended service area with MBS session. The finer granularity enables the UE to know whether a specific MBS session, e.g. the MBS session the UE is interested in or would to be interested in, is provided for a certain area. So that UE could decide whether to monitor MTCH for acquisition of interested MBS session data.

MBSBroadcastConfiguration is able to associate each MBS session with its intended service area naturally since MBSBroadcastConfiguration already has the MBS session ID list. Certainly, SIB20 could also be enhanced to indicate MBS Session ID list and corresponding intended area of each MBS session, but this incurs more specification impacts. So, MBSBroadcastConfiguration to convey the intended service area with each MBS session ID is a good choice.

Proposal 2: Provide the intended service area info per each MBS session via MBSBroadcastConfiguration message.

Apart from MBS session ID list of the serving cell, MBSBroadcastConfiguration also indicates the MBS session ID list provided by the neighbor cells. Therefore, if MBSBroadcastConfiguration is responsible for associating the MBS session with intended area, both current serving cell’s and neighbor cell’s MBS session specific intended area should be indicated to UEs. This allows the UE to request unicast reception of the interested service after moving to a cell not providing the MBS session in UE’s location area.

Proposal 3: Both current serving cell’s and neighbor cell’s MBS session specific intended area should be indicated to UEs via MBSBroadcastConfiguration.

Proposal 4: Based on the neighbor cell’s MBS session specific intended area, the UE requests unicast reception of the interested service if moving to a cell not providing the MBS session in UE’s location area.

SIB21 is used to guarantee the service continuity of MBS session when the UE initiates the cell reselection procedure. The UE could prioritize an MBS frequency on which MBS session of interest is provided when the UE performs the cell reselection. With the provision of intended service area per MBS frequency by SIB21, the UE know the exact intended service area where a given frequency provides the desirable MBS session. Therefore, it is beneficial for the UE to treat an MBS frequency as the highest priority based on UE’s location and intended area per MBS frequency during cell reselection.

Proposal 5: The UE determine to camp on an MBS frequency as the highest priority based on UE’s location and intended area info per MBS frequency via SIB21 during cell reselection.

In the previous discussion, we have identified that the intended area information is needed in multiple places for different usages, including SIB20 (for whether to obtain MCCH), MBSBroadcastConfiguration (for whether to monitor MTCH for MBS traffic), SIB21 (for service continuity).

The next issue is which SIB or message should define the area information, considering it could be represented by a set of reference Location and radius or by a set of polygon(s). It takes much signalling overhead and should not be repeated in several places. From our point of view, the area information can be defined in a new SIB or existing SIB20, together with an area ID for each area. By doing so, the aforementioned places that require area information can simply refers to the area ID (which incurs much smaller signalling overhead compared with signalling the whole area definition).

Proposal 6: The intended service area (composed of one or multiple circles or polygons) is defined in a new SIB or the existing SIB20, each area is associated with an area ID. By doing so, the aforementioned places requiring service area info can include the corresponding area ID.

Conclusion

In this paper we discuss the support of broadcast service via NR NTN, and propose the following:

Proposal 1: Provide the intended service area info via SIB20 for the power saving purpose.

Proposal 2: Provide the intended service area info per each MBS session via MBSBroadcastConfiguration message.

Proposal 3: Both current serving cell’s and neighbor cell’s MBS session specific intended area should be indicated to UEs via MBSBroadcastConfiguration.

Proposal 4: Based on the neighbor cell’s MBS session specific intended area, the UE requests unicast reception of the interested service if moving to a cell not providing the MBS session in UE’s location area.

Proposal 5: The UE determine to camp on an MBS frequency as the highest priority based on UE’s location and intended area info per MBS frequency via SIB21 during cell reselection.

Proposal 6: The intended service area (composed of one or multiple circles or polygons) is defined in a new SIB or the existing SIB20, each area is associated with an area ID. By doing so, the aforementioned places requiring service area info can include the corresponding area ID.

References

- RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3.

- R2-2405703, Report from Break-Out Session on NR NTN and IoT NTN, Session chair (ZTE)