3GPP TSG-RAN WG2 Meeting #127bis	R2-2408346

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.7.4

Source:		Lenovo

Title:			Enhanced Logical channel prioritization for XR

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss enhancements to the uplink scheduling for XR traffic. A detailed look is taking to the  logical channel prioritization (LCP) procedure.

# Discussion

The Rel-19 Work Item on XR Phase 3 was agreed in RP-240791.

The Rel-19 XR Phase 3 objectives are as follows: Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2].

In RAN2#126 the following agreements were reached:

- Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. - The solution should consider impact on UE complexity (as already indicated in SI objective description) - For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - Use additional priority configured to LCHs in case of these LCHs with delay-critical data - FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH - We try to avoid RAN1 impacts. - RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way. - LCP prioritization within a logical channel will not be considered in RAN2 discussions - FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR). - Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. - FFS whether DSR triggering is impacted - FFS whether PDU set importance needs to be included

In RAN2#127 the following agreements were reached:

RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR. Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

### LCP procedure

As concluded in RAN2#126 meeting, delay-aware LCP enhancements to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data are supported in Rel-19 XR.

As already discussed in the last meetings, the issue with the current LCP procedure is that only the logical channel priority (partly also Bj) is used to determine the order (and amount of data) in which packets from different LCHs are multiplexed in a TB/MAC PDU. The remaining delay of a data packet, e.g. RLC PDU, is not considered when distributing UL resources to the LCHs.

For XR applications, it is important to ensure that packets/PDU sets are received within the associated PDU set/packet delay budget, e.g. PSDB.  Since application layer doesn’t benefit from packets which are received beyond its PSDB, e.g. packets are dropped, it’s of vital importance that data packets are successfully received within the associated delay boundaries. Given that the LCP procedure currently prioritizes data of different LCHs only based on the associated static LCH priority, it may happen that UE is unable to transmit data of a lower priority LCHs having a small remaining delay within its PDSB/PDB requirements if there is also data of a higher priority LCH pending in UEs buffer for transmission. Even though the higher priority data may have a large remaining delay and hence there is sufficient time for the transmission of the data, UE will prioritize the high priority data and potentially not assign any UL resources to the lower priority data which is close to its delay boundary. In particular for scenarios, where pdu-SetDiscard is configured, this may in turn result in PDUs /PDU sets being discarded. UL transmissions which have been already carried out for PDUs of a PDU set which is finally discarded due to exceeding its PSDB have a negative impact on the system capacity.

Even though the DSR procedure, which was introduced in Rel-18 to provide the serving gNB with delay status information, is already a tool to improve the uplink scheduling efficiency and also the uplink capacity, the full potential will be only exploited if also the LCP procedure considers remaining delay/time of the data. When using the legacy LCP procedure, it may still happen that UE can’t transmit the complete delay-critical data – reported in a DSR - on the UL resources allocated by the gNB in response to the reception of the DSR MAC CE, if there is higher priority data sitting in UEs’ buffer which is not delay-critical (and hence not being reported within the DSR MAC CE).

Therefore, the remaining time of a PDU/PDU set should be considered to determine the priority of a LCH respectively PDU.

Proposal 1: RAN2 to agree that remaining time of a PDU/PDU set should be considered when determining the priority of (data of) a LCH during LCP procedure.

Enhanced LCP procedure

- In RAN2#126bis meeting it was concluded that the option of overriding/adjusting the priority of a LCH during LCP based on the remaining time of the data of a LCH is used as a baseline for a delay-aware LCP procedure. UE may use an additional LCH priority configured for a LCH in case this LCH has delay-critical data available for transmissions. Alternatively, the UE may be configured with an offset by how much the priority of a LCH is increased, e.g. LCH priority value is decreased by this offset, in case delay-critical data is pending for transmission. Both implementation options are very similar in our understanding, and it might be slightly simpler to configure a second LCH priority for a LCH which is eligible for a delay-aware prioritization mechanism.

Proposal 2: NW can configure a second logical channel priority for a LCH to prioritize delay-critical data of the LCH.

For the agreed baseline solution of an enhanced delay-aware LCP procedure, there could be different flavors on how the enhanced LCH prioritization is implemented. The general approach would be to adapt the LCH priority based on the remaining delay time of data within the LCH and perform the legacy LCP procedure with the (adapted) LCH priorities. UE may for example increase the priority of a LCH in case the remaining delay/time of data of this LCH is becoming lower than a threshold or use a second configured priority. It should be noted that RAN2 would need to further discuss when exactly MAC entity determines the priority of a LCH for the LCP procedure, e.g. when MAC determines which LCH priority to apply. MAC may for example only determine once the priority of a LCH - by considering the remaining time of data buffered for this LCH - before the (legacy) LCP procedure is executed. This determination step would be similar to determining a delay-critical LCH as mentioned above.

In order to provide more fairness, MAC may further determine the priority of a LCH additionally before executing the second step of the LCP procedure, i.e. assigning UL resource in a strict priority order regardless of Bj . This would ensure that a LCH is not further prioritized, e.g. using the second (higher) LCH priority, during the second step of the LCP procedure if all the delay-critical data of a LCH has been already multiplexed in the MAC PDU during the first step of LCP. This additional fairness would however come at the expense of a slightly increased complexity.

In our view, complexity is an important factor to consider when designing an enhanced LCP procedure, however it is also of equal importance to consider aspects such as fairness. Therefore, we slightly prefer the second option, where MAC determines the LCH priority to apply before every step of the LCP procedure.

Proposal 3: RAN2 should discuss when MAC determines the priority of a LCH for/during LCP by considering the delay status of the data of the LCH, e.g. only once before executing the LCP procedure or before each step/round of the LCP procedure to ensure more fairness.

RAN2 needs to also further discuss how to define a delay-critical LCH, e.g. when UE should apply a higher LCH priority for a LCH during LCP. In our understanding a LCH is considered a delay-critical LCH and should use a second higher LCH priority if there is data available for transmission for the LCH for which the remaining time is less than a configured threshold. We think that the threshold for determining a delay-critical LCH should be separate to the delay threshold used for the triggering of a DSR. The reason is that the DSR related remaining Time threshold needs to account for the delay incurred by SR transmission and reception of corresponding UL grant.

Proposal 4: RAN2 to agree that a delay-critical LCH is defined as a LCH having at least one PDU/PDU set available for transmission for which the remaining time till discardTimer expiry is less than the a configured threshold.  The threshold for determining a delay-critical LCH is a separate threshold than remainingTimeThreshold used for DSR triggering/reporting.

Regardless of which option is chosen as the detailed enhanced LCP procedure, we think that NW should control/configure if and for which LCHs the remaining time should be considered when executing the LCP. For example, the remaining time doesn’t need to be considered during LCP for every kind of service/LCH, e.g. only for specific LCHs carrying XR traffic. As another example, the enhanced LCP procedure may be only used when DSR has been reported to the gNB in order to ensure that delay-critical data – as reported in the DSR – is transmitted in the corresponding UL grant, e.g. received in response to the DSR. It should be also further discussed whether the enhanced LCP procedure is explicitly activated/deactivated by NW, e.g. similar to the PSI-based discarding procedure by a MAC CE.

Proposal 5: NW should control/configure whether the enhanced LCP procedure taking remaining delay/time into account is used or not by the UE. RAN2 to further discuss whether enhanced LCP procedure is explicitly activated/deactivated by NW.

Similar to the considerations for the LCP procedure, also for the intra-UE prioritization mechanism which was introduced in Rel-16 it should be also discussed whether to consider the remaining time/delay of the data. According to the current specification the priority of an overlapping UL grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, i.e., only the logical channel priority is considered to determine which UL grant is prioritized/deprioritized.

However, it doesn’t make sense to prioritize an UL grant which carries data for which the PSDB/PDB is already exceeded. Such UL grant should be rather deprioritized. Similarly, a MAC PDU/UL grant which carries delay-critical data or a DSR MAC CE should be treated as a prioritized grant in the intra-UE prioritization procedure.

Proposal 6: RAN2 should consider enhancements to the intra-UE prioritization procedure, e.g. considering the remaining delay of data when determining the priority of an UL grant (prioritized/deprioritized UL grant).

The PDU set importance based PDCP discard has been introduced for Rel-18 XR. In case of UL congestion, PSI based discard can be activated, and a lower discard timer value, e.g. discardTimerForLowImportance is applied to the low importance data.  When PSI based discarding is activated, UE only supports DSR reporting for high importance data. Following the same principle, when LCP procedure considers the remaining time/delay of data prioritization, the enhancement may only be applied to high importance data.

Proposal 7: RAN2 to discuss whether the LCP enhancements are only applicable for high importance packets in case of UL congestion, e.g., PSI-based discard is activated.

# Conclusion

In this contribution, we discuss enhancements to the uplink scheduling. We have the following proposals:

Proposal 1: RAN2 to agree that remaining time of a PDU/PDU set should be considered when determining the priority of (data of) a LCH during LCP procedure.

Proposal 2: NW can configure a second logical channel priority for a LCH to prioritize delay-critical data of the LCH.

Proposal 3: RAN2 should discuss when MAC determines the priority of a LCH for/during LCP by considering the delay status of the data of the LCH, e.g. only once before executing the LCP procedure or before each step/round of the LCP procedure to ensure more fairness.

Proposal 4: RAN2 to agree that a delay-critical LCH is defined as a LCH having at least one PDU/PDU set available for transmission for which the remaining time till discardTimer expiry is less than the a configured threshold.  The threshold for determining a delay-critical LCH is a separate threshold than remainingTimeThreshold used for DSR triggering/reporting.

Proposal 5: NW should control/configure whether the enhanced LCP procedure taking remaining delay/time into account is used or not by the UE. RAN2 to further discuss whether enhanced LCP procedure is explicitly activated/deactivated by NW.

Proposal 6: RAN2 should consider enhancements to the intra-UE prioritization procedure, e.g. considering the remaining delay of data when determining the priority of an UL grant (prioritized/deprioritized UL grant).

Proposal 7: RAN2 to discuss whether the LCP enhancements is only applicable for high importance packets in case of UL congestion, e.g., PSI-based discard is activated.

# References

### TS 38.321 v18.0.0