3GPP TSG-RAN WG2 Meeting #127bis	R2-2408347

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.7.3

Source:		Lenovo

Title:			Enabling TX/RX for XR during measurement gaps/restrictions

Document for:	Discussion and Decision

# Introduction

This contribution focuses on the following objective identified by the work item description (RP-241771):

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4]

  - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

In the following, we provide our views on potential RAN2 impacts.

# Discussion

The following working assumption was made in RAN1#118:

Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

An LS was sent to RAN4 in R1-2407561 to ask RAN4 to check the working assumption and inform RAN1 if there are any concerns that RAN1 needs to address. In particular RAN4 is looking into the impact of measurement (gap) skipping on the performance of RRM, e.g. potential performance degradation of the RRM measurements.

Furthermore, a processing time/timeline for the reception of the DCI indicating to skip a measurement gap, e.g. minimum processing time for reception of “MG skip” DCI, is expected to be defined by RAN4.

Depending on the reply from RAN4 there might be some impacts to RAN2 specifications, which we discuss in the following.

In our understanding, based on RAN1/RAN4 discussions, the skipping of measurement gaps may be restricted to specific types of measurement gaps or only applicable to specific measurement gap configurations in order to minimize the impact on the accuracy of RRM measurements. Furthermore, the number of consecutive MG skipping indications which UE is expected to receive may be limited, e.g. UE is not expected to receive more than X consecutive MG skipping indications. Those restrictions when being introduced by RAN4/RAN1 will have some impact to the RAN2 specifications, e.g. new RRC signalling implementing the MG skipping restrictions.

Observation 1: Based on RAN4/RAN1 decisions, new RRC signalling may need to be introduced to enable a restriction of MG skipping, e.g. MG skipping is only applicable to specific MG configurations or MG types.

However, in our view, RAN2 should wait for further progress in RAN1/4 before starting to discuss those new RRC configurations enabling MG skipping restrictions.

Proposal 1: RAN2 to wait for further progress in RAN1/4 before discussing new configurations, e.g. measurement gap skipping restrictions.

RAN1 also discussed about new UE assistance information related to measurements occasions and related channel conditions, traffic, UE mobility. As shown in the agreement below, RAN1 discussed whether to introduce some new UE assistance information for solutions to enable /x/Rx in measurement gaps/restrictions that are caused by RRM measurements. Since no conclusion on the feasibility and usefulness of such UAI could be reached in RAN1 a related LS was sent in R1-2405736 (Cc RAN2). Depending on whether new UAI signalling will be supported based on input from RAN4, RAN2 will have to implement the new UAI reporting in TS38.331.

RAN1#117 Agreement RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study: - FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme. Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information. Conclusion RAN1 does not further discuss new UE assistance information related to channel conditions, traffic, UE mobility. Conclusion There is no consensus in RAN1 to support UE assistance information related to measurements occasions. The reason for this situation is lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). It is up to other working groups to trigger further work in RAN1 on UE assistance information.. Particularly, it was discussed whether impact on RRM performance from skipping measurement occasions may be reduced if UE sends additional information: - Information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period. - Information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable.

Proposal 2: RAN2 to wait for further progress in RAN4 before discussing the introduction of new UAI signalling for enabling Tx/Rx in gaps/restrictions that are caused by RRM measurements.

In our view some new UE capability signalling should be introduced indicating the support of Measurement gap skipping. Similar to other features like PDCCH skipping, also MG skipping based on the reception of a DCI to enable data reception/transmission during such measurement gaps may not be supported by all UEs.

Proposal 3: RAN2 to discuss the introduction of UE capability signalling for the skipping of measurement gaps, e.g. based on DCI indication.

## Impact of MG skipping DCI and DSR reporting

DSR reporting was introduced in Rel-18 to provide gNB with information on delay-critical data pending in UEs buffer for transmission. In response to the reception of a DSR gNB may immediately allocate UL resource to the UE to ensure that delay critical data is being transmitted before exceeding the corresponding delay budget, e.g. PSDB. We think that RAN2 should discuss the transmission/triggering of a DSR in the context of RRM measurement skipping. In order to make a meaningful decision in the gNB whether to skip an upcoming MG, gNB should be aware of data for which the associated remaining delay/discard timer is going to expire within the MG as exemplified in the figure below. Therefore, it may be beneficial when UE expedites the triggering/transmission of a DSR triggering (triggers a DSR sooner than the configured DSR threshold remainingTimeThreshold) in proximity of an upcoming cancellable MG, such that the gNB has at least certain time to cancel the MG to enable data transmission and reception, as well as control channel monitoring during the MG (Figure 1 and Figure 2).

Figure 1: DSR is triggered sooner than the configured threshold in the proximity of cancellable MG.

Figure 2: Timeline for DSR reporting in proximity of a MG

Furthermore, the cancellation rules of a previously triggered DSR in the proximity of a cancellable MG should be further looked at. A triggered/pending DSR in general should not be cancelled unless that DSR if transmitted is not useful from an application perspective. Hence, to give as much chance as possible to the UE to send its delay-critical UL data, the UE cancels the associated pending DSR, if it is too close to the MG (that is not yet cancelled), in which case transmitting that DSR would not be useful. Assuming that there will be some minimum processing time for the reception of a “MG skipping” DCI, e.g. indication to skip a MG needs to be received at least a predefined time before the start of the MG, UE should cancel a triggered DSR in case the UL resources for transmission of a DSR are less than the minimum MG skip DCI processing time from the start of the MG.

Proposal 4: RAN2 to discuss impacts of a cancellable MG to the DSR procedure, e.g. triggering and cancellation of DSR.

If UE is configured with measurement reporting events, UE should evaluate whether the measurement report event is fulfilled during the TimeToTrigger period associated with the event. After entering condition for a measurement report event being satisfied, UE continues to evaluate the event even if no latest measurement result is obtained due to the cancelled measurement gap occasion. Obviously, it may cause the false evaluation result when UE moves between the cell edge of two cells. For example, UE triggers Measurement Report (MR) based on the inaccurate evaluation and NW may trigger the handover procedure. In addition, the similar issue may happen to the conditional configuration event evaluation procedure. In legacy measurement procedure, the measurement based on measurement gap is only cancelled due to random access procedure or gap collision. The probability to cancel a measurement gap is quite low. However, with R19 MG cancelling function, the probability to cancel gap occasion becomes higher. Therefore, it is proposed RAN2 to discuss the impact of a cancellable MG to the measurement event evaluation procedure, whether to enhance the measurement event evaluation procedure, e.g., enhance TimeToTrigger due to a cancelled MG occasion.

Proposal 5: RAN2 to discuss impacts of a cancellable MG to the measurement event evaluation procedure, e.g. enhance TimeToTrigger due to a cancelled MG occasion.

# Conclusion

In this contribution, we discuss potential impacts of RRM measurement gap skipping to RAN2. We have the following proposals:

Observation 1: Based on RAN4/RAN1 decisions, new RRC signalling may need to be introduced to enable a restriction of MG skipping, e.g. MG skipping is only applicable to specific MG configurations or MG types.

Proposal 1: RAN2 to wait for further progress in RAN1/4 before discussing new configurations, e.g. measurement gap skipping restrictions.

Proposal 2: RAN2 to wait for further progress in RAN4 before discussing the introduction of new UAI signalling for enabling Tx/Rx in gaps/restrictions that are caused by RRM measurements.

Proposal 3: RAN2 to discuss the introduction of UE capability signalling for the skipping of measurement gaps, e.g. based on DCI indication.

Proposal 4: RAN2 to discuss impacts of a cancellable MG to the DSR procedure, e.g. triggering and cancellation of DSR.

Proposal 5: RAN2 to discuss impacts of a cancellable MG to the measurement event evaluation procedure, e.g. enhance TimeToTrigger due to a cancelled MG occasion.

# References

### TS 38.321 v18.0.0