3GPP TSG-RAN WG2 Meeting #127bis	R2-2408352

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:		7.20.2

Source:				ASUSTeK

Title:		Discussion on supporting 8Tx in MAC specification

Document for: 	Discussion and Decision

- Introduction

8Tx UL has been introduced by RAN1 in release 18 MIMO. In the last meeting, some corresponding changes for TS 38.321 were discussed, and it was concluded that more time is needed to analyze the impact on MAC specification in order to support 8Tx:

R2-2406519 Discussion on remaining issue for 8Tx in MAC specification ASUSTeK discussion Rel-18 NR_MIMO_evo_DL_UL-Core Discussion - ZTE think we do not need to add the note, and think UE will generate two TBs according to the UL grant. - CATT think the issue is valid, and note is useful. - LG E not sure whether the wording ‘for the HARQ process’ is correct. Discussions in CB - Asustek reports that in offline there are common understanding that some note may be needed, exact changes FFS. - In addition, in the offline discussion, some companies pointed out that HARQ modelling (i.e., in LTE there are two HARQ process ID for two UL grants for the case of UL spatial multiplexing, but in NR there is only one) discussions are needed. Asustek think more time is needed, also may need to check with R1. ZTE think the potential issue was found in this meeting can think in R2 we can analyse the impact. Samsung think we need more time to check if any issue in R2 spec, can discuss based on company contribution. - Samsung not ready to send LS to R1. Postponed

In this contribution, we aim to discuss and determine the modelling of HARQ operation of 8Tx and discuss the way forward to support 8Tx in MAC.

- Discussion

HARQ information and HARQ process for  8Tx HARQ operation

In NR release 18, 8Tx UL was introduced for MIMO. According to PHY and RRC, the maxRank and maxMIMO-Layers could be configured above 4 (up to 8), which indicates the maximum MIMO layer to be used for PUSCH. Based on the configuration, there could be up to two code words for PUSCH. That is to say, the UE could transmit up to two TBs at a same timing.

In TS 38.212, the DCI for scheduling up to two TBs (i.e., DCI format 0_1) comprises one HARQ process ID and separate fields for HARQ information (i.e., MCS, NDI, and RV) for the first TB and the second TB respectively:

(38.212 v18.3.0) - 7.3.1.1.2 Format 0_1 DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE. The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI: … For transport block 1: - Modulation and coding scheme - 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214] - New data indicator - 1 bit if the number of scheduled PUSCH indicated by the Time domain resource assignment field is 1; otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined based on the maximum number of schedulable PUSCH among all entries in the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH, where each bit corresponds to one scheduled PUSCH as defined in clause 6.1.4 in [6, TS 38.214]. - Redundancy version - - number of bits determined by the following: - 2 bits as defined in Table 7.3.1.1.1-2 if the number of scheduled PUSCH indicated by the Time domain resource assignment field is 1; - otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined by the maximum number of schedulable PUSCHs among all entries in the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH, where each bit corresponds to one scheduled PUSCH as defined in clause 6.1.4 in [6, TS 38.214] and redundancy version is determined according to Table 7.3.1.1.2-34. For transport block 2 (only present if maxRank or maxMIMO-Layers is larger than 4): - Modulation and coding scheme - 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214] - New data indicator - 1 bit - Redundancy version - 2 bits as defined in Table 7.3.1.1.1-2 If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part, maxRank is larger than 4 or the value of maxMIMO-Layers for the indicated bandwidth part is larger than 4 and the value of maxRank or maxMIMO-Layers for the active bandwidth part is no more than 4, the UE assumes zeros are padded when interpreting the "Modulation and coding scheme", "New data indicator", and "Redundancy version" fields for transport block 2 according to Clause 12 of [5, TS38.213], and the UE ignores the "Modulation and coding scheme", "New data indicator", and "Redundancy version" fields of transport block 2 for the indicated bandwidth part. - Transform precoder indicator - 0 or 1 bit - 1 bit if the higher layer parameter dynamicTransformPrecoderIndicationDCI-0-1 is configured to 'enabled ' and if the UE is configured to monitor DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI, where the bit value of 0 indicates that transform precoder is enabled and the bit value of 1 indicates that transform precoder is disabled. For a DCI format 0_1 with CRC scrambled by CS-RNTI and the value indicated by new data indicator field is 0, or for a DCI format 0_1 with CRC scrambled by SP-CSI-RNTI, the bit is reserved. - 0 bit otherwise. - HARQ process number - 5 bits if higher layer parameter harq-ProcessNumberSizeDCI-0-1 is configured; otherwise 4 bits

Observation 1: For scheduling multiple PUSCHs in one Cell, the DCI indicates one HARQ process ID and two sets of HARQ information (MCS, NDI and RV) for two TBs.

In 38.300, it is indicated that a single HARQ process is used for DL and UL spatial multiplexing:

(38.300 v18.2.0) ### 6.2.4 HARQ The HARQ functionality ensures delivery between peer entities at Layer 1. A single HARQ process supports one TB when the physical layer is not configured for downlink/uplink spatial multiplexing, and when the physical layer is configured for downlink/uplink spatial multiplexing, a single HARQ process supports one or multiple TBs.

Which is added based on an agreement in RAN2#98:

Agreements:

- RAN2 aims to keep Multi-bit HARQ feedback and CBG-based retransmission transparent to the MAC for one TB

- A single HARQ process supports one TB when the physical layer is not configured for downlink/uplink spatial multiplexing.

- A single HARQ process supports one or multiple TBs when the physical layer is configured for downlink/uplink spatial multiplexing.

- One HARQ entity should be supported in one carrier

Modeling of UL grants for  8Tx HARQ operation

In RAN2#126 meeting, it was agreed to capture “Each HARQ process supports one or two TBs” in MAC for UL HARQ. In RAN2#127 meeting, it was raised by companies that there is some ambiguity in the modelling of HARQ operation (i.e., one or two UL grants for the two-TB transmission), and a formal agreement may be needed in order to determine how to proceed to adopt 8Tx to MAC specification.

That is, RAN2 should decide from the two options to model the UL grant(s) for 2-TB PUSCH transmissions on a Cell scheduling by a DCI:

- Option 1: for uplink spatial multiplexing, a DCI schedules a single uplink grant for two-TB transmission for a HARQ process. This would mean that the single uplink grant is in charge of providing HARQ information of the two TBs (in other words, two sets of MCS, NDI, and RV belong to the same uplink grant).

- Option 2: for uplink spatial multiplexing, a DCI schedules two uplink grants for two-TB transmission for a HARQ process, each uplink grant is associated with one TB; that is, each uplink grant is in charge of HARQ information (MCS, NDI, and RV) of one of the two TBs.

Based on companies’ feedback as option 1 being more aligned with RAN1 intention, we propose RAN2 can go with option 1 for 8Tx HARQ operation.

Proposal 1: For uplink grant modeling of HARQ operation for uplink spatial multiplexing, a DCI schedules a single uplink grant for two-TB transmission for a HARQ process.

After determination of UL grant modelling, an analysis of MAC changes that are needed to support 8Tx is provided below. Based on the selected model, different MAC spec changes may be needed to support 8Tx:

NDI consideration for different TBs

Due to HARQ process having more than one NDI for multiple TBs, MAC specification needs to take multiple NDIs into consideration when determining whether the NDI(s) are toggled. For example, in the current UL grant reception procedure in section 5.4.1, the UE considers the NDI for a HARQ process to have been toggled if an uplink grant if for MAC entity’s C-RNTI and the previous uplink grant for the same HARQ process is associated with configured uplink grant:

### 5.4.1 UL Grant reception 1> if an uplink grant for this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI or Temporary C-RNTI; or 1> if an uplink grant has been received in a Random Access Response: 2> if the uplink grant is for MAC entity's C-RNTI and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was either an uplink grant received for the MAC entity's CS-RNTI or a configured uplink grant: 3> consider the NDI to have been toggled for the corresponding HARQ process regardless of the value of the NDI.

If option 1 (i.e., single uplink grant model) is selected, when the uplink grant is received and the condition is fulfilled, the UE should consider both NDIs to have been toggled for the HARQ process; if option 2 (i.e., two uplink grant model) is selected, the UE should consider each NDI toggling separately for each uplink grant.

Per-TB new transmission determination

In the current specification, the HARQ entity considers whether a uplink grant is a new transmission or retransmission in a per-HARQ process manner, including determining whether an uplink grant is for a new transmission by considering whether the HARQ buffer is empty:

#### 5.4.2.1 HARQ Entity … Each HARQ process supports one or two TBs. … For each uplink grant, the HARQ entity shall: 1> identify the HARQ process associated with this grant, and for each identified HARQ process: 2> if the received grant was not addressed to a Temporary C-RNTI on PDCCH, and the NDI provided in the associated HARQ information has been toggled compared to the value in the previous transmission of this TB of this HARQ process; or 2> if the uplink grant was received on PDCCH for the C-RNTI and the HARQ buffer of the identified process is empty; or … 2> if the uplink grant is part of a bundle of the configured uplink grant, and may be used for initial transmission according to clause 6.1.2.3 of TS 38.214 [7], and if no MAC PDU has been obtained for this bundle: 3> if there is a MAC PDU in the MSGA buffer and the uplink grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload was selected; or 3> if there is a MAC PDU in the MSGA buffer and the uplink grant was received in a fallbackRAR and this fallbackRAR successfully completed the Random Access procedure: 4> obtain the MAC PDU to transmit from the MSGA buffer. 3> else if there is a MAC PDU in the Msg3 buffer and the uplink grant was received in a fallbackRAR: 4> obtain the MAC PDU to transmit from the Msg3 buffer. 3> else if there is a MAC PDU in the Msg3 buffer and the uplink grant was received in a MAC RAR; or 3> if there is a MAC PDU in the Msg3 buffer and the uplink grant was received on PDCCH for the C-RNTI in ra-ResponseWindow and this PDCCH successfully completed the Random Access procedure initiated for beam failure recovery: 4> obtain the MAC PDU to transmit from the Msg3 buffer. 4> if the uplink grant size does not match with size of the obtained MAC PDU; and 4> if the Random Access procedure was successfully completed upon receiving the uplink grant: 5> indicate to the Multiplexing and assembly entity to include MAC subPDU(s) carrying MAC SDU from the obtained MAC PDU in the subsequent uplink transmission; 5> obtain the MAC PDU to transmit from the Multiplexing and assembly entity. 3> else if this uplink grant is a configured grant configured with autonomousTx; and 3> if the previous configured uplink grant, in the BWP, for this HARQ process was not prioritized; and 3> if a MAC PDU had already been obtained for this HARQ process; and 3> if the uplink grant size matches with size of the obtained MAC PDU; and 3> if none of PUSCH transmission(s) of the obtained MAC PDU has been completely performed: 4> consider the MAC PDU has been obtained. 3> else if the MAC entity is not configured with lch-basedPrioritization; or 3> if this uplink grant is a prioritized uplink grant: 4> obtain the MAC PDU to transmit from the Multiplexing and assembly entity, if any; … 2> else (i.e. retransmission): …

In 8Tx UL transmission, one HARQ process handles transmissions of two TBs, and the corresponding HARQ buffer is used to store two MAC PDUs generated for the two-TB transmission. As a DCI indicates two NDIs for two TBs, it is possible that a DCI can indicate a new transmission for a first TB and a retransmission for a second TB by assigning corresponding NDI values for each TB.

Observation 2: A single HARQ buffer of a HARQ process is used for storing two MAC PDUs for two-TB transmission.

Observation 3: For a 2-TB transmission in one Cell, the DCI can indicate a new transmission and a retransmission for the two TBs respectively.

Therefore, an issue might occur when using the existing procedure in the current spec to determine new transmission or retransmission for an uplink grant, when there’s one MAC PDU waiting for retransmission stored in the HARQ buffer and the NW intends to schedule a two-TB transmission for retransmission of the MAC PDU and a new transmission of a second TB, as shown in Figure 1. The UE does not consider the NDI_2 of the uplink grant to be toggled, as there’s no previous uplink grant for the second TB to compare with, and also the HARQ buffer is not empty for the HARQ process, so the UE does not obtain a MAC PDU for this second TB transmission.

Figure 1

Per-TB HARQ buffer handling

In the current specification, the HARQ buffer of a HARQ process is flushed when a new transmission is indicated but no MAC PDU has been obtained for the uplink grant:

#### 5.4.2.1 HARQ Entity … For each uplink grant, the HARQ entity shall: … 3> if a MAC PDU to transmit has been obtained: … 3> else: 4> flush the HARQ buffer of the identified HARQ process.

This is to avoid unintended (re)transmission of outdated data stored in the HARQ buffer that might be misread by NW as new data. In 8Tx, another issue would occur if the entire HARQ buffer is flushed for the HARQ process as it could affect the transmission of the other TB that shares the same HARQ buffer of the HARQ process.

Ensuring proper 2-TB generation

In LTE, when the UE is configured to transmit up to two TBs at a same timing and receives two UL grants for two TBs, the UE needs to generate both MAC PDUs once one MAC PDU is to be generated. More specifically, according to TS 36.321 as below, when the PHY is configured for uplink spatial multiplexing, the MAC can receive two UL grants for two TBs for the same TTI. And if at least one MAC PDU is to be generated for this TTI, the MAC generates MAC PDUs corresponding to all UL grants for this TTI (even if a condition for UL skipping may be fulfilled).

Observation 4: In LTE, the UE generates both MAC PDUs if one MAC PDU is to be generated, when the UE receives two UL grant for the TTI.

5.4.1 UL Grant reception In order to transmit on the UL-SCH the MAC entity must have a valid uplink grant (except for non-adaptive HARQ retransmissions) which it may receive dynamically on the PDCCH or in a Random Access Response or which may be configured semi-persistently or preallocated by RRC or provided by RRC for transmission using PUR (see clause 5.4.7). To perform requested transmissions, the MAC layer receives HARQ information from lower layers. When the physical layer is configured for uplink spatial multiplexing, the MAC layer can receive up to two grants (one per HARQ process) for the same TTI from lower layers. … ### 5.4.3 Multiplexing and assembly #### 5.4.3.1 Logical channel prioritization The Logical Channel Prioritization procedure is applied when a new transmission is performed. … If the MAC PDU includes only the MAC CE for padding BSR or periodic BSR with zero MAC SDUs and there is no aperiodic CSI requested for this TTI, as specified in TS 36.213 [2], the MAC entity shall not generate a MAC PDU for the HARQ entity in the following cases: - in case the MAC entity is configured with skipUplinkTxDynamic and the grant indicated to the HARQ entity was addressed to a C-RNTI; or - in case the MAC entity is configured with skipUplinkTxSPS and the grant indicated to the HARQ entity is a configured uplink grant activated by the MAC entity's Semi-Persistent Scheduling C-RNTI or by the MAC entity's UL Semi-Persistent Scheduling V-RNTI; or - in case the grant indicated to the HARQ entity is a configured uplink grant activated by the MAC entity's AUL C-RNTI; or - in case the grant indicated to the HARQ entity is a preconfigured uplink grant. NOTE 1a: If at least one MAC PDU is to be generated for the HARQ entity for this TTI, the MAC entity generates MAC PDUs corresponding to all UL grants indicated to the HARQ entity for this TTI.

The intention for adding the NOTE in LTE is that PHY is unable to perform corresponding transmission properly when MAC generates only one TB for UL spatial multiplexing, as a two TB transmission is scheduled by DCI and expected by eNB. When UL spatial multiplexing is configured, MAC can receive two dynamic UL grants at a given TTI and PHY would expect to receive two TBs from MAC, as PHY is unable to perform corresponding transmission if only one TB is generated from MAC. Without NOTE 1a, whether to generate MAC PDU is determined separately for each MAC PDU, so that the HARQ entity would generate only one MAC PDU if there is insufficient available data at the TTI. The Multiplexing and assembly entity would not generate the MAC PDU comprising pure padding when skipUplinkTxDynamic is configured, resulting in only one MAC PDU being generated.

Observation 5: The intention for the NOTE 1a in LTE is that PHY is unable to perform corresponding transmission properly with only one TB when two TBs are expected for UL spatial multiplexing.

As a result, this NOTE is needed irrespective of how many UL grants are received. For 8Tx UL in NR, even if one UL grant is used to schedule two TBs (i.e., option 1 modeling), whether to generate MAC PDU is still determined separately for each MAC PDU so that the same issue in LTE UL spatial multiplexing occurs. That is, in NR, when UE receives one UL grant for two TBs and there is insufficient available data, the Multiplexing and assembly entity would not generate the MAC PDU comprising pure padding when skipUplinkTxDynamic is configured and thus results in only one MAC PDU generated. If two UL grant modelling is adopted (i.e., option 2), the issue would be more similar to LTE scenario and a NOTE should be added as well except the wording would be different from one-UL grant case. Therefore, to address the same issue, it is proposed to add a similar note in NR as shown in the TP in Annex. The difference between legacy LTE NOTE and the proposed change is due to lack of concept of ‘TTI’ in NR, and we also try to cover the case where a DCI schedules a new transmission and a retransmission.

Based on the above analysis, two TPs are drafted for option 1 and 2 respectively, and corresponding TP can be adopted when RAN2 makes a conclusion on using which modeling for HARQ operation for 8Tx.

Proposal 2: If Proposal 1 is agreed, RAN2 adopts TP in Annex 1 as baseline.

- Conclusion

In this contribution, we have following observations and proposals:

Observation 1: For scheduling multiple PUSCHs in one Cell, the DCI indicates one HARQ process ID and two sets of HARQ information (MCS, NDI and RV) for two TBs.

Proposal 1: For uplink grant modeling of HARQ operation for uplink spatial multiplexing, a DCI schedules a single uplink grant for two-TB transmission for a HARQ process.

Observation 2: A single HARQ buffer of a HARQ process is used for storing two MAC PDUs for two-TB transmission.

Observation 3: For a 2-TB transmission in one Cell, the DCI can indicate a new transmission and a retransmission for the two TBs respectively.

Observation 4: In LTE, the UE generates both MAC PDUs if one MAC PDU is to be generated, when the UE receives two UL grant for the TTI.

Observation 5: The intention for the NOTE 1a in LTE is that PHY is unable to perform corresponding transmission properly with only one TB when two TBs are expected for UL spatial multiplexing.

Proposal 2: If Proposal 1 is agreed, RAN2 adopts TP in Annex 1 as baseline.

- Reference

- Annex 1: Possible TP for Option 1 in TS 38.321

5.4	UL-SCH data transfer

5.4.1	UL Grant reception

Uplink grant is either received dynamically on the PDCCH, in a Random Access Response, configured semi-persistently by RRC or determined to be associated with the PUSCH resource of MSGA as specified in clause 5.1.2a. The MAC entity shall have an uplink grant to transmit on the UL-SCH. To perform the requested transmissions, the MAC layer receives HARQ information from lower layers. When maxRank or maxMIMO-Layers (as specified in TS 38.214 [7]) is more than 4, the MAC layer can receive an uplink grant for up to two TBs on the same HARQ process from lower layers. An uplink grant addressed to CS-RNTI with NDI = 0 is considered as a configured uplink grant. An uplink grant addressed to CS-RNTI with NDI = 1 is considered as a dynamic uplink grant.

…

1>	if an uplink grant for this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI or Temporary C-RNTI; or

1>	if an uplink grant has been received in a Random Access Response:

2>	if the uplink grant is for MAC entity's C-RNTI and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was either an uplink grant received for the MAC entity's CS-RNTI or a configured uplink grant:

3>	consider the NDI(s) to have been toggled for the corresponding HARQ process regardless of the value of the NDI.

…

5.4.2.1	HARQ Entity

…

For each uplink grant, the HARQ entity shall:

1>	identify the HARQ process associated with this grant, and for each identified HARQ process and for each TB associated to the identified HARQ process:

2>	if the received grant was not addressed to a Temporary C-RNTI on PDCCH, and the NDI provided in the associated HARQ information has been toggled compared to the value in the previous transmission of this TB of this HARQ process; or

2>	if the uplink grant was received on PDCCH for the C-RNTI and the HARQ buffer of the TB of the identified process is empty; or

2>	if the uplink grant was received in a Random Access Response (i.e. in a MAC RAR or a fallback RAR); or

2>	if the uplink grant was determined as specified in clause 5.1.2a for the transmission of the MSGA payload; or

2>	if the uplink grant was received on PDCCH for the C-RNTI in ra-ResponseWindow and this PDCCH successfully completed the Random Access procedure initiated for beam failure recovery; or

2>	if the uplink grant is part of a bundle of the configured uplink grant, and may be used for initial transmission according to clause 6.1.2.3 of TS 38.214 [7], and if no MAC PDU has been obtained for this bundle:

...

3>	if a MAC PDU to transmit has been obtained:

…

3>	else:

4>	flush the HARQ buffer of the TB of the identified HARQ process.

…

### 5.4.3	Multiplexing and assembly

…

5.4.3.1.3	Allocation of resources

…

The MAC entity shall:

1>	if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR:

3>	not generate a MAC PDU for the HARQ entity.

1>	else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR:

3>	not generate a MAC PDU for the HARQ entity.

NOTE 1a:	If at least one MAC PDU is to be generated or to be retransmitted for the HARQ process, the MAC entity generates MAC PDUs corresponding to the UL grant indicated to the HARQ process.

- Annex 2: Possible TP for Option 2 in TS 38.321

5.4	UL-SCH data transfer

5.4.1	UL Grant reception

Uplink grant is either received dynamically on the PDCCH, in a Random Access Response, configured semi-persistently by RRC or determined to be associated with the PUSCH resource of MSGA as specified in clause 5.1.2a. The MAC entity shall have an uplink grant to transmit on the UL-SCH. To perform the requested transmissions, the MAC layer receives HARQ information from lower layers. When maxRank or maxMIMO-Layers (as specified in TS 38.214 [7]) is more than 4, the MAC layer can receive up to two uplink grants on the same HARQ process from lower layers. An uplink grant addressed to CS-RNTI with NDI = 0 is considered as a configured uplink grant. An uplink grant addressed to CS-RNTI with NDI = 1 is considered as a dynamic uplink grant.

…

1>	if an uplink grant for this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI or Temporary C-RNTI; or

1>	if an uplink grant has been received in a Random Access Response:

2>	if the uplink grant is for MAC entity's C-RNTI and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was either an uplink grant received for the MAC entity's CS-RNTI or a configured uplink grant:

3>	consider the NDI of this TB to have been toggled for the corresponding HARQ process regardless of the value of the NDI.

…

5.4.2.1	HARQ Entity

…

For each uplink grant, the HARQ entity shall:

1>	identify the HARQ process associated with this grant, and for each identified HARQ process and for each TB associated to the identified HARQ process:

2>	if the received grant was not addressed to a Temporary C-RNTI on PDCCH, and the NDI provided in the associated HARQ information has been toggled compared to the value in the previous transmission of this TB of this HARQ process; or

2>	if the uplink grant was received on PDCCH for the C-RNTI and the HARQ buffer of the TB of the identified process is empty; or

2>	if the uplink grant was received in a Random Access Response (i.e. in a MAC RAR or a fallback RAR); or

2>	if the uplink grant was determined as specified in clause 5.1.2a for the transmission of the MSGA payload; or

2>	if the uplink grant was received on PDCCH for the C-RNTI in ra-ResponseWindow and this PDCCH successfully completed the Random Access procedure initiated for beam failure recovery; or

2>	if the uplink grant is part of a bundle of the configured uplink grant, and may be used for initial transmission according to clause 6.1.2.3 of TS 38.214 [7], and if no MAC PDU has been obtained for this bundle:

...

3>	if a MAC PDU to transmit has been obtained:

…

3>	else:

4>	flush the HARQ buffer of the TB of the identified HARQ process.

…

### 5.4.3	Multiplexing and assembly

…

5.4.3.1.3	Allocation of resources

…

The MAC entity shall:

1>	if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR:

3>	not generate a MAC PDU for the HARQ entity.

1>	else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR:

3>	not generate a MAC PDU for the HARQ entity.

NOTE 1a:	If at least one MAC PDU is to be generated or to be retransmitted for the HARQ process, the MAC entity generates MAC PDUs corresponding to all UL grants indicated to the HARQ process.