3GPP TSG RAN WG2 #127bis				R2-2408356

Hefei, China, Oct 14th – 18th, 2024	Revision of R2-2406520

Agenda Item:	8.2.3

Source:		ASUSTeK

Title:				Discussion on Ambient IoT paging message with multiple IDs

Document for: Discussion and Decision

- Introduction

In RAN2#126 meeting [1], the following agreements on Ambient IoT device paging were made:

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

In RAN1#117 meeting [2], the following agreements on Ambient IoT channel design were made:

Agreement

Scheduling information of PDRCH transmission is provided by a corresponding PRDCH.

Agreement

For R2D, the only physical channel is PRDCH.

- PRDCH carries any higher-layer payload

- PRDCH carries L1 R2D control information (if defined)

- FFS details of device behaviour(s) for receiving PRDCH

Agreement

For D2R

- PDRCH carries any higher-layer payload

- PDRCH carries L1 D2R control information (if defined)

- Note: PDRCH carries the response agreed at RAN1#116

- Discussion

In RAN2#126 meeting, it was agreed that A-IoT paging message indicates information from which the device can determine resources to be used for response. The A-IoT paging message may indicate information to a single device, a device group (or multiple devices) or all devices. That is, there may be a single device ID, a single group ID, no ID (for broadcasting to all devices) or multiple IDs (for multiple devices) in a A-IoT paging message.

The model of paging and access procedure is still under discussion. For example, according to the draft TP provided by the rapporteur [3], there may be signals (e.g. R2D trigger) additional to A-IoT paging message. The occasions for the device to perform D2R transmission after receiving the above paging-related signals are not determined, either. Nevertheless, it is acknowledged that A-IoT paging message contains ID(s) for device(s) (e.g. the device ID, the group ID of which the device belongs to, or no ID) to perform action(s) (e.g. command, inventory). In the case of multiple IDs contained in a A-IoT paging message, each indicated device should perform a corresponding D2R transmission.

Observation 1:  If there are multiple IDs included in a A-IoT paging message, each indicated device should perform a corresponding D2R transmission.

On the other hand, it was also agreed in RAN1 that the scheduling information of PDRCH transmission is provided by a corresponding PRDCH. For the case of A-IoT paging and its corresponding response, the scheduling information in the A-IoT paging message should be associated to an ID of the paging request. For example, if the A-IoT paging message indicates an ID of a single device, the A-IoT paging message should include the scheduling information for PDRCH transmission corresponding to the device ID. And if the A-IoT paging message indicates a group ID, the A-IoT paging message should include the scheduling information for PDRCH transmission corresponding to the group ID. Additionally, as demonstrated in other discussions [4][5], if more than one devices are supposed to perform D2R transmission without collision (e.g. CFRA), the D2R resource (e.g. time and/or frequency resource) should be provided to each device separately. As a result, if multiple IDs are included in a A-IoT paging message, there should be a D2R resource, which is indicated by the scheduling information in the paging message, mapped to each device ID.

Proposal 1:  The A-IoT paging message includes scheduling information for corresponding response, where each scheduling information is associated to an ID of the paging request.

Moreover, it is also possible to include multiple “requests” (e.g. command, inventor, inventory + command) in the paging message for a single device, at least in some conditions. For example, the reader would read some (first) information from a device group to which the device belongs, and write some (second) information to the device. It would be efficient to include both requests in the same paging message, if the two requests don’t conflict with or have impact on each other. As for which conditions the multiple “requests” are allowed to be sent to the same device can be further discussed.

Proposal 2:  A-IoT paging message could include multiple command(s), inventor(ies), and inventory(ies) plus command(s) for a single device, at least in some conditions. FFS on which conditions.

- Conclusion

Observation 1:  If there are multiple IDs included in a A-IoT paging message, each indicated device should perform a corresponding D2R transmission.

Proposal 1:  The A-IoT paging message includes scheduling information for corresponding response, where each scheduling information is associated to an ID of the paging request.

Proposal 2:  A-IoT paging message could include multiple command(s), inventor(ies), and inventory(ies) plus command(s) for a single device, at least in some conditions. FFS on which conditions.

- Reference

[1] Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan

[2] RAN1 Chair’s Notes for 3GPP TSG RAN WG1 #117 (Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024)

[3] Report of [POST127][033][AIoT] Random Access

[4] R2-2407283, “Discussion on A-IoT paging”

[5] R2-2406856, “Ambient-IoT Paging”