3GPP TSG-RAN WG2 Meeting #127bis	R2-2408360

Hefei, China, Oct 14th – 18th, 2024	Revision of R2-2406526

Agenda Item:	8.9.2

Source:			ASUSTeK

Title:	Discussion on information for Store & Forward

Document for: 	Discussion and Decision

- Introduction

In previous RAN2 meetings, we have made the following progress on Store & Forward operation. In RAN2 #125bis meeting:

RAN2 assumptions:

- S&F implies that at least the full eNB will be onboard

- An IoT NTN network shall be able to inform UE(s) whether S&F Satellite operation is applied, either via NAS or AS (wait for SA2 progress on this)

3.	The S&F satellite operation is common for NB-IoT and eMTC.

4.	The S&F satellite operation is applied to both CP solution and UP solution (for the UP solution pending on SA2 conclusions on the architecture)

In RAN2 #126 meeting:

Agreements:

1.	For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite):

1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it.

2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN.

3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available).

4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again

2.	S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed

3.	RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

In RAN2 #127 meeting:

Agreements:

1.	RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options)

2.	RAN2 will consider both single satellite pass and multiple satellite pass scenarios

3.	RAN2 will consider both MO and MT data within scope

4.	UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

5.	RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

6.	RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

In this contribution, we would like to further discuss the assistance information provided to the UE for S&F operation.

- Discussion

Store and Forward (S&F) operation is intended to provide some level of communication service for UEs under satellite coverage with intermittent/temporary satellite connectivity (e.g. when the satellite is not connected via a feeder link or via ISL to the ground network) for delay-tolerant communication service. As specified in TR 22.865 [1], S&F could be considered as another operation mode (as shown in Figure 1 below) in addition to the “normal/default satellite operation” mode (as shown in Figure 2 below).

Figure 1: “S&F satellite operation” mode [1]

Figure 2:“Normal/default satellite operation” mode [1]

Since the UE behaviour when the UE is in S&F operation mode should be different from UE behaviour when the UE is in normal/default operation mode, the UE needs to know whether the current serving network is operating in which mode. In the previous meetings, it was agreed that a S&F indication would be provided by SIB that an IoT NTN network shall inform UE(s) whether S&F satellite operation is applied on the serving satellite.

And in RAN2 #127 meeting, it was discussed that the IoT NTN network may switch between these two different operation modes. For example, when feeder link is available for a satellite, the IoT NTN network could operate in normal/default operation mode. And when feeder link becomes unavailable, the IoT NTN network could switch to S&F operation mode.

Since when to switch operation mode may depend on satellite movement or network decision, the switching should be predictable. It is beneficial to indicate the time information about when to switch operation mode between S&F operation mode and normal/default operation mode to the UE. In RAN2 #127 meeting, it has been discussed in [2] whether the UE is informed of the remaining time of the satellite’s current operation mode (normal / S&F) via system information. And it was decided to come back at the next meeting. With the time information, the UE could know when the S&F operation would be started or stopped. For example, based on the time information, the UE could determine whether to perform some non-delay-tolerant transmission, which is not suitable in S&F mode. The UE could also use the time information to determine whether to establish a connection in the current cell.

Proposal 1:  The S&F indication to the UE includes time information about when to switch to/from S&F operation mode.

- Conclusion

We have the following proposal for S&F indication:

Proposal 1:  The S&F indication to the UE includes time information about when to switch to/from S&F operation mode .

- Reference

[1] 3GPP TR 22.865 v19.2.0, “Study on satellite access Phase 3.”

[2] R2-2407237, “Support for store and forward in IoT NTN”, Ericsson.