3GPP TSG-RAN WG2 Meeting #127bis	R2-2408368

Hefei, China, October 14-18, 2024

Source: 			Qualcomm Incorporated, Ericsson

Title: 	Discussion on cell barring and associated behaviour

Document for: 	Decision

Agenda Item: 	7.24.2.2

# Introduction

Through multiple releases, cell barring mechanisms have been specified for different features. While many of them share similar behaviours, the way the corresponding behaviours are specified is not very consistent in the current specifications.

In this document, we review how the cell barring behaviour is specified for different features, and propose　specification clean-up.

# Discussion

  - Ignoring cellBarred in MIB

The field cellBarred in MIB is ignored by NTN UE, ATG UE, IAB-MT and NCR-MT. The behaviour is specified differently as shown in the table below.

| UE, MT type | 38.331 | 38.304 |
| --- | --- | --- |
| NTN UE | Section 5.2.2.4.1 Field description of MIB | Not specified |
| ATG UE |  |  |
| IAB-MT | Field description of MIB | NOTE in section 5.3.1 |
| NCR-MT |  |  |

  - Determination of cell barring

This section summarizes how the determination of cell barring status is defined in the current specifications. It should be noted that the specifications define how the UE determines that the cell is considered as “barred”, which only implicitly define when the cell is considered as “not barred”.

It can be seen that the descriptions in 38.304 are either duplication of 38.331 or incomplete.

NTN UE

| cellBarredNTN (SIB1) | cellBarredFixedVSAT / cellBarredMobileVSAT (SIB1) | Cell barred? | 38.331 | 38.304 |
| --- | --- | --- | --- | --- |
| Not present | Don’t care | Barred | Section 5.2.2.4.2 | Section 5.3.1 |
| Set to “barred” | Don’t care |  |  | Not specified |
| Set to “not barred” | Not present |  |  |  |
|  | Set to “barred” |  |  |  |
|  | Set to “not barred” | Not barred | Not specified | Not specified |

Note that cellBarred in MIB is ignored by the UE of this type.

ATG UE

| cellBarredATG (SIB1) | Cell barred? | 38.331 | 38.304 |
| --- | --- | --- | --- |
| Not present | Barred | Section 5.2.2.4.2 | Section 5.3.1 |
| Set to “barred” |  |  | Not specified |
| Set to “not barred” | Not barred | Not specified | Not specified |

Note that cellBarred in MIB is ignored by the UE of this type.

(e)RedCap UE

| cellBarred (MIB) | intraFreqReselectionRedCap (SIB1) | cellBarred(e)RedCap1Rx / cellBarred(e)RedCap2Rx (SIB1) | Cell barred? | 38.331 | 38.304 |
| --- | --- | --- | --- | --- | --- |
| Barred | Don’t care | Don’t care | Barred | Section 5.2.2.4.1 Section 5.2.2.4.2 | Not specified |
| Not barred | Not present | Don’t care |  |  | Section 5.3.1 |
|  | Present | Set to “barred” |  |  | Not specified |
|  |  | Not present | Not barred | Not specified |  |
|  |  | Set to “not barred” |  |  |  |

(e)RedCap half duplex UE

| cellBarred (MIB) | intraFreqReselectionRedCap (SIB1) | halfDuplexRedCapAllowed (SIB1) | Cell barred? | 38.331 | 38.304 |
| --- | --- | --- | --- | --- | --- |
| Barred | Don’t care | Don’t care | Barred | Section 5.2.2.4.1 Section 5.2.2.4.2 | Not specified |
| Not barred | Not present | Don’t care |  |  | Section 5.3.1 |
|  | Present | Set to “barred” |  |  | Not specified |
|  |  | Not present |  |  | Section 5.3.1 |
|  |  | Set to “not barred” | Not barred | Not specified | Not specified |

2RX XR UE

| cellBarred (MIB) | cellBarred2RxXR (SIB1) | Cell barred? | 38.331 | 38.304 |
| --- | --- | --- | --- | --- |
| Barred | Don’t care | Barred | Section 5.2.2.4.1 Section 5.2.2.4.2 | Not specified |
| Not barred | Set to “barred” |  |  | Section 5.3.1 |
|  | Not present | Not barred | Not specified | Not specified |

Not that the field cellBarred2RxXR has the single value “barred”.

NES UE

| cellBarred (MIB) | cellBarredNES (SIB1) | Cell barred? | 38.331 | 38.304 |
| --- | --- | --- | --- | --- |
| Barred | Not present | Barred | Section 5.2.2.4.1 Section 5.2.2.4.2 | Section 5.3.1 |
|  | Set to “not barred” | Not barred | Not specified | Not specified |
| Not barred | Don’t care |  |  |  |

Not that the field cellBarredNES has the single value “not barred”.

IAB-MT

| iab-Support / mobileIAB-Support (SIB1) | Cell barred? | 38.331 | 38.304 |
| --- | --- | --- | --- |
| Not present | Barred | Section 5.2.2.4.2 | Not specified |
| Present | Not barred | Not specified |  |

Note that cellBarred in MIB is ignored by the MT of this type.

NCR-MT

| ncr-Support (SIB1) | Cell barred? | 38.331 | 38.304 |
| --- | --- | --- | --- |
| Not present | Barred | Section 5.2.2.4.2 | Not specified |
| Present | Not barred | Not specified |  |

Note that cellBarred in MIB is ignored by the MT of this type.

  - Cell reselection on the same frequency when the cell is barred

When the cell is considered as “barred”, the UE determines if reselection to another cell on the same frequency is allowed or not. The table below summarizes how the UE behaviour is defined.

It can be seen that some descriptions in 38.331 and 38.304 are duplicated or inconsistent (while the final behaviour is the same). Also the descriptions in 38.331 are either duplication of 38.304 or even incomplete.

| UE type | 38.331 | 38.304 |
| --- | --- | --- |
| NTN | Section 5.2.2.4.2 - “According to 38.304”. Section 5.2.2.2 - May select another cell on the same frequency if unable to acquire MIB/SIB1. | Section 5.3.1 - Follow intraFreqReselection in MIB if acquired. - May select another cell on the same frequency if unable to acquire MIB/SIB1. |
| ATG | Section 5.2.2.4.2 - “According to 38.304”. Section 5.2.2.2 - May select another cell on the same frequency if unable to acquire MIB/SIB1. | Section 5.3.1 - Follow intraFreqReselection in MIB if acquired. - May select another cell on the same frequency if unable to acquire MIB/SIB1. |
| (e)RedCap | Section 5.2.2.4.2 - Follow intraFreqReselection(-e)RedCap if present. - Consider as if intraFreqReselection(-e)RedCap = allowed if not present. Section 5.2.2.2 - Consider as if intraFreqReselection(-e)RedCap = allowed if unable to acquire MIB/SIB1. | Section 5.3.1 - Follow intraFreqReselection(-e)RedCap if present. - Consider as if intraFreqReselection(-e)RedCap = allowed if the cell does not support (e)RedCap. - May select another cell on the same frequency if unable to acquire MIB/SIB1. |
| 2Rx XR | Section 5.2.2.4.2 - Follow intraFreqReselection2RxXR if present. - Consider as if intraFreqReselection2RxXR = allowed if not present. Section 5.2.2.2 - Consider as if intraFreqReselection2RxXR = allowed if unable to acquire MIB/SIB1. | Section 5.3.1 - Follow intraFreqReselection2RxXR if present. - Not specified. - May select another cell on the same frequency if unable to acquire MIB/SIB1. |
| NES | Section 5.2.2.4.2 - “According to 38.304”. Section 5.2.2.2 - May select another cell on the same frequency if unable to acquire MIB/SIB1. | Section 5.3.1 - Follow intraFreqReselection in MIB if acquired. - May select another cell on the same frequency if unable to acquire MIB/SIB1. |
| IAB-MT | Section 5.2.2.4.2 - Not specified. Section 5.2.2.2 - May select another cell on the same frequency if unable to acquire MIB/SIB1. | Section 5.3.1 - NOTE: Ignore intraFreqReselection (i.e. treat intraFreqReselection as if it was set to allowed) - May select another cell on the same frequency if unable to acquire MIB/SIB1. |
| NCR-MT | Section 5.2.2.4.2 - Not specified. Section 5.2.2.2 - May select another cell on the same frequency if unable to acquire MIB/SIB1. | Section 5.3.1 - NOTE: Ignore intraFreqReselection (i.e. treat intraFreqReselection as if it was set to allowed) - May select another cell on the same frequency if unable to acquire MIB/SIB1. |

  - Cell reservation

Cell reservations can result in cell barring. The basic behaviour is defined in 38.304. However some feature specific behaviours are defined as follows.

There are a few things to highlight:

- For IAB/NCR-MT, descriptions are duplicated between 38.331 and 38.304.

- The descriptions on the determined of NPN-only cell are inconsistent within 38.331 between NPN-capable normal UE and IAB/NCR-MT.

| UE type | 38.331 | 38.304 |
| --- | --- | --- |
| IAB-MT | Section 6.3.2, field description - Ignore cellReservedForOperatorUse, cellReservedForFutureUse, cellReservedForOtherUse for cell barring determination. - NPN capable MT considers cellReservedForOtherUse for determination of an NPN-only cell. | Section 5.3.1 - NOTE: Ignore cellReservedForOperatorUse, cellReservedForFutureUse, cellReservedForOtherUse for cell barring determination. - NPN capable MT considers cellReservedForOtherUse for determination of an NPN-only cell. |
| NCR-MT | Section 6.3.2, field description - Ignore cellReservedForOperatorUse, cellReservedForFutureUse, cellReservedForOtherUse for cell barring determination. - NPN capable MT considers cellReservedForOtherUse for determination of an NPN-only cell. | Section 5.3.1 - NOTE: Ignore cellReservedForOperatorUse, cellReservedForFutureUse, cellReservedForOtherUse for cell barring determination. - NPN capable MT considers cellReservedForOtherUse for determination of an NPN-only cell. |
| NPN capable UE | Section 3.1 (Definitions) - NPN-only Cell: […] An NPN-capable UE determines that a cell is NPN-only Cell by detecting that the cellReservedForOtherUse IE is set to true while the npn-IdentityInfoList IE is present in CellAccessRelatedInfo. (*) Describing how the UE determines NPN-only cell. | Section 5.3.1 - When cell broadcasts any CAG-IDs or NIDs and the cell status is indicated as "not barred" and "not reserved" for operator use and "true" for other use, and not "true" for future use: All NPN-capable UEs shall treat this cell as candidate during the cell selection and cell reselection procedures. (*) Describing if the cell should be considered as cell (re)selection candidate. |

  - Proposed principles for clean-up

As shown in the previous sections, the current specifications, 38.331 and 38.304, are not very consistent in describing the behaviours related to cell barring. Such inconsistency is prone to misinterpretation and errors in future specification maintenance. It was also found that the description of some behaviours are duplicated in 38.331 and 38.304, even though consistent.

Proposal:	Agree on the following principles with regards to the “work split” between 38.331 and 38.304.

| Behaviour | Specified in 38.331 | Specified in 38.304 |
| --- | --- | --- |
| Determination of cell baring status based on cell barring indication in MIB and SIB1. | Yes | No |
| Cell reservation (for cell barring determination) | No | Yes |
| Cell reselection to another cell on the same frequency as barred cell is allowed or not. | No (only pointer to 38.304) | Yes |
| Determination of NPN-only cell. | Yes | No |

# Proposal

In this document we reviewed how the cell barring behaviours are specified in TS38.331 and TS38.304, and identified inconsistencies and duplications. In order to avoid endless cycle of alignment work among different WIs/features, we proposed specification clean-up based on the following principles.

Proposal:	Agree on the following principles with regards to the “work split” between 38.331 and 38.304.

| Behaviour | Specified in 38.331 | Specified in 38.304 |
| --- | --- | --- |
| Determination of cell baring status based on cell barring indication in MIB and SIB1. | Yes | No |
| Cell reservation (for cell barring determination) | No | Yes |
| Cell reselection to another cell on the same frequency as barred cell is allowed or not. | No (only pointer to 38.304) | Yes |
| Determination of NPN-only cell. | Yes | No |

CRs to 38.331 and 38.304 are provided in [1] and [2].

# Reference

[1]	R2-2408369	Rel-18 CR to 38.331 on Clarifications on cell barring	Qualcomm Incorporated

[2]	R2-2408370	Rel-18 CR to 38.304 on Clarifications on cell barring	Qualcomm Incorporated