3GPP TSG-RAN WG2 Meeting #127bis	R2-2408371

Hefei, China, October 14-18, 2024

Source: 			Qualcomm Incorporated, AT&T

Title: 	Network signalling of maximum number of UL segments

Document for: 	Decision

Agenda Item: 	6.1.3.2

# Introduction

In this document, we discuss the issue we identified in the field regarding UL RRC message segmentation of UECapabilityInformation. We also propose a RRC signalling solution to address the issue.

# Discussion

  - Existing RRC message segmentation for UECapabilityInformation

The following steps are taken for RRC message segmentation for UECapabilityInformation.

- The UE indicates its support for UL RRC segmentation (ul-RRC-Segmentation) in RRCSetupComplete message.

- The network enables UL RRC segmentation by the field rrc-SegAllowed in UECapabilityEnquiry message.

- The UE initiates UL message segment transfer procedure to transmit ULDedicatedMessageSegment messages in which UECapabilityInformation message is embedded.

ULDedicatedMessageSegment message supports transmission of up to 16 RRC segments.

  - The field issue

We observed in the field that the network does not always support the maximum number of RRC segments. There is no good way for the UE to determine how many RRC segments a given cell (and associated network nodes or configuration) may support.

We believe that this is a practical limitation of network implementations and cannot be resolved simply by forcing the support of maximum number of RRC segments by the standard.

  - Proposed solution

Based on the observations above, we propose the following new signalling method for RRC message segmentation of UECapabilityInformation. The text in red highlights the additions to the existing procedure.

- The UE indicates its support for UL RRC segmentation (ul-RRC-Segmentation) and its support for network-requested maximum number of UL segments (ul-RRC-MaxCapaSegments) in RRCSetupComplete message.

- The network enables UL RRC segmentation by the field rrc-SegAllowed and indicates the maximum number of UL segments the UE is allowed to use (rrc-MaxCapaSegAllowed) in UECapabilityEnquiry message.

- The UE generates UECapabilityInformation ensuring the total size of the message does not exceed the maximum allowed size according to rrc-MaxCapaSegAllowed signalled by the network. The UE initiates UL message segment transfer procedure to transmit ULDedicatedMessageSegment messages in which UECapabilityInformation message is embedded.

Proposal 1:	Agree on the signalling solution for or RRC message segmentation of UECapabilityInformation where:

- The UE indicates its support for network-requested maximum number of UL segments.

- The network indicates the maximum number of UL segments the UE is allowed to use.

- The UE generates UECapabilityInformation ensuring the total size of the message does not exceed the maximum allowed size according to the maximum number of UL segments the UE is allowed to use.

We see the need of addressing the field issue as soon as possible. We therefore propose introducing the new signalling solution from release-17.

Proposal 2:	The proposed new signalling solution is introduced from release-17.

  - RRC processing delay

The existing RRC processing delay requirement for UE capability reporting with UL segmentation is 80ms. It should be noted however that the proposed solution increases the UE processing after the reception of UECapabilityEnquiry, on top of what is needed for the existing UE capability filters. It is not very practical to define the RRC processing delay requirement for such dynamic processing at the UE which depends on the actual capability set of the UE and the maximum number of UL segments allowed by the network.

Given that the UE capability enquiry / transfer procedure is rather infrequent event, we propose not to specify the RRC processing delay requirement for this particular case.

Proposal 3:	Not specify the RRC processing delay requirement for UE capability enquiry / transfer procedure when the network indicates the maximum allowed number of UL segments.

# Proposal

Based on the discussion in this document, we made the following proposals.

Proposal 1:	Agree on the signalling solution for or RRC message segmentation of UECapabilityInformation where:

- The UE indicates its support for network-requested maximum number of UL segments.

- The network indicates the maximum number of UL segments the UE is allowed to use.

- The UE generates UECapabilityInformation ensuring the total size of the message does not exceed the maximum allowed size according to the maximum number of UL segments the UE is allowed to use.

Proposal 2:	The proposed new signalling solution is introduced from release-17.

Proposal 3:	Not specify the RRC processing delay requirement for UE capability enquiry / transfer procedure when the network indicates the maximum allowed number of UL segments.

CRs to 38.331 and 38.306 are provided in [1] and [2] respectively.

# Reference

[1]	R2-2408372	Rel-17 CR to 38.331 on Network signalling of maximum number of UL segments	Qualcomm Incorporated

[2]	R2-2408373	Rel-17 CR to 38.306 on Network signalling of maximum number of UL segments	Qualcomm Incorporated