# 3GPP TSG RAN2 Meeging #127bis  	                                       R2-2408378

# Hefei, China, 14th – 18th Oct , 2024

Agenda item:	8.1.5

Title:	Considerations on Model Transfer/Delivery in RAN2

Source:	ZTE Corporation

Document for:	Discussion and Decision

Introduction

In Rel-18, RAN2 have discussed the following solutions for model transfer/delivery:

7.2.1.4 	Model transfer/delivery

Whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) is unclear from the outcome of the present study. Nonetheless, to support AI/ML model transfer/delivery, the following solutions are considered:

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling.

- Solution 2a: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling.

- Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling.

- Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data.

- Solution 2b: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via User Plane (UP) data.

- Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data.

- Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP).

- Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE.

And analyze the PROs and CONs, specification gap for each solution.

In Rel-19, RAN1 have analyzed the model transfer/delivery that the case z4 for two-side model is focused. In this contribution, we try to share our views on the model transfer/delivery from RAN2 perspective based on the outcome of RAN1 discussion.

Discussion

General Aspect

In RAN1 discussion, the case Z4 among the cases are discussed, and the following agreement are obtained:

======================= RAN1 agreements for model transfer ===================

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top. | Outside 3gpp Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format. | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format. | 3GPP Network | NW-side |
| z3 | model transfer in open format. | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support. | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE, i.e., any other model structure not covered in z4, including any model structure that is only partially known. | 3GPP Network | NW-side |

- Agreement

From RAN1 perspective, for model delivery/transfer Case z4, further study the following alternatives (including the necessity/feasibility/benefits):

- Alt. A

  - Step A-1: UE reports the supported known model structure(s) to network

  - Step A-2: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step A-1

  - FFS: whether some additional step(s), and/or whether other information is needed

- Alt. B

  - Step B-0: UE reports to NW its support of model transfer/delivery case z4

    - Note: Step B-0 may be before or after Step B-1, or not necessary

  - Step B-1: NW indicates to UE the candidate known model structure(s)

  - Step B-2: UE reports to NW which model structure(s) out of the candidate known model structure(s) indicated in Step B-1 is supported

  - Step B-3: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step B-2

  - FFS: whether some additional step(s), and/or whether other information is needed

- Note: Other alternative(s) is not precluded

- Note: Other method(s) of parameter exchange from NW to UE side is a separate discussion.

======================= RAN1 agreements for model transfer ===================

Besides, RAN 1 also concluded, the model transfer/delivery is at least for the CSI compression (e.g. two side model), for other use cases with one side model, there is no consensus that the model transfer/delivery shall be supported.

Conclusion

From RAN1 perspective, model transfer is needed at least for some (e.g., Option 3b) of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported)

In our understanding, RAN2 can follow the RAN1 discussion scope so that only one use case (e.g. CSI compression with two side model) is focused for RAN2 discussion, RAN2 can extend the discussion if there is any extra requirement in RAN1 have been identified.

Proposal 1: Based on the current progress made in RAN1, The RAN2 discussion on model transfer/delivery shall be focused on two-side model (e.g. CSI compression) with a known model structure unless there is any extra requirement from RAN1 is identified.

Case z4’s description is shown as below:

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| z4 | model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support. | 3GPP Network | NW-side |

The case z4 is for the model transfer from NW to the UE which is perfectly aligned with the functionality mapping that  model training mapping for the use case of CSI compression which is shown as below:

- Model Training: - For the two-sided CSI compression use case, training data can be generated by either the UE or the gNB, depending on specific requirements, while the termination point for training data may include the gNB, OAM, Over-The-Top (OTT) server or UE. Note: RAN2 identified the case in which Core Network may be used for model training. However, no study was conducted since this is beyond the scope of this Working Group.

Considering the Core Network to train the model for CSI feedback is out of the RAN2 scope, the model transfer which involves the CN can be deprioritized in RAN2 for the model transfer discussion of the CSI compression.

In this sense, among the candidate solutions provided in the introduction part, the solution 1a, solution 1b, solution 4b can be applied for the model transfer/delivery for use case of the CSI compression:

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling.

- Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data.

- Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE.

Proposal 2: For AI/ML CSI compression with two side model, solution 1a, solution 1b and solution 4b can be the candidate solutions for the model transfer/delivery with a known structure.

Solution 1a

The RRC signaling is a common solution for gNB to transport information to UE, such as, the NAS signaling, the RRC information,etc.

For DL transferring a model to the UE, some existing RRC message or procedures can be reused, for example, RRCReconfiguration, DLInformationTransfer, etc. However, in our understanding, the AI based feature is an enhancement to the legacy feature, there is no urgent delay requirement for applying AI based feature so that the model transfer do not have urgent delay requirement as well;

In addition ,the model size for CSI compression may reach to a few MBytes based on the RAN4’s conclusion, the RRCReconfiguration and DL information for model transfer may occupy the SRB1 or SRB 2 for a long time, may cause the RRC signaling /NAS delay, in this sense, we would like to introduce a new RRC procedure for model transfer dedicatedly, which occupy a new SRB with a lower priority, at least lower than the priority of SRB1, SRB2, SRB3..

Proposal 2: For model transfer with solution 1a, a new RRC signaling based procedure is introduced for the model transfer from gNB to UE. A new SRB with a priority lower than 3 is introduced for such RRC signaling.

As mentioned above, RAN4 have estimated that the model size for CSI Compression with a known structure may reach up to tens Mega bytes, but the current RRC signaling only support at most 45KB due to one single RRC message can be segment with 5 pieces. The extension of RRC message segments seems necessary, the actual extension numbers of segments shall ask RAN4.

Proposal 3: For model transfer with solution 1a, the DL RRC message segments shall be extended, the actual extension number of the segments is determined by RAN1 and RAN4.

Regarding the contents for the model transfer with a known model structure, there is no any clues about the format of the model,：

- one possible solution is to specify the parameter of a known structure

- another possible solution is to let it be a black box (e.g. container), in this solution, how to define format in the container in order to guarantee the format can be accurately decoded by UE is also an issue need to be resolved.

- No matter which solution to go for, RAN1 need to get involved and make a final decision.

Proposal 4: For model transfer with solution 1a, RAN2 assumes the format of the model in RRC signaling message can be one of 1) specified parameters; 2)container.

Solution 1b

In solution 1b, there is no direct UP tunnel that is terminated between the gNB and UE according to the current spec. For supporting the UP tunnel between gNB and UE, there are two options:

- CN based solution: gNB to request CN to establish a PDU session that is terminated between UE and gNB.

- RAN based solution: Introduce a new UP data tunnel between gNB and UE for model transfer

In the first option, SA2 shall get involved into the discussion since RAN2 cannot evaluate and study the feasibility. In the second option, the protocol stack for the user plane will be changed accordingly,to our understanding, which is too late to touch the UP stack at such late stage of the NR. In this sense, we propose that:

Proposal 5: For model transfer with solution 1b, RAN2 assumes that the gNB can request CN to establish a PDU session that is terminated between UE and gNB, the feasibility shall be confirmed by SA2.

Solution 4b

In solution 4b, the TR have been indicated there are two sub-solutions for solution 4b, one is CP based solution, the other one is IP address based solution.

Table 7.3.1.4-6 Analysis of current status and gaps, and potential 
RAN specification impact for Solutions 4b

| Discussion Area | Current status and Gaps | Potential RAN specification impact (Note: whether and how to support model transfer/delivery from OAM to gNB and OAM to UE directly is out of RAN scope) |
| --- | --- | --- |
| A1. Large, no upper limit model/model parameter size | - Over Control Plane (CP) signalling: model size >45kBytes is not supported based on existing number of RRC segments if OAM does not do segmentation for model transfer/delivery - Over, e.g., IP: no model size limitation, but direct connection between OAM and UE is not supported | - Over Control Plane (CP) signalling: If OAM does not do segmentation for model transfer/delivery, it may need RRC segmentation, and extend RRC segment number if model size larger than 45kBytes - Over, e.g., IP: NOTE: whether and how to support direct connection between OAM and UE is out of RAN scope |

CP based solution

In the CP based solution 4b, For air-interface, as same as the proposal 1, a new RRC signaling based procedure can be applied for transferring the model from gNB to the UE, and the priority level of such RRC signaling is lower SRB 1 and SRB2, SRB3, and how to transfer the model from OAM to the gNB is out of RAN2 scope which shall be discussed in SA5.

Proposal 6a: For model transfer with CP based solution 4b, the solution 1a is applied for the model transfer from gNB to the UE.

Proposal 6b: For model transfer with CP based solution 4b, how to transfer the model from OAM to the gNB is out of RAN2 scope which shall be discussed in SA5.

IP address based solution

For the UP based solution 4b, the OAM can transfer the model to the UE via IP network in advance to which the IP address shall be provided to the UE. In order to provide the IP address of the OAM to the UE, the IP address need to be forwarded from OAM to gNB firstly, then gNB configured/send the IP address of the OAM to the UE.

Proposal 7a: For the IP address based solution 4b, the IP address is forwarded from gNB to the UE via L3 signaling, FFS the detail L3 signaling.

Proposal 7b: For the IP address based solution 4b, how to transfer the IP address from OAM to the gNB is out of RAN2 scope which shall be discussed in SA5.

Conclusion

In this contribution, we provide our views on model transfer/delivery from RAN2 perspective:

General Aspect

Proposal 1: Based on the current progress made in RAN1, The RAN2 discussion on model transfer/delivery shall be focused on two-side model (e.g. CSI compression) with a known model structure unless there is any extra requirement from RAN1 is identified.

Proposal 2: For model transfer with solution 1a, a new RRC signaling based procedure is introduced for the model transfer from gNB to UE. A new SRB with a priority lower than 3 is introduced for such RRC signaling.

Solution 1a for CSI Compression

Proposal 2: For model transfer with solution 1a, a new RRC signaling based procedure is introduced for the model transfer from gNB to UE. A new SRB with a LCH priority value higher than 3 is introduced for such RRC signaling.

Proposal 3: For model transfer with solution 1a, the DL RRC message segments shall be extended, the actual extension number of the segments is determined by RAN4.

Proposal 3: For model transfer with solution 1a, RAN2 assumes the format of the model in RRC signaling message can be one of 1) specified parameters; 2)container.

Solution 1b for CSI Compression

Proposal 5: For model transfer with solution 1b, RAN2 assumes that the gNB can request CN to establish a PDU session that is terminated between UE and gNB, the feasibility shall be confirmed by SA2.

Solution 4b for CSI Compression

Proposal 6a: For model transfer with CP based solution 4b, the solution 1a is applied for the model transfer from gNB to the UE.

Proposal 6b: For model transfer with CP based solution 4b, how to transfer the model from OAM to the gNB is out of RAN2 scope which shall be discussed in SA5.

Proposal 7a: For the IP address based solution 4b, the IP address is forwarded from gNB to the UE via L3 signaling, FFS the detail L3 signaling.

Proposal 7b: For the IP address based solution 4b, how to transfer the IP address from OAM to the gNB is out of RAN2 scope which shall be discussed in SA5.

References

- TR 38.843