3GPP TSG-RAN WG2 Meeting #127bis	R2-2408379

Hefei, China, 14 – 18 October 2024

Agenda item:	8.10.2

Source:	Nokia

Title:	MRO for CHO with candidate SCG

WID/SID:	NR_ENDC_SON_MDT_Ph4-Core

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were reached on the topic of MRO for CHO with candidate SCG with regard to information to be logged in various SON reports:

RAN2#126 - RAN2 to enhance the RLF report with additional information regarding the state of the two execution conditions. We see later if we also can enhance the SCGFailureInformation report.

RAN2#127 - UE includes following information in RLF report:

b.        Time information regarding condition fulfilment for CHO with candidate SCGs. Details are FFS. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled.

c.        Measurement results of PCells and PSCells.

RAN3 also agreed on information to be logged in the RLF report and a list of scenarios which should be in the scope of MRO for CHO w/ candidate SCG:

RAN3#125 - The RLF report includes: The type of the first fulfilled execution condition e.g. CPAC or CHO, fulfilment means all events of the type are met.

RAN3#125-  MRO for Case 1a, 1b, 2a, 2b, 3a, 3b, 7a, 7b, 8a, 8b, 9 will be addressed.( as per R3-244223). MRO for case 4/5/6 is in the scope.

This paper discusses further details on information logging in SON reports for MRO for CHO w/ candidate SCG.

# 2	Enhancements of SON reports for MRO for CHO with candidate SCGs

Based on the agreements from the last two RAN2 meetings listed above, there are three possible scenarios that may lead to the generation of an RLF report when the UE is configured with CHO w/ candidate SCG.

Scenario A: RLF on MCG leg is detected before execution of CHO w/ candidate SCG.

Scenario B: HOF on MCG leg during handover execution of CHO w/ candidate SCG.

Scenario C: RLF on the MCG leg is detected shortly after successful execution of CHO w/ candidate SCG.

For each scenario we will further discuss which information should be logged in the RLF report. We will also discuss what happens in case SCG failure is detected instead of MCG failure and possible enhancements to SCGFailureInformation message.

## 2.1 	Scenario A: RLF on MCG leg is detected before execution of CHO w/ candidate SCG

In this scenario, one or both execution conditions associated with the CHO w/ candidate SCG configuration are not fulfilled. While waiting for the execution, RLF on the MCG leg is detected. This scenario corresponds to scenarios 1a, 2a and 3a as agreed by RAN3. In this case, the UE may log the following information in the RLF report:

- An indication of the first fulfilled event/execution condition (CHO or CPAC) (if any) – this is fully in accordance with current RAN2 and RAN3 agreements.

- Additional information on the state of the execution event that was not triggered, e.g. was TTT running for this event or not – this will help the network identify how close to fulfillment the event was and gives a good indication on possible corrective actions.

Observation 1: In case RLF on MCG is detected before execution of CHO w/ candidate SCG, the RLF report should be enhanced with the following information:

- An indication of the first fulfilled event/execution condition (CHO or CPAC) (if any)

- Additional information on the state of the execution event that was not triggered, e.g. was TTT running for this event or not

SCG failure may also be detected (instead of MCG failure) in the same scenario. This corresponds to scenarios 1b, 2b or 3b as agreed by RAN3. In this case, only SCGFailureInformation will be generated but no RLF report. The current content of SCGFailureInformation only includes PSCell measurements but no PCell measurements. Also, MN may not have up to date PCell measurements reported by the UE. This makes it hard for the network to know or deduce which execution condition (CHO or CPAC or both) is to blame for stalling the execution of CHO w/ candidate SCG.

Observation 2: in case SCG failure is detected before the execution of CHO w/ candidate SCGs, the only report generated will be the SCGFailureInformation which only contains PSCell measurements.

In order to aid the network in determining which execution condition/event needs reconfiguration to avoid failures in the future, the SCGFailureInformation message could also be enhanced with the same information as the RLF report in this case.

Observation 3: In case RLF on SCG is detected before execution of CHO w/ candidate SCG, SCGFailureInformation message should also be enhanced with the following information:

- An indication of the first fulfilled event/execution condition (CHO or CPAC) (if any)

- Additional information on the state of the execution event that was not triggered, e.g. was TTT running for this event or not

## 2.2 	Scenario B: HOF on MCG leg during handover execution of CHO w/ candidate SCG

In this scenario, both the execution conditions were fulfilled and handover to Target PCell and Target PSCell was initiated. However, the UE experiences a HOF on the MCG leg so the handover cannot be successfully completed, and the UE will generate an RLF report. This scenario corresponds to scenario 7a as agreed by RAN3.

If the CHO execution condition was fulfilled first, while waiting for CPAC execution condition to also be fulfilled (especially if for a long time), this may lead to artificially selecting the wrong PCell. In this case the CPAC execution condition needs to be corrected. If the situation is reversed and the CPAC execution condition is fulfilled first and UE has to wait for a long time until CHO execution condition is also fulfilled before execution the handover, it is clear that this is a CHO execution condition parametrization issue.

Observation 4: Additional information is needed in the RLF report to aid the network in deciding which component of the CHO w/ candidate SCG to update in case of HOF on MCG leg during execution.

The RLF report could thus be enhanced with the following information:

- An indication of the first fulfilled event/execution condition (CHO or CPAC)

- Time duration between the two execution conditions being fulfilled

Observation 5: In case HOF on MCG leg is detected during handover execution of CHO w/ candidate SCG, the RLF report should be enhanced with the following information:

- An indication of the first fulfilled event/execution condition (CHO or CPAC)

- Time duration between the two execution conditions being fulfilled

HOF on the SCG leg may also be detected (instead of HOF on the MCG leg) in the same scenario. This corresponds to scenario 7b as agreed by RAN3. In this case, the UE will continue the handover as a single connectivity handover, i.e. the MN connection will be kept. The UE will only generate SCGFailureInformation in this case, not RLF report. Similar to the situation where HOF on MCG leg is detected during handover, the SCGFailureInformation message could be enhanced with the same information as the RLF report in order to aid the network in deciding which execution condition delayed the execution of the handover and led to the failure.

Observation 6: In case HOF on SCG leg is detected during handover execution of CHO w/ candidate SCG, the SCGFailureInformation message should be enhanced with the following information:

- An indication of the first fulfilled event/execution condition (CHO or CPAC)

- Time duration between the two execution conditions being fulfilled

## 2.3 	Scenario C: RLF on the MCG leg is detected shortly after successful execution of CHO w/ candidate SCG

In this scenario, both the execution conditions were fulfilled and handover to Target PCell and Target PSCell was successfully completed. However, the UE experiences an RLF on the MCG leg shortly after and generates RLF report. This scenario corresponds to scenario 8a as agreed by RAN3. If SCG failure is detected shortly after handover, this corresponds to scenario 8b as agreed by RAN3.

Similar to the logic in scenario B discussed above, the network may benefit from knowing which execution condition was first fulfilled and the time between execution conditions being fulfilled in order to determine which part of the configuration is responsible for the failure and needs to be corrected. This holds true for both MCG and SCG failure being detected shortly after HO.

Observation 7: In case RLF on MCG leg is detected shortly after the successful execution of CHO w/ candidate SCG, the RLF report should be enhanced with the following information:

- An indication of the first fulfilled event/execution condition (CHO or CPAC)

- Time duration between the two execution conditions being fulfilled

Observation 8: In case RLF on SCG leg is detected shortly after the successful execution of CHO w/ candidate SCG, the SCGFailureInformation message should be enhanced with the following information:

- An indication of the first fulfilled event/execution condition (CHO or CPAC)

- Time duration between the two execution conditions being fulfilled

In light of the above analyses and observations, we propose that the RLF report and SCGFailureInformation message be enhanced with new information to enable root cause analysis and corrective actions.

Proposal 1: In case RLF is detected on the MCG before, during or shortly after successful completion of CHO w/ candidate SCG, the RLF report should be enhanced with the following information:

- An indication of the first fulfilled event/execution condition (CHO or CPAC)

- Additional information on the state of the execution event that was not triggered, e.g. was TTT running for this event or not

- Time duration between the two execution conditions being fulfilled when have been both fulfilled

Proposal 2: In case RLF is detected on the SCG before, during or shortly after successful completion of CHO w/ candidate SCG, the SCGFailureInformation message should be enhanced with the following information:

- An indication of the first fulfilled event/execution condition (CHO or CPAC)

- Additional information on the state of the execution event that was not triggered, e.g. was TTT running for this event or not

- Time duration between the two execution conditions being fulfilled when both have been fulfilled

# 3	Conclusion

This document has made the following observations:

Observation 1: In case RLF on MCG is detected before execution of CHO w/ candidate SCG, the RLF report should be enhanced with the following information:

- An indication of the first fulfilled event/execution condition (CHO or CPAC) (if any)

- Additional information on the state of the execution event that was not triggered, e.g. was TTT running for this event or not

Observation 2: In case SCG failure is detected before the execution of CHO w/ candidate SCGs, the only report generated will be the SCGFailureInformation which only contains PSCell measurements.

Observation 3: In case RLF on SCG is detected before execution of CHO w/ candidate SCG, SCGFailureInformation message should also be enhanced with the following information:

- An indication of the first fulfilled event/execution condition (CHO or CPAC) (if any)

- Additional information on the state of the execution event that was not triggered, e.g. was TTT running for this event or not

Observation 4: Additional information is needed in the RLF report to aid the network in deciding which component of the CHO w/ candidate SCG to update in case of HOF on MCG leg during execution.

Observation 5: In case HOF on MCG leg is detected during handover execution of CHO w/ candidate SCG, the RLF report should be enhanced with the following information:

- An indication of the first fulfilled event/execution condition (CHO or CPAC)

- Time duration between the two execution conditions being fulfilled

Observation 6: In case HOF on SCG leg is detected during handover execution of CHO w/ candidate SCG, the SCGFailureInformation message should be enhanced with the following information:

- An indication of the first fulfilled event/execution condition (CHO or CPAC)

- Time duration between the two execution conditions being fulfilled

Observation 7: In case RLF on MCG leg is detected shortly after the successful execution of CHO w/ candidate SCG, the RLF report should be enhanced with the following information:

- An indication of the first fulfilled event/execution condition (CHO or CPAC)

- Time duration between the two execution conditions being fulfilled

Observation 8: In case RLF on SCG leg is detected shortly after the successful execution of CHO w/ candidate SCG, the SCGFailureInformation message should be enhanced with the following information:

- An indication of the first fulfilled event/execution condition (CHO or CPAC)

- Time duration between the two execution conditions being fulfilled

This document has made the following proposals:

Proposal 1: In case RLF is detected on the MCG before, during or shortly after successful completion of CHO w/ candidate SCG, the RLF report should be enhanced with the following information:

- An indication of the first fulfilled event/execution condition (CHO or CPAC)

- Additional information on the state of the execution event that was not triggered, e.g. was TTT running for this event or not

- Time duration between the two execution conditions being fulfilled when both have been fulfilled

Proposal 2: In case RLF is detected on the SCG before, during or shortly after successful completion of CHO w/ candidate SCG, the SCGFailureInformation message should be enhanced with the following information:

- An indication of the first fulfilled event/execution condition (CHO or CPAC)

- Additional information on the state of the execution event that was not triggered, e.g. was TTT running for this event or not

- Time duration between the two execution conditions being fulfilled when both have been fulfilled