3GPP TSG-RAN WG2 Meeting #127bis	R2-2408382

Hefei, China, 14th – 18th Oct, 2024

Agenda Item:	8.10.2

Source: 	China Unicom

Title:  	Discussion on MRO enhancement for CHO with candidate SCGs

Document for:	Agreement

Introduction

During the previous meeting, RAN2#126 and RAN3#125 reached agreements on MRO enhancement for CHO with candidate SCGs as referenced [1] [2].

UE includes following information in RLF report: - b. Time information regarding condition fulfilment for CHO with candidate SCGs. Details are FFS. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled. - c. Measurement results of PCells and PSCells.

MRO for Case 1a, 1b, 2a, 2b, 3a, 3b, 7a, 7b, 8a, 8b, 9 will be addressed. MRO for case 4/5/6 is in the scope. The RLF report includes: - The type of the first fulfilled execution condition e.g. CPAC or CHO, fulfilment means all events of the type are met.

It’s noted that cases addressed and included in the scope by RAN3 are outlined in [3] clause 2, with specifics provided in the Annex of this document. Portions “highlighted green” indicate cases to be addressed in RAN3, while “highlighted yellow” indicates cases within RAN3’s scope.

This document focuses on “highlighted green” portions and proposes further insights into the MRO discussion and analysis involving MCG with SCG conditions.

Discussion

- 2.1	MRO enhancement for CHO with candidate SCGs

In alignment with both RAN2 and RAN3 agreements, the following table introduces a new classification of cases that will be given priority in discussions within both RAN2 and RAN3.

Table 1: Cases are preferable in both RAN2 and RAN3

| Scenario | Prerequisites | Failure cases |
| --- | --- | --- |
| No condition is fulfilled | Failure happens before CHO and/or CPAC execution. | 1a) RLF@srcPCell 1b) SCG failure |
| Only one condition is fulfilled | CPC/CPA execution condition is met. Failure happens before CHO execution condition is met. | 2a) RLF@srcPCell 2b) SCG failure |
| Only one condition is fulfilled | CHO execution condition is met. No CHO only or CHO with SCG configuration. Failure happens before CPAC execution condition is met. | 3a) RLF@srcPCell 3b) SCG failure |
| Only one condition is fulfilled | Failure due to the execution conditions for candidate PCell and the associated candidate PSCell cannot be met simultaneously. | 9a) RLF@srcPCell 9b) SCG failure 9c) MCG failure + SCG failure |
| Both conditions are fulfilled in failure scenario | Both CHO and CPAC execution conditions are met. | 7a) CHO execution to a candidate PCell failure. 7b) CPAC to a candidate PSCell failure. |
| Both conditions are fulfilled in sub-optimal scenario | Failure happens shortly after CHO and CPAC execution success | 8a) RLF@targetPCell 8b) SCG failure |

RAN3#125 has agreed to log the type of the first fulfilled execution condition in RLF reports. Additionally, RAN2#127 has agreed to log measurement results of PCells and PSCells in RLF reports. There is a necessity to discuss what additional information should be logged in RLF and other SON reports if required. Prior to this discussion, it is crucial to address the following three questions:

- How to distinguish the aforementioned cases from the network's perspective?

- What optimizations can the network perform in CHO with candidate SCGs scenarios?

- What information might precipitate an RLF or failure in CHO with candidate SCGs scenarios?

To respond to question 1), the agreed-upon cases are categorized into different classes as shown in:

Class 1: No conditions are met. E.g. case 1a, 1b.

Class 2a: Only one condition is met. E.g. case 2a, 2b, 3a, 3b, 9a, 9b, 9c.

Class 2b: Both CHO and CPAC conditions are met. E.g. 7a, 7b, 8a, 8b.

To differentiate between Class 1 and Classes 2a/2b, the agreed information suffices. If the first fulfilled condition is absent, the NW will recognize that cases 1a and 1b have occurred. With the logged measurement results of PCells and PSCells, the network can analyze whether poor channel quality leads to RLF.

To differentiate between Class 2a and Class 2b, the currently agreed information is insufficient. Even if the network knows only the first fulfilled execution condition is CHO or CPAC, it cannot determine if the other condition is fulfilled or not. Therefore, to distinguish between Class 2a and Class 2b, it is necessary to introduce an indication of the state of the second execution condition. If this indication is true, the network will know that both CHO/CPAC conditions are fulfilled. Otherwise, the network will understand that only one condition is fulfilled, as indicated by the first fulfilled execution condition. Thus, it is proposed to indicate whether one condition or both conditions are met in the RLF report, enabling the network to distinguish all three classes of cases. This addresses the question 1) raised earlier.

Proposal 1: To differentiate cases where one condition or both conditions are met prior to an RLF or failure, the indication of whether one condition or two conditions are met should be logged in the RLF report. FFS on explicit or implicit.

Questions 2) and 3) are interrelated; the causes that may lead to an RLF or failure include strict or loose configured conditions (CHO or CPAC), poor channel quality, excessive elapsed time between the first and last execution conditions being met, or a combination of these factors. The network's feasible action is to restrict or relax the configured conditions.

Proceeding with the analysis for cases with only one condition met (Class 2a as described above), in cases 2a and 3a, if the elapsed time between the first fulfilled condition and failure is too brief, it suggests that the initially fulfilled configured condition is too loose and should be tightened by the network. Conversely, if the elapsed time is excessively long, it implies that the unmet condition's configuration is overly strict, making it difficult to satisfy the second condition.

For cases 2b and 3b, the unmet second condition leading to failure indicates poor channel quality. In these instances, regardless of whether the elapsed time between the first fulfilled condition and failure is short or long, the failure is attributed to overly strict configured conditions by the network. Thus, it is proposed to log the elapsed time between the first fulfilled condition and failure in cases 2a and 3a. For cases 2b and 3b, no additional information beyond the type of the first fulfilled execution condition and the measurements of PCell and PSCell needs to be logged in the RLF report.

Proposal 2: For cases 2a and 3a, the elapsed time between the first fulfilled condition and failure should be logged in the RLF report.

Proposal 3: For cases 2b and 3b, there is no need for additional information beyond the type of the first fulfilled execution condition and the measurements of PCell and PSCell to be logged in the RLF report.

In cases 9a, 9b, and 9c, only one condition is fulfilled at the same time. The network needs to ascertain whether the first condition (e.g., CHO or CPAC) has been satisfied more than once. If so, the network only needs to adjust the condition that was not satisfied initially.

Proposal 4: For cases 9a, 9b, and 9c, an indication that the first execution condition has been fulfilled more than once should be logged in the RLF report.

For cases 7a, where both conditions are met but an MCG failure occurs, if the first fulfilled condition is CHO, the failure may be due to an excessively long elapsed time between the fulfillment of the CHO condition and the CPAC condition. If the first fulfilled condition is CPAC, the failure may be due to overly loose configured conditions for CHO. For case 7b, the scenario is similar. Thus, since the network already knows the first fulfilled condition, it is sufficient to introduce the elapsed time between the fulfillment of the CHO condition and the CPAC condition.

Proposal 5: For cases 7a and 7b, RAN2 to agree to log the elapsed time between the fulfillment of the CHO condition and the CPAC condition.

Cases 8a and 8b are analogous to cases 7a and 7b. The only difference is that the UE must log additional information in the SHR/SPR, including the elapsed time between the fulfillment of the CHO condition and the CPAC condition.

Proposal 6: For cases 8a and 8b, RAN2 to agree to log the elapsed time between the fulfillment of the CHO condition and the CPAC condition in SHR/SPR:

Conclusion

RAN2 is kindly asked to discuss the following observations and proposals:

Proposal 1: To differentiate cases where one condition or both conditions are met prior to an RLF or failure, the indication of whether one condition or two conditions are met should be logged in the RLF report. FFS on explicit or implicit.

Proposal 2: For cases 2a and 3a, the elapsed time between the first fulfilled condition and failure should be logged in the RLF report.

Proposal 3: For cases 2b and 3b, there is no need for additional information beyond the type of the first fulfilled execution condition and the measurements of PCell and PSCell to be logged in the RLF report.

Proposal 4: For cases 9a, 9b, and 9c, an indication that the first execution condition has been fulfilled more than once should be logged in the RLF report.

Proposal 5: For cases 7a and 7b, RAN2 to agree to log the elapsed time between the fulfillment of the CHO condition and the CPAC condition.

Proposal 6: For cases 8a and 8b, RAN2 to agree to log the elapsed time between the fulfillment of the CHO condition and the CPAC condition in SHR/SPR:

References

- R2_127_Maint CovEnh SON-MDT eRedCapn (Mattias)_20240823_1000

- RAN3_125_agenda_20240823EOM

- R3-244223	Failure scenarios on MRO for CHO with candidate SCGs Samsung, Cybercore, ZTE Discussion & Decision

- Annex: Failure scenarios (R3-244223)

| Cases | Figure: CHO with candidate SCG(s) is configured | Brief Description |
| --- | --- | --- |
| Case 1 |  | Failure happens before CHO and/or CPAC execution. 1a) RLF@srcPCell 1b) SCG failure 1c) MCG failure+SCG failure |
| Case 2 |  | CPC/CPA execution condition is met. Failure happens before CHO execution condition is met. 2a) RLF@srcPCell 2b) SCG failure 2c) MCG failure + SCG failure |
| Case 3 |  | CHO execution condition is met. No CHO only or CHO with SCG configuration. Failure happens before CPAC execution condition is met. 3a) RLF@srcPCell 3b) SCG failure 3c) MCG failure + SCG failure |
| Case 4 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. CHO execution to a candidate PCell fails. |
| Case 5 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. RLF shortly after CHO execution success |
| Case 6 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. SCG failure shortly after CHO execution success |
| Case 7 |  | Both CHO and CPAC execution conditions are met. 7a) CHO execution to a candidate PCell failure. 7b) CPAC to a candidate PSCell failure. 7c) Execution to SCG failure and toMCG failure. |
| Case 8 |  | Failure happens shortly after CHO and CPAC execution success 8a) RLF@targetPCell 8b) SCG failure 8c) MCG failure + SCG failure |
| Case 9 |  | Failure due to the execution conditions for candidate PCell and the associated candidate PSCell cannot be met simultaneously. 9a) RLF@srcPCell 9b) SCG failure 9c) MCG failure + SCG failure |