3GPP TSG-RAN WG2 #127bis	Tdoc R2-2408385

Hefei, China, 14th-18th October 2024

Agenda Item:	8.5.2

Source:	Ericsson

Title:	Discussion on on-demand SSB for NES

Document for:	Discussion, Decision

# 1	Introduction

In RAN#105, the WID for network energy savings in Rel-19 was revised [2]. The first objective concerns the on-demand SSB SCell operation, as shown below.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

The list of RAN1 agreements is available in the Appendix.

# 2	Configuration and signalling aspects

## This contribution focuses on the first objective highlighted in the above WID. In particular, the contribution addresses the aspects concerning the configuration, signaling, and activation/deactivation of on-demand SSB transmissions on SCells.

According to the WID, the UE is in connected mode and configured with CA and, hence, it is configured with a PCell and one or more SCell(s). Note that the PCell is always activated and not dormant. The SCell(s) can be activated and dormant/not dormant, or deactivated.

## 2.1	MAC CE for on-demand SSB

RAN1 has sent RAN2 LS on the MAC CE aspects with the following content:

RAN1 has discussed signaling mechanisms to indicate on-demand SSB transmission and has reached agreements to support indication by both RRC and MAC CE signaling.

Agreement (RAN1#118)

For a cell supporting on-demand SSB SCell operation,

- Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration.

  - FFS: Whether to support RRC based signaling for other cases.

- Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A.

Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

Agreement (RAN1#118)

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- Periodicity of the on-demand SSB

- FFS: Any other relevant parameters

Agreement (RAN1#116bis)

For a cell supporting on-demand SSB SCell operation, further study the following options.

- Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication.

- Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided.

  - FFS: Details of the signaling

- Other options are not precluded.

- FFS: Details on On-demand SSB transmission indication

To provide more context for RAN2, the following are relevant RAN1 agreements on Cases and Scenarios for on-demand SSB SCell operation.

Agreement (RAN1#116)

Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

- FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted.

FFS: Which scenario the above applies for

Agreement (RAN1#116)

For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios.

- Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

- Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321)

  - This does not preclude SCell for which activation is completed

  - FFS: The case where SCell activation is completed

FFS: Application timing between NW triggering message and on demand SSB transmission

Agreement (RAN1#116bis)

For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases:

- Scenario #2 and Case #1

- Scenario #2 and Case #2

- Scenario #2A and Case #1

- Scenario #2A and Case #2

- FFS: Scenario #3A and Case #1

- FFS: Scenario #3A and Case #2

- FFS: Scenario #3B and Case #1

- FFS: Scenario #3B and Case #2

- For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner.

  - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered.

- Notes:

  - Scenario #2A refers to

    - “When UE receives SCell activation command (e.g., as defined in TS 38.321)”

  - Scenario #3A refers to

    - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed”

  - Scenario #3B refers to

    - “When SCell activation is completed and SCell is activated” or

    - “After SCell activation is completed and SCell is activated”

  - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications.

  - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

RAN1 respectfully requests RAN2 to design MAC CE based signaling to indicate on-demand SSB transmission for Scenarios #2 and #2A.

RAN1 is working on the further details needed for RRC and MAC CE based signaling to indicate on-demand SSB transmission and will inform RAN2 as further agreements are made.

In the context of the MAC CE, RAN2 made the following agreements:

- Scenarios on OD-SSB:

  - RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

- OD-SSB transmission indication:

  - RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

  - New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

RAN1 has asked RAN2 to design the MAC CE based signalling to indicate on-demand SSB transmission for #2 and #2A. The RAN1 agreements with relevant content as input for the design are:

Agreement (RAN1#118)

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- Periodicity of the on-demand SSB

- FFS: Any other relevant parameters

Seems that RAN1 would like to see separate handle for each value of the OnDemand SSB configuration and has agreed that at least the periodicity is one such parameter. This type of design means that there is a separate field in the MAC CE for each RRC parameter of the on-demand SSB configuration. However, since the only parameter indicated so far is periodicity it is difficult to assess if this type of design is feasible or not. Another type of design would be that there are N amount of OnDemand SSB configurations, configured via RRC signalling, with different parameter values and the MAC CE points out one of the configurations to be activated/deactivated. Yet another option is to have M “base configurations” configured by RRC and a few separate parameters to be selected/configured directly by the MAC CE. Further, The MAC CE may activate OnDemand SSB, and/or indicate specific parameters for it, per SCell, or for a list of SCells. Which way to design the MAC CE depends on the complexity needed for each activated OnDemand SSB. Our preference would be to aim to simple MAC CE where control of all OnDemand SSBs of all SCells could be indicated in one MAC CE.

For now, RAN1 has concluded on only one OnDemand SSB related configuration parameter, OnDemand SSB periodicity.

Depending on the number of individual parameters related to OnDemand SSB configuration, it may not be feasible to separately control each of those in a MAC CE.

RAN2 should aim for a MAC CE where control of all OnDemand SSBs of all SCells could be indicated in one MAC CE.

Wait for RAN1 to progress with OnDemand SSB design before discussing the MAC CE details further.

## 2.2	RRC configuration for OnDemand SSB

Concerning the configuration aspects, the discussion in RAN1#117 meeting concluded with the following agreements:

Agreement

- For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported.

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

  - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE

  - FFS: Whether the RRC is newly introduced or existing RRC is reused

As highlighted in the agreement above, one of the possible solutions is that the network provides the UE with different options for the OnDemand SSB transmission patterns by an RRC configuration. These options for the SSB transmission patterns could be given outside of the SCell configuration since we think that the SSB patterns can be commonly configured for several SCells. Furthermore, to support additional flexibility, the configurations could be different for FR1 and FR2. In this way, configurations for FR1 and FR2 would be shared among serving cells operating on FR1 and FR2, respectively. It is important to note that while the options for the SSB transmission patterns would be per FR, the activation of a certain SSB transmission pattern could be different for different SCells. This way of configuring on-demand SSB per FR requires fewer bits compared to, for example, configuring OnDemand SSB per SCell. Additionally, we think that FR1 and FR2 have different characteristics (e.g., number of potential beams employed in each FR, SCS) and as such this scheme allows for more flexibility than, for example, configuring the same OnDemand SSB for both FR1 and FR2. For example, as the maximum number of beams is different in FR1 and FR2, the potential number of deployed OnDemand SSB beams would vary. This could lead to different periodicities and therefore different starting point/offsets for the OnDemand SSBs. Therefore, configuring OnDemand SSB per FR can be seen as a solution that is compact while still provides a reasonable level of flexibility.

Define OnDemand SSB configuration per FR which is applicable for all SCells in that FR.

# 3	L3 measurements

RAN2 agreements are:

- Measurement on OD-SSB:

  - Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

  - For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

  - RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

R2-2407414: Proposal 3: RAN2 WG to discuss the issue of false measurement report triggering due to no SSB transmission when on-demand SSB is deactivated.

- RAN2 does not handle this issue in OD-SSB

  - RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

Via its

serving cell configuration the UE has the configuration for OnDemand SSB, and receives the MAC CE for activation of the OnDemand SSB reception. Since the UE has this information, it is not evident any enhancement is needed for L3 RRM configuration.

Current RAN1 agreement on this always on and OnDemand SSB is as follows:

RAN1 #116bis Agreement

- For a cell supporting on-demand SSB SCell operation,

  - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not.

  - For on-demand SSB on the cell, downselect between the following alternatives

    - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not.

    - Alt-2: On-demand SSB is limited to non-cell-defining SSB.

      - FFS: Further limitations to on-demand SSB

RAN1 #118 Agreement

- For a cell supporting on-demand SSB SCell operation, at least the following is supported

  - On-demand SSB on the cell is not located on synchronization raster.

  - On-demand SSB on the cell is non-cell-defining SSB

- FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

As can be seen the discussion is not finished. Further aspect is whether these different SSBs are on same channel raster or not. Since especially this frequency aspect is open, it is premature to try to conclude on this matter in RAN2:

RAN2 to wait for RAN1/RAN4 to progress on the design for OnDemand SSB

# 4	Conclusion

In this contribution we discuss the second objective highlighted in the WID. Based on the discussion in the previous sections we made the following observations:

Observation 1	For now, RAN1 has concluded on only one OnDemand SSB related configuration parameter, OnDemand SSB periodicity.

Observation 2	Depending on the number of individual parameters related to OnDemand SSB configuration, it may not be feasible to separately control each of those in a MAC CE.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 should aim for a MAC CE where control of all OnDemand SSBs of all SCells could be indicated in one MAC CE.

Proposal 2	Wait for RAN1 to progress with OnDemand SSB design before discussing the MAC CE details further.

Proposal 3	Define OnDemand SSB configuration per FR which is applicable for all SCells in that FR.

Proposal 4	RAN2 to wait for RAN1/RAN4 to progress on the design for OnDemand SSB

# References

RP-242354, “ Revised WID: Enhancements of network energy savings for NR”, RAN#105, October 2024.

# Appendix

### RAN1#116 Agreements

Agreement

Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

- FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted.

FFS: Which scenario the above applies for

Agreement

RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations.

Agreement

For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios.

- Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

- Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321)

  - This does not preclude SCell for which activation is completed

  - FFS: The case where SCell activation is completed

FFS: Application timing between NW triggering message and on demand SSB transmission

Agreement

Support on-demand SSB SCell operation triggered by gNB.

- FFS Details of associated signaling/indication/configuration provided to UE

Agreement

- For SSB burst(s) triggered by on-demand SSB SCell operation, study at least the following options.

  - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

  - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

  - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

  - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

  - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

  - FFS: The combination of above options

  - FFS: How to define time instance A/B and the value of N per option

  - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

### RAN1#116bis Agreements

Agreement

For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases:

- Scenario #2 and Case #1

- Scenario #2 and Case #2

- Scenario #2A and Case #1

- Scenario #2A and Case #2

- FFS: Scenario #3A and Case #1

- FFS: Scenario #3A and Case #2

- FFS: Scenario #3B and Case #1

- FFS: Scenario #3B and Case #2

- For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner.

  - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered.

- Notes:

  - Scenario #2A refers to

    - “When UE receives SCell activation command (e.g., as defined in TS 38.321)”

  - Scenario #3A refers to

    - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed”

  - Scenario #3B refers to

    - “When SCell activation is completed and SCell is activated” or

    - “After SCell activation is completed and SCell is activated”

  - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications.

  - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not.

  - For on-demand SSB on the cell, downselect between the following alternatives

    - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not.

    - Alt-2: On-demand SSB is limited to non-cell-defining SSB.

      - FFS: Further limitations to on-demand SSB

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - L1 and/or L3 measurement based on on-demand SSB is supported for the cell.

    - FFS further details on L1 and/or L3 measurement

Agreement

The following agreement from RAN1#116 is modified (in red)

- For SSB burst(s) triggeredindicated by on-demand SSB SCell operation, study at least the following options.

  - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

  - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

  - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

  - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

  - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

  - FFS: The combination of above options

  - FFS: How to define time instance A/B and the value of N per option

  - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

Agreement

For a cell supporting on-demand SSB SCell operation, further study the following options.

- Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication.

- Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided.

  - FFS: Details of the signaling

- Other options are not precluded.

- FFS: Details on On-demand SSB transmission indication

### RAN1#117 Agreements

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - Support RRC based signaling to indicate on-demand SSB transmission on the cell.

  - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell.

  - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell.

    - This DCI signaling does not provide SCell activation/deactivation.

    - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc.

  - FFS: Scenarios where the above signalings are applicable

Agreement

- For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported.

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

  - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE

  - FFS: Whether the RRC is newly introduced or existing RRC is reused

Agreement

- At least support L1 measurement based on on-demand SSB

  - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported.

    - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

Agreement

For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

- Alt 3-1: Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission

  - The SSB time domain positions of on-demand SSB burst are configured by gNB.

- FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components

- Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

- FFS: Whether the value of T is predefined or indicated/configured by gNB

- FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB”

Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Agreement

- For a cell supporting on-demand SSB SCell operation, at least the followings for on-demand SSB are known to UE.

  - Sub-carrier spacing of the on-demand SSB

  - Physical Cell ID of the on-demand SSB

  - Location of on-demand SSB burst

  - Downlink transmit power of on-demand SSB

  - FFS: Other parameters

  - FFS: Whether each of above parameters is configured/indicated explicitly or not

### RAN1#118 Agreements

Agreement

- Update the previous RAN1 agreement as follows.

  - At least support L1 measurement based on on-demand SSB

    - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported.

      - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

      - The support of LTM is a separate discussion point

Agreement

For a cell supporting on-demand SSB SCell operation,

- Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration.

  - FFS: Whether to support RRC based signaling for other cases.

- Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A.

Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- Periodicity of the on-demand SSB

- FFS: Any other relevant parameters

Agreement

For a cell supporting on-demand SSB SCell operation, at least the following is supported

- On-demand SSB on the cell is not located on synchronization raster.

- On-demand SSB on the cell is non-cell-defining SSB

FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

Agreement

Support L3 measurement based on on-demand SSB

- Further work on L3 measurement is up to RAN2/RAN4

Agreement

LS to RAN2 for on-demand SSB SCell operation is agreed. Final LS in R1-2407438.

Agreement

The previous RAN1 agreement made in RAN1#117 is revised as follows.

- For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

  - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] of on-demand SSB burst [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is at least T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission

    - The SSB time domain positions of on-demand SSB burst are configured by gNB.

  - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components

  - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

  - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification.

    - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1

  - FFS: Whether the value of T is predefined or indicated/configured by gNB

  - (Working assumption) T=T_min

  - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB”

- Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Agreement

LS on timeline for On-demand SSB operation on SCell is agreed in R1-2407565.