3GPP TSG-RAN WG2 #127-bis	R2-2408386

Hefei, China, Oct 14th– 18th, 2024

Agenda Item:	8.5.3

Source:	Ericsson

Title:	Discussion on on-demand SIB1 for NES

Document for:	Discussion, Decision

# 1	Introduction

The WID for network energy savings in Rel-19 was revised two times in RAN#103 [1], and RAN#105 [5] meetings, respectively, with the objective to study on-demand SIB1 for idle/inactive UEs as highlighted below:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this paper, we discuss the remaining issues raised in the previous RAN WG2 meetings. In the Appendix we also provide the list of RAN1 agreements.

# 2	UL WUS configuration and UE behaviour

As agreed in the latest WID RAN#105 [5], RAN2 should:

- Specify procedures and signaling method(s) for Case 2 [RAN1/2]

- Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell.

Taking this into consideration, in this section we address the following aspects:

- Provisioning of the UL WUS configuration,

- UL WUS configuration in the case of multiple NES Cells, and

- RA procedure for on-demand SIB1 acquisition.

## 2.1		Provisioning of the UL WUS configuration

The aspects related to the provisioning of the UL WUS configuration and associated updates were discussed during RAN WG2 Meeting #126 [3] and RAN WG2 Meeting #127. The discussion concluded with the following agreements in RAN WG2 Meeting #126 [3]:

Information of WUS configuration

- For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Transmission of WUS configuration

- Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

and in RAN WG2 Meeting #127:

WUS configuration of NES cell from NES cell:

- Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

As highlighted in the agreement on “transmission of WUS configuration” from RAN WG2 Meeting #126, a pending question that still needs to be addressed is whether Cell A provides the UL WUS configuration for NES Cell via an existing SIBx or a new SIB. One of the considered options is to include the UL WUS configuration for NES Cell into SIB1 or some other existing SIBx of Cell A. In our opinion, using SIB1 of Cell A to provide the information concerning the UL WUS configuration of NES Cell is not the best choice for the following reasons. SIB1 carries the essential information concerning the cell in which it is transmitted, and the UL WUS configuration of NES Cell cannot be considered essential information of Cell A. There are also practical concerns with extending SIB1 with non-essential contents as SIB1 already has a considerable size, especially in multi-PLMN deployments where many information elements are provided per PLMN. It would therefore be unwise to further extend SIB1 message and compromise SIB1 coverage unless it is absolutely necessary. Finally, reusing SIB1 or an existing SIBx of Cell A may impose unnecessary constrains on the size of the UL WUS configuration, which may be especially problematic in cases where the same Cell A assists multiple NES Cells in OD-SIB1 provisioning, i.e., when the same Cell A needs to provide UL WUS configurations for multiple NES Cells. Therefore, for the above-mentioned reasons, we prefer to define a new SIB-y for Cell A for the purpose of providing the UL WUS configuration(s) of the NES Cell(s). When applicable, this new SIB-y is provided in the Cell A for assisting the NES cell. This new SIB-y is also provided in the NES Cell itself when there are UL WUS configuration updates.

SIB1 carries the essential information (of Cell A), and the UL WUS configuration of NES Cell cannot be considered essential information of Cell A.

SIB1 has already a considerable size, especially in multi-PLMN deployments. Extending SIB1 message has impact on coverage.

One Cell A may assist multiple NES cells, including multiple WUS configurations. Extending an existing SIBx may be unnecessarily limiting on number of UL WUS configuration that can be broadcast.

UL WUS configuration for the NES Cell is provided in a new SIB-y of Cell A.

UL WUS configuration updates for the NES Cell is provided in the new SIB-y of NES Cell.

## 2.2		UL WUS configuration in case of multiple NES Cells

The versions of Scenario 1a with multiple Cells A and NES Cells were discussed during the WG2 Meeting #126 [3], and the following agreements were made:

Further clarification

- Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

Information of WUS configuration

- Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with NES Cell.

As highlighted in the agreement on “further clarification”, the same Cell A may assist multiple NES Cells. One of the remaining questions that needs to be addressed is whether the same UL WUS configuration can be used for multiple NES Cells. The benefit of using the same UL WUS configuration for multiple NES Cells is that it allows for minimizing the size of the message that carries the UL WUS configuration(s). However, using different UL WUS configurations for different NES Cells may be needed if Cell A assists multiple NES Cells that are in different bands (e.g., NES Cells in FR1 and FR2) or have different deployment characteristics such as different cell sizes. The benefit of using different UL WUS configurations is that it allows for the flexibility to provide different UL WUS configuration parameters for different NES Cells. Furthermore, in order to avoid waking up multiple NES cells by the same WUS configuration/transmission, distinguished WUS configurations for various NES cell are favorable. Therefore, in order to achieve the balance between the message size, unnecessary invocations in multi-NES-Cell deployments, and configuration flexibility, the NW should be able to decide whether to use the same or different UL WUS configurations for multiple NES Cells.

Using the same UL WUS configuration for multiple NES Cells allows for minimizing the size of the message that carries the UL WUS configuration(s).

Using different UL WUS configurations for different NES Cells avoids unnecessary/false NES Cell wakeups, allows for the flexibility to provide different UL WUS configuration parameters for different NES Cells when needed (e.g., if Cell A assists both NES cells in FR1 and FR2, different cell sizes, etc.).

It should be possible for the network to indicate one UL WUS configuration for multiple NES cells and/or separate UL WUS configurations.

## 2.3		On-demand SIB1 acquisition procedure

The aspects related to the acquisition of on-demand SIB1 and its updates were discussed during RAN WG2 Meeting #125bis [2], RAN WG2 Meeting #126 [3] and RAN WG2 Meeting #127. The discussion concluded with the following agreements in RAN WG2 Meeting #125bis [2]:

Agreements on on-demand SIB1:

- Existing Msg 1 based on-demand procedure is reused for on-demand SIB1 acquisition procedure. FFS on Msg 3. FFS if / when the UE monitors the OD-SIB1 upon reception of RAR. FFS: whether introduce specified UE behavior if RACH failure of OD-SIB1 request.

RAN WG2 Meeting #126 [3]:

SIB1 acquisition upon SIB change notification in NES cell

- Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

Paging and SIB1 update in NES cell

- After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

RAR monitoring for MSG1 based OD-SIB1 REQ

- RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

UE behaviour on RACH failure of OD-SIB1 REQ

- As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation

and RAN WG2 Meeting #127:

OD-SIB1 Validity:

- We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

- Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

MSG3 based OD-SIB1 REQ:

- Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

According to the highlighted agreements from the previous meetings, RAN2 agreed to support only MSG1 based on-demand SIB1 request procedure. The details concerning the RAR for MSG1 based on-demand SIB1 request procedure require, however, further discussion.

According to the current TS 38.331 [4], the following is specified:

“SIB1 includes information regarding the availability and scheduling (e.g. mapping of SIBs to SI message, periodicity, SI-window size) of other SIBs with an indication whether one or more SIBs are only provided on-demand and, in that case, the configuration needed by the UE to perform the SI request…. Each SI message is associated with an SI-window and the SI-windows of different SI messages do not overlap. That is, within one SI-window only the corresponding SI message is transmitted.”

The above specification text states that SIB1 carries the information regarding the availability and scheduling of the other SIBs. However, similar information is not available for SIB1, and given that SIB1 carries the essential information, there is a need for mechanisms that would allow the NW to convey the information to the UE regarding the requested on-demand SIB1 transmissions.

The existing mechanisms cannot be directly used for defining the UE behaviour upon transmitting MSG1 that carries the request for on-demand SIB1.

According to the above-highlighted agreement on “RAR monitoring for MSG1 based OD-SIB1 REQ”, the assumption is that as baseline the UE is expected to receive a RAR message as a response to the UE’s on-demand SIB1 request contained in MSG1. It is important to note that a NES Cell can send the same RAR message to all UEs that requested on-demand SIB1 since all requests are intended for the same functionality. Consequently, sending a RAR message as a response to on-demand SIB1 request(s) does not cause excessive signalling nor significant energy consumption in the NES Cell.

NES Cell can send the same RAR message to all UEs that requested on-demand SIB1 since all requests are intended for the same functionality, and hence sending a RAR message as a response to on-demand SIB1 request(s) does not cause excessive signalling nor significant energy consumption in the NES Cell.

Furthermore, similarly as the UE can use MSG1 to request on-demand SIB1, the NES Cell can use RAR as a feedback message to instruct the UE how to behave upon its reception. For example, the RAR can indicate the information when the UE can expect on-demand SIB1 transmissions relative to the RAR transmission (e.g., an offset value to on-demand SIB1 transmission and/or a time window during which the UE can expect on-demand SIB1 transmission can be introduced for this purpose), whether on-demand SIB1 is transmitted in all directions or only in some SSBs, and the information concerning the UE behaviour if the UE does not receive on-demand SIB1 according to the timeline specified in RAR message (e.g., whether the UE should attempt to repeat the request for on-demand SIB1 and if yes for how long the UE should wait before repeating the request). The RAR can contain the explicit parameter values or a pointer to the values preconfigured in the UL WUS configuration.

RAR message is used for indicating the UE behaviour concerning when to expect on-demand SIB1 transmission, in which directions the transmission of on-demand SIB1 can be expected, and how to proceed if on-demand SIB1 is not received when expected.

# 4	Cell barring, reselection, and camping aspects

In this section we address the aspects related to:

- Barring of legacy and Rel-19 UEs, and

- Cell reselection and camping aspects.

## 4.1		Barring aspects for legacy and Rel-19 NES UEs

We start with discussing the remaining questions concerning the barring of legacy UEs. During RAN WG2 Meeting #126 [2], RAN2 reached the following agreement on legacy UE impact:

Legacy UE impact

- NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

The discussion about the barring of legacy UEs continued during RAN WG2 Meeting #127, and concluded with the following agreements:

Access restriction for legacy UEs:

- Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Impacts to RRC connected UEs:

- RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

The above agreement on “Legacy UE impact” from During RAN WG2 Meeting #126 states that there is a need to bar legacy UEs from accessing a NES Cell with on-demand SIB1. The exact solution for barring is, however, still an open question, and according to the above agreement on “Access restriction for legacy UEs” from RAN WG2 Meeting #126, three options for barring are being considered for a NES Cell that transmits on-demand SIB1. Observe that in the case of Option 1, the legacy UEs that are capable of features for which the barring flag is in SIB1, would ignore the MIB barring flag, and instead need SIB1 to understand that the cell is barred. Therefore, Option 1 is not viable as the only mechanism for barring legacy UEs. In the case of Option 3, new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs would need to be introduced, and the UE would need to acquire SIB3 for the purpose of checking the list of exclude-listed inter-frequency neighboring cells and SIB4 for the purpose of checking the list of exclude-listed inter-frequency neighboring cells. This, however, may not be feasible in some cases, e.g., in the case of a UE that powers on and finds OnDemand-SIB1 NES cell. Compared to Option 1 and Option 3, Option 2 has less specification impact and does not require unnecessary signaling since it relies on the implicit barring of legacy UEs.

Legacy UEs that are capable of features for which the barring flag is in SIB1, would ignore the MIB barring flag, and instead need SIB1 to understand that the cell is barred. Therefore, Option 1 is not viable as the only mechanism for barring legacy UEs.

Legacy UEs that enter the OnDemand-SIB1 NES cell via cell selection do not have a list of excluded cells, therefore Option 3 is not a viable mechanism for restricting legacy UEs from NES cells.

A legacy UE is implicitly barred from a NES cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

For the purpose of discussing the barring mechanisms for Rel-19 NES UEs capable of on-demand SIB1, let us recall the following agreement from RAN WG2 Meeting #125bis [2]:

The UE first should acquire valid SIB1 (e.g. via SIB1 request) for camping to NES cell (if the UE knows the cell doesn’t broadcast SIB1 and supports on-demand SIB1).

The above agreement implies that a Rel-19 UE capable of on-demand SIB1 is implicitly barred from a NES OnDemand-SIB1 Cell in case it cannot acquire a valid SIB1. On the contrary, if a Rel-19 UE capable of on-demand SIB1 obtains a valid SIB1, then it follows the legacy mechanisms to determine whether the cell is barred or not. That is, if a Rel-19 UE capable of on-demand SIB1 is also capable of some other feature that has an associated barring flag in SIB1, then the UE ignores the barring flag in MIB and checks the barring flag in SIB1. Otherwise, if a Rel-19 UE capable of on-demand SIB1 is not capable of the features that are associated with a barring flag in SIB1, then the UE determines whether the cell is barred or not based on the barring flag in MIB. The latter implies that Option 2 “barring based on no SIB1 indication” is agreed.

A Rel-19 UE capable of on-demand SIB1 is implicitly barred from a NES OnDemand-SIB1 cell in case it cannot acquire a valid SIB1.

Upon acquisition of a valid SIB1, a Rel-19 UE capable of on-demand SIB1 and some other feature that is associated with a barring flag in SIB1 determines whether the cell is barred or not based on a barring flag in SIB1.

Upon acquisition of a valid SIB1, a Rel-19 UE capable of on-demand SIB1 but not capable of any of the other features that are associated with a barring flag in SIB1, determines whether the cell is barred or not based on a barring flag in MIB(assumption Option2 is adopted).

We continue with discussing the question related to barring relaxation for Rel-19 NES UEs in case the UE is unable to acquire SIB1. During RAN WG2 Meeting #126 [3], RAN2 reached the following agreements concerning the barring of Rel-19 NES UEs:

Barring for R19 NES UE

- If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

The above agreement on “barring for R19 NES UE” implies that the legacy behavior according to which UE considers the cell as barred if it is unable to acquire the SIB1 should be relaxed for Rel-19 NES UEs supportive of the on-demand SIB1 feature. However, an open question related to the above agreement is how long a UE should wait before initiating on-demand SIB1 procedure, i.e., once it identifies a NES Cell and obtains the corresponding UL WUS configuration. It is important to observe that even in a NES Cell with on-demand SIB1, a UE may still be able to acquire SIB1 without requesting it, i.e., if the NES cell is transmitting on-demand SIB1 as a response to another UE’s request, or if the gNB decides to revert the NES cell to its legacy mode of transmitting SIB1 periodically. Acquiring SIB1 without initiating on-demand SIB1 procedure implies fewer UL WUS transmissions, and hence the potential for further minimization of the energy consumption on both, the NW, and the UE side. One way of supporting SIB1 acquisition without an explicit on-demand SIB1 request is to instruct the UE (e.g., in the UL WUS configuration) to wait for a certain time before requesting on-demand SIB1, and/or to cancel the UL WUS process in case it finds SIB1 in the meantime. To avoid prolonged waiting, and hence a negative impact in terms of latency at the UE, the UE could be instructed, for example, to wait with on-demand SIB1 request for the periodicity at which the cell would have been transmitting SIB1 if it was operating in a legacy mode. In this case, the impact on the UE in terms of latency would not be larger than that of legacy periodic SIB1 transmissions. Furthermore, the UE could while waiting for the upcoming WUS transmission occasion, decode potential OD-SIB1 RAR occasions in case another UE has already requested OD-SIB1 in the previous WUS occasion.

Acquiring already ongoing SIB1 transmission without initiating on-demand SIB1 procedure implies fewer UL WUS transmissions, and hence the potential for further minimization of the energy consumption on both, the NW, and the UE side.

Discuss how to support SIB1 acquisition without transmitting UL WUS for NES Cell with on-demand SIB1, e.g., before WUS transmission the UE attempts to decode SIB1 and/or OD-SIB1 RAR at given occasions according to information in WUS configuration.

## 4.2		Cell reselection and camping aspects

Cell reselection and camping aspects were discussed during RAN WG2 Meeting #126 [3], and the following related agreements were reached:

Triggering condition of WUS transmission

- If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

- After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

UE request SIB1 to perform initial access in NES cell

- RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

Legacy UE impact

- How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

The above agreements on “triggering condition of WUS transmission” imply that the UE can trigger the request for on-demand SIB1 upon intending to camp on NES cell, i.e., before cell (re-)selection to NES cell. However, as highlighted in the agreement on the “legacy UE impact”, one of the pending questions from the previous meeting concerns cell reselection priorities for the legacy and Rel-19 UEs capable of on-demand SIB1 feature. In what follows we discuss the question of NES Cell prioritization/deprioritization.

In certain scenarios it could be beneficial to prioritize or deprioritize the reselection of NES Cell and Cell A. From the perspective of the UEs, the benefit is in preventing unnecessary energy consumption and delays, which occur while the UE is applying barring procedures for the NES Cell and searching for another suitable cell. Similarly, at the NW side, it could be beneficial in terms of energy consumption to deprioritize the reselection to the NES Cell if a NES Cell experiences an excessive number of wakeups without or with very few UEs accessing the NES cell. On the contrary, if for example, the NW have had intended the use of NES Cell, but configuration is done in such a way that only a few UEs are connected or paged for transmission in the DL, it could be beneficial to incentivize the reselection of the NES Cell.

Reusing the existing reselection parameters for incentivizing/disincentivizing the reselection of a NES OnDemand-SIB1 Cell is not suitable for the following reason. The existing reselection parameters are broadcasted for the cell reselection, and hence the same for all UEs camping on the cell. This implies that the existing cell reselection mechanisms do not support partitioning of the UEs in groups that have different reselection criteria (e.g., one for legacy and one for Rel-19 NES UEs). Therefore, in order to provide the NW with the desired flexibility in terms of cell reselection, one of the possible approaches is to define new NES-specific cell reselection priority parameters. These new NES-specific cell reselection priority parameters can be provided, for example, by Cell A.

The existing parameters for cell reselection do not support partitioning of the UEs in groups that have different reselection criteria (e.g., one for legacy and one for Rel-19 NES UEs capable of OnDemand-SIB1).

New NES-specific reselection priority parameters are defined for the purpose of prioritizing/deprioritizing a NES OnDemand-SIB1 Cell.

# 5	Indication of missing UL WUS configuration

In what follows we discuss the problems that may arise with missing UL WUS configuration and the means that can be used to address this type of problems. The problem of a missing UL WUS configuration is illustrated in Figure 1, and it occurs in a scenario where a part of the coverage area of the NES Cell is not covered by a Cell A, but it is covered by another cell, here denoted Cell X. In this case, the UE can find the NES Cell, but it cannot find (any) Cell A. Consequently, the UE cannot camp on the NES Cell. Such incomplete Cell A coverage effectively reduces the coverage area of a NES Cell, causing suboptimal performance and a waste of energy for both network and UE, since the UE cannot connect to the nearest cell when needed.

Figure 1: An example of how missing UL WUS configuration indication can reach the NES-Cell gNB.

Lack of Cell A coverage in the coverage area of the NES Cell causes suboptimal performance and wastes energy for both network and UE.

Besides the problem of a missing UL WUS configuration, there are other types of potential on-demand SIB1 problems that may arise with on-demand SIB1 provisioning, e.g., in the case where the UE has an UL WUS configuration and sends an UL WUS but does not receive on-demand SIB1. Consequently, it is important to provide the means to detect and report the problems with on-demand SIB1 provisioning to the NW so that the NW can take the appropriate actions and resolve the problem. Our RAN3 paper ‎[7] discusses the actions that the NW can take to resolve this type of problems. To support the detection of problems with missing UL WUS configuration and other similar problems with on-demand SIB1 provisioning, it is essential that a UE can detect that a cell is a NES Cell (i.e., that is using on-demand SIB1 transmission) from the SSB/MIB of the cell itself, which falls in RAN1 scope. In addition, to support a complete solution to the missing UL WUS configuration and similar problems with on-demand SIB1 provisioning, RAN2 needs to specify means for a UE to inform the NW about problems with on-demand SIB1 provisioning.

Discuss the means that would allow a UE to inform the NW about problems with on-demand SIB1 provisioning.

Furthermore, it is important to observe that the means that would allow a UE to inform the NW about problems with on-demand SIB1 provisioning could be used for enabling automatic Cell A relations. For example, an operator may deliberately deploy NES cells without manually configuring the NES Cell/Cell A relations, and through the reporting about problems with on-demand SIB1 provisioning, the relations can be automatically set up by different NG-RAN nodes. This can simplify NES Cell deployment for the operator.

The means that would allow a UE to inform the NW about problems with on-demand SIB1 provisioning could be used for enabling automatic Cell A relations, which can simplify NES Cell deployment for the operator.

We discuss this is issue also in [7]

# 6	On-demand SIB1 and Automatic Neighbour Relation

One of the important features that may be affected by on-demand SIB1 is Automatic Neighbour Relation (ANR). Introduction of on-demand SIB1 affects the ANR in a way that NES Cell with on-demand SIB1 cannot be discovered by the ANR. According to the current specification (c.f., Section 15.3.3 of 38.330 [5]), if a cell is not broadcasting SIB1, the UE aborts the Cell Global Identity (CGI) reporting procedure and reports "noSIB1":

“In case the detected NR cell does not broadcast SIB1, the UE may report noSIB1 indication as specified in TS 38.331 [12].”

Therefore, if the coexistence of on-demand SIB1 and ANR is not addressed, operators may need to manually provision and manage NES cells with on-demand SIB1 in the neighbour relation tables. Since this is not a practical and desirable solution, there is a need for new mechanisms that would allow for a functional coexistence of on-demand SIB1 and ANR. For example, one solution could be that the NW configures the UE whether CGI-Info is to be captured for NES cells with OD-SIB1, another solution could be that CGI-Info (or parts of it) of the NES cell is included in the WUS configuration such that the UE already knows or only needs to capture the WUS config and hence avoids asking for SIB1 for the sake of ANR.

Discuss how to support a functional coexistence of on-demand SIB1 and ANR. Example solutions could be that NW configured the UE with whether SIB1 is to be requested for the sake of ANR, another is that CGI-Info is included as part of WUS config so that UE avoids asking for SIB1.

# 7	Conclusion

In this contribution we discuss the second objective highlighted in the WID. Based on the discussion in the previous sections we made the following observations:

Observation 1	SIB1 carries the essential information (of Cell A), and the UL WUS configuration of NES Cell cannot be considered essential information of Cell A.

Observation 2	SIB1 has already a considerable size, especially in multi-PLMN deployments. Extending SIB1 message has impact on coverage.

Observation 3	One Cell A may assist multiple NES cells, including multiple WUS configurations. Extending an existing SIBx may be unnecessarily limiting on number of UL WUS configuration that can be broadcast.

Observation 4	Using the same UL WUS configuration for multiple NES Cells allows for minimizing the size of the message that carries the UL WUS configuration(s).

Observation 5	Using different UL WUS configurations for different NES Cells avoids unnecessary/false NES Cell wakeups, allows for the flexibility to provide different UL WUS configuration parameters for different NES Cells when needed (e.g., if Cell A assists both NES cells in FR1 and FR2, different cell sizes, etc.).

Observation 6	The existing mechanisms cannot be directly used for defining the UE behaviour upon transmitting MSG1 that carries the request for on-demand SIB1.

Observation 7	NES Cell can send the same RAR message to all UEs that requested on-demand SIB1 since all requests are intended for the same functionality, and hence sending a RAR message as a response to on-demand SIB1 request(s) does not cause excessive signalling nor significant energy consumption in the NES Cell.

Observation 8	Legacy UEs that are capable of features for which the barring flag is in SIB1, would ignore the MIB barring flag, and instead need SIB1 to understand that the cell is barred. Therefore, Option 1 is not viable as the only mechanism for barring legacy UEs.

Observation 9	Legacy UEs that enter the OnDemand-SIB1 NES cell via cell selection do not have a list of excluded cells, therefore Option 3 is not a viable mechanism for restricting legacy UEs from NES cells.

Observation 10	A Rel-19 UE capable of on-demand SIB1 is implicitly barred from a NES OnDemand-SIB1 cell in case it cannot acquire a valid SIB1.

Observation 11	Upon acquisition of a valid SIB1, a Rel-19 UE capable of on-demand SIB1 and some other feature that is associated with a barring flag in SIB1 determines whether the cell is barred or not based on a barring flag in SIB1.

Observation 12	Upon acquisition of a valid SIB1, a Rel-19 UE capable of on-demand SIB1 but not capable of any of the other features that are associated with a barring flag in SIB1, determines whether the cell is barred or not based on a barring flag in MIB(assumption Option2 is adopted).

Observation 13	Acquiring already ongoing SIB1 transmission without initiating on-demand SIB1 procedure implies fewer UL WUS transmissions, and hence the potential for further minimization of the energy consumption on both, the NW, and the UE side.

Observation 14	The existing parameters for cell reselection do not support partitioning of the UEs in groups that have different reselection criteria (e.g., one for legacy and one for Rel-19 NES UEs capable of OnDemand-SIB1).

Observation 15	Lack of Cell A coverage in the coverage area of the NES Cell causes suboptimal performance and wastes energy for both network and UE.

Observation 16	The means that would allow a UE to inform the NW about problems with on-demand SIB1 provisioning could be used for enabling automatic Cell A relations, which can simplify NES Cell deployment for the operator.

Based on the discussion in the previous section, we propose the following:

Proposal 1	UL WUS configuration for the NES Cell is provided in a new SIB-y of Cell A.

Proposal 2	UL WUS configuration updates for the NES Cell is provided in the new SIB-y of NES Cell.

Proposal 3	It should be possible for the network to indicate one UL WUS configuration for multiple NES cells and/or separate UL WUS configurations.

Proposal 4	RAR message is used for indicating the UE behaviour concerning when to expect on-demand SIB1 transmission, in which directions the transmission of on-demand SIB1 can be expected, and how to proceed if on-demand SIB1 is not received when expected.

Proposal 5	A legacy UE is implicitly barred from a NES cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

Proposal 6	Discuss how to support SIB1 acquisition without transmitting UL WUS for NES Cell with on-demand SIB1, e.g., before WUS transmission the UE attempts to decode SIB1 and/or OD-SIB1 RAR at given occasions according to information in WUS configuration.

Proposal 7	New NES-specific reselection priority parameters are defined for the purpose of prioritizing/deprioritizing a NES OnDemand-SIB1 Cell.

Proposal 8	Discuss the means that would allow a UE to inform the NW about problems with on-demand SIB1 provisioning.

Proposal 9	Discuss how to support a functional coexistence of on-demand SIB1 and ANR. Example solutions could be that NW configured the UE with whether SIB1 is to be requested for the sake of ANR, another is that CGI-Info is included as part of WUS config so that UE avoids asking for SIB1.

# References

RP-240170, “Revised WID: Enhancements of network energy savings for NR”, RAN103, Maastricht, Netherlands, March 2024.

R2-2404102, “Report of 3GPP TSG WG2 meeting #125bis”, 3GPP TSG-RAN WG2 Meeting #125bis, Changsha, China, April 15 – 19, 2024.

R2-2406202, “Report of 3GPP TSG WG2 meeting #126”, 3GPP TSG-RAN WG2 Meeting #126, Fukuoka, Japan, May 20 – 24, 2024.

3GPP TS 38.331 V18.1.0, Technical Specification Group Radio Access Network; NR; Radio Resource Control (RRC) protocol specification, Release 18.

3GPP TS 38.300 V18.1.0, Technical Specification Group Radio Access Network; NR; NR and NG-RAN Overall Description, Release 18.

RP-242354, “ Revised WID: Enhancements of network energy savings for NR”, RAN105,  Melbourne, Australia, September 2024.

R3-245498 “Xn impact of On-demand SIB1 for UEs in idle/inactive mode”, Ericsson, Hefei

# Appendix A

### RAN1#116 Agreements

Agreement

For discussion purpose, the following assumption will be used in RAN1

- Cell A: A cell that is periodically transmitting at least its own SIB1

- NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 studies the following options.

On target cell of UL WUS transmission:

- Option 1: UE transmits UL WUS to NES Cell

- Option 2: UE transmits UL WUS to Cell A

On configuration provision for UL WUS transmission

- Option A: UE obtains the UL WUS configuration from NES Cell

- Option B: UE obtains the UL WUS configuration from Cell A

Other options are not precluded

Agreement

For further study of achievable NES gain with on-demand SIB1 for idle/inactive mode UE,

- Assume the following for network energy evaluation of non-NES cell in FR1:

  - Empty/low/medium cell load as defined in 38.864

  - Cat 1/Cat 2 BS as defined in 38.864

  - 30kHz SCS, DDDSU TDD pattern

    - Case A: 20ms SSB period with 20ms SIB1 period;

    - Case C: 20ms SSB period with 160ms SIB1 period;

    - Case D: 20ms SSB period with 40ms SIB1 period;

Note: Other SSB/SIB1 periodicity assumptions are not precluded (up to companies to report)

  - 4 or 8 SSBs in a SSB burst with SSB pattern case C

  - 20ms or 160ms PRACH monitoring period

- Assume the following for network energy evaluation of NES cell in FR1:

  - Empty/low/medium cell load as defined in 38.864

  - Cat 1/Cat 2 BS as defined in 38.864

  - 30kHz SCS, DDDSU TDD pattern

    - Case 1: 20ms SSB period with no SIB1 transmitted;

Note: Other SSB/SIB1 assumptions are not precluded (up to companies to report)

  - 4 or 8 SSBs in a SSB burst with SSB pattern case C

  - 20ms/160ms UL WUS monitoring period

- Note: SSB/CORESET0 multiplexing pattern 1 is used

Agreement

- For study of UL WUS design, consider at least PRACH as a starting point

  - FFS: Whether there is dedicated PRACH resource for SIB1 request

- Other option(s) not precluded

Agreement

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 to discuss triggering conditions for sending UL-WUS.

Agreement

For the study of on-demand SIB1 for idle/inactive mode UE, RAN1 to further study whether feedback from gNB in response to the SIB1 request is supported including associated details.

Agreement

For the further study on UL WUS configuration among the following options:

- Option 1: Pre-defined UL WUS configuration

- Option 2: UL WUS configuration that applies to multiple NES cell

- Option 3: UL WUS configuration that applies to a single NES cell

### RAN1#116bis Agreements

Agreement

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 focuses its studies on the following cases:

- Case 1: Option 1+A+X

- Case 2: Option 1+B+X

- Case 3: Option 2+B+Y

Where the options 1/2/A/B/X/Y are defined below:

- On target cell of UL WUS transmission:

  - Option 1: UE transmits UL WUS to NES Cell

  - Option 2: UE transmits UL WUS to Cell A

- On configuration provision for UL WUS transmission

  - Option A: UE obtains the UL WUS configuration from NES Cell

  - Option B: UE obtains the UL WUS configuration from Cell A

- On receiving of SIB1

  - Option X: UE receives on-demand SIB1 from NES Cell

  - Option Y: UE receives on-demand SIB1 from Cell A

Agreement

RAN1 to further study the following UE operation scenarios in the UL WUS design:

- Scenario 1: UE requests SIB1 to camp on NES cell

- Scenario 2: UE request SIB1 to perform random access procedure to make RRC connection to NES cell

Agreement

RAN1 to further study UE identification of NES cell with on-demand SIB1 based on one, both, or combination of the following options:

- Option 1: By WUS configuration

- Option 2: By PBCH payload of NES cell

Agreement

Companies to report at least the following key settings used in the evaluation/simulation of achievable NES gain with on-demand SIB1 in idle/inactive mode

- Setting A: SIB1 period (20ms/40ms/160ms)

- Setting B1: Cell load (Empty/low/medium)

- Setting B2: Traffic model

- Setting C: SIB1 PDSCH time domain resource index in 38.214 Table 5.1.2.1.1-2

- Setting D: CORESET0/SSB multiplexing pattern including controlResourceSetZero (index) in 38.213 Table 13-6, and searchSpaceZero (index) in 38.213 Table 13-11

- Setting E: PRACH configurations (including PRACH configuration index in 38.211 Table 6.3.3.2-3) for WUS and initial/random access

- Setting F: Cat1/Cat2 BS

- Setting G: Number of SSB beams

- Setting H: NES gain/loss on Cell A

- Setting I: On-demand SIB1 transmission rate (how often UE requests on-demand SIB1)

Agreement

For further study of the NES gain/loss evaluation assumption on Cell A with on-demand SIB1 on NES cell for idle/inactive mode UE,

- Assume the following for network energy evaluation of Cell A in FR1:

  - Company to report among empty/low/medium cell load as defined in 38.864

  - Same Cat BS as the Non-NES cell

  - 30kHz SCS, DDDSU TDD pattern

    - Same SSB period as the Non-NES cell and company to report SIB1 period

  - Same number of SSBs in a SSB burst as the Non-NES cell with SSB pattern case C

  - 20ms PRACH configuration periodicity for WUS and/or initial access RACH and company to report RACH configuration index in 38.211 Table 6.3.3.2-3

  - Same SSB/CORESET0 multiplexing pattern and same SIB1 PDSCH time domain resource allocation as the Non-NES cell

  - Same traffic model as the Non-NES cell

  - Companies to report the assumption of WUS configuration provision or UL WUS monitoring or on-demand SIB1 transmission on Cell A if Case 2 (Option 1+B+X) or Case 3 (Option 2+B+Y) is considered

Agreement

For UL WUS design for SIB1 request, at least dedicated PRACH resource is the assumption for further study in RAN1

- FFS: Details on time, frequency, and/or PRACH preamble resources for UL WUS

- FFS: whether RACH resource for SIB1 request could be used for an initial access procedure and/or an on-demand SI procedure

Agreement

Conditions for triggering UL WUS transmission is up to RAN2. Any related work in RAN1 to be triggered by RAN2 LS. Send an LS to RAN2. Final LS in R1-2403779.

### RAN1#117 Agreements

Agreement

For SIB1 in idle/inactive mode, prioritize RAN1 discussions on Case 2 and Case 3

- Case 2 (Option 1+B+X) is feasible from RAN1 perspective.

- Further study Case 3, focusing on the additional NES benefits over Case 2, feasibility, complexity, and spec impact.

Agreement

For further study of on-demand SIB1 in idle/inactive mode, it is assumed that always-on SSB is transmitted on the NES cell with on-demand SIB1.

Agreement

At least for Case-2: For further study of type 0 PDCCH monitoring occasions for on demand SIB1, after UE transmits the UL WUS in idle/inactive mode, RAN1 assumes following as a starting point:

- Option 1: One or more type 0 PDCCH monitoring occasions for on demand SIB1 within a time window

  - FFS: How the search space zero configuration is provided (e.g. from  searchSpaceZero  in  MIB  or  from a new search space that is indicated by UL-WUS configuration)

  - FFS: Details of the time window, including at least the starting time and duration

  - FFS: Whether/how to support transmission of on-demand SIB1 with the association with SSB(s) based on a received UL-WUS

Agreement

From RAN1 point of view, the following is feasible. It is up to RAN2 to decide whether/how to support it.

At least for Case 2 (Option 1+B+X) design, a unified configuration format that can support both Option 2 (i.e. a UL-WUS configuration applies to multiple NES cells) and Option 3 (i.e. a UL-WUS configuration applies to a single NES cell).

Agreement

For further study of on-demand SIB1 in idle/inactive mode, on the spatial relationships among PDCCH/PDSCH of on-demand SIB1, SSB, and UL WUS, as UL WUS is using dedicated PRACH resource, it is assumed that spatial relationships among PDCCH/PDSCH of on-demand SIB1, SSB and UL WUS can follow legacy mechanism.

Agreement

For further study of on-demand SIB1 in idle/inactive mode, use the following Table I (from R1-2405106, Ericsson) as a starting point to discuss the required parameters/contents inside the UL WUS configuration.

Table I.

| Purpose | Parameters |  |  |
| --- | --- | --- | --- |
| To which cell does the config applies | NES-CellId | PhysCellId |  |
|  |  | ARFCN-ValueNR |  |
| WUS transmission | SIB1-RequestConfig | ss-PBCH-BlockPower |  |
|  |  | rach-OccasionsSIB1 | Prach-ConfigurationIndex |
|  |  |  | msg1-FDM |
|  |  |  | msg1-FrequencyStart |
|  |  |  | zeroCorrelationZoneConfig |
|  |  |  | preambleReceivedTargetPower |
|  |  |  | preambleTransMax |
|  |  |  | powerRampingStep |
|  |  |  | ra-ResponseWindow |
|  |  |  | ssb-perRACH-Occasion |
|  |  | sib1-RequestPeriod |  |
|  |  | sib1-RequestResource | ra-PreambleStartIndex |
|  |  |  | ra-AssociationPeriodIndex |
|  |  |  | ra-ssb-OccasionMaskIndex |
|  |  | rsrp-ThresholdSSB |  |
|  |  | prach-RootSequenceIndex |  |
|  |  | msg1-SubcarrierSpacing |  |
|  |  | restrictedSetConfig |  |
|  |  | tdd-UL-DL-ConfigurationCommon |  |
|  | frequencyInfoUL | frequencyBandList |  |
|  |  | absoluteFrequencyPointA |  |
|  |  | offsetToCarrier |  |
|  |  | p-Max |  |
|  |  | frequencyShift7p5khz |  |
| SIB1 reception | pdcch-ConfigSIB1 | ssb-SubcarrierOffset |  |
|  |  | controlResourceSetZero |  |
|  |  | searchSpaceZero |  |
| RAR Reception | pdcch-ConfigOD-SIB1-RAR | controlResourceSet |  |
|  |  | monitoringSlotPeriodicityAndOffset |  |
|  |  | Duration |  |
|  |  | monitoringSymbolsWithinSlot |  |
|  |  | aggregationLevels |  |

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, empty load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 39.37% - 47.69% from the following 7 sources

    - R1-2404463, R1-2405595, R1-2405341, R1-2404859, R1-2404507, R1-2405363, R1-2404625

  - One source (R1-2405162) reports 25% NES gain with PRACH configuration index = 152

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 24.6% with one beam of SIB1 PDSCH transmitted in each slot

    - 17.4% with two beams of SIB1 PDSCH transmitted in each slot

  - One source (R1-2404122) reports the following with FDMed SSB and PDSCH of SIB1

    - 9.27%~18.94% NES gain with UL WUS configuration transmitted in legacy SIB on Cell A

    - 0%~9.66% NES gain with UL WUS configuration transmitted in separated SIB on Cell A. The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource.

    - 0% NES gain with Case 1 (Options 1+A+X) and WUS configuration transmitted in separate SIB on the NES cell

    - 13.25% NES gain with Case 1 (Options 1+A+X) and pre-configured for fixed WUS configuration

      - NES cell does not transmit the WUS configuration

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 22.1% - 42.6% from the following 5 sources

    - R1-2404408, R1-2405341, R1-2404758, R1-2405363, R1-2404625

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 16.8% with one beam of SIB1 PDSCH transmitted in each slot

    - 10.9% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 30.43% - 45.79% from the following 4 sources

    - R1-2404463, R1-2404859, R1-2405363, R1-2404625

  - One source (R1-2405162) reports the following NES gain with PRACH configuration index = 152

    - 25% NES gain with Case 3 (Options 2+B+Y) assuming the UE can camp on the NES cell and the SIB1 is only transmitted in a single beam corresponding to the UL WUS beam

    - 23% with Case 2 (Options 1+B+X)

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 21.79% - 33.11% from the following 2 sources

    - R1-2405363, R1-2404625

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 20.5% - 39.34% from the following 8 sources

    - R1-2404463, R1-2403942, R1-2405341, R1-2404224, R1-2404859, R1-2404507, R1-2404561, R1-2405595

  - One source (R1-2404224) reports 11.9% NES gain with medium cell load on Cell A

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 19.31 – 19.39% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 29.19% - 36.87% from the following 3 sources

    - R1-2404463, R1-2403942, R1-2404859

  - One source (R1-2404561) reports 15.87%~20.18% NES gain with PRACH configuration index = 148

- For Cat 1 BS, 4/8 beams, 4% < on-demand SIB1 transmission rate < 24%, the NES gain is 17.98%~24.48% from one source (R1-2404408).

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 160ms SIB1 period (Case C), FR1, empty load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 2.21 – 10.6% from the following 10 sources

  - R1-2405595, R1-2403979, R1-2405341, R1-2404859, R1-2405162, R1-2404122, R1-2404507, R1-2405363, R1-2404625, R1-2404561

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 1.7% - 6.23% from the following 5 sources

    - R1-2403979, R1-2405341, R1-2405363, R1-2404625, R1-2404758

  - One source (R1-2404758) reports 28.9% NES gain with Case 3 where no PRACH and paging is activated for the NES cell

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.75% - 8.48% from the following 6 sources

  - R1-2405106, R1-2405162, R1-2404859, R1-2405363, R1-2404625, R1-2404561

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.01% - 5.8% from the following 3 sources

  - R1-2405106, R1-2405363, R1-2404625

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 3.24% - 7.76% from the following 5 sources

  - R1-2403942, R1-2405341, R1-2404859, R1-2404507, R1-2404561

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 2.96% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 2.5% - 7.44% from the following 3 sources

  - R1-2404561, R1-2403942, R1-2404859

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, low load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 30.56% - 41.8% from the following 5 sources

    - R1-2405595, R1-2405341, R1-2404859, R1-2405363, R1-2404625

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 13.9% with one beam of SIB1 PDSCH transmitted in each slot

    - 7.9% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 16% - 36.2% from the following 5 sources

    - R1-2405247, R1-2405341, R1-2404033, R1-2405363, R1-2404625

  - One source (R1-2404033) reports 10% NES gain with FDMed SSB and PDSCH of SIB1

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 5.2% with one beam of SIB1 PDSCH transmitted in each slot

    - 1.9% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 22.44% - 37.64% from the following 3 sources

    - R1-2404859, R1-2405363, R1-2404625

  - One source (R1-2404122) reports the following with FDMed SSB and PDSCH of SIB1

    - 4.73%~9.64% NES gain with UL WUS configuration transmitted in legacy SIB on Cell A

    - 0%~4.8% NES gain with UL WUS configuration transmitted in separated SIB on Cell A. The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource.

    - 0% NES gain with Case 1 (Options 1+A+X) and WUS configuration transmitted in separate SIB on the NES cell

    - 7% NES gain with Case 1 (Options 1+A+X) and pre-configured for fixed WUS configuration

      - NES cell does not transmit the WUS configuration

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 13.17% - 36.2% from the following 3 sources

    - R1-2404625, R1-2405247, R1-2405363

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 16.8% - 28.77% from the following 6 sources

    - R1-2403942, R1-2405341, R1-2404224, R1-2404859, R1-2404561, R1-2405595

  - One source (R1-2404224) reports 10.5% NES gain with medium cell load on Cell A

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 12.71% – 12.88% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 14.43% - 27.47% from the following 3 sources

  - R1-2404561, R1-2403942, R1-2404859

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 160ms SIB1 period (Case C), FR1, low load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 1.2 – 8.3% from the following 7 sources

    - R1-2405595, R1-2403979, R1-2405341, R1-2404859, R1-2405363, R1-2404625, R1-2404561

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.2% - 5.46% from the following 4 sources

  - R1-2403979, R1-2405341, R1-2405363, R1-2404625

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 2.87% - 7.38% from the following 4 sources

  - R1-2404859, R1-2405363, R1-2404625, R1-2404561

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.97% - 4.48% from the following 2 sources

  - R1-2405363, R1-2404625

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 3.04% - 5.57% from the following 4 sources

  - R1-2403942, R1-2405341, R1-2404859, R1-2404561

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 1.79% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 2.12% - 5.26% from the following 3 sources

  - R1-2404561, R1-2403942, R1-2404859

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, medium load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 9.88% - 14.4% from the following 2 sources

    - R1-2405341, R1-2405363

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 4.5% with one beam of SIB1 PDSCH transmitted in each slot

    - 2.3% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 4.92% - 8.4% from the following 2 sources

    - R1-2405341, R1-2405363

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 1.5% with one beam of SIB1 PDSCH transmitted in each slot

    - 0.5% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 11.5% - 12.88% from one source (R1-2405363)

  - One source (R1-2404122) reports the following with FDMed SSB and PDSCH of SIB1

    - 1.94%~4% NES gain with UL WUS configuration transmitted in legacy SIB on Cell A

    - 0%~1.94% NES gain with UL WUS configuration transmitted in separated SIB on Cell A. The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource.

    - 0% NES gain with Case 1 (Options 1+A+X) and WUS configuration transmitted in separate SIB on the NES cell

    - 2.64% NES gain with Case 1 (Options 1+A+X) and pre-configured for fixed WUS configuration

      - NES cell does not transmit the WUS configuration

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 6%~7.53% from one source (R1-2405363)

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 8.08% - 15.47% from the following 4 sources

  - R1-2405341, R1-2404224, R1-2404561, R1-2403942

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 4.09% – 4.19% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 10.78% - 15.3% from the following 2 sources

  - R1-2404561, R1-2403942

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 160ms SIB1 period (Case C), FR1, medium load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.3 – 2.44% from the following 4 sources

  - R1-2403979, R1-2405341, R1-2405363, R1-2404561

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.1% - 1.3% from the following 3 sources

  - R1-2403979, R1-2405341, R1-2405363

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.24% - 2.26% from the following 2 sources

  - R1-2405363, R1-2404561

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 0.67% - 1.6% from one source (R1-2405363)

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.95% - 4.13% from the following 3 sources

  - R1-2403942, R1-2405341, R1-2404561

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.48% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.44% - 4.02% from the following 2 sources

  - R1-2404561, R1-2403942

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 40ms SIB1 period (Case D), FR1, empty load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 24.51% - 31.3% from the following 6 sources

    - R1-2405595, R1-2405341, R1-2404859, R1-2404507, R1-2405363, R1-2404625

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 16.3% with one beam of SIB1 PDSCH transmitted in each slot

    - 10.6% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 12.4% - 35.5% from the following 5 sources

    - R1-2404408, R1-2405341, R1-2404758, R1-2405363, R1-2404625

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 10.1% with one beam of SIB1 PDSCH transmitted in each slot

    - 6.1% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 19.44% - 28.81% from the following 3 sources

  - R1-2404859, R1-2405363, R1-2404625

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 12.87% - 20.09% from the following 2 sources

  - R1-2405363, R1-2404625

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 11.77% - 25.46% from the following 5 sources

  - R1-2403942, R1-2405341, R1-2404859, R1-2404507, R1-2404561

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 10.69 – 10.74% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 16.46% - 24.24% from the following 2 sources

    - R1-2403942, R1-2404859

  - One source (R1-2404561) reports 8.92%~10.37% NES gain with PRACH configuration index = 148

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 40ms SIB1 period (Case D), FR1, low load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 18.38% - 26.4% from the following 5 sources

    - R1-2405595, R1-2405341, R1-2404859, R1-2405363, R1-2404625

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 8.1% with one beam of SIB1 PDSCH transmitted in each slot

    - 4.3% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 9.84% - 17.41% from the following 3 sources

    - R1-2405341, R1-2405363, R1-2404625

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 2.8% with one beam of SIB1 PDSCH transmitted in each slot

    - 1% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 13.36% - 23.68% from the following 3 sources

  - R1-2404859, R1-2405363, R1-2404625

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 7.27% - 15.59% from the following 2 sources

  - R1-2405363, R1-2404625

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 10.48% - 17.72% from the following 4 sources

  - R1-2403942, R1-2405341, R1-2404859, R1-2404561

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 7.01% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 7.7% - 17.05% from the following 3 sources

  - R1-2403942, R1-2404859, R1-2404561

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 40ms SIB1 period (Case D), FR1, medium load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 5.07% - 7.8% from the following 2 sources

    - R1-2405341, R1-2405363

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 2.4% with one beam of SIB1 PDSCH transmitted in each slot

    - 1.2% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 2.46% - 4.4% from the following 2 sources

    - R1-2405341, R1-2405363

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 0.7% with one beam of SIB1 PDSCH transmitted in each slot

    - 0.3% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 5.62~7.01% from one source (R1-2405363)

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 2.77% - 3.85% from one source (R1-2405363)

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 4.08%~10.1% from the following 3 sources

  - R1-2403942, R1-2405341, R1-2404561

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 2.02%~2.09% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 5.61% - 9.94% from the following 2 sources

  - R1-2404561, R1-2403942

Agreement

For the evaluation of NES loss on cell A, the following is observed

  - NES loss of 0%~0.98% from the following 15 sources with UL WUS configuration transmitted in legacy SIB on Cell A (with different assumptions on SIB periodicity and cell loading):

    - R1-2404408, R1-2404463, R1-2403942, R1-2403979, R1-2405341, R1-2404224, R1-2404859, R1-2404758, R1-2405162, R1-2404122, R1-2404507, R1-2404033, R1-2405363, R1-2404625, R1-2404561 (for Case 2)

  - One source (R1-2405106) reports for the case with UL WUS configuration transmitted in separated SIB on Cell A where the separated SIB is TDMed with legacy SIB and transmitted in a 320ms period using dedicated resource.

    - 0.62%~2.65% NES loss for Case 2 (Options 1+B+X) with empty load

    - 0.73%~3.20% NES loss for Case 3 (Options 2+B+Y) with empty load

  - One source (R1-2404122) reports

    - 2.95%(medium load)~15.49%(empty load) NES loss with UL WUS configuration transmitted in separated SIB on Cell A

      - The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource.

    - 1.28%(medium load)~3.62%(empty load) NES loss with UL WUS configuration transmitted in legacy SIB on Cell A

Conclusion

For further study of on-demand SIB1 in idle/inactive mode, enabling or disabling specific operations (e.g. paging, RACH receiving, OSI request …) of the NES cell with on-demand SIB1 is up to RAN2.

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, 8 beams, on-demand SIB1 transmission rate > 30%

- For Cat 1 BS, empty load, the NES gain is 37.77% from one source (R1-2404463)

- For Cat 2 BS, empty load,

  - the NES gain is 8.9% - 28.71% from the following 3 sources

    - R1-2404224, R1-2404561, R1-2404463

  - The NES gain is 1.47% from one source (R1-2404561) with 75% on-demand SIB1 transmission rate

- For Cat 2 BS, low load,

  - the NES gain is 7.9% - 14.65% from the following 2 sources

    - R1-2404224, R1-2404561

  - The NES gain is 0.42% from one source (R1-2404561) with 75% on-demand SIB1 transmission rate

- For Cat 2 BS, medium load,

  - the NES gain is 6.2% - 10.4% from the following 2 sources

    - R1-2404224, R1-2404561

  - The NES gain is 0.12% from one source (R1-2404561) with 75% on-demand SIB1 transmission rate

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 40ms SIB1 period (Case D), FR1, 8 beams, on-demand SIB1 transmission rate > 30%

- For Cat 2 BS, empty/low/medium load, the NES gain is 3.77% - 5.97% from one source (R1-2404561)

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 160ms SIB1 period (Case C), FR1, 8 beams, on-demand SIB1 transmission rate > 30%

- For Cat 1/Cat 2 BS, empty/low/medium load, the NES gain is 0.31%~3.33% from the following 2 sources

  - R1-2404561, R1-2404122

Agreement

For the evaluation of the energy consumption to transmit WUS configuration on NES Cell:

- One source (R1-2405595) reports that for broadcasting WUS configuration using DCI occupying 2 symbols per SSB beam) on NES cell every 80 ms on CORESET#0 (48 RBs):

  - For BS category 1, empty/low load, the energy consumption is increased by 6.26%~8.51% over SSB transmission only. The energy saving gain with Case 1 is 40.52% / 32.41% for empty/low load over the baseline with 8 beams, SIB-1 period 20ms (baseline), 0% SIB-1 transmission rate (for on demand SIB1).

  - For BS category 2, empty/low load, the energy consumption is increased by 1.55%~2.11% over SSB transmission only. The energy saving gain with Case 1 is 28.25% / 23.03% for empty/low load over the baseline with 8 beams, SIB-1 period 20ms (baseline), 0% SIB-1 transmission rate (for on demand SIB1).