3GPP TSG-RAN WG2 #127bis	Tdoc R2-2408388

Hefei, China, 14th-18th October 2024

Agenda Item:	7.25.1

Source:	Ericsson

Title:	On ATG timing advance reporting procedure

Document for:	Discussion, Decision

# 1	Introduction

In RAN2#127, clarification for ATG timing advance reporting procedure was discussed:

R2-2407530	Clarifications for ATG timing advance reporting procedures	Samsung	discussion	Rel-18	NR_ATG-Core

[AT127][024][ATG] Clarification CR (Samsung)

Intended outcome: agree to CR

Deadline:  08-23-24

R2-2407806	Report of [AT127][024][ATG] Clarification on SCS for Timing Advance reporting	Samsung	discussion	Rel-18	NR_ATG

=>	The issue is postponed to next meeting

=>	Noted

In that offline, the following conclusions were attempted:

Proposal 1: SCS of offsetThresholdTA is assumed to be based on the SCS of the active BWP when the UE is configured with Timing Advance reporting. Agree CR R2-2406604.

Proposal 2: Agree MAC CR with the following change:

For ATG in FR1, the Timing Advance field indicates the least integer number of symbols greater than or equal to the Timing Advance value (see TS 38.211 [8], clause 4.3.1). The symbol duration is based on the subcarriers spacing of the active BWP the UE is currently configured with. In this release of the specification, only 15 kHz and 30 kHz SCS are applicable and only values 1 … 56 are used.

- Proposal 3: TAR MAC CE reporting in another BWP than it was triggered in is considered to handled via network implementation.

However, there is one company which promotes for simpler solution to agree a fixed SCS 30kHz, hence the discussion was postponed to next meeting.

In this contribution, we present our view on the matter.

# 2	Discussion

The root cause of the issue which lead to the proposals cited in the previous section was that the current field description of offsetThresholdTA-r18 in RRC specification only states that the value is in the unit of symbols, which could lead to problems when there is BWP switching that involves changing the SCS. Meanwhile, the TAR MAC CE has specified a specific SCS to be used to interpret the TAR value. As a result, the question was how to update the field description of offsetThresholdTA-r18 to make it robust to BWP switching and whether any update is needed for the TAR MAC CE.

In our view, the end goal of any specs update is there should be no ambiguity in interpreting the values signalled by offsetThresholdTA-r18 and by the TAR MAC CE at the UE and the NW. There are different ways to achieve that goal, but we see two main options:

- Option 1: Keep the description of TAR MAC CE (in TS 38.321) intact, only update field description of offsetThresholdTA-r18 (in TS 38.331). In this case, since the TAR MAC CE text says that the symbol length is based on the SCS being used when the MAC CE is transmitted by the UE, the most coherent approach would be to apply the same principle for offsetThresholdTA-r18. That means, the symbol length signalled by offsetThresholdTA-r18 shall be based on the SCS of the active BWP at the time the TAR configuration is sent to the UE. In other words, always based on the SCS of the active BWP when the respective signalling is sent. Technically, other options such as SCS of the currently active BWP or the initial BWP also work, but they will lead to different ways of interpreting TAR MAC CE and offsetThresholdTA-r18.

- Option 2: Update the description of both TAR MAC CE and offsetThresholdTA-r18. In this case, it would be convenient to use an explicit SCS, either 15 kHz and 30 kHz. In case RAN2 adopts this approach, we slightly prefer 30 kHz since it provides better granularity for the TA. However, one drawback of this approach is that the specs might need to be updated in the future if later on some other SCS like 60 kHz is supported for ATG.

Updating only the field description of offsetThresholdTA-r18 requires least effort than also updating TAR MAC CE.

Among the proposed solutions for updating offsetThresholdTA-r18, the solution based on the SCS of the active BWP when the UE is configured with Timing Advance reporting is most coherent with the current TAR MAC CE description.

Last but not least, since the most important thing is to conclude on this matter, we propose according to the majority view expressed in [1]:

Keep the TAR MAC CE description as it is now in TS 38.321 and update the field description of offsetThresholdTA-r18.

SCS of offsetThresholdTA is assumed to be based on the SCS of the active BWP when the UE is configured with Timing Advance reporting.

Below is the change as proposed in R2-2406604:

#### –	TAR-Config

The IE TAR-Config is used to configure Timing Advance reporting in non-terrestrial networks and ATG network.

TAR-Config information element

-- ASN1START

-- TAG-TAR-CONFIG-START

TAR-Config-r17 ::=                      SEQUENCE {

offsetThresholdTA-r17               ENUMERATED {ms0dot5, ms1, ms2, ms3, ms4, ms5, ms6 ,ms7, ms8, ms9, ms10, ms11, ms12,

ms13, ms14, ms15, spare13, spare12, spare11, spare10, spare9, spare8, spare7,

spare6, spare5, spare4, spare3, spare2, spare1}          OPTIONAL,    -- Need R

timingAdvanceSR-r17                 ENUMERATED {enabled}                                                OPTIONAL,    -- Need R

...

}

TAR-Config-r18 ::=                      SEQUENCE {

offsetThresholdTA-r18               INTEGER (1..56)                                                     OPTIONAL,    -- Need R

timingAdvanceSR-r18                 ENUMERATED {enabled}                                                OPTIONAL,    -- Need R

...

}

-- TAG-TAR-CONFIG-STOP

-- ASN1STOP

TAR-Config field descriptions

offsetThresholdTA Offset for TA reporting as specified in TS 38.321 [3]. Network only configures this parameter for MCG. For ATG, network only configures offsetThresholdTA-r18, which is in unit of symbols. For ATG, the offset is based on the SCS of the active BWP where the configuration was received and does not change with BWP switching.

timingAdvanceSR Used to configure whether a Timing Advance report may trigger a Scheduling Request as specified in TS 38.321 [3].

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Updating only the field description of offsetThresholdTA-r18 requires least effort than also updating TAR MAC CE.

Observation 2	Among the proposed solutions for updating offsetThresholdTA-r18, the solution based on the SCS of the active BWP when the UE is configured with Timing Advance reporting is most coherent with the current TAR MAC CE description.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Keep the TAR MAC CE description as it is now in TS 38.321 and update the field description of offsetThresholdTA-r18.

Proposal 2	SCS of offsetThresholdTA is assumed to be based on the SCS of the active BWP when the UE is configured with Timing Advance reporting.

# References

R2-2407806, Report of [AT127][024][ATG] Clarification on SCS for Timing Advance reporting