3GPP TSG-RAN WG2 Meeting #127bis		     R2-2408390

Hefei, China: 14th October – 18th October 2024

Source:	Qualcomm Incorporated

Title:	On LCM for UE-sided Models for Beam Management

Agenda Item:	8.1.2.2 LCM for UE-sided model  for Beam Management use case

Document for:	Discussion and Decision

# Introduction & Background

The Rel-18 study on AI/ML Air Interface [1] has been concluded. The study has been captured in the Technical Report TR 38.843 [2]. Based on the Rel-18 study outcome, at RAN #102, a new Work Item on artificial intelligence/machine learning for NR air interface has been approved in [3].

This paper discusses the “LCM of UE-sided models for beam management use case” considering recent RAN1/RAN2 agreements.

# Discussion on LCM of UE-sided Models for Beam Management

## 2.1 Signaling of Associated ID and RS Configuration

In RAN2#127 [4], RAN2 agreed on the following agreement for determining the applicable functionalities at the UE,

Agreements on procedures - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

In the RAN1#118 meeting [5], RAN1 agreed to support associated ID, for the UE-side model in beam management as captured below

Agreement For UE sided model in beam management, support associated ID

Note that the above RAN1 / RAN2 agreements ensure consistency during model training and the inference, i.e. to ensure that AI/ML models are used in the network side additional condition(s) for which they are trained. Therefore, the associated ID and RS configuration should be configured/signaled during both UE side data collection and when UE is configured with inference. This section discussed the procedures for configuring/signaling the network-side additional condition and RS Configuration for UE-side data collection, how a common procedure for signaling configuring/signaling for training data collection and inference, and the need for unique associated ID at least within a PLMN.

Observation 1:  RAN1 has agreed to support associated IDs for the UE-sided model for the beam management use case.

#### 2.1.1	Configuration/signaling of Associated ID and RS Configuration for UE side Training Data Collection

A UE should be able to collect data for model training on the UE side in both of these scenarios

- UE does not support inference for beam management use cases,

- UE supports inference for beam management use cases, but UE is not configured with inference for beam management.

- UE supports inference for beam management use cases and UE is configured with inference for beam management.

Proposal 1: Data collection at the UE for UE-side model training is supported in the following scenarios

- UE does not support inference for beam management use cases and UE wants to performe the data collection for model training on the UE side

- UE supports inference for beam management use cases but UE is not configured with inference for beam management and UE wants to performe the data collection for model training/retraining/finetuning on the UE side.

- UE supports inference for beam management use cases and UE is configured with inference for beam management, and UE wants to performe the data collection for model training/retraining/finetuning on the UE side.

We will discuss the configuration/signaling of associated ID and RS configuration for UE side training data collection in the two scenarios in Proposal 1 separately in this section.

##### 2.1.1.1  Configuration/signaling of Associated ID and RS Configuration for UE side Training Data Collection when UE does not Support Inference for Beam Management

As discussed in Proposal 1, data collection at the UE for UE-side model training should be supported even if UE does not support inference for beam management use cases. Consider the procedure described in Fig. 2.1.1.1-1 for requesting [by the UE] the associated ID and RS configuration, and network signaling [by serving cell] for providing the associated ID and RS configuration for supporting UE side data collection for model training.

Fig. 2.1.1.1-1: Signaling/procedure for requesting [from UE] and Providing [by serving cell] associated IDs and RS configuration for model training.

Proposal 2: If the UE does not support inference for beam management

- Consider Fig. 2.1.1.1-1 for configuration/signaling of associated ID and RS configuration for UE side training data collection when UE does not support inference for beam management.

As described in Fig. 2.1.1.1-1, the triggers for data collection at the UE for UE-side model training should be left up to the UE implementation, i.e., UE may prefer to collect the training data when UE is not overloaded, UE has memory/computational resources available, etc. Therefore, the interest in receiving associated ID and RS configuration for measurements should be UE initiated and there is no need for UE capability signaling for data collection for UE-side model training. The UE should be allowed to send UE assistance information for requesting the network to provide the associated ID and RS configuration for model training.

Proposal 3: If the UE does not support inference for beam management

- The UE is allowed to send UE assistance information for requesting the network to provide the associated ID and RS configuration for model training.

Proposal 4: If the UE does not support inference for beam management

- Triggers for UE to send UE assistance information for requesting the network to provide the associated ID and RS configuration for model training are left up to UE implementation.

##### 2.1.1.2  Configuration/signaling of Associated ID and RS Configuration for UE side Training Data Collection when UE supports inference for beam management use cases but UE is not configured with inference for beam management

As discussed in Proposal 1, data collection at the UE for UE-side model training should be supported if UE supports inference for beam management use cases but UE is not configured with inference for beam management. Proposal 2, Proposal 3, and Proposal 4 should apply to this case too.

Proposal 5: When UE supports the inference for beam management but UE is not configured with inference for beam management

- Proposal 2, Proposal 3, and Proposal 4 should apply.

##### 2.1.1.3  Configuration/signaling of Associated ID and RS Configuration for UE side Training Data Collection when UE supports inference for beam management use cases and UE is configured with inference for beam management

When the UE is configured with inference, the UE may be configured with RS configuration for inference and monitoring. Also, the network may signal the associated ID to the UE upon the change in associated IDs for determining the applicable functionalities. Therefore, there may not be a need for UE to request associated IDs. In this case, UE may still be allowed to request RS Configuration for training, as described in Fig. 2.1.1.3-1.

Fig. 2.1.1.3-1: Signaling/procedure for requesting [from UE] and Providing [by serving cell] RS configuration for model training.

Observation 2: When the UE is configured with inference, the UE may be configured with RS configuration for inference and monitoring. The network may also signal the associated ID to the UE upon the change in associated IDs for determining the applicable functionalities.

Observation 3: When the UE is configured with inference, the RS configuration for training may still be different from the RS configuration for inference and RS configuration for monitoring.

Proposal 6: When UE is configured with inference, the UE should still be allowed

- To request RS configuration for training using UE Assistance Information.

### 2.1.2 Indicating Associated ID for Training Data Collection and Inference

As discussed previously, RAN1 agreed that for UE sided model in beam management, support associated IDs. For the training data collection, the associated ID is signaled from the network to the UE to achieve consistency between training and inference [5]. In the RAN2#127 meeting [4], RAN discussed in the associated ID is provided for determining the applicable functionalities. We believe that associated IDs need to be signaled from the network to the UE for training data collection, therefore, we do not need separate procedures where UE signals applicable functionalities without considering network-side additional conditions (i.e., associated ID). Therefore, consider a common solution where the associated ID is signaled by the serving cell to the UE for training data collection and inference operations (for determining applicable functionality and inference configuration [if performed separately]).

Proposal 7: RAN2 is requested to consider a common signaling framework for providing associated ID to the UE from the network for training data collection and inference operations (for determining applicable functionality and inference configuration [if performed separately]).

### 2.1.3 Uniqueness of Associated IDs within A PLMN

In the RAN1#117 meeting [10], RAN1 had the following working assumption

Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell. - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study).

Observation 4: RAN1 assumed that NW-side additional conditions with the same associated ID are consistent at least within a cell.

While consistency within a cell is useful for UE-side for model development and runtime LCM (e.g., model selection/switching/activation/deactivation), it is not scalable. If the associated ID at different cells represents different network-side additional conditions, then it will be hard for the UE side to generalize the model across multiple cells. This poses challenges not only on UE-side model development but also on runtime LCM. Without sufficient generalization, the UE side has to download/get the model frequently causing unnecessary interruptions. Therefore, the uniqueness of associated IDs extended to the PLMN. How the uniqueness of the associated IDs within a PLMN is achieved can be left up to OAM implementation.

Observation 5: The uniqueness of associated ID within a single cell is not scalable, as it will create challenges for the UE side to generalize models across multiple cells.

Observation 6: The lack of uniqueness of the associated ID poses challenges not only on UE-side model development but also on runtime LCM.

Observation 7: Without sufficient generalization, the UE side has to download/get the model frequently causing unnecessary interruptions.

Proposal 8: NW-side additional conditions with the same associated ID are consistent within a PLMN.

Proposal 9: How the uniqueness of the associated IDs within a PLMN is achieved can be left up to OAM implementation.

## 2.2   Monitoring and Life Cycle Management Signaling

### 2.2.1 Monitoring of UE-side models

In RAN1#117 meeting [6], RAN1 agreed on following for performance monitoring

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

For option 2, RAN1 agreed on following in meeting RAN1#118 [5]

Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

Observation 8: RAN1 is still discussing different options and respective alternatives for performance monitoring.

Proposal 10: Wait for RAN1 progress before discussing signaling for reporting performance monitoring.

### 2.2.1 Life Cycle Management Signaling

In the RAN2#125bis meeting [7], RAN2 agreed the following

Agreements:

1	For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline.  The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach).

2	“UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19

In this section, we discuss procedures for supporting

- “Network decision, network initiated” AI/ML Management,

- “UE autonomous, decision reported to the network”, AI/ML Management, and

- “Network decision, UE-initiated” (i.e. proactive approach)” AI/ML Management.

#### 2.1.3.1	“Network decision, network initiated” AI/ML Management

Note that the network’s AI/ML management decision can be based on several factors, e.g., network issues, performance issues, etc. The AI/ML management decision at the network can be due to

- Performance issue,

- Change in additional condition (or network configuration) at the network, or

- Temporary network overload/failure to support AI/ML-based feature/feature groups.

Observation 9: The network’s AI/ML management decision can be based on several factors, e.g., network issues, performance issues, etc. The AI/ML management decision at the network can be due to

- Performance issue,

- Change in additional condition (or network configuration) at the network, or

- Temporary network overload/failure to support AI/ML-based feature/feature groups.

To facilitate appropriate action (for example, model retraining, etc.), the network should signal the cause of management instruction.

Proposal 11: To facilitate appropriate action (for example, model retraining, etc.), the network should signal the cause of AI/ML management together with the AI/ML management signal.

#### 2.1.3.2	“UE autonomous, decision reported to the network”, AI/ML Management

The AI/ML Management decisions at the UE can be due to

- Performance issue, or

- Change in applicable functionalities, due to changes in

  - Model availability at the UE, or

  - Additional conditions at the UE or network.

Observation 10: The AI/ML management decisions at the UE can be due to

- Performance issue, or

- Change in applicable functionalities, due to changes in

  - Model availability at the UE, or

  - Additional conditions at the UE or network.

Proposal 12: To facilitate “UE autonomous, decision reported to the network”, AI/ML Management, the network provides assistance information (e.g., network side additional condition) reporting from the network to the UE.

Proposal 13: UE reports the management decision (without cause) to the network upon taking the AI/ML management decisions.

#### 2.1.3.3	“Network decision, UE-initiated” (i.e. proactive approach)” AI/ML Management

In some cases, when the AI/ML functionalities were deactivated, or fallback to legacy procedures based on “Network decision, network initiated” AI/ML Management, or “UE autonomous, decision reported to the network”, AI/ML Management, the UE can request network to activate a functionality. In our understanding, as the functionality is already configured, the UE can evaluate the applicable functionalities and request activation of the application functionality.

Proposal 14: When the AI/ML functionalities were deactivated, or fallback to legacy procedures based on “Network decision, network-initiated” AI/ML Management, or “UE autonomous, decision reported to the network”, AI/ML Management,

- The UE can evaluate the applicable functionalities and request activation of the application functionality

# Conclusion

Observation 1:  RAN1 has agreed to support associated IDs for the UE-sided model for the beam management use case.

Proposal 1: Data collection at the UE for UE-side model training is supported in the following scenarios

- UE does not support inference for beam management use cases and UE wants to performe the data collection for model training on the UE side

- UE supports inference for beam management use cases but UE is not configured with inference for beam management and UE wants to performe the data collection for model training/retraining/finetuning on the UE side.

- UE supports inference for beam management use cases and UE is configured with inference for beam management, and UE wants to performe the data collection for model training/retraining/finetuning on the UE side.

Proposal 2: If the UE does not support inference for beam management

- Consider Fig. 2.1.1.1-1 for configuration/signaling of associated ID and RS configuration for UE side training data collection when UE does not support inference for beam management.

Proposal 3: If the UE does not support inference for beam management

- The UE is allowed to send UE assistance information for requesting the network to provide the associated ID and RS configuration for model training.

Proposal 4: If the UE does not support inference for beam management

- Triggers for UE to send UE assistance information for requesting the network to provide the associated ID and RS configuration for model training are left up to UE implementation.

Proposal 5: When UE supports the inference for beam management but UE is not configured with inference for beam management

- Proposal 2, Proposal 3, and Proposal 4 should apply.

Observation 2: When the UE is configured with inference, the UE may be configured with RS configuration for inference and monitoring. The network may also signal the associated ID to the UE upon the change in associated IDs for determining the applicable functionalities.

Observation 3: When the UE is configured with inference, the RS configuration for training may still be different from the RS configuration for inference and RS configuration for monitoring.

Proposal 6: When UE is configured with inference, the UE should still be allowed

- To request RS configuration for training using UE Assistance Information.

Proposal 7: RAN2 is requested to consider a common signaling framwork for providing associated ID to the UE from the network for training data collection and inference operations (for determining applicable functionality and inference configuration [if performed separately]).

Observation 4: RAN1 assumed that NW-side additional conditions with the same associated ID are consistent at least within a cell.

Observation 5: The uniqueness of associated ID within a single cell is not scalable, as it will create challenges for the UE side to generalize models across multiple cells.

Observation 6: The lack of uniqueness of the associated ID poses challenges not only on UE-side model development but also on runtime LCM.

Observation 7: Without sufficient generalization, the UE side has to download/get the model frequently causing unnecessary interruptions.

Proposal 8: NW-side additional conditions with the same associated ID are consistent within a PLMN.

Proposal 9: How the uniqueness of the associated IDs within a PLMN is achieved can be left up to OAM implementation.

Observation 8: RAN1 is still discussing different options and respective alternatives for performance monitoring.

Proposal 10: Wait for RAN1 progress before discussing signaling for reporting performance monitoring.

Observation 9: The network’s AI/ML management decision can be based on several factors, e.g., network issues, performance issues, etc. The AI/ML management decision at the network can be due to

- Performance issue,

- Change in additional condition (or network configuration) at the network, or

- Temporary network overload/failure to support AI/ML-based feature/feature groups.

Proposal 11: To facilitate appropriate action (for example, model retraining, etc.), the network should signal the cause of AI/ML management together with the AI/ML management signal.

Observation 10: The AI/ML management decisions at the UE can be due to

- Performance issue, or

- Change in applicable functionalities, due to changes in

  - Model availability at the UE, or

  - Additional conditions at the UE or network.

Proposal 12: To facilitate “UE autonomous, decision reported to the network”, AI/ML Management, the network provides assistance information (e.g., network side additional condition) reporting from the network to the UE.

Proposal 13: UE reports the management decision (without cause) to the network upon taking the AI/ML management decisions.

Proposal 14: When the AI/ML functionalities were deactivated, or fallback to legacy procedures based on “Network decision, network-initiated” AI/ML Management, or “UE autonomous, decision reported to the network”, AI/ML Management,

- The UE can evaluate the applicable functionalities and request activation of the application functionality.

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec. 2021

- TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18),” Dec. 2023

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #102, Dec. 2023

- RAN2#127 meeting report

- RAN1#118 meeting report

- RAN1#117 meeting report