3GPP TSG-RAN WG2 Meeting #127bis		     R2-2408391

Hefei, China: 14th October – 18th October 2024

Source:	Qualcomm Incorporated

Title:	On Network Side Data Collection

Agenda Item:	8.1.3 NW Side Data Collection

Document for:	Discussion and Decision

# Introduction & Background

The Rel-18 study on AI/ML Air Interface [1] has been concluded. The study has been captured in the Technical Report TR 38.843 [2]. Based on the Rel-18 study outcome, at RAN #102, a new Work Item on artificial intelligence/machine learning for NR air interface has been approved in [3].

This contribution paper discusses the data collection for the network side model training for beam management and positioning enhancements use cases.

# Data Collection for Network Side Models

This section discusses the following:

- Type of data required to be collected for NW-sided model and UE-sided model (common to NW-sided and different),

- Data collection for network-side model training for beam management use case, and

- Data collection for network-side model training for positioning enhancements use case.

## 2.1   Type of Data Required for NW-side model and UE-side models

In LS replies R2-2309435 and R2-2311720 from RAN1 on data collection requirements and assumptions, RAN1 provided the list of data content for different use cases. These data contents apply to both network-side model training and UE-side model training. RAN1 further replied in the LS reply R2-2311720 that there may be further information for training not included in the tables.

Observation 1: In LS replies R2-2309435 and R2-2311720 from RAN1 on data collection requirements and assumptions, RAN1 provided the list of data content for different use cases that apply to both network-side model training and UE-side model training.

Observation 2: RAN1 further replied in LS replies R2-2309435 and R2-2311720 that there may be further information for training not included in the tables.

In both RAN1 and RAN2, there have been significant discussions on the types of data required for model training on the UE side and network side. For example, RAN2 has previously discussed different flavors/granularity of RSRP measurements that can be used for UE side training. Furthermore, RAN2 has also discussed the need for additional data collection at the UE for UE-side offline engineering. Considering all the previous discussions, we believe that further discussions on the type of data required to be collected for the NW-sided model and UE-sided model (common to NW-sided and different) may not be desired and will be a waste of both RAN2 and RAN1 TUs.

Proposal 1: Based on prior discussions in RAN2 and LS R2-2309435 and R2-2311720, the data collection for network-side model training and UE-side model training differ by

- Additional data that may be useful for UE-side model training, and

- Triggers for data collection. For UE side data collection, the triggers should be left to UE implementation.

## 2.2   Data Collection for Network-side Models for the Beam Management

In the RAN2#126 meeting [4], RAN2 made the following agreements for the network-side model training for the beam management use case:

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature. FFS how to handle case when single RRC message is not sufficient. FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting. FFS whether and what event-based reporting is supported and FFS on network request reporting

During the RAN2#126 meeting [4], the events for data collection for network-side model training were discussed briefly, however, no agreements were made on the events definitions for data collection/reporting. Furthermore, RAN2 agreed to use immediate MDT for OAM-centric data collection.

In the RAN2#127 meeting [5], RAN2 further agreed on the following for the beam management use case.

Agreements - As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is supported. - UE stores the logged training data at the AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. This is across all use cases - When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging. - Measurements for data collection purposes and logging based can be controlled based on power state of the UE. It is up to UE implementation how the UE determines power state. FFS whether the UE stops autonomously or if it reports to the network . - FFS whether AS buffer event based reporting is supported. FFS if we send availability indication or full report if it is supported - FFS on event based data collection/logging - On-demand request from the network is supported. FFS details on signalling

Considering the above agreements in RAN2#126 [4] and RAN2#127 [5] meetings, we discuss the different FFS issues associated with data collection configuration and data collection reporting in this section.

### 2.2.1	Discussion on FFS Issues on Data Collection Configuration

In the RAN2, the following are FFS issues associated with data collection configuration for network-side model training,

- Whether UE can be configured with multiple RS configurations for network-side model training,

- Whether dynamic activation/deactivation is supported, and

- Whether event-based data collection/logging is supported.

Note that the performance of AI/ML functionalities/models depends on the quality/distribution of the training data. For example, with the periodic data collection, the network maybe able to collect the training data across different beams of the serving cells. However, if a UE is static and configured with periodic data collection, then the collected training data may be redundant. Note that when redundant data is used for training, it may create significant biases. One solution may be that redundancy is removed during the training by the NW-side training server. But, this will require huge number of training data to be collected at the UE, as after the removing the reducdency the network-side may end-up with significantly low amount of meaningful training data. Therefore, the following issues may happen with periodic data collection:

- Huge amount of training data collection at the UEs,

- Significantly high redundancy causing biases during the training, if not handled by the training server properly,

- Would be hard to get a representative sample of events.

Observation 3: Periodic data collection has following issues,

- Requires huge amount of training data collection at the UEs, as there may exist significantly high redundancy,

- Network side training server needs to handled redundancy in the training data, otherwise, it may cause biases in the trained models,

- Would be hard to get representative samples of events.

Another possible method to collect the meaningful training data is that UE is configured with events for training data colletion and logging. However, if the UE is configured with events, then the collected training data may be biased towards those events. The biases in the training data may result in poor performance of AI/ML models. In some case such biases may be desired, as training data is expected to capture events properly, for example, if the AI/ML models are designed for detecting beam failure or maing bema switching decisions. While in another case, such biases in the training data may not be desired, for example, if AI/ML models are designed for predicting radio measurements (e.g., RSRP prediction on SSBs/CSI-RSs). Therefore, the training data collection should be performed with the AI/ML output objectives.

As described in Fig. 2.2.1-1, for the L1-RSRP predictions of top-1/top-K beams, the training data should be independent and identically distributed (i.i.d) across beams. We need a solution that alleviates/minimizes the redudency in the data collection and captures a good representation across beams, across beam center vs beam center, and others. This can be achieved by introducing randomness in the training data logging/reporting.

Fig. 2.2.1-1: An example of training data collection where the training data is distributed (equally) across the beams, across the beam center beam edge, etc.

This section further discusses FFS issues (discussed above) associated with data collection configuration.

#### 2.2.1.1	 On Supporting Multiple RS Configuration

In the legacy CSI framework, CSI-ResourceConfig indicates the RS configuration, and CSI-ReportConfig is provided for configuring the CSI report. From TS 38.331 [6], we can see that up to 48 CSI-ReportConfigurations can be provided. Similarly, up to 112 RS Configurations can be provided. Therefore, following the legacy CSI report framework, multiple CSI RS configurations and reporting/logging configurations can be provided.

Observation 4: In the legacy CSI framework, UE can be configured with up to 112 RS configurations and 48 CSI report configurations.

Proposal 2: For the training data collection, UE can be configured with multiple CSI RS configurations and CSI report configurations.

–	CSI-ReportConfig

The IE CSI-ReportConfig is used to configure a periodic or semi-persistent report sent on PUCCH on the cell in which the CSI-ReportConfig is included or to configure a semi-persistent or aperiodic report sent on PUSCH triggered by DCI received on the cell in which the CSI-ReportConfig is included (in this case, the cell on which the report is sent is determined by the received DCI). See TS 38.214 [19], clause 5.2.1.

CSI-ReportConfig information element

-- ASN1START

-- TAG-CSI-REPORTCONFIG-START

CSI-ReportConfig ::=                SEQUENCE {

reportConfigId                          CSI-ReportConfigId,

/* omitted

-- TAG-CSI-REPORTCONFIG-STOP

-- ASN1STOP

–	CSI-ReportConfigId

The IE CSI-ReportConfigId is used to identify one CSI-ReportConfig.

CSI-ReportConfigId information element

-- ASN1START

-- TAG-CSI-REPORTCONFIGID-START

CSI-ReportConfigId ::=              INTEGER (0..maxNrofCSI-ReportConfigurations-1)

-- TAG-CSI-REPORTCONFIGID-STOP

-- ASN1STOP

–	CSI-ResourceConfig

The IE CSI-ResourceConfig defines a group of one or more NZP-CSI-RS-ResourceSet, CSI-IM-ResourceSet and/or CSI-SSB-ResourceSet.

CSI-ResourceConfig information element

-- ASN1START

-- TAG-CSI-RESOURCECONFIG-START

CSI-ResourceConfig ::=      SEQUENCE {

csi-ResourceConfigId        CSI-ResourceConfigId,

csi-RS-ResourceSetList      CHOICE {

nzp-CSI-RS-SSB              SEQUENCE {

nzp-CSI-RS-ResourceSetList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need R

csi-SSB-ResourceSetList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-SSB-ResourceSetId  OPTIONAL  -- Need R

},

csi-IM-ResourceSetList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId

},

bwp-Id                      BWP-Id,

resourceType                ENUMERATED { aperiodic, semiPersistent, periodic },

...,

[[

csi-SSB-ResourceSetListExt-r17      CSI-SSB-ResourceSetId                                                  OPTIONAL  -- Need R

]]

}

-- TAG-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

–	CSI-ResourceConfigId

The IE CSI-ResourceConfigId is used to identify a CSI-ResourceConfig.

CSI-ResourceConfigId information element

-- ASN1START

-- TAG-CSI-RESOURCECONFIGID-START

CSI-ResourceConfigId ::=            INTEGER (0..maxNrofCSI-ResourceConfigurations-1)

-- TAG-CSI-RESOURCECONFIGID-STOP

-- ASN1STOP

6.4	RRC multiplicity and type constraint values

-- ASN1START

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-START

/*omitted

maxNrofCSI-ReportConfigurations         INTEGER ::= 48      -- Maximum number of report configurations

maxNrofCSI-ResourceConfigurations       INTEGER ::= 112     -- Maximum number of resource configurations

/*omitted

maxNrofNZP-CSI-RS-ResourceSetsPerConfig INTEGER ::= 16      -- Maximum number of resource sets per resource configuration

/*omitted

maxNrofCSI-SSB-ResourceSetsPerConfig    INTEGER ::= 1       -- Maximum number of CSI SSB resource sets per resource configuration

/*omitted

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-STOP

-- ASN1STOP

#### 2.2.1.2	On Supporting Dynamic Activation/Deactivation

From TS 38.214 [7], we have the following,

- The Reporting configuration for CSI can be aperiodic (using PUSCH), periodic (using PUCCH), or semi-persistent (using PUCCH, and DCI activated PUSCH).

- The CSI-RS Resources can be periodic, semi-persistent, or aperiodic.

Furthermore, Table 5.2.1.4-1 in TS 38.214 illustrates supported combinations of CSI reporting configuration and CSI-RS Resource configuration (also copied below).

| CSI-RS Configuration | Periodic CSI Reporting | Semi-Persistent CSI Reporting | Aperiodic CSI Reporting |
| --- | --- | --- | --- |
| Periodic CSI-RS | No dynamic triggering/activation | For reporting on PUCCH, the UE receives an activation command, as described in clause 6.1.3.16 of [10, TS 38.321]; for reporting on PUSCH, the UE receives triggering on DCI | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |
| Semi-Persistent CSI-RS | Not Supported | For reporting on PUCCH, the UE receives an activation command, as described in clause 6.1.3.16 of [10, TS 38.321]; for reporting on PUSCH, the UE receives triggering on DCI | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |
| Aperiodic CSI-RS | Not Supported | Not Supported | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |

Observation 5: When the Resource Configuration(CSI-RS Configuration = NZP CSI RS) is periodic, the Report Configuration can be set to be any type (periodic, semi-persistent, or aperiodic).

Observation 6: When the Resource Configuration(CSI-RS Configuration = NZP CSI RS) is semi-periodic, the Report Configuration can be Semi-persistent or Aperiodic.

Observation 7: When the Resource Configuration (CSI-RS Configuration = NZP CSI RS) is aperiodic, the Report Configuration can only be Aperiodic.

As discussed previously, to achieve adequate AI/ML functionality/model performance, the redundancy in the training data should be minimized. This can be achieved by introducing randomness in the training data logging/reporting. Following the above observation 2 – observation 4 from the legacy CSI framework, Fig. 2.2.1.2-1 illustrates a potential solution for redundancy in the training data collection.

Fig. 2.2.1.2-1: Training data collection: UE can be configured with periodic/semi-persistent/aperiodic RS configuration, but the CSI report configuration is semi-persistent/aperiodic.

Proposal 3: For NW-side training data collection for beam management, the dynamic activation/deactivation of training data collection and logging/reporting should be supported to reduce redundancy in training data. The dynamic activation/deactivation of training data collection and logging/reporting should be randomazied to reduce redundancy in training data.

#### 2.2.1.3	On event-based data collection/logging

Events-based data collection/logging is an effective tool for collecting meaningful data while reducing the power and memory consumption at the UE. However, as discussed above, collected data may be biased towards the configured events. Therefore, the suitability of event-based data collection may depend on the AI/ML objectives. For example, event-based data collection may not be suitable if AI/ML functionality/model is used for predicting L1-RSRP of Top-1/Top-K beams, as there may be missing data (depending on how events are defined). For example, if the events are defined to collect the data when the top-1/top-k beam changes, then there may be missing L1-RSRP (for beam management use case) measurements associated with the center of the beams. In such a case, the AI/ML may perform poorly in predicting the L1-RSRP (for beam management use case) measurements of top-1/top-k beams when the UE is in the center of the beams. In another scenario, where the AI/ML functionality/model objective is to predict beam failures/switching, event-based data collection may be a suitable solution for collecting meaningful data while reducing the power and memory consumption at the UE.

Observation 8: Events-based data collection/logging is an effective tool for collecting meaningful data while reducing the power and memory consumption at the UE.

Observation 9: Events-based data collection/logging may result in a scenario, where collected data may be biased towards the configured events.

Observation 10: The suitability of the event-based data collection/logging may depend on AI/ML functionality/model objectives. For example, if the AI/ML functionality/model is used for predicting the L1-RSRP of top-1/top-k beams, the event-based data collection/logging may not be suitable. While it may be suitable if AI/ML functionality/model is expected to predict beam failures/switching.

Proposal 4: Event-based data collection/logging is supported, as it helps in collecting representative data/samples of events, for example, beam failure detection, beam switching occurrences/events.

### 2.2.2	Handling Data Collection Configuration and Reporting during UE Mobility

In the RAN2#126 meeting [4], RAN2 agreed that “For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.” In this section, we discuss several issues on handling data collection configuration and reporting of collected training data during UE mobility.

Observation 11: For the beam management use case, RAN2 agreed that logging of L1-RSRP measurement for gNB-centric / OAM-centric data collection for NW-side model training is an optional feature for the UE.

#### 2.2.2.1	SRB for Training Data Reporting

In the RAN2#127 meeting [4], RAN2 agreed that the data collection report will not be transmitted over SRB1. Which SRB to use for logged training data is FFS in RAN2.

Agreements

- Data collection report will not be transmitted over SRB1.  FFS which SRB is used.

Note that SRB2 is used for carrying high-priority RRC messages (e.g., DLInformationTransfer carrying DL NAS messages; ULInformationtransfer carrying UL NAS messages; UEInformationResponse carrying SON reports, logged MDT reports, and IDLE Model Measurement reports) transmitted over the DCCH logical channel. As the reporting of the logged training data collection is not time-critical, therefore, we believe that SRB2 should not be used for the reporting of logged training data.

Observation 11: SRB2 is used for carrying high-priority RRC messages (e.g., DLInformationTransfer carrying DL NAS messages; ULInformationtransfer carrying UL NAS messages; UEInformationResponse carrying SON reports, logged MDT reports, and IDLE Model Measurement reports) transmitted over the DCCH logical channel.

Observation 12: Reporting of the logged training data collection is not time-critical, therefore, SRB2 should not be used for the reporting of logged training data.

Proposal 5: Reporting of the training data should be performed using SRB4 / SRB5 / a new SRB, irrespective of whether the logging is supported or not.

#### 2.2.2.2 	Leakage of Training Data across PLMN and Infra-Vendor

This section discusses some concerns associated with the leakage of the training data from one PLMN to another and one infra-vendor to another infra-vendor.

##### 2.2.2.2.1  From one PLMN to another PLMN

In the legacy immediate MDT measurement collection and reporting, data logging is not supported. This implies that serving gNB configures UE with measurement/reporting configuration based on the configuration received from the OAM, and receives the measurements which is then forwarded to TCE. During the handover / Reestablishment / Resume / Reconnection, the immediate MDT configuration is forwarded from the source gNB / AMF to the target gNB if they belong MDT PLMN List configured by the OAM. While the reporting of measurement collected is immediate UE is not required to check PLMN before reporting. UE is not aware of the MDT PLMN List.

Let us consider the following scenario (see Fig. 2.2.2.2.1-1) where a cell gNB1 (belonging to vendor 1 and PLMN 1) configures UE with training data collection (measurement and reporting configuration), but cannot retrieve the logged training data because of the following reasons

- The gNB1 was overloaded and couldn’t retrieve the training data before handover, or

- UE has RLF before the training data can be retrieved by gNB1.

Fig. 2.2.2.2.1-1: OAM-centric data collection, whether the logged training data can be reported to other PLMN or other-infra-vendor cells if they belong to the same/different PLMN.

We prefer RAN2 to clarify the UE behavior when UE has unretrieved logged training data during handover, RRCReestablishment, or RRC state transitions. RAN2 is further requested to clarify if there are any concerns associated with the leakage of collected training data from one PLMN to another PLMN (also one infra-vendor to another infra-vendor for OAM-centric data collection) if UE reports the collected training data target cell / Reestablishment / Reconnection cell / Resume cell if it belongs to the different PLMN.

##### 2.1.2.2.2  From one infra-vendor to another infra-vendor

In the legacy L3 measurement collection and reporting, data logging is not supported. This implies that serving gNB configures UE with measurement/reporting configuration, and receives the measurements. During the handover, the measurement/reporting configuration is handled by the following methods

- UE maintains the configuration if configured with need code M

- UE releases the configuration if configured with need code R

- Serving/target cell can explicitly release the configuration by sending a reconfiguration message

Let us consider the following scenario (see Fig. 2.2.2.1-1) where a single PLMN deploys the cells/gNBs from two different infra-vendors. Let us assume that gNB1 from vendor 1 configures the data collection at the UE, but cannot retrieve the logged training data because of the following reasons

- The gNB1 was overloaded and couldn’t retrieve the training data before handover, or

- UE has RLF before the training data can be retrieved by gNB1.

Fig. 2.2.2.2.2-1: gNB-centric data collection, whether the logged training data can be reported to other infra-vendor cells if they belong to the same/different PLMN.

We prefer RAN2 to clarify the UE behavior when UE has unretrieved logged training data during handover, RRCReestablishment, or RRC state transitions. RAN2 is further requested to clarify if there are any concerns associated with the leakage of collected training data from one infra-vendor to another infra-vendor if UE reports the collected training data target cell / Reestablishment / Reconnection cell / Resume cell if it belongs to the same/different PLMN. Alternatively, UE can provided with the AreaConfig. (e.g., list of cells/gNBs) where UE can report the logged training data.

In our understanding, the following can be the possible solutions:

- UE discards the unretrieved training data upon handover / RLF / RRC state transition.

- UE can report the unretrieved training data to any cell within a list of PLMNs. FFS How UE determines in which PLMN UE can report unretrieved logged training data.

- UE can report the unretrieved training data to any cell if they belong to the same PLMN.

- UE can report the unretrieved training data to cells configured. Configuring gNB should provide the list of cells/gNBs where the unretrieved logged training data should be reported.

Discarding the unretrieved training data upon handover / RLF / RRC state transition is not desired, as it is a waste of UE power and memory spent towards NW-side data collection.

Observation 13: In the following scenarios, the configuring gNB may not be able to retrieve the logged training data

- The configuring gNB was overloaded and could not retrieve the training data before handover, or

- UE has RLF before the training data can be retrieved by the configuring gNB1.

Proposal 6: For OAM-centric data collection, RAN2 is requested to clarify if there are any concerns associated with the leakage of collected training data when it is reported from

- Configuring PLMN (or MDT PLMN List) to another PLMN. FFS How does UE know which PLMNs UE can report unretrieved logged training data

- From one infra-vendor (configuring) to another infra-vendor

Proposal 7: For gNB-centric data collection, RAN2 is requested to clarify if there are any concerns associated with the leakage of collected training data when it is reported from

- From one infra-vendor (configuring) to another infra-vendor.

Proposal 8: RAN2 is requested to clarify the UE behavior when UE has unretrieved logged training data during handover, RRCReestablishment, or RRC state transitions, among the following

- UE discards the unretrieved training data upon handover / RLF / RRC state transition.

- UE can report the unretrieved training data to any cell within a list of PLMNs. FFS How UE determines in which PLMN UE can report unretrieved logged training data.

- UE can report the unretrieved training data to any cell if they belong to the same PLMN.

- UE can report the unretrieved training data to cells configured. Configuring gNB should provide the list of cells/gNBs where the unretrieved logged training data should be reported.

Proposal 9: Discarding the unretrieved training data upon handover / RLF / RRC state transition is not desired, as it is a waste of UE power and memory spent towards NW-side data collection.

#### 2.2.2.3 Handing of Training Data Collection Configuration during UE Mobility

##### 2.2.2.3.1 Handing of Training Data Collection Configuration for gNB-centric Data Collection

In the legacy CSI framework, the CSI-MeasCOnfig is used to configure CSI-RS (reference signal) belonging to the serving cell. Following the legacy CSI framework, the training data collection configuration for the beam management may be released upon handover/RLF/RRC state transitions from CONNECTED to IDLE/INACTIVE states.

Observation 14:  In the legacy CSI framework, the CSI-MeasCOnfig is used to configure CSI-RS (reference signal) belonging to the serving cell.

Proposal 10: For the gNB-centric training data collection, the training data collection configuration for the beam management is released upon handover/RLF/RRC state transitions from CONNECTED to IDLE/INACTIVE states.

##### 2.2.2.3.2 Handing of Training Data Collection Configuration for gNB-centric Data Collection

Legacy immediate MDT can be signaling-based MDT or management-based MDT. Signaling-based immediate MDT is used for collecting data from specific UEs, and the UE selection for immediate MDT is performed at the OAM. We believe that for the network-side model training, there may not be a need for collecting training from specific UEs. Therefore, the signaling-based immediate MDT for network-side model training may not be supported.

Observation 15: Signaling-based immediate MDT is used for collecting data from specific UEs, and the UE selection for immediate MDT is performed at the OAM.

Observation 16: For the network-side model training, there may not be a need for collecting training from specific UEs.

Proposal 11: RAN2 is requested to discuss if signaling-based immediate MDT data collecting is supported for NW-side training data collection.

On the other hand, the management-based immediate MDT is used for collecting data in a specific area, and UE selection for immediate MDT is performed at the gNB. For the management based MDT, the Management Based MDT PLMN List IE is sent HANDOVER REQUEST from the source to target NG-RAN, and from AMF to the target NG-RAN. Therefore, if the serving cell belongs to the Management Based MDT PLMN List IE, then they should, if supported, select UEs for configuring training data collection configuration. Furthermore, note that Management Based MDT PLMN List IE is stored in the UE context for management-based MDT data collection.

Observation 17: For management-based MDT, the data collection is performed at cells/gNBs if they belong to the Management Based MDT PLMN List IE and support MDT data collection.

There may be two possible methods for supporting management-based training data collection.

- Training data collection configuration is maintained at the UE during the handover/RLF/RRC state transitions; the serving NG-RAN forwards the training data collection configuration to the target/new NG-RAN.

- Training data collection configuration is not maintained at the UE during the handover/RLF/RRC state transitions; the target/new NG-RAN provides the training data collection configuration after the handover/Reestablishment/RRC state transition from IDLE/INACTIVE to CONNECTED state.

Proposal 12: RAN2 discusses what is the baseline procedure for supporting management-based training data collection, among the following

- Training data collection configuration is maintained at the UE during the handover/RLF/RRC state transitions; the serving NG-RAN forwards the training data collection configuration to the target/new NG-RAN.

- Training data collection configuration is not maintained at the UE during the handover/RLF/RRC state transitions; the target/new NG-RAN provides the training data collection configuration after the handover/Reestablishment/RRC state transition from IDLE/INACTIVE to CONNECTED state.

#### 2.2.2.4 Override Protection of unretrieved Training Data

As the UE may allocate sufficient resources and power for NW-side training data collection, the override protection of unretrieved training data should be supported. RAN2 is requested to discuss whether/how override protection of unretrieved training data collection should be supported.

Observation 18: UEs allocate sufficient resources and power for NW-side training data collection, thus override protection of unretrieved training data should be supported.

Proposal 13: RAN2 is requested to discuss whether/how override protection of unretrieved training data collection should be supported.

#### 2.2.2.5 Training Data Availability Indication

In the RAN2#127 meeting [4], RAN2 agreed that “UE stores the logged training data at the AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases”. Furthermore, the following is captured as FFS,

- FFS whether AS buffer event-based reporting is supported.  FFS if we send availability indication or full report if it is supported

As pointed out by some companies during the online session, the UE should not send the logged training data blindly, without the network indication as the serving cell may be overloaded.

Proposal 14: UE should not send the logged training data blindly, without the network indication as the serving cell may be overloaded.

Proposal 15: When the AS buffer is filled, the availability indication from the UE is sent tothe serving cell.

## 2.3   Data Collection for Network-side Model for Positioning Enhancements

### 2.3.1	UE-assisted positioning with LMF-side model (Case 2b)

For (2nd priority) Case 2b, the UE reports DL-PRS-based measurements to an LMF which may be any of the existing measurement types (e.g., RSTD, UE Rx-Tx Time Difference, etc.) or any new measurement types defined by RAN1 (e.g., channel impulse response measurements, etc.). For Case 2b data collection, SA2 agreed the following general procedure ([8], clause 5.18.1).

Fig. 2.3.1-1: Data collection by LMF to train the AI/ML based positioning model using UE measurements (Case 2b)

At Step 1, the LMF subscribes to an AMF to retrieve the list of SUPIs located in an area of interest using the Namf_EventExposure_Subscriber_Request operation. The AMF then responds with a Namf_EventExposure_Subscriber_Response operation including a list of SUPIs in the area of interest. After the LMF verified the user consent for data collection for model training for each SUPI, the LMF requests necessary data from the UE to train the LMF-side models at Step 4.

To obtain the input data for model training from the UE, any of the existing positioning procedures can be used [8],[9]. For example, the LMF may use a UE-based/UE-assisted LPP positioning procedure at Step 4. If periodic or event triggered measurement reporting is desired, the LMF may instigate a deferred MT-LR procedure as specified in TS 23.273 [8], using a desired event type (e.g., periodic location, motion event, area event, etc.).

Proposal 16: For LMF-side data collection in Positioning Enhancements Case 2b, existing LCS and UE positioning procedures (e.g., NI-LR, MT-LR and/or deferred MT-LR) are used to obtain the desired measurements from a UE/PRU.

For the label generation for both, model training data generation and model performance metric calculation in label-based model monitoring, the UE/PRU 'ground-truth' location would be needed at the LMF. However, obtaining a UE/PRU location by an LMF is an existing functionality. A LMF could use any of the existing UE-based, UE-assisted, or network-based positioning methods to obtain the UE/PRU location (dependent on UE and network capability). The UE/PRU location determination must be independent on the data collected for model training. For example, the LMF may use a RAT-independent location method (e.g., GNSS) to obtain the label, and using DL-TDOA, DL-AoD, or Multi-RTT positioning procedures to collect the desired data for model training (the details on the measurements depend on RAN1 progress). For PRU's, the PRU location may be a surveyed location provided to an LMF during PRU registration.

Proposal 17: For Positioning Enhancements Case 2b ground-truth label generation (for training and label-based monitoring), the entity which obtains the UE/PRU location is an LMF and existing LCS [8] and UE positioning [9] procedures are used to obtain the UE/PRU location estimate by an LMF.

### 2.3.2	Network-based positioning with LMF-side model (Case 3b)

For case 3b, the NG-RAN node reports SRS-based measurements to an LMF which may be any of the existing measurement types (e.g., RTOA, gNB Rx-Tx Time Difference, etc.) or any new measurement types defined by RAN1 (e.g., channel impulse response measurements, etc.). For Case 3b data collection, SA2 noted the following ([8], clause 5.18.1):

Editor's note:The procedure to collect the input data for AI/ML based positioning from NG-RAN is FFS.

Similar to Case 2b discussed in Section 2.3.1 above, existing LCS and UE-positioning procedures can be used for data collection from NG-RAN. An example is illustrated in Fig. 2.3.2-1.

Fig. 2.3.2-1: Data collection by LMF to train the AI/ML based positioning model using NG-RAN measurements (Case 3b)

Step 1 in Fig. 2.3.2-1 comprises Steps1-3 in Fig. 2.3.1-1, which may only be needed for a UE (i.e., may not be needed for a PRU). At Step 2, the LMF configures a desired SRS in the UE/PRU using existing NRPPa and RRC procedures [9]. With the UE/PRU SRS transmission, the LMF requests the desired measurements for data collection from NG-RAN using the existing NRPPa measurement procedures at Step 3/4 [9].

For the label generation for both, model training data generation and model performance metric calculation in label-based model monitoring, the same as discussed in section 2.3.1 above applies: A LMF could use any of the existing UE-based, UE-assisted, or network-based positioning methods to obtain the UE/PRU 'ground-truth' location at Step 5 in Fig. 2.3.2-1.

The details of the data collection for Case 3b are up to RAN3. However, Step 5 in Figure 5 involves the UE/PRU and may be in RAN2's scope. Therefore:

Proposal 18: For Positioning Enhancements Case 3b ground-truth label generation (for training and label-based monitoring), the entity which obtains the UE/PRU location is an LMF and existing LCS [8] and UE positioning [9] procedures are used to obtain the UE/PRU location estimate by an LMF.

### 2.3.3	Network-based positioning with gNB-side model (Case 3a)

For Case 3a, the AI/ML model is deployed at the NG-RAN node (gNB/TRP). The NG-RAN node reports AI/ML enhanced "SRS-based measurements", which may be AI/ML enhanced legacy measurements (e.g., RTOA, gNB Rx-Tx Time Difference, etc.) or new measurements.

For training data generation, the measurements and related attributes (e.g., timestamp) are generated by the TRP/NG-RAN node. Therefore, the training data are directly available at the NG-RAN node (gNB/TRP) and it appears natural that the model training is performed by the gNB. An example is illustrated in Fig. 2.3.3-1.

Fig. 2.3.3-1: Data collection by NG-RAN to train the AI/ML based positioning model using NG-RAN measurements (Case 3a)

The desired SRS for model training can be configured in the UE/PRU directly by the gNB. The gNB then measures the UE transmission and determines any desired data for model training at Step 2 in Fig. 2.3.3-1. The label for the training data can be provided by an LMF upon request at Step 3. The LMF could then use any of the existing LCS [8] and UE positioning [9] procedures to obtain the ground-truth label at Step 4, which the LMF provides at Step 5 to the NG-RAN. The user consent for data collection can be checked by the LMF like in Fig. 2.3.1-1 or 2.3.2-1. In addition, since existing positioning procedures are used at Step 4, existing privacy procedures for UE-positioning also apply.

Similar to the discussion for Case 3b in section 2.3.2 above, the details of the data collection for Case 3a are up to RAN3. However, Step 4 in Fig. 2.3.3-1 involves the UE/PRU and may be in RAN2's scope. Therefore:

Proposal 19: For Positioning Enhancements Case 3a ground-truth label generation (for training and label-based monitoring), the entity which obtains the UE/PRU location is an LMF and existing LCS [8] and UE positioning [9] procedures are used to obtain the UE/PRU location estimate by an LMF.

# Conclusion

Type of Data Required for NW-side model and UE-side models

Observation 1: In LS replies R2-2309435 and R2-2311720 from RAN1 on data collection requirements and assumptions, RAN1 provided the list of data content for different use cases that apply to both network-side model training and UE-side model training.

Observation 2: RAN1 further replied in LS replies R2-2309435 and R2-2311720 that there may be further information for training not included in the tables.

Proposal 1: Based on prior discussions in RAN2 and LS R2-2309435 and R2-2311720, the data collection for network-side model training and UE-side model training differ by

- Additional data that may be useful for UE-side model training, and

- Triggers for data collection. For UE side data collection, the triggers should be left to UE implementation.

Data collection and reporting for beam management use case:

Observation 3: Periodic data collection has following issues,

- Requires huge amount of training data collection at the UEs, as there may exist significantly high redundancy,

- Network side training server needs to handled redundancy in the training data, otherwise, it may cause biases in the trained models,

- Would be hard to get representative samples of events.

Observation 4: In the legacy CSI framework, UE can be configured with up to 112 RS configurations and 48 CSI report configurations.

Proposal 2: For the training data collection, UE can be configured with multiple CSI RS configurations and CSI report configurations.

Observation 5: When the Resource Configuration(CSI-RS Configuration = NZP CSI RS) is periodic, the Report Configuration can be set to be any type (periodic, semi-persistent, or aperiodic).

Observation 6: When the Resource Configuration(CSI-RS Configuration = NZP CSI RS) is semi-periodic, the Report Configuration can be Semi-persistent or Aperiodic.

Observation 7: When the Resource Configuration (CSI-RS Configuration = NZP CSI RS) is aperiodic, the Report Configuration can only be Aperiodic.

Proposal 3: For NW-side training data collection for beam management, the dynamic activation/deactivation of training data collection and logging/reporting should be supported to reduce redundancy in training data. The dynamic activation/deactivation of training data collection and logging/reporting should be randomazied to reduce redundancy in training data.

Observation 8: Events-based data collection/logging is an effective tool for collecting meaningful data while reducing the power and memory consumption at the UE.

Observation 9: Events-based data collection/logging may result in a scenario, where collected data may be biased towards the configured events.

Observation 10: The suitability of the event-based data collection/logging may depend on AI/ML functionality/model objectives. For example, if the AI/ML functionality/model is used for predicting the L1-RSRP of top-1/top-k beams, the event-based data collection/logging may not be suitable. While it may be suitable if AI/ML functionality/model is expected to predict beam failures/switching.

Proposal 4: Event-based data collection/logging is supported, as it helps in collecting representative data/samples of events, for example, beam failure detection, beam switching occurrences/events.

Observation 11: SRB2 is used for carrying high-priority RRC messages (e.g., DLInformationTransfer carrying DL NAS messages; ULInformationtransfer carrying UL NAS messages; UEInformationResponse carrying SON reports, logged MDT reports, and IDLE Model Measurement reports) transmitted over the DCCH logical channel.

Observation 12: Reporting of the logged training data collection is not time-critical, therefore, SRB2 should not be used for the reporting of logged training data.

Proposal 5: Reporting of the training data should be performed using SRB4 / SRB5 / a new SRB, irrespective of whether the logging is supported or not.

Observation 13: In the following scenarios, the configuring gNB may not be able to retrieve the logged training data

- The configuring gNB was overloaded and could not retrieve the training data before handover, or

- UE has RLF before the training data can be retrieved by the configuring gNB1.

Proposal 6: For OAM-centric data collection, RAN2 is requested to clarify if there are any concerns associated with the leakage of collected training data when it is reported from

- Configuring PLMN (or MDT PLMN List) to another PLMN. FFS How does UE know which PLMNs UE can report unretrieved logged training data

- From one infra-vendor (configuring) to another infra-vendor

Proposal 7: For gNB-centric data collection, RAN2 is requested to clarify if there are any concerns associated with the leakage of collected training data when it is reported from

- From one infra-vendor (configuring) to another infra-vendor.

Proposal 8: RAN2 is requested to clarify the UE behavior when UE has unretrieved logged training data during handover, RRCReestablishment, or RRC state transitions, among the following

- UE discards the unretrieved training data upon handover / RLF / RRC state transition.

- UE can report the unretrieved training data to any cell within a list of PLMNs. FFS How UE determines in which PLMN UE can report unretrieved logged training data.

- UE can report the unretrieved training data to any cell if they belong to the same PLMN.

- UE can report the unretrieved training data to cells configured. Configuring gNB should provide the list of cells/gNBs where the unretrieved logged training data should be reported.

Proposal 9: Discarding the unretrieved training data upon handover / RLF / RRC state transition is not desired, as it is a waste of UE power and memory spent towards NW-side data collection.

Observation 14:  In the legacy CSI framework, the CSI-MeasCOnfig is used to configure CSI-RS (reference signal) belonging to the serving cell.

Proposal 10: For the gNB-centric training data collection, the training data collection configuration for the beam management is released upon handover/RLF/RRC state transitions from CONNECTED to IDLE/INACTIVE states.

Observation 15: Signaling-based immediate MDT is used for collecting data from specific UEs, and the UE selection for immediate MDT is performed at the OAM.

Observation 16: For the network-side model training, there may not be a need for collecting training from specific UEs.

Proposal 11: RAN2 is requested to discuss if signaling-based immediate MDT data collecting is supported for NW-side training data collection.

Observation 17: For management-based MDT, the data collection is performed at cells/gNBs if they belong to the Management Based MDT PLMN List IE and support MDT data collection.

Proposal 12: RAN2 discusses what is the baseline procedure for supporting management-based training data collection, among the following

- Training data collection configuration is maintained at the UE during the handover/RLF/RRC state transitions; the serving NG-RAN forwards the training data collection configuration to the target/new NG-RAN.

- Training data collection configuration is not maintained at the UE during the handover/RLF/RRC state transitions; the target/new NG-RAN provides the training data collection configuration after the handover/Reestablishment/RRC state transition from IDLE/INACTIVE to CONNECTED state.

Observation 18: UEs allocate sufficient resources and power for NW-side training data collection, thus override protection of unretrieved training data should be supported.

Proposal 13: RAN2 is requested to discuss whether/how override protection of unretrieved training data collection should be supported.

Proposal 14: UE should not send the logged training data blindly, without the network indication as the serving cell may be overloaded.

Proposal 15: When the AS buffer is filled, the availability indication from the UE is sent to the serving cell.

Data collection and reporting for positioning enhancements use case:

Proposal 16: For LMF-side data collection in Positioning Enhancements Case 2b, existing LCS and UE positioning procedures (e.g., NI-LR, MT-LR and/or deferred MT-LR) are used to obtain the desired measurements from a UE/PRU.

Proposal 17: For Positioning Enhancements Case 2b ground-truth label generation (for training and label-based monitoring), the entity which obtains the UE/PRU location is an LMF and existing LCS [8] and UE positioning [9] procedures are used to obtain the UE/PRU location estimate by an LMF.

Proposal 18: For Positioning Enhancements Case 3b ground-truth label generation (for training and label-based monitoring), the entity which obtains the UE/PRU location is an LMF and existing LCS [8] and UE positioning [9] procedures are used to obtain the UE/PRU location estimate by an LMF.

Proposal 19: For Positioning Enhancements Case 3a ground-truth label generation (for training and label-based monitoring), the entity which obtains the UE/PRU location is an LMF and existing LCS [8] and UE positioning [9] procedures are used to obtain the UE/PRU location estimate by an LMF.

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec. 2021

- TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18),” Dec. 2023

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #102, Dec. 2023

- Meeting Report, “RAN2#126 meeting report”

- Meeting Report, “RAN2#127 meeting report”

- TS 38.331, RRC Specification

- TS 38.214

- 3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

- 3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".