3GPP TSG-RAN WG2 Meeting #127bis		     R2-2408392

Hefei, China: 14th October – 18th October 2024

Source:	Qualcomm Incorporated

Title:	Discussion on Model Delivery/Transfer

Agenda Item:	7.16.2.3 Control and LCM other

Document for:	Discussion and Decision

# Introduction & Background

In the TR 38.843 [1], the following solutions were captured for model delivery/transfer,

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling.

- Solution 2a: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling.

- Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling.

- Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data.

- Solution 2b: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via User Plane (UP) data.

- Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data.

- Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP).

- Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE.

In RAN2 discussions during the study item, the model delivery/transfer was considered for delivering an AI/ML model from the MNO network/OTT to the UE for running the inference [1]. However, with the recent progress in RAN1 for two-sided models, the discussion on model delivery/transfer needs to be extended to support two-sided model training.

This paper discusses model delivery/transfer for inference and two-sided model training, considering discussions during the SI phase and recent agreements in RAN1 as the baseline.

# Model Delivery/Transfer Methods

As mentioned above, during the SI phases, the model delivery/transfer was discussed in the context of supporting AI/ML inference at the UE, where the required model is downloaded by the UE or delivered/transferred to the UE. This is still true for the one-side model, i.e., there is no need for model delivery/transfer for the training. However, with the recent progress in RAN1, the discussion of model transfer/delivery solutions should be extended to two-sided model training, where the training dataset, parameter set, or the reference model for UE-side training can be provided from the network to the UE or UE-side training server [2][3].

Observation 1: During the SI phases, the model delivery/transfer was discussed in the context of supporting AI/ML inference at the UE, where the required model is downloaded by the UE or delivered/transferred to the UE.

Observation 2: Recently, for two-sided model training, the RAN1 has discussed the delivery/transfer of the training dataset, parameter set, or the reference model for UE-side training from the network to the UE or UE-side training server [2][3].

Proposal 1: For the model transfer/delivery, RAN2 is requested to consider the following:

- To support inference at the UE, transfer/delivery of UE side models from the network to UE,

- To support UE-sided training of the UE-part model of the two-sided model, the delivery/transfer of the training dataset, parameter set, or the reference model from the network to the UE or UE-side training server.

- To support inference at the UE, the delivery/transfer of the UE-part model (trained at the UE-side or parameter or reference model trained at the NW-side) of the two-sided model from the network to the UE.

## 2.1 One-Side Model: Model Delivery/Transfer during Inference

## 2.1.1 One-Side Model: Model Training, Testing, Validation, and Compilation

The TR 38.843 [1] captures different model delivery/transfer cases as below:

Table 4.3-1: Model delivery/transfer cases

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top. | Outside 3GPP Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format. | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format. | 3GPP Network | NW-side |
| z3 | model transfer in open format. | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support. | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE, i.e., any other model structure not covered in z4, including any model structure that is only partially known. | 3GPP Network | NW-side |
| Note: The definition of various Cases is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization. |  |  |  |

Furthermore, the following has been captured for model delivery/transfer to UE (for UE-side models and UE-part of two-sided models):

-	Model delivery/transfer to UE, if feasible, may be beneficial to handle scenario/configuration specific (including site-specific configuration/channel conditions) models (i.e., when a single model cannot generalize well to multiple scenarios/configurations/sites), to reduce the device storage requirement.

-	Model delivery/transfer to UE after offline compiling and/or testing may be friendlier from UE’s implementation point of view compared to the case without offline compiling and/or testing. On the other hand, the case without offline compiling and/or testing (that can update parameter with known model structure), may have benefit at least in terms of shorter model parameter update timescale.

-	Model transfer/delivery of an unknown structure at UE has more challenges related to feasibility (e.g. UE implementation feasibility) compared to delivery/transfer of a known structure at UE.

-	For model trained at network side, Case y (w/ NW-side training) and Case z2 may incur the burden of offline cross-vendor collaboration such as sending a model to the UE-side and/or compiling a model.

-	For model trained at UE side/neutral site, Case z1 and Case z3 may incur the burden of offline cross-vendor collaboration to send the trained model from the UE-side to the network, compared to Case y (w/ UE-side training) which does not have such burden.

-	Model storage at the 3GPP network, compared to storing the model outside the 3GPP network, may come with 3GPP network side burden on model maintenance/storage.

-	Proprietary design disclosure concern may arise from model training and/or model storage at the network side compared to other cases (such as case y with UE side training) which does not have such issue.

Considering the above, the following agreements were made in RAN#116bis [2][3] meeting to deprioritize the model delivery/transfer options z2, z3, and z5.

Agreements/conclusions from RAN1#116 meeting [2]:

Conclusion From RAN1 perspective, the model transfer/delivery Case z5 is deprioritized for Rel-19.

Agreements/conclusions from RAN1#116bis meeting [3]:

Conclusion From RAN1 perspective, the model transfer/delivery Case z2 is deprioritized at least for UE-sided model in Rel-19 due to the following reasons: - Risk of proprietary design disclosure - Burden of offline cross-vendor collaboration Conclusion From RAN1 perspective, the model transfer/delivery Case z3 is deprioritized for Rel-19 due to the following reasons (compared to Case y): - No much benefit compared to Case y - Risk of proprietary design disclosure - Large burden of offline cross-vendor collaboration - Additional burden on model storage within in 3GPP network

Observation 3: RAN1 has deprioritized model delivery/transfer cases z2, z3, and z5 for supporting AI/ML inference.

Based on the discussions on model delivery/transfer captured in TR 38.843 [1], and the above agreements/conclusions in RAN1 [2][3], the UE side models are trained, tested, validated, and complied with for the target devices by the UE-side. While there is FFS if

- Z1: The trained, tested, validated, and compiled AI/ML models can be stored in the 3GPP MNO network for model delivery/transfer, and

- Z4: The network can train and transfer parameter sets for known and identified model structures, conditioned on testing and validation of the parameter set is not required.

Observation 4: Based on the discussions on model delivery/transfer captured in TR 38.843 [1], and the above agreements/conclusions in RAN1 [2][3], the UE side models are trained, tested, validated, and complied with for the target devices by the UE-side.

Observation 5: The following are FFS for model delivery/transfer cases for supporting AI/ML inference at the UE,

- Z1: The trained, tested, validated, and compiled AI/ML models can be stored in the 3GPP MNO network for model delivery/transfer, and

- Z4: The network trains and delivers/transfers parameter sets for known and identified model structures to the UE, conditioned on testing and validation of the parameter set is not required.

In our understanding, the model delivery/transfer case, z1, is kept FFS, as this may provide some potential benefits such as

- Lower latency in model transfer/delivery, and

- QoS guarantees from the 3GPP network.

Observation 6: Model delivery/transfer case z1 may provide some potential benefits such as

- Lower latency in model transfer/delivery,

- Segregation of traffic due to model transfer/delivery from user traffic for charging policies, and

- QoS guarantees from the 3GPP network.

Note that as captured in TR 38.843 [1], model transfer/delivery case z1 comes with additional complexity such as

- Offline cross-vendor collaboration to send the trained model from the UE side to the network,

- Model storage at the 3GPP network.

Observation 7: Model transfer/delivery case z1 comes with additional complexity such as

- Offline cross-vendor collaboration to send the trained model from the UE side to the network,

- Model storage at the 3GPP network.

Proposal 2: RAN2 is requested to discuss if model delivery/transfer case z1 is supported for inference for one-sided models, considering the benefits in observation 6 and complexities in observation 7.

There was not much interest in the model delivery/transfer option, z4, For one-sided model inference at the UE. Therefore, we believe that the model delivery/transfer option z4 should be deprioritized for the one-sided model.

Proposal 3: Deprioritize model delivery/transfer case z4, for one-side UE models.

## 2.1.2 One-Side Model: Model Delivery/Transfer Solutions

Whether a standardized solution for model delivery/transfer is needed depends on RAN2's conclusion on whether the model delivery/transfer case z1 is supported.

Observation 8: Whether a standardized solution for model delivery/transfer is needed depends on RAN2's conclusion on whether the model delivery/transfer case z1 is supported.

#### 2.1.2.1 If Model Delivery/Transfer Case Z1 is not Supported

A standard-based model delivery/transfer solution is not required, if RAN2 determines that model delivery/transfer case z1 is not supported. Then, solution 4a is considered the default model delivery/transfer solution for inference.

Proposal 4: Solution 4a is the model delivery/transfer solution for inference, if RAN2 determines that model delivery/transfer case z1 is not supported.

#### 2.1.2.2 If Model Delivery/Transfer Case Z1 is Supported

During the SI phase, RAN2 evaluated different model transfer/delivery solutions, considering the following, and captured it in TS 38.843 [1]:

- A1: Large, no upper limit model/model parameter size,

- A2: Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs),

- A3: Network controllability on model transfer/delivery (e.g., management decision at gNB),

- A4: Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB).

Before going to further discussion on the model delivery/transfer solutions, we want to take a step back and highlight the following,

- To reduce the air interface traffic, a model should be used across multiple cells/gNBs/locations,

- A model should be used throughout multiple connections (i.e., one or more transitions between RRC IDLE/INACTIVE states to RRC CONNECTED).

Observation 9: For the UE-side inference,

- An AI/ML model should be used across multiple cells/gNBs/locations,

- An AI/ML model should be used throughout multiple connections (i.e., one or more transitions between RRC IDLE/INACTIVE states to RRC CONNECTED).

A control plane-based AI/ML model transfer/delivery assumes that the network is responsible for delivering/transferring the model when required. This has the following implications

- Requires model delivery/transfer at every inference configuration. Otherwise, the network entity (depending on the solution flavor) needs to store the model information (e.g., identity, metadata) that is transferred to the UE to reduce Uu overhead,

- UE should be able to indicate to the network when it releases a model due to memory issues if the model is used across multiple cells/gNBs/locations. This may need steps for the model identification of the physical models,

- UE may need to provide the network with proprietary information such as currently available memory, computation resources, and others, which is not desired. Furthermore, even by providing the aforementioned information, the network may not be able to determine if certain AI/ML models can be used at the UE.

Observation 10: A control plane-based AI/ML model transfer/delivery assumes that the network is responsible for delivering/transferring the model when required. This has the following implications

- Requires model delivery/transfer at every inference configuration. Otherwise, the network entity (depending on the solution flavor) needs to store the model information (e.g., identity, metadata) that is transferred to the UE to reduce Uu overhead,

- UE should be able to indicate to the network when it releases a model due to memory issues if the model is used across multiple cells/gNBs/locations. This may need steps for the model identification of the physical models,

- UE may need to provide the network with proprietary information such as currently available memory, computation resources, and others, which is not desired. Furthermore, even by providing the aforementioned information, the network may not be able to determine if certain AI/ML models can be used at the UE.

During the evaluation of different model delivery/transfer solutions, the following was agreed upon as the complexity associated with control-plane-based model transfer/delivery

- Delivery/transfer of model sizes larger than 45 KBs is not supported, at least without extending the number of RRC segments,

- Model delivery/transfer continuity is not supported, as SRB is restarted during every cell change,

- Xn and NG-AP enhancements are required to support model delivery/transfer continuity,

- Requirement for new SRB, and others.

Observation 11: During the evaluation of different model delivery/transfer solutions, the following was agreed upon as the complexity associated with control-plane-based model transfer/delivery

- Delivery/transfer of model sizes larger than 45 KBs is not supported, at least without extending the number of RRC segments,

- Model delivery/transfer continuity is not supported, as SRB is restarted during every cell change,

- Xn and NG-AP enhancements are required to support model delivery/transfer continuity,

- Requirement for new SRB, and others.

Also, note that control-plane-based solutions are also not in line with the discussions/agreements we had in agenda item 8.1.2.2 for applicable functionality determination and inference configuration.

Observation 12: Control-plane-based solutions are also not in line with the discussions/agreements we had in agenda item 8.1.2.2 for applicable functionality determination and inference configuration.

Proposal 5: For one-sided models, deprioritize all control-plane-based model delivery/transfer solutions if RAN2 determines that model delivery/transfer case z1 is supported.

##### 2.1.2.2.1 Positioning Enhancements: If Model Delivery/Transfer Case Z1 is Supported

Note that the LMF is a CN entity, whether LMF can be enhanced with model storage, model management, and model delivery/transfer procedure is SA2 decision. In our understanding, if model delivery/transfer case, z1, is supported, then a common NF should be provisioned handling model storage, model management, and model transfer delivery/transfer for all of the use cases.

Observation 13: The LMF is a CN entity, whether LMF can be enhanced with model storage, model management, and model delivery/transfer procedure is SA2 decision.

Proposal 6: If model delivery/transfer case, z1, is supported, then a common NF should be provisioned handling model storage, model management, and model transfer delivery/transfer for all of the use cases.

## 2.2 Two-Sided Models: Model Delivery/Transfer

### 2.2.1 Two-Sided Models: Model Delivery/Transfer for Supporting Training at the UE Side

In the RAN1#116 meeting to alleviate/resolve the issues related to infra-vendor collaboration of AI/ML-based CSI compression using two-side models, the following were agreed for study [2],

Agreement To alleviate / resolve the issues related to inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, study the following options: - Option 1: Fully standardized reference model (structure + parameters) - Option 2: Standardized dataset - Option 3: Standardized reference model structure + Parameter exchange between NW-side and UE-side - Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side - Option 5: Standardized model format + Reference model exchange between NW-side and UE-side Note 1: The above options may not be mutually exclusive and may be used together. Note 2: Other options are not precluded. Note 3: The study should consider how different methods of exchanging the parameters / dataset / reference model would affect the feasibility and collaboration complexity of options 3 / 4 / 5 respectively, e.g., over the air-interface, offline delivery, etc. Note 4: “Dataset” refers to a set of data samples of CSI feedback and associated target CSI.

Agreement For the study of inter-vendor collaboration issues for AI/ML-based CSI compression using a two-sided model, consider at least the following aspects when comparing different options: - Inter-vendor collaboration complexity, e.g., whether bilateral collaboration is required between vendors. - Performance. - Interoperability and RAN4 / testing related aspects. - Feasibility.

In RAN1#116bis meeting [3], RAN1 further made the following agreements

Agreement - For Option 3, further define the two sub-options: - 3a: Parameters received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing. - 3b: Parameters received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations. - For Option 5, further define the two sub-options: - 5a: Model received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing. - 5b: Model received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations. - For Option 4, it is clarified that: - Dataset received at the UE or UE-side goes through offline engineering at the UE- side (e.g., UE-side OTT server), e.g., model training or offline testing. - Note: The descriptions under each option are only for the purpose of simplified discussion and do not mean deprioritizing any other flavours (such as an exchange originating from the UE-side and ending at the NW-side) from potential specification.

Agreement (amended as shown in red in Wednesday session) - For Option 3/4/5, focus further discussion on the following assumptions: - Option 3a/5a - The model(5a)/parameter(3a) exchange originates from the NW-side and ends at the UE-side. - Model(5a)/parameters(3a) exchanged from the NW-side to UE-side is either CSI generation or reconstruction part or both. - Option 3a-1/5a-1: Model/Parameters exchanged from the NW-side to UE-side is CSI generation part. - Option 3a-2/5a-2: Model/Parameters exchanged from the NW-side to UE-side is CSI reconstruction part. - Option 3a-3/5a-3: Model/Parameters exchanged from the NW-side to UE-side are both CSI generation part and CSI reconstruction part. - Some additional information, if necessary, may be shared from the NW-side to help UE-side offline engineering and provide performance guidance. - Performance target - Dataset or information related to collecting dataset - Study different methods of exchanging, e.g., over the air-interface, offline delivery, etc. - Option 3b - The method of exchanging is over the air-interface via model transfer/delivery Case z4. - The parameter exchange is from NW to UE. - Parameters exchanged from the NW-side to UE-side is CSI generation part. - Option 5b - The method of exchanging is over the air-interface via model transfer/delivery Case z4, assuming that the model structure is aligned based on offline inter-vendor collaboration. - The model exchange is from NW to UE. - Model exchanged from the NW-side to UE-side is CSI generation part. - Option 4: - The dataset exchange originates from the NW-side and ends at the UE-side. - Option 4-1: Dataset exchanged from the NW-side to UE-side consists of (target CSI, CSI feedback). - Option 4-2: Dataset exchanged from the NW-side to UE-side consists of (CSI feedback, reconstructed target CSI). - Option 4-3: Dataset exchanged from the NW-side to UE-side consists of (target CSI, CSI feedback, reconstructed target CSI). - Some additional information, if necessary, may be shared from the NW-side to help UE-side offline engineering and provide performance guidance. - Performance target - Study different methods of exchanging, e.g., over the air-interface, offline delivery, etc. - Note: For each option/sub-option of interest, companies to bring discussion on how inter-vendor collaboration complexity, interoperability, and feasibility may be addressed. Companies to strive to provide solution(s) that can address all the following aspects: inter-vendor collaboration complexity, performance, interoperability, and feasibility. - Note: The descriptions under each option are only for the purpose of simplified discussion and do not mean deprioritizing any other flavours (such as an exchange originating from the UE-side and ending at the NW-side) from potential specification.

Conclusion: - Conclude, from RAN1 perspective, that Option 1, if feasible for specification, eliminate the inter-vendor collaboration complexity (e.g., whether bilateral collaboration is required between vendors). - It is RAN1’s understanding that Option 1 corresponds to RAN4 options, e.g., RAN4-Option3, or RAN4-Option4. Further study and final conclusion on interoperability and RAN4 testing of the RAN4-Option3 and RAN4-Option4 is up to RAN4. Observation - Option 1 and 2 may have limited performance in the field compared to Options 3, 4, and 5, further study is needed - Option 1 and 2 may require high specification effort from RAN1 perspective. Conclusion - Deprioritize Option 2 for inter-vendor training collaboration. - Note: This de-prioritization shall not affect the ongoing discussion in RAN4 on RAN4-Option3 and RAN4-Option4.

From RAN1 agreements in meeting RAN1#116 [2] and RAN1#116bis [3], we can see the dataset, parameter, and reference model exchange can be supported using

- Over-the-Air Signaling,

- Offline delivery/transfer.

Observation 14: Based on RAN1 agreements [2][3], the dataset, parameter, and reference model exchange can be supported using

- Over-the-Air,

- Offline delivery/transfer.

This section further discusses, dataset, parameter, and reference model exchange using the over-the-air signaling and offline delivery/transfer.

#### 2.2.1.1 Dataset, Parameter, and Reference Model Exchange Over the Air

Note that training at the UE-side is offline training, therefore the training dataset, parameter, or reference model must be sent to the UE-side training server. Fig. 2.2.1.1-1 illustrates the dataset, parameter, or reference model transfer/delivery over the air. In Fig. 2.2.1.1-1, the RAN delivers/transfers the required dataset, parameter, or reference model, after that, the UE transfers/delivers the dataset, parameter, or reference model (sent by the RAN) to the UE side training server. This method of exchanging the dataset, parameter, or reference model for training is not suitable, as the significantly large amount of training data is duplicated across multiple UEs, and at the same time it causes the Uu overhead twice: (i) for delivering/transferring the dataset, parameter, or reference model from the RAN to the UE, and (ii) for delivering/transferring the dataset, parameter, or reference model from the UE to the UE side server.

Fig. 2.2.1.1-1: Dataset, parameter, or reference model transfer/delivery over the air

As discussed above, the UE part of the two-side model will not be trained at the UE, rather it will be trained at a UE side server.

Observation 15: The UE part of the two-side model is trained at a UE side server not at the UE, i.e., training is not performed by the UE rather it is performed by the UE vendors.

Observation 16: Over-the-air delivery of the training dataset, parameter, or reference model for supporting UE-side model training causes the following issues

- No standard support for determining the UE vendor identity: currently there is no standard procedure to determine the UE vendor identity at the serving cell/gNB. Therefore, there is no method by which gNB can ensure the required training dataset, parameter, or reference model is exchanged with the desired/participating UE vendor(s).

- Need for inter-vendors collaboration for the selection/indication of the representative UEs: without the representative UEs in the network, significant duplication may exist, as the same training dataset, parameter, or reference model may need to be sent to multiple UEs CONNECTED to a cell/gNB. Therefore, there may be a need for inter-vendor collaboration to select/indicate representative UEs.

- UE implementation burden: Without the inter-vendor collaboration for selecting the representative UEs for UE vendor, the UE vendors must implement the signaling/procedure for exchanging training dataset, parameter, or reference model at all UEs in the network.

- Concerns with cell-specific NW-part model of the two-sided models: If the NW-part of the two-sided models is cell-specific, then the selection/indication of representative UEs needs to be performed per cell. This causes huge overhead for inter-vendor collaboration.

- Double the Uu overhead: (i) for delivering/transferring the dataset, parameter, or reference model from the RAN to the UE, and (ii) for delivering/transferring the dataset, parameter, or reference model from the UE to the UE side server.

Considering the issues associated with over-the-air delivery/transfer of the training dataset, parameter, or reference model to support UE-side model training, delivering/transferring the training dataset, parameter, or reference model is a not scalable solution. RAN2 is requested to develop a solution that minimizes/alleviates the Uu overhead associated with delivering/transferring the training dataset, parameter, or reference model to the UE side. One possible solution is that serving cell(s) may configure UEs to provide the information for delivering/transferring the training dataset, parameter, or reference model to the UE side. The serving cell(s) can provide OAM or a Network Function (NF) with this information for delivering/transferring the training dataset, parameter, or reference mode.

Proposal 7: Over-the-air delivery/transfer of the training dataset, parameter, or reference model to support UE-side model training is not a suitable or scalable solution. Please refer to the issues mentioned in observation 16.

Proposal 8: To minimize the inter-vendor collaboration for transferring the training dataset, parameter, or reference model to support UE-side model training while reducing the Uu overhead, the RAN is requested to discuss a solution, where

- The UE can provide required information for sharing the training dataset, parameter, or reference model with the UE-side to the serving gNB, thereafter

- Serving gNB can provide OAM or NF with the required information for sharing the training dataset, parameter, or reference model to the UE-side, thereafter

- OAM or NF then delivers/transfers the training dataset, parameter, or reference model with the UE-side using the information provided from the serving gNB.

#### 2.2.1.2 Dataset, Parameter, and Reference Model Exchange using Offline Transfer/Delivery

In the SA WGs, we have precedence where different types of data are shared with application function (within/outside the 3GPP network) and or edge cloud, for example,

- Target UE location to AF [TS 23.273],

- Analytics exposure from NWDAF to AF, e.g., network status reporting [TS 23.288],

- Traffic volume, UL/DL data rates, PDU session status, and others [TS 23.501].

Observation 17: Within the SA WGs, we have precedence where different types of data are shared with application function (within/outside the 3GPP network) and or edge cloud, for example,

- Target UE location to AF [TS 23.273],

- Analytics exposure from NWDAF to AF, e.g., network status reporting [TS 23.288],

- Traffic volume, UL/DL data rates, PDU session status, and others [TS 23.501].

To minimize the inter-vendor collaboration while reducing the Uu overhead, RAN2 is requested to discuss how a standardized solution for offline delivery/transfer of training dataset, parameter, or reference model from the network entity (e.g., RAN/OAM/NF) to the UE-side training server can be supported.

Proposal 9: To minimize the inter-vendor collaboration while reducing the Uu overhead, RAN2 is requested to discuss how a standardized solution for offline delivery/transfer of training dataset, parameter, or reference model from the network entity (e.g., RAN/OAM/NF) to the UE-side training server can be supported.

### 2.2.2 Two-Sided Models: Model Delivery/Transfer for Supporting Inference at the UE

#### 2.2.2.1	Model Delivery/Transfer of the UE-part model of the two-sided model

The discussions on model transfer/delivery of the one-sided model for supporting inference at the UE should be extended to the model delivery/transfer of the UE-part model of two-sided models.

Proposal 10: RAN2 is requested to discuss if model delivery/transfer of the UE-part model of two-sided models, i.e. z1, is supported for inference, considering the benefits in observation 6 and complexities in observation 7.

Proposal 11: Solution 4a is the model delivery/transfer solution for delivering/transferring the UE-part model of the two-sided models to the UE (at least when the UE-part model of the two-sided model is trained by the UE-side) for inference if RAN2 determines that model delivery/transfer case z1 is not supported.

Proposal 12: Deprioritize all control plane-based solutions for the delivery/transfer of the UE-part model of two-sided models (at least when the UE-part model of the two-sided model is trained by the UE-side) if RAN2 determines that model delivery/transfer case z1 is supported.

#### 2.2.2.2	Two-Sided Models: Model delivery of the UE-part model of the two-sided model

In RAN1 discussions, the following two options were considered where the parameter or model can be used by the UE without offline engineering. RAN2 is requested to wait for the conclusion on these options.

Observation 17: The following two options were considered where the parameter or model can be used by the UE without offline engineering

- 3b: Parameters received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations.

- 5b: Model received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations.

Proposal 13: Wait for RAN1 conclusion on whether options 3b/5b are supported for two-sided models, for the inference at the UE.

# Conclusion

Observation 1: During the SI phases, the model delivery/transfer was discussed in the context of supporting AI/ML inference at the UE, where the required model is downloaded by the UE or delivered/transferred to the UE.

Observation 2: Recently, for two-sided model training, the RAN1 has discussed the delivery/transfer of the training dataset, parameter set, or the reference model for UE-side training from the network to the UE or UE-side training server [2][3].

Proposal 1: For the model transfer/delivery, RAN2 is requested to consider the following:

- To support inference at the UE, transfer/delivery of UE side models from the network to UE,

- To support UE-sided training of the UE-part model of the two-sided model, the delivery/transfer of the training dataset, parameter set, or the reference model from the network to the UE or UE-side training server.

- To support inference at the UE, the delivery/transfer of the UE-part model of the two-sided model from the network to the UE.

Observation 3: RAN1 has deprioritized model delivery/transfer cases z2, z3, and z5 for supporting AI/ML inference.

Observation 4: Based on the discussions on model delivery/transfer captured in TR 38.843 [1], and the above agreements/conclusions in RAN1 [2][3], the UE side models are trained, tested, validated, and complied with for the target devices by the UE-side.

Observation 5: The following are FFS for model delivery/transfer cases for supporting AI/ML inference at the UE,

- Z1: The trained, tested, validated, and compiled AI/ML models can be stored in the 3GPP MNO network for model delivery/transfer, and

- Z4: The network trains and delivers/transfers parameter sets for known and identified model structures to the UE, conditioned on testing and validation of the parameter set is not required.

Observation 6: Model delivery/transfer case z1 may provide some potential benefits such as

- Lower latency in model transfer/delivery,

- Segregation of traffic due to model transfer/delivery from user traffic for charging policies, and

- QoS guarantees from the 3GPP network.

Observation 7: Model transfer/delivery case z1 comes with additional complexity such as

- Offline cross-vendor collaboration to send the trained model from the UE side to the network,

- Model storage at the 3GPP network.

Proposal 2: RAN2 is requested to discuss if model delivery/transfer case z1 is supported for inference, considering the benefits in observation 6 and complexities in observation 7.

Proposal 3: Deprioritize model delivery/transfer case z4, for one-side UE models.

Observation 8: Whether a standardized solution for model delivery/transfer is needed depends on RAN2's conclusion on whether the model delivery/transfer case z1 is supported.

Proposal 4: Solution 4a is the model delivery/transfer solution for inference, if RAN2 determines that model delivery/transfer case z1 is not supported.

Observation 9: For the UE-side inference,

- An AI/ML model should be used across multiple cells/gNBs/locations,

- An AI/ML model should be used throughout multiple connections (i.e., one or more transitions between RRC IDLE/INACTIVE states to RRC CONNECTED).

Observation 10: A control plane-based AI/ML model transfer/delivery assumes that the network is responsible for delivering/transferring the model when required. This has the following implications

- Requires model delivery/transfer at every inference configuration. Otherwise, the network entity (depending on the solution flavor) needs to store the model information (e.g., identity, metadata) that is transferred to the UE to reduce Uu overhead,

- UE should be able to indicate to the network when it releases a model due to memory issues if the model is used across multiple cells/gNBs/locations. This may need steps for the model identification of the physical models,

- UE may need to provide the network with proprietary information such as currently available memory, computation resources, and others, which is not desired. Furthermore, even by providing the aforementioned information, the network may not be able to determine if certain AI/ML models can be used at the UE.

Observation 11: During the evaluation of different model delivery/transfer solutions, the following was agreed upon as the complexity associated with control-plane-based model transfer/delivery

- Delivery/transfer of model sizes larger than 45 KBs is not supported, at least without extending the number of RRC segments,

- Model delivery/transfer continuity is not supported, as SRB is restarted during every cell change,

- Xn and NG-AP enhancements are required to support model delivery/transfer continuity,

Observation 12: Control-plane-based solutions are also not in line with the discussions/agreements we had in agenda item 8.1.2.2 for applicable functionality determination and inference configuration.

Proposal 5: For one-sided models, deprioritize all control-plane-based model delivery/transfer solutions if RAN2 determines that model delivery/transfer case z1 is supported.

Observation 13: The LMF is a CN entity, whether LMF can be enhanced with model storage, model management, and model delivery/transfer procedure is SA2 decision.

Proposal 6: If model delivery/transfer case, z1, is supported, then a common NF should be provisioned handling model storage, model management, and model transfer delivery/transfer for all of the use cases.

Observation 14: Based on RAN1 agreements [2][3], the dataset, parameter, and reference model exchange can be supported using

- Over-the-Air Signaling,

- Offline delivery/transfer.

Observation 15: The UE part of the two-side model is trained at a UE side server not at the UE, i.e., training is not performed by the UE rather it is performed by the UE vendors.

Observation 16: Over-the-air delivery of the training dataset, parameter, or reference model for supporting UE-side model training causes the following issues

- No standard support for determining the UE vendor identity: currently there is no standard procedure to determine the UE vendor identity at the serving cell/gNB. Therefore, there is no method by which gNB can ensure the required training dataset, parameter, or reference model is exchanged with the desired/participating UE vendor(s).

- Need for inter-vendors collaboration for the selection/indication of the representative UEs: without the representative UEs in the network, significant duplication may exist, as the same training dataset, parameter, or reference model may need to be sent to multiple UEs CONNECTED to a cell/gNB. Therefore, there may be a need for inter-vendor collaboration to select/indicate representative UEs.

- UE implementation burden: Without the inter-vendor collaboration for selecting the representative UEs for UE vendor, the UE vendors must implement the signaling/procedure for exchanging training dataset, parameter, or reference model at all UEs in the network.

- Concerns with cell-specific NW-part model of the two-sided models: If the NW-part of the two-sided models is cell-specific, then the selection/indication of representative UEs needs to be performed per cell. This causes huge overhead for inter-vendor collaboration.

- Double the Uu overhead: (i) for delivering/transferring the dataset, parameter, or reference model from the RAN to the UE, and (ii) for delivering/transferring the dataset, parameter, or reference model from the UE to the UE side server.

Proposal 7: Over-the-air delivery/transfer of the training dataset, parameter, or reference model to support UE-side model training is not a suitable or scalable solution. Please refer to the issues mentioned in observation 16.

Proposal 8: To minimize the inter-vendor collaboration for transferring the training dataset, parameter, or reference model to support UE-side model training while reducing the Uu overhead, the RAN is requested to discuss a solution, where

- The UE can provide required information for sharing the training dataset, parameter, or reference model with the UE-side to the serving gNB, thereafter

- Serving gNB can provide OAM or NF with the required information for sharing the training dataset, parameter, or reference model to the UE-side, thereafter

- OAM or NF then delivers/transfers the training dataset, parameter, or reference model with the UE-side using the information provided from the serving gNB.

Observation 17: Within the SA WGs, we have precedence where different types of data are shared with application function (within/outside the 3GPP network) and or edge cloud, for example,

- Target UE location to AF [TS 23.273],

- Analytics exposure from NWDAF to AF, e.g., network status reporting [TS 23.288],

- Traffic volume, UL/DL data rates, PDU session status, and others [TS 23.501].

Proposal 9: To minimize the inter-vendor collaboration while reducing the Uu overhead, RAN2 is requested to discuss how a standardized solution for offline delivery/transfer of training dataset, parameter, or reference model from the network entity (e.g., RAN/OAM/NF) to the UE-side training server can be supported.

Proposal 10: RAN2 is requested to discuss if model delivery/transfer of the UE-part model of two-sided models, i.e. z1, is supported for inference, considering the benefits in observation 6 and complexities in observation 7.

Proposal 11: Solution 4a is the model delivery/transfer solution for delivering/transferring the UE-part model of the two-sided models to the UE (at least when the UE-part model of the two-sided model is trained by the UE-side) for inference if RAN2 determines that model delivery/transfer case z1 is not supported.

Proposal 12: Deprioritize all control plane-based solutions for the delivery/transfer of the UE-part model of two-sided models (at least when the UE-part model of the two-sided model is trained by the UE-side) if RAN2 determines that model delivery/transfer case z1 is supported.

Proposal 13: Wait for RAN1 conclusion on whether options 3b/5b are supported for two-sided models, for the inference at the UE.

# Reference

- TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”

- RAN1#116 meeting report

- RAN1#116bis meeting report