3GPP TSG-RAN WG2 Meeting #127-bis	R2-2408394

Hefei, China, Oct 14th – Oct 18th, 2024

Agenda item:	8.3.3

Source:	Qualcomm Incorporated

Title:	Measurement Event prediction

WID/SID:	FS_NR_AIML_Mob – Release 19

Document for:	Discussion and Decision

# Introduction

In the last RAN2 meeting (RAN2 #127 [1]), the focus was on two topics – RLF prediction, Simulation results for RRM measurement prediction – and very good progress was made on both the topics [1].

In this contribution, we discuss Measurement Event prediction, which is the main component of handover prediction. We discuss the following aspects of Measurement Event prediction:

- AI/ML model for Measurement Event prediction including:

  - Indirect Measurement Event prediction,

  - Direct Measurement Event prediction,

- Performance KPIs of Measurement Event prediction,

- Use cases and scenarios of Measurement Event prediction.

# Measurement event prediction

Chair notes, RAN2 #125-bis [2] meeting

- Measurement event evaluation based on RRM measurement prediction will be studied. Direct measurement event prediction is also allowed.

- Start with Event A3 as a baseline.

- For the evaluation study, clarifications on the inputs to the model (e.g., UE position, if used) should be provided along with the results.

- Measurement event prediction study can start after some progress on RRM measurement prediction has been made.

In the previous RAN2 meeting (RAN2 #125-bis [2]), the following agreements were made on Measurement Event prediction.

In legacy L3 handover, network takes the handover decision based on measurement reports provided by the UE, e.g., event triggered measurement reports. Events A3 and A5 are the events that are relevant for handover. For the discussion below we consider Event A3, though we will have the proposals including both events.

Event A3 is satisfied at the UE when the following (entry) condition is met.

Candidate cell RSRP > Source cell RSRP + A3 offset + A3 hysteresis.

If the A3 condition is satisfied for the configured period of time timeToTrigger, then UE transmits a measurement report to the network. Upon receiving the measurement report, the source gNB performs handover preparation of the candidate cell with the target gNB and transmits an RRC reconfiguration message to the UE containing the handover command. UE performs handover on the candidate cell upon receiving the handover command. The parameters A3 offset, A3 hysteresis, and timeToTrigger are configured by the network.

There is a delay between the time the measurement report is transmitted by the UE and the handover command is received by the UE, which includes the time for handover preparation on the network side (see Figure 1 for the timeline related to the event, Event A3).

Figure 1: Event A3 and related timeline

In the previous RAN2 meeting (RAN2 #125-bis [2]), it was agreed to have both Indirect and Direct Measurement Event predictions.

Indirect Measurement Event prediction is based upon RRM measurement prediction (see Figure 2).

Proposal 1. In Indirect Measurement Event prediction, UE evaluates whether a configured measurement event will be triggered in the future, i.e., the measurement event condition is met for the configured timeToTrigger duration of time, based on RRM predictions of measurements of the source cell and the candidate cell. Only the triggering of measurement events related to handover – A3, A5 – are considered and studied.

Figure 2: Indirect Measurement Event prediction

Based on measurement predictions of the source cell and the candidate cell, UE can predict a time in the future when a configured event, e.g., Event A3, is likely to be triggered, i.e., when the measurement event condition is met for the configured timeToTrigger duration of time.

Proposal 2. In Indirect Measurement Event prediction, if the UE determines that a configured measurement event will be triggered in a prediction window of time into the future, then an output of Indirect Measurement Event prediction can be the predicted time of triggering of the measurement event.

The prediction window parameter is the same as has been agreed for RRM measurement prediction and is configured by the network.

It is assumed that the measurement predictions of the source cell and the candidate cell during the prediction window satisfy RRM prediction accuracy requirements that will have to be agreed upon by RAN2; otherwise, the measurement event prediction cannot be reliable.

Proposal 3. For the simulation studies, for the UE-side AI/ML model for Indirect Measurement Event prediction (see Figure 2), the possible inputs and outputs of the model are:

- Inputs: Beam and cell measurements of the source cell and the candidate cell.

- Outputs:

  - Whether a configured measurement event will be triggered in a prediction window of time into the future.

  - If a configured measurement event will be triggered in a prediction window of time into the future, the predicted time of triggering of the measurement event, or information related to the distribution (e.g., CDF) of the predicted time.

If the UE predicts that a configured measurement event will be triggered in the future as discussed in the above proposal, the UE should then transmit a message to the network providing this information, including the predicted time of triggering of the measurement event, which can be useful for the network, and the RRM measurement predictions of the source cell and the candidate cell for the prediction window. Upon receiving this message, the source gNB may perform handover preparation and transmit a handover command so that the UE can receive it on time, i.e., the source cell conditions are good enough that the UE can receive the handover command.

Observation 1. If the UE predicts that a configured measurement event will be triggered in the future as discussed in the above Proposal 3, the UE should then transmit a message to the network providing this information. Upon receiving this message, the source gNB may perform handover preparation and transmit a handover command so that the UE can receive it on time.

In Direct Measurement Event prediction, RRM measurement predictions of the source cell and the candidate cell are not used. In Direct Measurement Event prediction, we think that the inputs and outputs of the UE-side AI/ML model should be the same as for Indirect Measurement Event prediction (see Figure 3).

Figure 3: Direct Measurement Event prediction

Proposal 4. In Direct Measurement Event prediction, RRM measurement predictions of the source cell and the candidate cell are not used. The possible inputs and outputs of the UE-side AI/ML model can be the same as for Indirect Measurement Event prediction (see Figure 3).

Observation 2. It is up to UE implementation to consider other inputs, e.g., UE location, UE motion, for Indirect or Direct Measurement Event prediction.

Regarding handover performance KPIs, we think we should consider some system level KPIs and compare with the baseline case where AI/ML models are not used.

Proposal 5. The following are some of the handover performance KPIs to consider. These metrics are compared to the case where no AI/ML models are used.

- Rate of RLF occurrences,

- Difference between the predicted time and the actual time of triggering of the measurement event,

- Difference between the source and candidate cell RSRPs when the handover command is received by the UE.

Regarding the use cases and scenarios for simulation study of Measurement Event prediction, we think that we can start with the use cases and scenarios agreed for the RRM measurement prediction study.

Proposal 6. The simulation study of Measurement Event prediction is performed for the use cases and scenarios that have been agreed for RRM measurement prediction, e.g., for intra-frequency and inter-frequency mobility, FR1-to-FR1 mobility, and FR2-to-FR2 mobility.

The agreed list of parameters for the RRM measurement prediction simulation studies should be taken as baseline for the Measurement Event prediction studies.

Proposal 7. Regarding the deployment model, UE trajectory model, and UE speed, similar assumptions and parameters for the simulation study as for RRM measurement prediction can be adopted for the Measurement Event prediction study.

- The 7 sites, 3 sectors/cell per site deployment should be considered, and the UE should be allowed to move across the sites and cells to trigger multiple mobility events.

- SoftLoS update and shadow fading update modeling should be considered since UE moves across multiple sites and cells.

The initial simulation studies can focus on inter-cell mobility with different cells of the same frequency.

In RRM measurement prediction studies so far, inter-frequency, inter-cell simulation has been considered for the collocated scenario, where the cells are the same sector and are at the same site but correspond to different frequency layers.

In a later phase of the simulation studies, inter-cell mobility with different cells in different carrier frequencies should be considered. For this, RRM measurement prediction studies need to be extended to consider the more general case where the cells belong to different sites that are transmitting at different frequencies.

Proposal 8. The initial simulation studies can focus on inter-cell mobility with different cells of the same frequency.

In a later phase of the simulation studies, inter-cell mobility with different cells in different carrier frequencies should be considered (where the cells belong to different sites that are transmitting at different frequencies).

Proposal 9. For the simulation studies, Table 1 provides some recommended values of Mobility Event A3 parameters.

Table 1: Mobility Event A3 parameters

| Items | Description |
| --- | --- |
| TimeToTrigger [ms] | 40, 80, 128, 160, 256, 320, 480 |
| a3-offset [dB] | -1, 0, 1, 2, 3 |

# 3 Conclusions

Based on the above discussions, we recommend that RAN2 discuss the following observations and proposals.

Proposal 1. In Indirect Measurement Event prediction, UE evaluates whether a configured measurement event will be triggered in the future, i.e., the measurement event condition is met for the configured timeToTrigger duration of time, based on RRM predictions of measurements of the source cell and the candidate cell. Only the triggering of measurement events related to handover – A3, A5 – are considered and studied.

Proposal 2. In Indirect Measurement Event prediction, if the UE determines that a configured measurement event will be triggered in a prediction window of time into the future, then an output of Indirect Measurement Event prediction can be the predicted time of triggering of the measurement event.

The prediction window parameter is the same as has been agreed for RRM measurement prediction and is configured by the network.

Proposal 3. For the simulation studies, for the UE-side AI/ML model for Indirect Measurement Event prediction (see Figure 2), the possible inputs and outputs of the model are:

- Inputs: Beam and cell measurements of the source cell and the candidate cell.

- Outputs:

  - Whether a configured measurement event will be triggered in a prediction window of time into the future.

  - If a configured measurement event will be triggered in a prediction window of time into the future, the predicted time of triggering of the measurement event, or information related to the distribution (e.g., CDF) of the predicted time.

Observation 1. If the UE predicts that a configured measurement event will be triggered in the future as discussed in the above Proposal 3, the UE should then transmit a message to the network providing this information. Upon receiving this message, the source gNB may perform handover preparation and transmit a handover command so that the UE can receive it on time.

Proposal 4. In Direct Measurement Event prediction, RRM measurement predictions of the source cell and the candidate cell are not used. The possible inputs and outputs of the UE-side AI/ML model can be the same as for Indirect Measurement Event prediction (see Figure 3).

Observation 2. It is up to UE implementation to consider other inputs, e.g., UE location, UE motion, for Indirect or Direct Measurement Event prediction.

Proposal 5. The following are some of the handover performance KPIs to consider. These metrics are compared to the case where no AI/ML models are used.

- Rate of RLF occurrences,

- Difference between the predicted time and the actual time of triggering of the measurement event,

- Difference between the source and candidate cell RSRPs when the handover command is received by the UE.

Proposal 6. The simulation study of Measurement Event prediction is performed for the use cases and scenarios that have been agreed for RRM measurement prediction, e.g., for intra-frequency and inter-frequency mobility, FR1-to-FR1 mobility, and FR2-to-FR2 mobility.

The agreed list of parameters for the RRM measurement prediction simulation studies should be taken as baseline for the Measurement Event prediction studies.

Proposal 7. Regarding the deployment model, UE trajectory model, and UE speed, similar assumptions and parameters for the simulation study as for RRM measurement prediction can be adopted for the Measurement Event prediction study.

- The 7 sites, 3 sectors/cell per site deployment should be considered, and the UE should be allowed to move across the sites and cells to trigger multiple mobility events.

- SoftLoS update and shadow fading update modeling should be considered since UE moves across multiple sites and cells.

Proposal 8. The initial simulation studies can focus on inter-cell mobility with different cells of the same frequency.

In a later phase of the simulation studies, inter-cell mobility with different cells in different carrier frequencies should be considered (where the cells belong to different sites that are transmitting at different frequencies).

Proposal 9. For the simulation studies, Table 1 provides some recommended values of Mobility Event A3 parameters.

# References

[1] Chair notes, RAN2 #127 meeting.

[2] Chair notes, RAN2 #125-bis meeting.