3GPP TSG-RAN WG2 Meeting #127bis	R2-2408401

Hefei, China, Oct 14th – 18th, 2024

Source:         ZTE Corporation

Title:            Consideration on the intra-band EN-DC Channel Spacing

Agenda item:    7.24.1

Document for:   Discussion and Decision

- Introduction

About the capability for the intra-band EN-DC Channel Spacing, a LS [1] was received. Meanwhile, RAN4 has agreed a CR for the non-contiguous CA with nominal spacing case [2] as below.

### 5.4B.1 Channel spacing for intra-band EN-DC carriers The spacing between carriers will depend on the deployment scenario, the size of the frequency block available and the channel bandwidths. The nominal channel spacing between E-UTRA carrier and an adjacent NR carrier for intra-band contiguous EN-DC is defined as following: - For NR operating bands with 100 kHz channel raster, Nominal Channel spacing = (BWE-UTRA_Channel + BWNR_Channel)/2 - For NR operating bands with 15 kHz channel raster, - Nominal Channel spacing = (BWE-UTRA_Channel + BWNR_Channel)/2+{-5kHz, 0kHz, 5kHz} for ∆FRaster equals to 15 kHz - Nominal Channel spacing = (BWE-UTRA_Channel + BWNR_Channel)/2+{-10 kHz, 0 kHz, 10 kHz} for ∆FRaster equals to 30 kHz where BWE-UTRA_Channel and BWNR_Channel are the channel bandwidths of the E-UTRA and NR carriers, ∆FRaster is the band dependent channel raster granularity defined in TS38.101-1[2]. The channel spacing can be adjusted depending on the channel raster to optimize performance in a particular deployment scenario. For intra-band non-contiguous EN-DC the channel spacing between E-UTRA and NR carriers shall be larger than the nominal channel spacing defined in this clause. UE indicating [intraBandENDC-NominalSpacing] for a non-contiguous intra-band EN-DC configuration shall meet requirements for intra-band non-contiguous EN-DC with equal to or greater than nominal channel spacing.

In this paper, we’d like to share our further consideration based on the LS and RAN4’s CR.

- Discussion

In this chapter, we first focus on the DL case, then discuss both the DL and UL case.

For the DL capability defined below (or for the case that the DL and UL share the same capability): the UE can report 3 cases, i.e. contiguous only, non-contiguous only, and both.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| intraBandENDC-Support Indicates whether the UE supports intra-band (NG)EN-DC with only non-contiguous spectrum, or with both contiguous and non-contiguous spectrum for the (NG)EN-DC combination as specified in TS 38.101-3 [4]. If the UE does not include this field for an intra-band (NG)EN-DC combination the UE only supports the contiguous spectrum for the intra-band (NG)EN-DC combination. If intrabandENDC-Support-UL is absent and the band combination supports intra-band (NG)EN-DC only in DL, this field indicates the DL capability. If intrabandENDC-Support-UL is absent and the band combination supports intra-band (NG)EN-DC in DL and UL, this field indicates the common capability for both DL and UL. If intrabandENDC-Support-UL is included, intraBandENDC-Support indicates the DL capability. | BC | No | N/A | N/A |

With this legacy signalling:

- For the contiguous only case, it means that the UE only supports the nominal spacing with contiguous requirements;

- For the non-contiguous only case, it means that the UE only supports the spacing larger than the nominal spacing with non-contiguous requirements;

- For the “Both”, it means both the above 2 cases are supported.

Now, when a new capability (i.e. intraBandENDC-NominalSpacing) is introduced, there would be 4 cases summarized as below:

Table 1: Cases with New Capability Introduced

| Case | Requirement for the case with nominal spacing | Requirement for the case with spacing larger than nominal spacing |
| --- | --- | --- |
| Case 1: Contiguous only | Contiguous Requirement | Not supported |
| Case 2: non-contiguous only without supporting the new capability | Not supported | Non-contiguous requirement |
| Case 3: non-contiguous only with supporting the new capability (new) | Non-Contiguous Requirement | Non-contiguous requirement |
| Case 4: Both | Contiguous Requirement | Non-contiguous requirement |

From the table 1, we can see that the new case was introduced for that the non-contiguous requirements can be used for the nominal spacing. For the discussion convenience, we’d like to confirm that the companies share the same understanding on the new capability (i.e. intraBandENDC-NominalSpacing defined in the RAN4 spec).

Proposal 1: RAN2 to confirm whether the understanding of the legacy intraBandENDC-Support and the new capability (i.e. intraBandENDC-NominalSpacing defined in the RAN4 spec) in the Table 1 is correct.

Table 1: Cases with New Capability Introduced

| Case | Requirement for the case with nominal spacing | Requirement for the case with spacing larger than nominal spacing |
| --- | --- | --- |
| Case 1: Contiguous only | Contiguous Requirement | Not supported |
| Case 2: non-contiguous only without supporting the new capability | Not supported | Non-contiguous requirement |
| Case 3: non-contiguous only with supporting the new capability (new) | Non-Contiguous Requirement | Non-contiguous requirement |
| Case 4: Both | Contiguous Requirement | Non-contiguous requirement |

If the Table 1 is confirmed, we can further discuss it for the intrabandENDC-Support-UL.

| intrabandENDC-Support-UL Indicates whether the UE supports intra-band (NG)EN-DC in UL with only non-contiguous spectrum, or with both contiguous and non-contiguous spectrum for the intra-band (NG)EN-DC combination as specified in TS 38.101-3 [4]. The UE includes this field only if the UE supports different UL and DL capabilities for the intra-band (NG)EN-DC band combination. When 'both' is indicated in intrabandENDC-Support and in intraBandENDC-Support-UL, the UE supports the following three cases of intra-band (NG)EN-DC: contiguous DL/contiguous UL, non-contiguous DL/non-contiguous UL, contiguous DL/non-contiguous UL. For the inter-band (NG)EN-DC band combination with multiple intra-band (NG)EN-DC components as defined in clause 5.5B in the TS 38.101-3 [4]: - This field is applicable only if the UE supports the same spectrum contiguity capability in UL for all the intra-band (NG)EN-DC components. - If the UE supports different spectrum contiguity capabilities in UL for the intra-band (NG)EN-DC components, the UE shall not include this field. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

For the UL, if we take the new capability into consideration, the same logic as in the Table 1 shall be used, i.e. there would be 4 cases, the newly added case is intrabandENDC-Support-UL(non-contiguous) with new capability, which means for the spacing equal or larger than the nominal spacing, the non-contiguous requirement would be used.

Proposal 2: If the Table 1 was confirmed, RAN2 to confirm whether the same logic would be used for the UL case.

If the proposal 2 was confirmed, we need to further discuss whether we need to introduce the new capability for the DL and UL separately. Similar to the discussion on the introduction of the intrabandENDC-Support-UL, we need to analyse different combination cases.

For the legacy intrabandENDC-Support-UL, many combinations were discussed and the below Table was confirmed by the companies:

Table 2: intrabandENDC-Support for DL and UL case

| Scenario# | intraBandENDC-Support | intraBandENDC-Support-UL | UE supports in DL / UL (if applicable) |
| --- | --- | --- | --- |
| 1 | Absent (Contiguous) | Absent | - Contiguous/Contiguous |
| 2 | Absent (Contiguous) | Non-contiguous | - Contiguous/Non-contiguous NOTE: “Case 3” in R2-2300060 (R4-2220837) |
| 3 | Non-contiguous | Absent | - Non-contiguous/Non-contiguous |
| 4 | Both | Absent | - Contiguous/contiguous - Non-contiguous/Non-contiguous |
| 5 | Both | Non-contiguous | - Contiguous/Non-contiguous - Non-contiguous/Non-contiguous |
| 6 | Absent (Contiguous) | Both | - Contiguous/Contiguous - Contiguous/Non-contiguous NOTE: “Case 4” in R2-2300060 (R4-2220837) |
| 7 | Both | Both | - Contiguous/Contiguous - Non-contiguous/Non-contiguous - Contiguous/Non-contiguous |

Now when a new case was introduced as in the Table 1, we need to further discuss the DL and UL cases combination based on the above Table 1 and Table 2.

Table 3: intrabandENDC-Support for DL and UL case with new Capability

| Scenario# | intraBandENDC-Support | intraBandENDC-Support-UL | UE supports in DL / UL (if applicable) |
| --- | --- | --- | --- |
| 1 | Absent (Contiguous) | Absent | - Contiguous/Contiguous |
| 2 | Absent (Contiguous) | Non-contiguous without new capability | - Contiguous/Non-contiguous NOTE: “Case 3” in R2-2300060 (R4-2220837) |
| 3 | Absent (Contiguous) | Non-contiguous with new capability | - Contiguous/Non-contiguous requirement for the nominal spacing (Note 1) - Contiguous/Non-contiguous |
| 4 (original 6) | Absent (Contiguous) | Both | - Contiguous/Contiguous - Contiguous/Non-contiguous NOTE: “Case 4” in R2-2300060 (R4-2220837) |
| 5 (original 3) | Non-contiguous (without new Capability) | Absent | - Non-contiguous/Non-contiguous |
| 6 | Non-contiguous (with new capability) | Contiguous (absent ?) | - Non-contiguous requirement for the nominal spacing/ Contiguous (Note 1) |
| 7 | Non-contiguous (with new capability) | Non-contiguous (without new Capability) Or absent | Non-contiguous requirement for the nominal spacing / non-contiguous (Note 2) Non-contiguous/Non-contiguous |
| 8 | Non-contiguous (with new capability) | Non-contiguous (with new Capability) | non-contiguous requirements for the nominal spacing/ non-contiguous requirements for the nominal spacing non-contiguous requirements for the nominal spacing/ non-contiguous (Note 2) non-contiguous/ non-contiguous |
| 9 | Non-contiguous (with new capability) | Both | non-contiguous requirements for the nominal spacing/ contiguous (Note 1) non-contiguous requirements for the nominal spacing/ non-contiguous non-contiguous/ non-contiguous |
| 10 (original 4) | Both | Absent | - Contiguous/contiguous - Non-contiguous/Non-contiguous |
| 11 (original 5) | Both | Non-contiguous (without new capability) | - Contiguous/Non-contiguous - Non-contiguous/Non-contiguous |
| 12 (original 7) | Both | Both | - Contiguous/Contiguous - Non-contiguous/Non-contiguous - Contiguous/Non-contiguous |
| 13 | Both | Non-contiguous (with new capability) | - Contiguous/ non-contiguous requirements for the nominal spacing (Note 1) - Non-contiguous/Non-contiguous - Contiguous/Non-contiguous |
| Note 1: We need to discuss whether it’s allowed to adopt different requirements for DL and UL for the nominal spacing case. Note 2: We need to discuss whether it’s allowed to support nominal spacing with non-contiguous requirement on the DL but doesn’t support nominal spacing on the UL |  |  |  |

As Note1 described, for the case 3/6/9/13, different requirements would be adopted for the DL and UL on the nominal spacing case. Thus, we first need to confirm whether it’s allowed.

Proposal 3: RAN2 to confirm whether it’s allowed to report different requirements for DL and UL for the nominal spacing case (i.e. DL non-contiguous requirements and UL with contiguous requirements and vice visa).

If not allowed, then the case 3/9/13 would be the same as the case 1/7/11 respectively, while the case 6 is disabled.

If allowed, there would be some signalling issue for the case 6, the current signalling can’t indicate contiguous case explicitly but the absent would means the same capability as the DL.

intraBandENDC-Support-UL ENUMERATED {non-contiguous, both} OPTIONAL

Thus, from the signalling aspect, at least the case 6 can’t be supported.

Observation 4: From the signalling aspect, the case 6 is unable to be supported.

Based on the observation 4, to simplify the discussion, we prefer not to support the UE to report different requirements for DL and UL for the nominal spacing case if there is no practical use case.

Proposal 3a: Not to support the UE to report different requirements for DL and UL for the nominal spacing case (i.e. DL non-contiguous requirements and UL with contiguous requirements and vice visa) if there is no practical use case.

For the Note 2, it means that the UE can support the DL with non-contiguous requirement for the nominal spacing but doesn’t support UL with nominal spacing. For such kind of case, it seems reasonable thus the case 7/8 shall be kept and the table can be further updated as below:

Table 4: intrabandENDC-Support for DL and UL case with new Capability updated

| Scenario# | intraBandENDC-Support | intraBandENDC-Support-UL | UE supports in DL / UL (if applicable) |
| --- | --- | --- | --- |
| 1 | Absent (Contiguous) | Absent | - Contiguous/Contiguous |
| 2 | Absent (Contiguous) | Non-contiguous without new capability | - Contiguous/Non-contiguous NOTE: “Case 3” in R2-2300060 (R4-2220837) |
| 3 (original 6) | Absent (Contiguous) | Both | - Contiguous/Contiguous - Contiguous/Non-contiguous NOTE: “Case 4” in R2-2300060 (R4-2220837) |
| 4 (original 3) | Non-contiguous (without new Capability) | Absent | - Non-contiguous/Non-contiguous |
| 5 | Non-contiguous (with new capability) | absent | - Non-contiguous requirement for the spacing / non-contiguous - Non-contiguous/Non-contiguous |
| 6 | Non-contiguous (with new capability) | Non-contiguous (with new Capability) | - non-contiguous requirements for the nominal spacing/ non-contiguous requirements for the nominal spacing - non-contiguous requirements for the nominal spacing/ non-contiguous - non-contiguous/ non-contiguous |
| 7 (original 4) | Both | Absent | - Contiguous/contiguous - Non-contiguous/Non-contiguous |
| 8 (original 5) | Both | Non-contiguous (without new capability) | - Contiguous/Non-contiguous - Non-contiguous/Non-contiguous |
| 9 (original 7) | Both | Both | - Contiguous/Contiguous - Non-contiguous/Non-contiguous - Contiguous/Non-contiguous |

Proposal 4: If the proposal 3a was confirmed, Ran2 further confirm the Table 4 with the new capability introduced.

If the proposal 4 was confirmed, the new capability shall only be used for the case 5 and case 6 in the Table 4, and obviously, we only need to introduce 1 capability for the both DL and UL and it’s only used for the case that the non-contiguous was explicitly indicated.

Proposal 5: If the proposal 4 was confirmed, RAN2 further confirm that introduce 1 new capability for the both DL and UL and it’s only used for the case that the non-contiguous was explicitly indicated.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: RAN2 to confirm whether the understanding of the legacy intraBandENDC-Support and the new capability (i.e. intraBandENDC-NominalSpacing defined in the RAN4 spec) in the Table 1 is correct.

Table 1: Cases with New Capability Introduced

| Case | Requirement for the case with nominal spacing | Requirement for the case with spacing larger than nominal spacing |
| --- | --- | --- |
| Case 1: Contiguous only | Contiguous Requirement | Not supported |
| Case 2: non-contiguous only without supporting the new capability | Not supported | Non-contiguous requirement |
| Case 3: non-contiguous only with supporting the new capability (new) | Non-Contiguous Requirement | Non-contiguous requirement |
| Case 4: Both | Contiguous Requirement | Non-contiguous requirement |

Proposal 2: If the Table 1 was confirmed, RAN2 to confirm whether the same logic would be used for the UL case.

Proposal 3: RAN2 to confirm whether it’s allowed to report different requirements for DL and UL for the nominal spacing case (i.e. DL non-contiguous requirements and UL with contiguous requirements and vice visa).

Observation 4: From the signalling aspect, the case 6 in the Table 3 is unable to be supported.

Proposal 3a: Not to support the UE to report different requirements for DL and UL for the nominal spacing case (i.e. DL non-contiguous requirements and UL with contiguous requirements and vice visa) if there is no practical use case.

Proposal 4: If the proposal 3a was confirmed, Ran 2 further confirm the Table 4 with the new capability introduced.

Table 4: intrabandENDC-Support for DL and UL case with new Capability updated

| Scenario# | intraBandENDC-Support | intraBandENDC-Support-UL | UE supports in DL / UL (if applicable) |
| --- | --- | --- | --- |
| 1 | Absent (Contiguous) | Absent | - Contiguous/Contiguous |
| 2 | Absent (Contiguous) | Non-contiguous without new capability | - Contiguous/Non-contiguous NOTE: “Case 3” in R2-2300060 (R4-2220837) |
| 3 (original 6) | Absent (Contiguous) | Both | - Contiguous/Contiguous - Contiguous/Non-contiguous NOTE: “Case 4” in R2-2300060 (R4-2220837) |
| 4 (original 3) | Non-contiguous (without new Capability) | Absent | - Non-contiguous/Non-contiguous |
| 5 | Non-contiguous (with new capability) | absent | - Non-contiguous requirement for the spacing / non-contiguous - Non-contiguous/Non-contiguous |
| 6 | Non-contiguous (with new capability) | Non-contiguous (with new Capability) | - non-contiguous requirements for the nominal spacing/ non-contiguous requirements for the nominal spacing - non-contiguous requirements for the nominal spacing/ non-contiguous - non-contiguous/ non-contiguous |
| 7 (original 4) | Both | Absent | - Contiguous/contiguous - Non-contiguous/Non-contiguous |
| 8 (original 5) | Both | Non-contiguous (without new capability) | - Contiguous/Non-contiguous - Non-contiguous/Non-contiguous |
| 9 (original 7) | Both | Both | - Contiguous/Contiguous - Non-contiguous/Non-contiguous - Contiguous/Non-contiguous |

Proposal 5: If the proposal 4 was confirmed, RAN2 further confirm that introduce 1 new capability for the both DL and UL and it’s only used for the case that the non-contiguous was explicitly indicated.

- Reference

- R4-2410693 LS on intra-band EN-DC channel spacing  To RAN2

- R4-2414423 R18 Cat-F CR 38.101-3 channel spacing for intra-band EN-DC[Intra-Band_EN-DC_Channelspacing]