3GPP TSG-RAN WG2#127bis											   R2-24xxxxx

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.9.4

Source: 	NEC

Title:	        Discussion on PWS broadcast for NB-IoT

Document for:	 Discussion and Decision

- Introduction

Considering the nature characteristic of satellite broadcast, supporting emergency broadcast message is beneficial. Hence, for NB-IoT, broadcast of PWS message can be supported as below:

The aim of this WI is enhancement of IoT-NTN with the following objectives: - Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In this contribution, we will discuss how to support broadcast of PWS message and provide our views.

- Discussion

In LTE, paging message is used to inform the UE other than BL UE or UE in CE about the presence of an ETWS primary notification, ETWS secondary notification, and one or more CMAS notifications. The corresponding indication is as below in paging message.

Paging ::=					SEQUENCE {

pagingRecordList				PagingRecordList					OPTIONAL,	-- Need ON

systemInfoModification			ENUMERATED {true}					OPTIONAL,	-- Need ON

etws-Indication					ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension			Paging-v890-IEs						OPTIONAL

}

Paging-v920-IEs ::=			SEQUENCE {

cmas-Indication-r9				ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension			Paging-v1130-IEs					OPTIONAL

}

When the UE receives a Paging message including the etws-Indication/cmas-Indication , it shall start receiving the ETWS primary notification/ETWS secondary notification/CMAS notification according to schedulingInfoList contained in SystemInformationBlockType1.

For BL UE and UE in CE, direct indication information can be used to inform the presence of an ETWS primary notification/ETWS secondary notification/one or more CMAS notifications. The details for ETWS and CMAS indication are as below:

Table 6.6-1: Direct Indication information using P-RNTI

| Bit | Direct Indication information |
| --- | --- |
| 1 | systemInfoModification |
| 2 | etws-Indication |
| 3 | cmas-Indication |
| 4 | eab-ParamModification |
| 5 | systemInfoModification-eDRX |
| 6 | uac-ParamModification |
| 6, 7, 8 | Not used, and shall be ignored by UE if received. |

- Observation 1: The ETURAN informs UE other than BL UE and UE in CE about the presence of ETWS and CMAS with the indication field in the paging message.

- Observation 2: The ETURAN informs BL UE and UE in CE about the presence of ETWS and CMAS with direct indication information.

According to the WID, broadcast of PWS can reuse LTE mechanism. Then for NB-IoT UE, two indication ways above may be considered, such as paging message, direct indication information. As we know, direct indication information is transmitted by DCI. If it is used, the UE does not need to decode the PDSCH, such that receiving heavy repetitions of paging message can be avoided similar to eMTC. Besides, for NB-IoT, direct indication information has been introduced to indicate system information update and there are some reserved bits. Hence, for NB-IoT, it is better to add ETWS and CMAS indication fields in the Direct indication information as below.

Table 6.7.5-1: Direct Indication information

| Bit | Field in Direct Indication information |
| --- | --- |
| 1 | systemInfoModification |
| 2 | systemInfoModification-eDRX |
| 3 | etws-Indication |
| 4 | cmas-Indication |
| 5, 6, 7, 8 | Not used, and shall be ignored by UE if received |

- Proposal 1: For NB-IoT, the indication field for ETWS and CMAS should be added in the current Direct Indication information table using unused bit fields.

- In LTE, ETWS primary notification, secondary notification, and CMAS notification can be transmitted by SIB 10/11/12. Then for NB-IoT UE, similarly, the corresponding ETWS and CMAS notifications are also needed by system information. Hence, new SIB messages including ETWS primary notification, ETWS secondary notification, and CMAS notification should be introduced.

- Proposal 2: New SIB messages including ETWS primary notification, ETWS secondary notification, and CMAS notification should be introduced.

- Conclusion

This contribution provides our analysis and views for broadcast of PWS message, the following observations and proposals are listed below:

- Observation 1: The ETURAN informs UE other than BL UE and UE in CE about the presence of ETWS and CMAS with the indication field in paging message.

- Observation 2: The ETURAN informs BL UE and UE in CE about the presence of ETWS and CMAS with direct indication information.

- Proposal 1: For NB-IoT, indication field for ETWS and CMAS should be added in the current Direct Indication information table using unused bit fields.

- Proposal 2: New SIB messages including ETWS primary notification, ETWS secondary notification, and CMAS notification should be introduced.

- References

[1] RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3.