3GPP TSG-RAN WG2 Meeting #127bis	R2-2408413

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.9.3

Source: 	NEC

Title:	        Discussion on UL Capacity Enhancement for IoT-NTN

Document for:	 Discussion and Decision

- Introduction

The objective of the support of capacity enhancements for uplink in the WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3 is given below [1]:

- Support of Capacity enhancements for uplink - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

In RAN2#127, the Msg3 transmission without msg1/ Random Access Response (RAR) has reached the following agreements [2]:

Agreements:

1.	RAN2 will continue working on a CB-msg3 EDT-like mechanism

2.	RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts

3.	In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1.

4.	For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition.

In RAN1#118 and RAN4#112, RAN1[3] and RAN2 [4] reply to our LS R2-2407463, respectively. The reply can be found in the appendix section.

In this contribution, we continue discussing the CB-msg3 EDT-like mechanism on 1) the performance comparison between SA, DSA, CRDSA and OCC, 2) the down selection on the CB-msg3 mechanisms, 3) the question to ask to RAN1, 4) the remaining issues on UL synchronization for CB-msg3 without Msg1/Msg2, 5) CB-msg3 resource configuration, 6) contention resolution, 7) efficient delivery of DL message and 8) failure handling.

- Discussion

  - SA, DSA and CRDSA performance comparison

To compare the performance of SA, DSA, and CRDSA CB-msg3, we adopt the system assumption from Table 1 of [5] as the baseline. Since link-level simulation is beyond the scope of RAN2, the parameters for evaluating throughput and packet loss rate under Config#1 are listed in the table below.

Table. 1 System-level simulation assumption

| Sub-carrier spacing (SCS) | 3.75 kHz |
| --- | --- |
| Number of sub-carriers ( Nsc ) | 48 |
| CB-msg3 TBS ( Nbitspkt ) | 328 bits |
| Modulation | p/4-QPSK |
| Config#1 |  |
| Single CB-msg3 duration ( TCB-msg3 ) | 96 ms |
| FEC Coding rate ( r ) | 0.61 |
| Number of CB-msg3 occasions ( Nopp ) | 3 |
| Maximum number of replicas ( Kmax ) | 3 |
| CRDSA SIC iterations ( Niter ) | 4 |

It's important to note that the packet loss rate mentioned here is a different metric from the BLER in PHY. It reflects the rate of unsuccessful user transmissions. Therefore, it would be more appropriate to illustrate the overload factor versus packet loss rate on a linear axis rather than a logarithmic axis.

From a system-level perspective, 1) the maximum throughput shows the capacity of a scheme in all working circumstances; 2) the maximum overload factor with a target packet loss rate (e.g. 50%) shows the robustness of a scheme against a large number of users accessing the network simultaneously.

Only a robust scheme with high capacity could enhance the CB-msg3 capacity and cut off unnecessary retransmissions and delays.

SA/DSA/CRDSA without repetition

In the coverage enhancement study for Rel-17/18, the target Block Error Rate (BLER) for the PUSCH channel is 10%. For the coverage-limited IoT NTN scenario, especially when repetition is not considered, it is reasonable for RAN2 to simulate the system-level performance by assuming a BLER of 10% as the SA/DSA/CRDSA CB-msg3 link-level performance (especially without comprehensive simulations in RAN1). In our simulations, we consider both cases of the performance with link-level errors (BLER = 10%) and without errors.

Regarding DSA/CRDSA, there are 3 CB-msg3 occasions (Nopp = 3), so the maximum number of replicas is limited to 3. To further reduce user interference, the terminal randomly choosing the replica number within the maximum number of replicas (i.e. K = 1, 2, or 3) is proposed. Thus, fixing the replica number as 3 (K=3) and choosing the replica number randomly within 3 (KMAX=3) are evaluated accordingly.

For CRDSA, the SIC interaction number is 4 (Niter = 4), which fully exploits the CRDSA capacity. Due to the receiver errors (BLER = 10%) and CRC miss detections in each iteration, a false detection can cause the entire iteration cancellation chain to fail.

Observation 1: An error in decoding one CRDSA attempt, whether in the header part or the payload part, will lead to errors in the interference cancellation chain and affect the decoding of other users.

For the header part of the CRDSA, using the same header of all the replicas benefits lower the false detection rate and 18 bits is required to indicate the time-frequency position of the 3 occasions with 48 subcarriers.

Observation 2: CRDSA requires an 18-bit additional overhead (18/328 = 5.5%) for each packet to enable the receiver to do interference cancellation for Nopp =3 cases.

The overload factor is defined as the number of users divided by the maximum number of users supported by a physical resource to access orthogonally in the time-frequency domain, which reflects the active user density. Using the parameters in Table. 1, the overload factor is calculated as active user number / (48*3) = active user number / 144.

The comprehensive simulation results for throughput and packet loss rate against the overload factor are illustrated in Fig. 1 and Fig. 2.

Fig. 1 SA, DSA, and CRDSA throughput comparison without repetition

Fig. 2 SA, DSA, and CRDSA packet loss rate comparison without repetition

Based on simulation results in Fig.1 and Fig.2, we have the following observations and proposals:

Observation 3: Both the DSA and CRDSA show better performance with KMax configuration than with the K fixed configuration from the middle to high user density region (DSA > 40%, CRDSA > 75%) due to the time diversity gain.

Proposal 1: Support UE in randomly selecting the replica number under the maximum replica number to enhance DSA/CRDSA performance in the IoT-NTN high-user density scenario.

Observation 4: Detection errors negatively impact the following cancellation iterations in CRDSA, reducing overall system throughput and increasing packet loss rate, especially in the best CRDSA throughput region  (60% < Overload Factor < 90%).

Proposal 2: CB-msg3 performance evaluation should take into account the BLER constraint from PHY.

In the following comparisons, we will only consider schemes with realistic restrictions, such as a BLER of 10%.

Observation 5: CRDSA demonstrates the best throughput performance under the overload factor region of around 70% when the CRDSA transmission opportunity and the maximum attempt number are both 3.

Observation 6: After setting a packet loss rate lower than 50% as the target, DSA can only work when the overload factor is lower than 52%, while SA and CRDSA can work under 60% and 88% overloading factor, respectively.

Observation 7: The maximum throughput of DSA is 44 kbps, while SA and CRDSA are 54 kbps and 74 kbps, respectively.

Observation 8: Both SA and CRDSA demonstrate more robust throughput and packet loss rate performance across all overloading factor regions compared to DSA.

If the IoT-NTN network implements strict initial user access control to allow DSA to work in a less overloaded region, it means that excluded users will have to perform the CB-msg3 at a later occasion. This user access control introduces delays in the system, similar to high overload factor schemes with the HARQs / retransmissions, but it also loses the network's guidance on which UE should perform the retransmission. To build a resilient IoT-NTN network and support a larger number of IoT devices accessing the network simultaneously, it is essential to have robust performance against various overloading factors.

Observation 9: A scheme with strict initial user access control will introduce delays similar to high overload factor schemes, but it also loses the network's guidance on which UE should back off or retransmit.

Proposal 3: For Rel-19 CB-msg3 in IoT-NTN, RAN2 deprioritize DSA.

More simulation results and observations under Nopp =3 are attached in Appendix 5.1.

SA with repetition

IoT-NTN has limited uplink coverage due to the constrained uplink budget of an IoT device, and it is important to use repetition to ensure the uplink PHY performance and meet the NPUSCH coverage requirement.

OCC expands the orthogonal resources from the time-frequency domain to the time-frequency-code domain. Currently, RAN1 is studying NPUSCH capacity enhancement with OCC. Slot-level and/or symbol-level OCC will be specified.

Here, the SA with repetition number =4 and SA with OCC-length =4 on the 4 repetitions are evaluated for CB-msg3.  The evaluation results of the throughput and the packet loss rate against the overload factor of the SA and SA with OCC are presented in Fig. 3 and Fig. 4. Here, the overloading factor is still equal to active user number / 144, as the orthogonal time-frequency resource for one CB-msg3 transmission with repetition is still the same. The repetition will enhance the BLER performance from PHY, and we assume the BLER is reduced to 1% after 4 repetitions.

Fig. 3 SA without OCC and SA with OCC throughput performance comparison

Fig. 4 SA without OCC and SA with OCC packet loss ratio performance comparison

Observation 10: The packet loss rate of SA with OCC increases linearly as the overload factor rises from 0% to 200%. This rate is lower than that of SA without OCC in all overload factor ranges and less than half of SA without OCC's packet loss rate when the overload factor exceeds 100%.

Observation 11: SA with OCC demonstrates superior throughput compared to SA without OCC across all overload factor ranges. The throughput is twice as high when the overload factor is at 90% and five times higher when the overload factor reaches 200%.

Proposal 4: RAN2 should consider implementing OCC to further improve the CB-msg3 capacity, especially if SA/CRDSA CB-msg3 repetition is supported.

Other Mechanism

Under Configure#1, the single CB-msg3 duration is 96ms. When considering repetition and multiple replicas for CRDSA, the NPUSCH transmission duration will be 96*repetition number*KMAX ms (e.g. 96*4*4 = 1536ms ). The half-duplex NB-IoT device might lose the downlink synchronization or miss the downlink signaling from the network during such a long CB-msg3 transmission.

Observation 12: CRDSA with repetition will require a long uplink transmission duration which might result in desynchronization of the DL and missing detection of the DL signling.

Here, we provide an alternative mechanism, which uses code domain diversity instead of the time domain diversity in legacy SA/DSA/CRDSA. For a Nopp=4, Kmax=4 system configuration, a terminal will choose one subcarrier and one OCC codeword index to transmit the CB-msg3 multiplexing with an OCC-length =4 codewords. Then for a 96*4 ms and 48 subcarrier time-frequency resource, it supports 48*4 = 192 users to orthogonally access in code-frequency domain or time-frequency domain. This mechanism exchanges the time-diversity gain for repetition gain and uses the code domain freedom to compensate for the time-domain freedom, which is named as code-domain ALOHA (CA).

The throughput and the packet loss performance of SA, DSA, CRDSA, and CA are evaluated and presented in Fig. 5 and Fig.6.

Fig. 5 SA, DSA. CRDSA and CA throughput performance comparison

Fig. 6 SA, DSA. CRDSA and CA packet loss rate performance comparison

- Observation 13: CA demonstrates packet loss rates of lower than 50%, when the overload factor is lower than 90%. For DSA, SA, and CRDSA, the corresponding overload factors are 70%, 80%, and 115%.

- Observation 14: In a large overload factor region (80%-200%), CA maintains a robust throughput of around 60 kbps and achieves higher throughput than SA and DSA in almost all overloading regions. CRDSA reaches peak throughput when the overload factor is under 90% but experiences throughput loss with an overload factor larger than 125%.

- Observation 15: The main differences between SA, DSA, CRDSA, and CA are listed in Table.2

Table. 2

| Scheme | SA | DSA | CRDSA | CA |
| --- | --- | --- | --- | --- |
| Robust performance with high user density | Yes | No | No | Yes |
| No additional assistance overhead for interference cancellation | Yes | Yes | No | Yes |
| No RAN1 specification impact on repetition | No | No | No | Yes |

- Proposal 5: RAN2 should further discuss the use of CA as the CB-msg3 mechanism, especially if SA/CRDSA CB-msg3 repetition is not supported.

- Proposal 6: Ask RAN1 whether the discussion on NPUSCH capacity enhancement with OCC applies to CB-msg3 without msg1/2.

- Proposal 7: Ask RAN1 whether channel estimation will pose a challenge for implementing interference cancellation for CRDSA CB-msg.

- Proposal 8: Ask RAN1, for IoT-NTN CB-msg3, whether the repetition of SA/DSA/CRDSA will result in downlink desynchronization or violate other physical layer constraints.

- Proposal 9: Ask RAN1, whether the 10% BLER for SA/DSA/CRDSA without repetition and 1% BLER for CA and SA/DSA/CRDSA with repetition are suitable for the system-level evaluation of CB-msg3 throughput and packet loss ratio. If not, please recommend the proper value from link-level.

  - UL synchronization for CB-msg3 transmission without Msg1/Msg2

From the RAN1 reply[3], the pre-compensated TA can meet the requirement for CB-Msg3, if the CB-Msg3 is NB-IoT with 3.75kHz SCS or eMTC CE mode A

From the RAN4 reply[4], using pre-compensated TA may result in a timing error larger than half of the cyclic prefix (CP) for NB-IoT with a 15kHz SCS.

- Observation 16: With NTA = 0, pre-compensated TA leads to a timing error larger than half CP for NB-IoT with 15kHz SCS and eMTC CE mode B. RAN1 may further discuss the cases of NB-IoT with 15kHz SCS and eMTC CE mode B.

For the requirement for uplink capacity enhancement, the WID emphasizes that IoT-NTN, in particular NB-IoT, will have to support massive capacity in terms of number and types of UE, some of which with worse characteristics than others (e.g. low cost devices, wearables, etc) [1]. For different types of UEs, NB-IoT with 3.75 KHz SCS, NB-IoT with 15 KHz SCS, eMTC CE mode A and eMTC CE mode B should be considered.

- Proposal 10: RAN2 should consider NB-IoT with 3.75 KHz SCS, NB-IoT with 15 KHz SCS, eMTC CE mode A and eMTC CE mode B for further discussion regarding CB-msg3.

  - CB PUSCH resource

Regarding CB-msg3 resource, it should be a contention based PUSCH resource according to the past agreement. For CB-msg3 resource configuration, the following two options may be considered:

- Option 1: CB PUSCH resource is configured by broadcast signaling similar to PRACH resource of EDT procedure.

- Option 2: CB PUSCH resource is configured by dedicated signaling, such as RRC release message similar to PUR procedure.

For option 1, the PUSCH resource is configured by system information, so when the camped cell changes, the CB-msg3 EDT procedure is still used. However, this option can increase collision probability because common resources can be used for all UEs within the cell.

For option 2, the collision probability may be controlled by configuring a shared UE number for a configured PUSCH resource using dedicated signaling. However, compared with option 1, when the camping cell changes, dedicated PUSCH resource configuration will become invalid, especially in NGSO systems. Some corresponding solving schemes are needed, for example, how to make dedicated PUSCH resource valid in a cell other than the one that configured it.

- Proposal 11: For CB-msg3 PUSCH resource configuration, consider the following two options:

- Option 1: CB PUSCH resource is configured by broadcast signaling

- Option 2: CB PUSCH resource is configured by dedicated signaling

For CB PUSCH resource configuration, similar to EDT, different coverage levels should be supported. Additionally, considering multi-carrier operation in NB-IoT, CB PUSCH resources in anchor carrier and different non-anchor carriers should also be taken into account.

- Proposal 12: Support CB PUSCH resources for different coverage levels and different carriers.

  - Contention resolution

After the first UL message is transmitted, the UE needs to determine whether the transmission using CB-msg3 is successful or not by the contention resolution during a configured time window. For legacy EDT procedure, RA-RNTI and preamble can identify a group of UEs, and then contention resolution ID is used to further identify each UE. For PUR, a PUR-RNTI can be configured for each UE per PUR resource.

For CB-msg3, a new RNTI may be introduced in two ways:

- Option 1: A pre-configured RNTI may be used for one UE or multiple UEs (further contention resolution is needed).

- Option 2: RNTI is calculated according to a predefined rule.

After sending CB-msg3, the UE needs a contention resolution timer and a corresponding defined UE behavior to start and stop the timer.

If a CB-msg3 RNTI is shared by multiple UEs, a contention resolution ID should be used for further contention resolution. The legacy 48-bit contention resolution ID may then be reused for CB-msg3.

- Proposal 13: For CB-msg3 RNTI, consider the following two options:

- Option 1: A pre-configured RNTI for UE using CB-msg3

- Option 2: RNTI determined by PUSCH resource

- Proposal 14: Introduce contention resolution timer to receive the DL message after CB-msg3

  - Efficient delivery of DL message

In the context of CP CIoT optimizations, if there is no pending DL data and signaling, the NW may send L1 ACK containing TAC or L2 MAC containing TAC to the UE to update the TAC and terminate the PUR procedure. If there is DL data, the NW can send the RRCEarlyDataComplete message to the UE. For UP CIoT optimizations, the NW can send t RRCConnectionRelease message to keep the UE in RRC_IDLE state.

In the context of the CB-msg3 procedure, if there is no pending DL data or signaling and contention resolution ID is needed, L2 ACK may be used. If a dedicated RNTI is used, contention resolution ID is not needed, and then L1/L2 ACK may be used.

For CP CIoT optimizations with pending DL data and UP CIoT optimizations, the NW sends the RRCEarlyDataComplete or RRCConnectionRelease message to the UE.

- Proposal 15: For CP CIoT optimizations, when there is no pending DL data and signaling, L1/L2 ACK may be used for efficient delivery.

Besides, if calculated RNTI or RNTI shared by multiple UEs is used, DL messages from multiple UEs with the same RNTI may be multiplexed into one MAC PDU for more DL efficient delivery.

- Proposal 16: If calculated RNTI or pre-configured RNTI shared by multiple UEs is used, data multiplexing from multiple UEs may be used for efficient delivery.

  - Failure handling and fallback

The behavior of the UE needs to be defined if it fails to receive the corresponding contention resolution/response in the contention resolution window, similar to the 2 or 4-step RACH procedure. One potential solution is to allow the UE to re-initiate CB-msg3 transmission for a configured number of times before declaring complete failure. Then, whether and how to implement power control and power ramping for CB-msg3 transmission and retransmission should be investigated.

The network must control and distribute the load over the CB resources for CB-msg3 transmission. For instance, when the load is temporarily too high, the network could instruct the UE to apply a backoff for upcoming transmission over CB resources. A procedure for the network to generally control a reasonable amount of UEs to use CB-msg3 transmission and configure a proper amount of CB PUSCH resources for it should be introduced. However, this may not solve the issue of high user density in the long term.

If CB-msg3 fails, the UE can also fall back to the normal RACH-based solution, similar to the 2-step RACH falling back to 4-step RACH. The conditions and triggers of this fallback need to be discussed, for example, after the maximum time of CB-msg3 retransmission or after receiving the fallback command from the network.

- Proposal 17: RAN2 to discuss the following failure handling and fallback issues:

- CB-msg3 retransmission

- NW controlled backoff

- The condition/trigger for a UE to fallback from CB-msg3 to RACH-based solution

- Conclusion

This contribution provides our analysis and views for the CB-msg3, the following observations and proposals are listed below:

Observation 1: An error in decoding one CRDSA attempt, whether in the header part or the payload part, will lead to errors in the interference cancellation chain and affect the decoding of other users.

Observation 2: CRDSA requires a 18-bit additional overhead (18/328 = 5.5%) for each packet to enable the receiver to do interference cancellation for Nopp =3 cases.

Observation 3: Both the DSA and CRDSA show better performance with KMax configuration than with the K fixed configuration from the middle to high user density region (DSA > 40%, CRDSA > 75%) due to the time diversity gain.

Observation 4: Detection errors negatively impact the following cancellation iterations in CRDSA, reducing overall system throughput and increasing packet loss rate, especially in the best CRDSA throughput region  (60% < Overload Factor < 90%).

Observation 5: CRDSA demonstrates the best throughput performance under the overload factor region of around 70% when the CRDSA transmission opportunity and the maximum attempt number are both 3.

Observation 6: After setting a packet loss rate lower than 50% as the target, DSA can only work when the overload factor is lower than 52%, while SA and CRDSA can work under 60% and 88% overloading factor, respectively.

Observation 7: The maximum throughput of DSA is 44 kbps, while SA and CRDSA are 54 kbps and 74 kbps, respectively.

Observation 8: Both SA and CRDSA demonstrate more robust throughput and packet loss rate performance across all overloading factor regions compared to DSA.

Observation 9: A scheme with strict initial user access control will introduce delays similar to high overload factor schemes, but it also loses the network's guidance on which UE should back off or retransmit.

Observation 10: The packet loss rate of SA with OCC increases linearly as the overload factor rises from 0% to 200%. This rate is lower than that of SA without OCC in all overload factor ranges and less than half of SA without OCC's packet loss rate when the overload factor exceeds 100%.

Observation 11: SA with OCC demonstrates superior throughput compared to SA without OCC across all overload factor ranges. The throughput is twice as high when the overload factor is at 90% and five times higher when the overload factor reaches 200%.

Observation 12: CRDSA with repetition will require a long uplink transmission duration which might result in desynchronization of the DL and missing detection of the DL signaling.

- Observation 13: CA demonstrates packet loss rates of lower than 50%, when the overload factor is lower than 90%. For DSA, SA, and CRDSA, the corresponding overload factors are 70%, 80%, and 115%.

- Observation 14: In a large overload factor region (80%-200%), CA maintains a robust throughput of around 60 kbps and achieves higher throughput than SA and DSA in almost all overloading regions. CRDSA reaches peak throughput when the overload factor is under 90% but experiences throughput loss with an overload factor larger than 125%.

- Observation 15: The main differences between SA, DSA, CRDSA, and CA are listed in Table.2

Table. 2

| Scheme | SA | DSA | CRDSA | CA |
| --- | --- | --- | --- | --- |
| Robust performance with high user density | Yes | No | No | Yes |
| No additional assistance overhead for interference cancellation | Yes | Yes | No | Yes |
| No RAN1 specification impact on repetition | No | No | No | Yes |

- Observation 16: With NTA = 0, pre-compensated TA leads to a timing error larger than half CP for NB-IoT with 15kHz SCS and eMTC CE mode B. RAN1 may further discuss the cases of NB-IoT with 15kHz SCS and eMTC CE mode B.

Proposal 1: Support UE in randomly selecting the replica number under the maximum replica number to enhance DSA/CRDSA performance in the IoT-NTN high-user density scenario.

Proposal 2: CB-msg3 performance evaluation should take into account the BLER constraint from PHY.

Proposal 3: For Rel-19 CB-msg3 in IoT-NTN, RAN2 deprioritize DSA.

Proposal 4: RAN2 should consider implementing OCC to further improve the CB-msg3 capacity, especially if SA/CRDSA CB-msg3 repetition is supported.

- Proposal 5: RAN2 should further discuss the use of CA as the CB-msg3 mechanism, especially if SA/CRDSA CB-msg3 repetition is not supported.

- Proposal 6: Ask RAN1 whether the discussion on NPUSCH capacity enhancement with OCC applies to CB-msg3 without msg1/2.

- Proposal 7: Ask RAN1 whether channel estimation will pose a challenge for implementing interference cancellation for CRDSA CB-msg.

- Proposal 8: Ask RAN1, for IoT-NTN CB-msg3, whether the repetition of SA/DSA/CRDSA will result in downlink desynchronization or violate other physical layer constraints.

- Proposal 9: Ask RAN1, whether the 10% BLER for SA/DSA/CRDSA without repetition and 1% BLER for CA and SA/DSA/CRDSA with repetition are suitable for the system-level evaluation of CB-msg3 throughput and packet loss ratio. If not, please recommend the proper value from link-level.

- Proposal 10: RAN2 should consider NB-IoT with 3.75 KHz SCS, NB-IoT with 15 KHz SCS, eMTC CE mode A and eMTC CE mode B for further discussion regarding CB-msg3.

- Proposal 11: For CB-msg3 PUSCH resource configuration, consider the following two options:

- Option 1: CB PUSCH resource is configured by broadcast signaling

- Option 2: CB PUSCH resource is configured by dedicated signaling

- Proposal 12: Support CB PUSCH resources for different coverage levels and different carriers.

- Proposal 13: For CB-msg3 RNTI, consider the following two options:

- Option 1: A pre-configured RNTI for UE using CB-msg3

- Option 2: RNTI determined by PUSCH resource

- Proposal 14: Introduce contention resolution timer to receive the DL message after CB-msg3.

- Proposal 15: For CP CIoT optimizations, when there is no pending DL data and signaling, L1/L2 ACK may be used for efficient delivery.

- Proposal 16: If calculated RNTI or pre-configured RNTI shared by multiple UEs is used, data multiplexing from multiple UEs may be used for efficient delivery.

- Proposal 17: RAN2 to discuss the following failure handling and fallback issues:

- CB-msg3 retransmission

- NW controlled backoff

- The condition/trigger for a UE to fallback from CB-msg3 to RACH-based solution

- References

[1] RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3.

[2] Draft_R2_126_Meeting_Report_v1.

[3] R1-2407548, Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2.

[4] R4-2414114, Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2.

[5] R2-2407502, Discussion on DSA and CRDSA Performance, ESA, Eutelsat Group, Viasat, Inmarsat, Novamint, Echostar, Sateliot.

- Appendix

  - More Simulation Results

In this section, we have compiled all the simulation results with Nopp = 3 together in the two figures below.

It's important to remember that the packet loss rate mentioned here is a different metric from the BLER in PHY. It reflects the rate of unsuccessful user transmissions. Therefore, it would be more appropriate to illustrate the overload factor vs packet loss rate on a linear axis rather than a logarithmic axis.

If we consider the target that over half of the users should be able to successfully access the network and only take into account schemes with realistic restrictions (i.e. BLER = 10%), CRDSA (Kmax =3) allows more users to access the network simultaneously. Among the other schemes, the order in terms of maximum user number would be CRDSA (Kmax =2), CRDSA (K =2), SA, DSA (Kmax = 3), and DSA (K=3).

The comparison also shows similar results in terms of throughput. We are only considering schemes with realistic restrictions (i.e. BLER = 10%). Among these, CRDSA (Kmax = 3) achieves the highest maximum throughput of 82 kbps. In comparison, the other schemes rank in terms of maximum throughput as follows: CRDSA (K=2), CRDSA (Kmax =2), SA, and DSA (K=3).

When comparing CRDSA CRDSA (Kmax =2) with CRDSA (K =2), the maximum throughput difference is only around 4 kbps. However, CRDSA is more robust in high overload conditions, which is crucial for building a resilient network.

For DSA, the maximum throughput is 40 kbps, which is less than half of the CRDSA (Kmax=3) and around 72% of the SA. It's important to note that DSA can only operate in an extremely low overload factor region, which can cause significant issues for the network in terms of user initial access control and preventing breakdowns when dealing with a large number of users.

  - WID

Need for Uplink capacity enhancement: NB-IoT NTN is already being deployed live at this very moment. In these early and upcoming deployments, it is clearly emerging that IoT-NTN, in particular NB-IoT, will have to support massive capacity in terms of number and types of UE, some of which with worse characteristics than others (e.g. low cost devices, wearables, etc). Multiplexing of UEs by usage of orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH should therefore be studied and if beneficial being specified.

Therefore, in order to unlock the additional UL capacity potential, there is a need to identify methods to de-couple the UL from the DL as much as possible.

Apart from enhancements above, reducing the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction could be beneficial, therefore should be studied and if beneficial, specified.

- Support of Capacity enhancements for uplink

  - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]:

    - Msg3 transmission without msg1/ Random Access Response (RAR)

    - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

    - Study and specify RRM requirement, if identified [RAN4]

  - LS reply from RAN1

1. Overall Description: RAN1 would like to thank RAN2 for the LS in R1-2405786/R2-2405769 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2. Regarding RAN2’s questions, following is the RAN1’s response: Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2? RAN1’s response: From RAN1 perspective, assuming Note 1 and Note 2 are satisfied, an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy requirement for Msg3 transmission/reception without Msg1/Msg2 at least in the following cases: - NB-IoT with 3.75kHz SCS. - eMTC CE mode A. Note 1: RAN1 assumes NTA=0 for Msg3 transmission without Msg1/Msg2 Note 2: RAN1 assumes that the RAN4 UL synchronization requirements specified in Table 7.20A.2-1 and Table 7.24A.2-1 of TS 36.133 apply to the Msg3 transmission without Msg1/Msg2 in NB-IoT over NTN and eMTC over NTN, respectively. Note 3: RAN1 may further discuss the cases of NB-IoT with 15kHz SCS and eMTC CE mode B

  - LS reply from RAN4

# 1. Overall Description: RAN4 would like to thank RAN2 for the LS R2-2405769 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2. RAN2 asked for RAN4 following questions: Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2? Q2: If the answer for Q1 is no, from RAN4 and RAN1 perspective, how the required timing accuracy for Msg3 transmission can be satisfied in this case? RAN4’s feedback: For Msg3 transmission without Msg1/Msg2, UE can meet UE UL transmit timing requirements defined for IoT NTN in TS 36.133[attached in Annex] from UE RRM requirement definition point of view. Note 1: RAN4 assumed that NTA_Ref is not configured by the network for MSG3 transmission without Msg1/Msg2. RAN4 will follow the conclusion of the other working groups in the future, if any. Note 2: For NB-IoT with 15kHz SCS, the timing error may be larger than half CP.