3GPP TSG-RAN WG2 Meeting #127b	 R2-2408416

Hefei, China, Oct 14-18, 2024

Agenda Item:	8.4.3

Source: 	NEC

Title:  	Discussion on LP-WUS RRM

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#127 meeting, the following agreement related to LP-WUS RRM are made:

RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs.

For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

    - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

    - The exit condition is based on the LR measurement results.

In this contribution, we further discuss remaining issue related to criteria for measurement offloading and relaxation.

- 2 Discussion

- 2.1 General measurement framework

According to RAN4#111 meeting, RAN4 made the following agreement:

Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |

RAN4 to further discuss case #2 to #4:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |
| #4 Relaxed case c | Off | On, FFS the condition and the details | ON |

In next RAN4#112 meeting, further agreement are made:

Agreement:

- Support case #3

  - For serving cell measurement, further discuss:

    - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR

    - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR

    - Option 3: It is up to UE implementation whether to combine the measurement across the two radios.

    - Other options are not precluded.

| RRM measurement case index | MR serving cell measurement | MR neighbouring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

  - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility.

Based on RAN4 understanding, we can see that measurement (fully) offloading (i.e., no any MR measurement for both serving cell and neighbour cell) and relaxed case 3 (i.e., relaxed MR measurement for both serving cell and neighbour cell) are selected for further discussions, therefore, we RAN2 should further consider the case related criterion and other potential issue based on selected case.

Proposal-1: RAN2 to consider measurement offloading case (i.e., no any MR measurement for both serving cell and neighbour cell) and relaxed case 3 (i.e., relaxed MR measurement for both serving cell and neighbour cell) identified by RAN4.

- 2.2 Measurement relaxation/offloading criterion

Fig. 1 measurement offloading/relaxation framework

From RAN2 perspective, we need to specify the criterion for measurement offloading and relaxation. Furthermore according to TS 38.304 and 38.133, there is already specified measurement rule and relaxation as shown below:

General principle of legacy measurement rule for cell re-selection

- If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ:

- the UE may not perform intra-frequency measurements;

- If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ:

- the UE may not to perform measurements of NR inter-frequency cells of equal or lower priority;

- the UE shall search for inter-frequency of higher priority at least every Thigher_priority_search ([60 * Num of higher priority frequency] seconds).

General principle of legacy measurement relaxation

If the UE has not stopped measurement based on measurement rule for cell re-selection, then we go with the following:

- if one of the low-mobility and not-at-cell-edge criterion is fulfilled:

- the UE may choose to perform relaxed measurements according to relaxation factor K1 (3);

- if both low-mobility and not-at-cell-edge criterion are fulfilled:

- the UE is not required to meet the measurement requirement for 1 hour.

NOTE: higher priority frequency measurement relaxation further depends on whether Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ.

Back to LP-WUS measurement offloading and relaxation, the intention is to design a further relaxed measurement for LP-WUS capable UE compared with legacy measurement mechanism. To our understanding, baseline principle should be as long as the UE enters LP-WUS monitoring mode, the UE should not turn on the MR frequently for measurement reason.

Measurement relaxation

It is noted that RAN4 currently only support case 3, in this case measurement relaxation means the UE still need to turn on MR but in a less frequent way to perform MR measurement on both serving cell and neighbor cell for cell re-selection. As said before, as long as the UE is entering LP-WUS monitoring mode, MR should not turn on as regular measurement requirement, therefore, we can consider LP-WUS monitoring entry/exit condition as measurement relaxation criterion, i.e., the UE perform measurement relaxation when the LP-WUS monitoring entry condition is fulfilled and exit from measurement relaxation when exit condition is fulfilled.

Proposal-2: RAN2 to consider LP-WUS monitoring entry/exit condition as measurement relaxation criterion for measurement relaxation case 3 (i.e., relaxed measurement on both serving cell and neighbor cell).

However, some impact from the legacy measurement relaxation (i.e., not-at-cell-edge) could be foreseen, for example, if both legacy measurement relaxation and LP-WUS specific measurement relaxation are configured, what if the UE fulfills the LP-WUS specific measurement relaxation but not fulfil the legacy measurement relaxation (it means the UE cannot perform the legacy relaxed measurement), what is the UE behavior then? Therefore, RAN2 is suggested to consider the relationship between legacy measurement relaxation and LP-WUS specific measurement relaxation.

Proposal-3: RAN2 to further consider the relationship between legacy measurement relaxation and LP-WUS specific measurement relaxation.

Measurement offloading

Measurement offloading means there is no any MR measurement for both serving cell and neighbour cell, it should be careful. We need to ensure that the serving cell signal quality of the UE should be good enough to support measurement offloading. Otherwise, if the UE cannot derive neighbour cell signal quality but the serving cell signal quality is not stable, mobility problem could happen. There is no doubt that criterion of measurement offloading should be stricter than measurement relaxation. Since we consider using LP-WUS monitoring entry/exit condition as the criterion of measurement relaxation, the threshold of measurement offloading should be higher than the threshold of LP-WUS monitoring entry/exit condition.

Proposal-3: the entry condition of measurement offloading should be stricter than that one of measurement relaxation.

Again, the impact from legacy measurement rule for cell re-selection should also be considered, for example, if both legacy measurement rule for cell re-selection and measurement offloading are configured, what if the UE fulfills the measurement offloading but not fulfil the legacy measurement rule (it means the UE still need to perform intra/inter-frequency measurement), what is the UE behavior then? Therefore RAN2 is suggested to consider the relationship between legacy measurement rule for cell re-selection and measurement offloading.

Proposal-4: RAN2 to further consider the relationship between legacy measurement rule for cell re-selection and measurement offloading.

- 3 Conclusions

Proposal-1: RAN2 to consider measurement offloading case (i.e., no any MR measurement for both serving cell and neighbour cell) and relaxed case 3 (i.e., relaxed MR measurement for both serving cell and neighbour cell) identified by RAN4.

Proposal-2: RAN2 to consider LP-WUS monitoring entry/exit condition as measurement relaxation criterion for measurement relaxation case 3 (i.e., relaxed measurement on both serving cell and neighbor cell).

Proposal-3: RAN2 to further consider the relationship between legacy measurement relaxation and LP-WUS specific measurement relaxation.

Proposal-3: the entry condition of measurement offloading should be stricter than that one of measurement relaxation.

Proposal-4: RAN2 to further consider the relationship between legacy measurement rule for cell re-selection and measurement offloading.

- 4 References