3GPP TSG-RAN WG2 Meeting #127b	 R2-2408417

Hefei, China, Oct 14-18, 2024

Agenda Item:	8.4.4

Source: 	NEC

Title:  	Discussion on LP-WUS in RRC_CONNECTED

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#127 meeting, the following agreement related to LP-WUS in RRC_CONNECTED are made:

For option 1-2,

    - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.)

    - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

In this contribution, we further discuss the detailed design for LP-WUS in RRC_CONNECTED.

- 2 Discussion

- 2.1 Option 1-2

In last RAN2#127 meeting, we merge option 1-2-1 and option 1-2-2 into one solution, i.e., option 1-2. The general understanding of option 1-2 is to say the LP-WUS monitoring is occurred outside of legacy C-DRX active timer and LP-WUS can trigger a new active timer during which UE monitors PDCCH.

There is one FFS for the active timer triggered by LP-WUS, whether it is to reuse legacy timer (e.g., drx-InactivityTimer) or introduce a new timer. To our understanding, there is no too much difference between legacy timer and new timer, it both means the UE start this timer and monitor PDCCH until timer expiration or some mechanism to stop it. But we slightly prefer introducing new active timer, the reason is that when we reuse legacy timer, e.g., drx-InactivityTimer, it also means we need to reuse the same value as drx-InactivityTimer, which makes thing not flexible. If new active timer is configured, the NW could flexibly adjust different timer value, therefore it is more promising.

Proposal-1: for option 1-2, after LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts a newly defined timer, when the timer is running, the UE monitors PDCCH.

- 2.2 The general procedure of LP-WUS monitoring

Working Assumption

From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

- Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

- Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

- FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE

Note: Above can be revisited considering RAN2 decisions.

Based on RAN1 working assumption, now we have two clear solution, solution 1-1 is similar to DCP mechanism, and solution 1-2 is to monitor LP-WUS outside of active timer to trigger a new active timer for PDCCH monitoring. There is one FFS to consider whether/how to combine option 1-1 and option 1-2.

To our understanding, there is no problem to support a combination of optional 1-1 and 1-2, because no matter the active timer is drx-onDurationTimer or new active timer, there is no unnecessary power consumption due to the fact that both active timer should be control by LP-WUS indication. Furthermore drx-onDurationTimer should be configured as it reflect UE traffic mode which is determined by NW implementation, and new active timer triggered by LP-WUS could alleviate the latency issue.

One issue to support this combination is how to configure LP-WUS mapping relationship between new active timer and drx-onDurationTimer. Some LP-WUS will trigger a new active timer, and some LP-WUS will trigger drx-onDurationTimer. Do we need two types of LP-WUS occasion configuration (i.e., part of LP-WUS is offset specific to drx-onDurationTimer, part of LP-WUS is located anywhere outside of C-DRX active time)? We think this makes things complicated and not needed. In fact we can just provide only one type of LP-WUS occasion configuration, and the periodicity of C-DRX cycle is integer multiple of periodicity of LP-WUS. Then the closet LP-WUS to next drx-onDurationTimer will be used to trigger drx-onDurationTimer and other LP-WUS is used to trigger new active timer. Details about what is closet LP-WUS could be FFS. The whole picture is shown as Fig.1.

Fig.1 combination of option 1-1 and 1-2

Proposal-2: support combination of option 1-1 and option 1-2, i.e., LP-WUS can be used to trigger new active timer and legacy drx-onDurationTimer, when the new active timer and legacy drx-onDurationTimer is running, the UE monitors PDCCH. FFS the LP-WUS triggering details.

- 3 Conclusions

Proposal-1: for option 1-2, after LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts a newly defined timer, when the timer is running, the UE monitors PDCCH.

Proposal-2: support combination of option 1-1 and option 1-2, i.e., LP-WUS can be used to trigger new active timer and legacy drx-onDurationTimer, when the new active timer and legacy drx-onDurationTimer is running, the UE monitors PDCCH. FFS the LP-WUS triggering details.

- 4 References