3GPP TSG-RAN WG2 Meeting #127b	 R2- 2408419

Hefei, China, Oct 14-18, 2024

Agenda Item:	8.3.2

Source: 	NEC

Title:  	Discussion on some clarification for RRM measurement prediction

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#127 meeting, the following agreements related to RRM measurement prediction are made:

Agreements

=>	companies are encouraged to considers both prediction from low-frequency cell to high-frequency cell and prediction from high-frequency cell to low-frequency cell, but only low to high is expected.

=>	For the agreed frequencies for inter-frequence case, only one UE speed is considered for inter-frequency prediction in simulation, e.g., 30km/h.  Companies can consider other speeds for other frequencies if they chose to simulate them.

=>	For temporal domain case B prediction the input is historical measurement values and the output is the values at the subsequent time instances that measurement is skipped, i.e., the prediction is always after the measurement and is at future time instance(s).

Agreements

1 	To keep two filtering options on the table and up to company to report.

2	One fixed sampling period of FR2 is introduced for L1/L3 filtering option 1 to replace existing one i.e., 20ms. The detail value is FFS.

3	In the definition of 3 RRM sub-cases, all cell level measurement result(s) refers to L3 filtered cell level measurement

4 	continue to discuss following issues in the post email discussion:

1, Further clarification of intra-frequency of temporal domain case A and case B.

2, The set of observation vs prediction window parameters for intra-frequency temporal domain case A and case B

3, The number of TX and RX for FR1 and FR2

4, Filtering co-efficient for beam level prediction

Agreements

Companies can consider to do L3 filtered beam level results for any of this cases.  L3 filtered beam level prediction cases are lower priority.

Case 1: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results.

Case 2: To directly predict L3 filtered beam level results based on the L3 beam level measurement results.

Case 3: To directly predict L3 filtered beam level results based on the L1 beam level measurement results.

If companies do L3 filtered beam level prediction simulations, they should focus on FR2-to-FR2 intra-frequency temporal domain prediction case A

In this contribution, we want to provide non-simulation aspects, to make some clarifications for RRM prediction aspects.

- 2 Discussion

- 2.1 Measurement prediction type

RAN2 has agreed that the cell level measurement prediction as following:

For cell level measurement prediction model, at least consider the following cases:

Case 1: To predict beam level results, then generate cell level results based on the predicted beam results;

Case 2: To directly predict cell level results based on cell level results.

Case 3: To directly predict cell level results based on beam level results

Fig.1 measurement model

The agreed use cases can facilitate the AI model framework design, but more details are still needed. According to TS 38.300 measurement model (Fig.1), for case 1, only L1 beam level result can be used as model input to inference L1 beam level result. For case 2, in last meeting, we agreed that “in the definition of 3 RRM sub-cases, all cell level measurement result(s) refers to L3 filtered cell level measurement”, therefore, only L3 cell level result can be used as model input/output to inference L3 cell level result and normally the cell level mentioned in our RRC spec only means L3 cell level. For case 3, only L1 beam level result can be used as model input to inference L3 cell level result, note that in this case, the layer 3 filtering procedure is inside AI inference. Furthermore, there is still raw beam (i.e., A) can be considered instead of L1 beam, our understanding is that since layer 1 filtering procedure is UE implementation, it is not clear whether/how we use it, therefore, it is preferred that assume L1 beam level result as model input. Based on the analysis we can modify the agreement as the following to make it clearer.

Proposal-1: modify the cell level measurement prediction case as following:

For cell level measurement prediction model, at least consider the following cases:

Case 1: To predict L1 beam level results, then generate cell level results based on the predicted L1 beam level results;

Case 2: To directly predict cell level results based on cell level results;

Case 3: To directly predict cell level results based on L1 beam level results.

NOTE: for case 3, the layer 3 filtering procedure is internal implementation of AI inference;

- 2.2 Clarifications for prediction scenarios

Based on the discussion in RAN2 so far, basically we have the following scenarios related to cell level prediction:

| Intra-frequency-intra-cell temporal domain case A/B | intra-F-intra-C-T-Case A/B |
| --- | --- |
| Intra-frequency-intra-cell spatial domain | intra-F-intra-C-S |
| Intra-frequency-inter-cell temporal domain case A/B | intra-F-inter-C-T-Case A/B |
| Intra-frequent-inter-cell spatial domain | intra-F-inter-C-S |
| Inter-frequency-inter-cell temporal domain case A/B | inter-F-inter-C-T-Case A/B |
| Inter-frequency-inter-cell spatial domain (frequency domain) | inter-F-inter-C-S |

Besides these scenarios, we also need to combine them with the prediction cases mentioned above. We give some examples below:

Intra-F-intra-C-T-Case A/B with case 1: for the same cell (could be serving cell and neighbour cell), by measuring historical L1 beam results to predict future L1 beam results, and the difference between case A and B can follow the definition of case A and B discussed in [Post-127][30] email discussion [1].

Intra-F-intra-C-T-Case A/B with case 2: for the same cell, by measuring historical L3 cell results to predict future L3 cell results, and the difference between case A and B can follow the definition of case A and B discussed in [Post-127][30] email discussion [1].

Intra-F-intra-C-T-Case A/B with case 3: for the same cell, by measuring historical L1 beam results to predict future L3 cell results, different type of input and output is involved, definition of case A and B need FFS.

Intra-F-intra-C-S with case 1: similar to RAN1 BM-Case 1

Intra-F-intra-C-S with case 2: invalid

Intra-F-intra-C-S with case 3: for the same cell, by measuring subset of L1 beam results, to inference L3 cell results.

Intra-F-inter-C-T-Case A/B with case 1: whether it is still worth to study? For different cell (e.g., cell-1 as input and cell-2 as output), by measuring cell-1 beam results, to inference future cell-2 beam results, different cell result is involved, definition of case A and B need FFS.

Intra-F-inter-C-T-Case A/B with case 2: whether it is still worth to study? For different cell (e.g., cell-1 as input and cell-2 as output), by measuring cell-1 results, to inference future cell-2 results, different cell result is involved, definition of case A and B need FFS.

Intra-F-inter-C-T-Case A/B with case 3: whether it is still worth to study? for different cell (cell-1 as input and cell-2 as output), by measuring cell-1 beam results, to inference future cell-2 level result, different cell result is involved, definition of case A and B need FFS.

Here we just listed some example from all scenarios, as we can see there are so many scenarios combination, and different company may have different understanding on each scenario, which makes the discussion so messed. Noted that even though we give the clear definition for each scenario combination, it does not necessarily mean we need to study all of the scenarios, in fact some cases are not worth to study, but down-selection can be made after we have clear definition. Therefore, RAN2 is suggested to give the clear definition of the following prediction scenarios.

Proposal-2: RAN2 is suggested to give the clear definition of the following prediction scenarios:

intra-F-intra-C-T-Case A/B + Case 1/2/3

intra-F-intra-C-S + Case 1/3

intra-F-inter-C-T-Case A/B + Case 1/2/3

intra-F-inter-C-S + Case 1/2/3

inter-F-inter-C-T-Case A/B + Case 1/2/3

inter-F-inter-C-S (frequency-domain) + Case 1/2/3

- 3 Conclusions

Proposal-1: modify the cell level measurement prediction case as following:

For cell level measurement prediction model, at least consider the following cases:

Case 1: To predict L1 beam level results, then generate cell level results based on the predicted L1 beam level results;

Case 2: To directly predict cell level results based on cell level results;

Case 3: To directly predict cell level results based on L1 beam level results.

NOTE: for case 3, the layer 3 filtering procedure is internal implementation of AI inference;

Proposal-2: RAN2 is suggested to give the clear definition of the following prediction scenarios:

intra-F-intra-C-T-Case A/B + Case 1/2/3

intra-F-intra-C-S + Case 1/3

intra-F-inter-C-T-Case A/B + Case 1/2/3

intra-F-inter-C-S + Case 1/2/3

inter-F-inter-C-T-Case A/B + Case 1/2/3

inter-F-inter-C-S (frequency-domain) + Case 1/2/3

- 4 References

[1] R2-2407849