3GPP TSG-RAN WG2 Meeting #127bis											R2-2408420

Hefei, China, October 14th – 18th, 2024

Agenda Item:	8.11.2

Source: 	Sharp

Title:  	Random Access Procedures for SBFD

Document for: 	Discussion

# Introduction

In RAN2#127 meeting, RAN2 made the following working assumption and agreements:

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS. FFS whether/how early indication is used during a SBFD RA procedure. RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed.

Meanwhile, in RAN1#118 meeting, RAN1 made some relevant agreements:

Agreement For SBFD-aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs Agreement For SBFD-aware UEs and RACH configuration Option 2, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs Agreement For SBFD aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure Agreement For SBFD aware UEs and RACH configuration Option 2, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure

This document discusses open issues on random access procedure for SBFD.

# Discussion:

## Early Indication

The first issue is whether an early indication of SBFD capability during the random access using legacy RO is needed. We see that an early indicating of SBFD capability during the random access is beneficial. One of the most important benefits of SBFD is latency reduction compared to legacy TDD. An early indication enables the benefit of latency reduction during the random access.

An early indication in legacy ROs during RA procedure is supported to indicate UE’s SBFD capability.

Regarding how to support the early indication, we need to consider the earliest way which is during Msg1 transmission. If the early indication is provided during Msg1 transmission, then gNB can allocate an uplink grant in SBFD symbols for Msg3 transmission. Moreover, the early indication during Msg1 transmission is useful if RAN2 agrees to support RO fallback operation from additional RO in SBFD symbols to legacy RO. In short, an early indication during Msg1 is desirable. Specifically, we can consider RACH preamble partitioning by a simple extension of existing RACH preamble partitioning.

One may argue that an early indication in Msg3 by LCID differentiation can be considered. However, the latency reduction gain is smaller than Msg1. Also, LCID differentiation may require multiple LCID consumptions, so it is not desirable.

As an early indication, RACH preamble partitioning of legacy ROs to indicate UE’s SBFD capability is used.

## RO Selection Condition for Initial Transmission

For RO selection among additional RO in SBFD symbols and legacy RO, an issue is not to cause too strong cross link interference. If the UE selects an additional RO without consideration of the interference or coverage, it will lead unnecessarily many RA attempts with power ramp-up.

We think that potential cross link interference aggressor UEs should not utilize additional ROs in SBFD symbols. To achieve this, we can reuse well-known RSRP-based criteria for RO selection. More specifically, if the measured RSRP is higher than a new RSRP threshold for RO selection, which is provided upon PRACH configuration, the UE can select additional ROs. Otherwise, the UE selects legacy ROs. With this approach, SBFD-aware UE will not select additional ROs if the UE is expected to use high transmission power for PRACH by checking the RSRP threshold for RO selection.

An SBFD-aware UE selects either additional RO or legacy RO based on comparing measured RSRP with a RSRP threshold for RO selection in case of initial RA attempt.

## Fallback to Different RO

If a UE selects an additional RO for the initial PRACH transmission attempt and cannot change the RO type in the subsequent PRACH transmission reattempts, probability of getting random access problems will become higher. For example, if the UE experiences strong cross link interference while the UE keeps selecting the additional ROs, even if the channel quality of the legacy ROs is good enough to successfully complete the random access, the UE will issue random access problem.

At least for CBRA, this issue cannot be solved by the network control or network configuration. This issue needs to be solved by the UE side. In order to avoid random access problem, the UE should try both RO types before declaring the random access problem.

We see a fallback approach is beneficial. More specifically, if a few first PRACH transmission attempts use additional ROs in SBFD symbols which would be affected by cross link interference, then it is natural and straightforward to fall back to the legacy ROs. With this approach, gNB can control the degree of power ramp-up while using additional ROs to some extent. By an appropriate threshold configuration, the gNB can reduce too much power ramp-up in the PRACH transmission attempts using additional ROs.

A fallback procedure on RO type selection from additional RO in SBFD symbols to legacy RO is supported when the number of PRACH transmission attempts exceed a threshold.

Conversely, additional ROs in SBFD symbols can be less crowded as legacy UEs are not aware of them. Thus, a fallback procedure from legacy RO to additional RO can be also useful from offloading perspective.

RAN2 to discuss whether to support a fallback procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold.

# Conclusions

Based on the discussion above, RAN2 is requested to discuss and agree the following proposals:

- An early indication in legacy ROs during RA procedure is supported to indicate UE’s SBFD capability.

As an early indication, RACH preamble partitioning of legacy ROs to indicate UE’s SBFD capability is used.

An SBFD-aware UE selects either additional RO or legacy RO based on comparing measured RSRP with a RSRP threshold for RO selection in case of initial RA attempt.

A fallback procedure on RO type selection from additional RO in SBFD symbols to legacy RO is supported when the number of PRACH transmission attempts exceed a threshold.

RAN2 to discuss whether to support a fallback procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold.