3GPP TSG-RAN WG2 Meeting #127bis											R2-2408422

Hefei, China, October 14th – 18th, 2024

Agenda Item:	8.7.4.2

Source: 	Sharp

Title:  	Considerations for DSR Enhancements

Document for: 	Discussion

# Introduction

In RAN#105, WID on Phase 3 XR has been updated to have a normative objective for enhanced DSR reporting as follows [1]:

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

In RAN2#127 during the study phase of DSR enhancement, RAN2 made the following agreements:

For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

This document discusses detailed procedures of the DSR enhancements.

# Discussion

Figure 1. DSR MAC CE Format defined in 3GPP 38.321 [2]

Figure 1 shows a DSR MAC CE format defined in Rel-18 XR WI. In the MAC CE, each logical channel group can include at most one pair of remaining time and buffer size. A common remaining time threshold is used for both DSR triggering and determination of contents of the DSR. The new objective approved in RAN#105 is to report multiple pairs. An open issue is how the Rel-18 remaining time threshold is associated with the multiple pairs of remaining time and buffer size.

In our view, it is possible to de-couple contents of the new DSR with multiple pairs from existing DSR triggering and definition of delay-critical data defined in Rel-18. More specifically, new delay thresholds used for the new DSR with multiple pairs can be separately configured by an RRC message. Figure 2 shows how the separate multiple thresholds work. In the example, two threshold values 10ms and 30ms for DSR contents are configured whereas Rel-18 remaining time threshold used for triggering DSR is 20ms. After a DSR is triggered by data with remaining time below the Rel-18 remaining time threshold (=20ms), the buffer size and remaining time are calculated when the DSR is multiplexed in the MAC PDU. Since two threshold values are configured for DSR contents, pairs of remaining time and buffer size are reported for two zones. The first zone is for data with remaining time is greater than or equal to 0ms (Threshold 0) but smaller than 10ms (Threshold 1). The corresponding shortest remaining time is 4ms and buffer size is 210 bytes (= 100 + 150 + 30). The second zone is for data with remaining time is greater than or equal to 10ms (Threshold 1) and smaller than 30ms (Threshold 2). The corresponding shortest remaining time is 12ms and buffer size is 319 bytes.

It is possible that one of threshold value is the same as Rel-18 remaining time threshold. Also, the network may configure threshold value greater than remaining time threshold. It can mandate the UE to report remaining time and buffer size for non-delay critical data. We can leave all flexibility up to the network.

There is no impact on definition of delay critical data and DSR triggering condition defined in Rel-18. Rel-18 DSR triggering condition triggers Rel-19 DSR.

Separate delay threshold values are configured for Rel-19 DSR MAC CE with multiple pairs of remaining time and buffer size.

It is up to the network whether to configure the delay threshold for UE to report buffer size of non-delay critical data.

Figure 2. Example of DSR reporting with multiple pairs of remaining time and buffer size

# Conclusions

Based on the discussion given above, RAN2 is requested to agree the following proposals:

- There is no impact on definition of delay critical data and DSR triggering condition defined in Rel-18. Rel-18 DSR triggering condition triggers Rel-19 DSR.

Separate delay threshold values are configured for Rel-19 DSR MAC CE with multiple pairs of remaining time and buffer size.

It is up to the network whether to configure the delay threshold for UE to report buffer size of non-delay critical data.

# Reference

[1] RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur)

[2] 3GPP TS 38.321 NR MAC specification