3GPP TSG-RAN WG2 Meeting #127bis	R2-2408431

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.3.3

Source:	Mediatek Inc.

Title:	Evaluation methodology, scenario and simulation assumption for measurement event prediction

Document for:	Discussion, Decision

Introduction

For AI/ML RRM prediction, in the #125bis meeting, RAN2 made the following agreement：

Agreements:

- At least measurement event evaluation based on RRM measurement prediction result will be studied.   Direct measurement event prediction is also allowed.

- Clarifications on what is being as input should be provided with results.

- Start with A3 as a baseline.

- Measurement event prediction study can start after some further progress on RRM measurement prediction has been made.

In this contribution, we explore the motivations behind measurement event prediction, define the concepts of indirect and direct event prediction, examine how AI-driven predictions can enhance overall mobility performance, delve into the mechanisms underlying both indirect and direct event prediction, and clarify the metrics and simulation assumptions for performance evaluation.

Discussion

Motivation

The goal of RRM measurement event prediction is to improve mobility system performance, the question is what the potential scenarios are where AI-driven event prediction can be beneficial, what measurement event prediction entails and how to utilize the outcome of measurement event prediction to improve mobility performance in these scenarios.

With the evolution of cellular technology and the adoption of higher frequency bands like FR2 and beamforming, the handover can increase even further due to dynamic channel conditions, dense deployments, and smaller cell sizes. Legacy L3 HO mechanism introduces TTT to reduce unnecessary HO and ping-pong, which also leads to additional latency. This reactive approach, involving measurements, reporting, and handover execution, may still result in suboptimal mobility performance, as it may not always facilitate the seamless transition of UE to the best cell in a timely manner.

Observation 1: As cellular technology advances with higher frequency bands like FR2 and beamforming are utilized, prompting the usage of TTT within legacy L3 handover mechanisms to mitigate unnecessary transitions and reduce "ping-pong" effects, though at the cost of introducing latency and achieving only suboptimal mobility performance.

In mobility procedure, the UE experiences continuous handover, some of them are unnecessary handover, for example, from cell A to cell B, and then back to cell A; or from cell A to B and then change to cell C at a short time; or UE received handover Command in cell A to switch to Cell B. However, a Fail occurred on the way handover to cell B. Such continuous and unnecessary handovers may significantly affect the mobility performance and user experience, leading to issues like game freezing, delays, and unsmooth video services.

Observation 2: UE may experiences continuous and unnecessary handover and significantly affect the mobility performance and user experience, leading to issues like game freezing, delays, and unsmooth video services.

Following two applications of measurement event prediction can be considered:

- To enable a proactive measurement report(MR) and handover: the UE predict measurement during the TTT period and proactively trigger the measurement report without the necessity of actually waiting for the entire TTT duration, thus reducing the HO latency.

- To avoid unnecessary handover: the UE avoids sending unnecessary measurement reports based on the prediction and consequently avoids the execution of unnecessary handover.

Proposal 1: In order to achieve the 2nd goal, i.e., mobility enhancement, measurement event prediction can be utilized to: 1. enable proactive measurement reporting, thereby reducing handover latency; 2. prevent the transmission of unnecessary measurement reports that could lead to unnecessary handovers.

Considering above applications, AI approaches could be applied to predict the measurement event in the following aspects:

- Option 1: Utilize AI to predict if certain event(s) is about to happen, i.e., the outcome of the predictions is whether the measurement event(s) condition is fulfilled in the future/the prediction window.

- Option 2: Utilize AI to predict if certain event(s) are valid, i.e., the outcome of the prediction is whether the occurring event is valid/invalid in the future/the prediction window. An event that leads to unnecessary HO or HOF is regarded as an invalid event, otherwise, it is considered a valid event.

The occurring event means that the entering condition of the event is fulfilled. We define the corresponding event which leads to unnecessary handover or handover failure as invalid. Otherwise, the event is considered as valid.

Proposal 2: RAN2 study the following two options of measurement event prediction, i.e. what to predict:

- Optoin 1: UE predicts if certain event is about to happen in the future/the prediction window.

- Option 2: UE predicts if the occurring event is valid in the future/ the prediction window. The occurring event is the event that the entering condition is fulfilled.

In measurement event prediction, the event(s) to be predicted is configured by the network, e.g., Event A1/A2/A3/A4/A5.

Proposal 3: For measurement event prediction, the event(s) to be predicted is configured by the network.

With the measurement event prediction, if the UE predicts the occurring event is valid or the entering condition of the event is fulfilled during the whole prediction window (which can be longer than TTT), the UE determine to trigger an early MR to reduce the handover latency and enhance overall mobility performance. If the UE predicts the occurring event is invalid or predicts the entering condition of the event is fulfilled during TTT, but the not satisfied after TTT, the UE determines to refrain from sending the measurement report (MR) or still send the measurement report with information to hint NW the invalid event to reduce the unnecessary handover. One top of the prediction, the UE considers the link quality of the serving cell. For example, only when the link quality of the serving cell is about a threshold to guarantee the connection with the network, the UE can prevent the measurement report.

Proposal 4: Following the measurement event prediction, the UE further determines whether to send the MR or prevent transmission of the MR.

Direct measurement event prediction

In the previous meeting, RAN2 agree to study both direct and indirect measurement event prediction. In this section, we discuss following solutions on how to use the direct measurement event prediction results to achieve the two options mentioned in proposal 2.

- The UE predicts if the configured event(A1/A2/A3/A4/A5/B1/B2) is about to occur (entering condition is fulfilled or not) or the probability of its occurrence in the future/prediction window based on measurement results in observation window.

- The UE predicts whether the occurring measurement event (entering condition is fulfilled) is valid in the future/prediction window.

In direct measurement event prediction, the prediction window should equal to or even longer than TTT for UE/gNB to determine that the entering condition can be fulfilled at least during the TTT duration.

Proposal 5: For direct mesurement event prediction, the prediction window should be equal to or longer than TTT.

According to option 1, the direct measurement event predicts if the certain event(A1/A2/A3/A4/A5/B1/B2) is about to happen or the probability of it occurrence in the future prediction window at a time sample or window based on the measurement results in observation window. The concerned events to predict is configured by network. The AI Model inputs are serving cell and/or neighbour measurement signal quality (RSRP/SINR) within observation window. The AI model outputs is the probability or binary flag (e.g., {Yes(event occurs); No(event not occur)}) of the event occurrence in the future/the prediction window if one measurement event is configured. If multiple events are configured, the AI model is designed as a multi-classification model. The AI model output is the vector encompassing the probability distribution of occurrence of all configured event (e.g., [0.1,0.2,0.1,0.6], 0.1 for no event, 0.2 for EventA1, 0.1 for EventA1, 0.6 for EventA3) or multi-class flag (e.g.,0 for no event,1 for EventA1, 2 for EventA1, 3 for EventA3). With multi-classification model, it’s possible that only 1 model is needed for multiple events.

Proposal 6: For direct measurement event prediction on if certain event is about to happen (option 1) in the future, AI predicts if the event is about to happen or the probability of it occurrence in the future/ the prediction window.

Proposal 7: For direct measurement event prediction on if certain event is about to happen (option 1) in the future, the AI Model inputs is serving cell and/or neighbour measurement signal quality (RSRP/RSRQ/SINR) within observation window. Other inputs are not excluded. The AI model output is the probability or binary flag (e.g., {Yes(event occurs); No(event not occur)}, if one measurement event is configured) /multi-class flag (e.g.,0 for no event,1 for EventA1, 2 for EventA1, 3 for EventA3, if multiple events are configured) of the event(s) occurrence in the future/the prediction window.

According to option 2, the direct measurement event prediction predicts whether the occurring event keeps being valid or not in the future/the prediction window. The AI Model inputs is a serious of serving cell and neighbor cell quality (RSRP/SINR) within observation window. The AI model output is the probability or binary flag ({Yes (event is valid), no (event not valid)}) of the event that is considered valid.

Proposal 8: For direct measurement event prediction on if certain occurring event is valid (option 2) in the future, AI predicts whether the occurring event is considered valid. The AI Model inputs is serving cell and/or neighbour measurement signal quality (RSRP/RSRQ/SINR) within observation window. Other inputs are not excluded. The AI model output is the probability or binary flag ({Yes (event is valid), no (event not valid)}) of the event that is considered valid.

Indirect measurement event prediction

In this section, we discussed how to use the indirect measurement event prediction results to achieve the two options (proposal 2) of measurement event prediction in section 2.1.

For indirect prediction that discussed in RAN2 meeting, our understanding is:

- For indirect measurement event prediction, UE predicts the L3-cell level RSRPs based on the RRM measurement prediction case A with different sub use cases.

Observation 3: For indirect prediction, AI model is used to predict future signal quality, instead of measurement event directly.

The AI model input is the historical measurement results, and the model output is the future measurement results. The UE determines whether and when to trigger MR based on prediction on the measurement results without further AI/ML models.

Observation 4: The output of RRM measurement prediction can be use to judge the entering condition of configured event. Together with the serving cell quality, the UE further determines whether to trigger the MR.

Proposal 9: For indirect measurement event prediction, the UE utilizes the output of the RRM measurement prediction results to determine whether and when to trigger MR without further AI/ML models.

As showed in figure 1 and 2, there are two step of indirect measurement event prediction. UE predicts the measurement in prediction window firstly and then determine whether and when to trigger MR.

If the UE judges that the entering condition for configured event(s) is satisfied during the whole prediction window or the event is valid, UE can trigger the MR to the NW at the present moment. If the UE judges the entering condition for configured event(s) is satisfied during the predicted TTT window, but not satisfied during additional prediction window. However, the predicted serving cell quality is below certain threshold in the prediction window. In this case, UE need to trigger the MR to NW to request a handover and prevent data throughput drop or RLF.UE can trigger the MR to the NW at the present moment.

Figure 1 Indirect measurement event prediction for Early Measurement Report

Based on the predicted measurement results (RSRP/RSRQ/SINR), even if UE judges the event is invalid or the event is satisfied during TTT, when the predicted source cell quality is better than certain threshold, but the entering condition is no longer satisfied after TTT during prediction window. In this case, UE can skip trigger MR to the network at present. In this way, the unnecessary MR and handover can be avoided due to frequent cell switches caused by the fluctuation of the source cell being inferior to the target cell for a very short time.

Figure 2 Indirect measurement event prediction for skipping unnecessary Measurement Report

Proposal 10: In indirect measurement event prediction, the UE determines the time instance the measurement event occurs based on the predicted RSRPs of the serving cell and neighbouring cells after AI prediction.

Proposal 11: The indirect measurment event prediction results can be used to adjust(early/skipping) the timing for measurement report for early MR or avoid unnecessary handover.

Methodology for simulation

And in RAN2#127 post meeting email discussion, the following definition and example Figures for temporal domain case A are proposed. The description of case A is as follows [1]:

Intra-frequency temporal domain case A:

In case A, continuous measurement results in prediction window are predicted by continuous historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either one or more sampling periods (with sliding L1/L3 filtering option) or one or more measurement periods (with non-sliding L1/L3 filtering option), where measurement result(s) are actually measured before sliding.

Figure 3 Example of temporal domain case A

With the certain specific observation window and prediction window, the sliding length impact the inference times. For instance, in units of sampling intervals or measurement period, the observation window=4 and prediction window =3, to predict the signal quality in future 7, with sliding length equals to 1, 5 times model inference is needed; with sliding length equals to 2, 3 times model inference is needed; while with sliding length equals to 4, 2 times model inference is needed. The model inference consumes computation resources, the increase of sliding length can reduce the number of model inferences, thus saving computing resources.

Besides, for case A with no measurement reduction and continuous prediction, the sliding length should be no exceeding than observation window or prediction window, i.e., 0<sliding length<= min (observation window, prediction window).

Figure 4 Case A with different sliding length

Observation 5: For case A, with no measurement reduction, the sliding length has to be no exceeding than observation window or prediction window, i.e., 0 < sliding length <= min(observation window, prediction window).

Proposal 12: For temporal domain case A, the sliding lenth should with range to be no exceeding than observation windowor prediction window, i.e., 0 < sliding length <= min(observation window, prediction window).

Performance metric

For both direct and indirect prediction, the goal is to predict possible measurement event in the future. The accuracy KPI including two types:

- Type 1: for direct prediction accuracy KPIs, e.g., event prediction accuracy/recall/precision at time sample or window.

- Type 2: for indirect prediction accuracy KPIs, e.g., average L3-RSRP difference.

Recall is expressed by the formula: True positive/(True Positive + False Negatives). Precision is expressed by the formula: True positive/(True Positive + False Positives). Accuracy is expressed by the formula: Correct Predictions/Total Predictions.

True Positive represents the cases where the actual class is positive (1), and the model correctly predicts it as positive (1). False Negative (FN) represents the cases where the actual class is positive (1), but the model incorrectly predicts it as negative (0). False Positive (FP) represents the cases where the actual class is negative (0), but the model incorrectly predicts it as positive (1).

The typical KPI for the classification model can also be considered for direct measurement event prediction, e.g., precision and recall.

Proposal 13: For both direct and indirect prediction, consider two types of accuracy performance metric:

- Type 1: for direct prediction accuracy KPIs, e.g., event prediction recall/precision at time sample or window.

- Type 2: for indirect prediction accuracy KPIs, e.g., average L3-RSRP difference.

The temporal domain case A is for 2nd study goal, which aim to improve the handover performance. The intermediate KPIs cannot truly reflect the prediction impact on mobility performance. Therefore, we propose:

Proposal 14: System-level performance KPIs is needed to reflect the impact of RRM measurement event prediction results(inidrect prediciton) and event prediction recall/precision (direct prediction) on handover performance for the 2nd study goal.

Simulation assumption

The value range for TTT is from 0ms to 5120ms ({ms0, ms40, ms64, ms80, ms100, ms128, ms160, ms256, ms320, ms480, ms512, ms640, ms1024, ms1280, ms2560, ms5120}).

Proposal 15: The recommended value for handover parameters i.e. event A3 offset=2dB, hysteresis=0dB , TTT Timer=160ms.

Conclusion

Motivation

Observation 1: As cellular technology advances with higher frequency bands like FR2 and beamforming are utilized, prompting the usage of TTT within legacy L3 handover mechanisms to mitigate unnecessary transitions and reduce "ping-pong" effects, though at the cost of introducing latency and achieving only suboptimal mobility performance.

Observation 2: UE may experiences continuous and unnecessary handover and significantly affect the mobility performance and user experience, leading to issues like game freezing, delays, and unsmooth video services.

Proposal 1: In order to achieve the 2nd goal, i.e., mobility enhancement, measurement event prediction can be utilized to: 1. enable proactive measurement reporting, thereby reducing handover latency; 2. prevent the transmission of unnecessary measurement reports that could lead to unnecessary handovers.

Proposal 2: RAN2 study the following two options of measurement event prediction, i.e. what to predict:

- Optoin 1: UE predicts if certain event is about to happen in the future/the prediction window.

- Option 2: UE predicts if the occurring event is valid in the future/ the prediction window. The occurring event is the event that the entering condition is fulfilled.

Proposal 3: For measurement event prediction, the event(s) to be predicted is configured by the network.

Proposal 4: Following the measurement event prediction, the UE further determines whether to send the MR or prevent transmission of the MR.

Direct measurement event prediction

Proposal 5: For direct mesurement event prediction, the prediction window should be equal to or longer than TTT.

Proposal 6: For direct measurement event prediction on if certain event is about to happen (option 1) in the future, AI predicts if the event is about to happen or the probability of it occurrence in the future/ the prediction window. The events to be predicted is configured by the network.

Proposal 7: For direct measurement event prediction on if certain event is about to happen (option 1) in the future, the AI Model inputs is serving cell and/or neighbour measurement signal quality (RSRP/SINR) within observation window. The AI model output is the probability or binary flag (e.g., {Yes(event occurs); No(event not occur)}, if one measurement event is configured) /multi-class flag (e.g.,0 for no event,1 for EventA1, 2 for EventA1, 3 for EventA3, if multiple events are configured) of the event(s) occurrence in the future/the prediction window.

Proposal 8: For direct measurement event prediction on if certain occurring event is valid (option 2) in the future, AI predicts whether the occurring event is considered valid. The AI Model inputs is serving cell and/or neighbour measurement signal quality (RSRP/SINR) within observation window. The AI model output is the probability or binary flag ({Yes (event is valid), no (event not valid)}) of the event that is considered valid.

Indirect RRM measurement event prediction

Observation 3: For indirect prediction, AI model is used to predict future signal quality, instead of measurement event directly.

Observation 4: The output of RRM measurement prediction can be use to judge the entering condition of configured event. Together with the serving cell quality, the UE further determines whether to trigger the MR.

Proposal 9: For indirect measurement event prediction, the UE utilizes the output of the RRM measurement prediction results to determine whether and when to trigger MR without further AI/ML models.

Proposal 10: In indirect measurement event prediction, the UE determines the time instance the measurement event occurs based on the predicted RSRPs of the serving cell and neighbouring cells after AI prediction.

Proposal 11: The indirect measurment event prediction results can be used to adjust(early/skipping) the timing for measurement report for early MR or avoid unnecessary handover.

Methodology for simulation

Observation 5: For case A, with no measurement reduction, the sliding length has to be no exceeding than observation window or prediction window, i.e., 0 < sliding length <= min(observation window, prediction window).

Proposal 12: For temporal domain case A, the sliding lenth should with range to be no exceeding than observation windowor prediction window, i.e., 0 < sliding length <= min(observation window, prediction window).

Proposal 13: For both direct and indirect prediction, consider two types of accuracy performance metric:

- Type 1: for direct prediction accuracy KPIs, e.g., event prediction recall/precision at time sample or window.

- Type 2: for indirect prediction accuracy KPIs, e.g., average L3-RSRP difference.

Proposal 14: System-level performance KPIs is needed to reflect the impact of RRM measurement event prediction results(inidrect prediciton) and event prediction recall/precision (direct prediction) on handover performance for the 2nd study goal.

Proposal 15: The recommended value for handover parameters i.e. event A3 offset=2dB, hysteresis=0dB , TTT Timer=160ms.

Reference

[1]	R2-2407849 Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

[2]  R2_127_ChairNotes_24-08-23_17-00_eom

[3]	3GPP TR 36.839.