3GPP TSG-RAN WG2 Meeting #127-bis	R2-2408435

Hefei, China,  October 14th – 18th, 2024

Agenda item:	8.10.2

Source:	Samsung

Title:	MRO enhancements for Rel-18 mobility features

Document for:	Discussion and Decision

# Introduction

As per [1], one of the objectives of R19 SON WI is to support the MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs and subsequent CPAC. From RAN2, this involves identification and specification of necessary UE reporting to enhance the mobility parameter tuning as given below.

The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

# Discussion

## MRO for subsequent CPAC

In RAN2#126 [1], the following agreement was made.

RAN2 will look into failure and near failure scenarios for subsequent execution of SCPAC and see if/what enhancements are needed.

Subsequent CPAC allows subsequent cell group change after changing cell group without reconfiguration and re-initiation of CPC/CPA. Like legacy CPAC, robustness of subsequent CPAC also depends on the configuration of thresholds, offsets, timers etc. related to measurement reporting and mobility. Thus it is straightforward that R18 SCGFailureInformation and SPR are the baseline for MRO for subsequent CPAC.

Proposal 1-1: R18 MRO for SCGFailureInformation/SPR is the baseline for MRO for subsequent CPAC.

Now let us look at the differences between CPAC and subsequent CPAC.

Subsequent CPAC introduces condExecutionCondToAddModList and condExecutionCondToReleaseList as shown below.

CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16

CondReconfigToAddMod-r16 ::=     SEQUENCE {

condReconfigId-r16               CondReconfigId-r16,

condExecutionCond-r16            SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Need M

condRRCReconfig-r16              OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Cond condReconfigAdd

...,

[[

condExecutionCondSCG-r17         OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL     -- Need M

]],

[[

condExecutionCondPSCell-r18      SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Cond condReconfigCHO-WithSCG

subsequentCondReconfig-r18       SubsequentCondReconfig-r18                            OPTIONAL,    -- Need M

securityCellSetId-r18            SecurityCellSetId-r18                                 OPTIONAL,    -- Need M

scpac-ConfigComplete-r18         ENUMERATED {true}                                     OPTIONAL     -- Cond CPAC

]]

}

CondReconfigExecCondSCG-r17 ::=  SEQUENCE (SIZE (1..2)) OF MeasId

SubsequentCondReconfig-r18 ::=   SEQUENCE {

condExecutionCondToReleaseList-r18   CondExecutionCondToReleaseList-r18                OPTIONAL,   -- Need N

condExecutionCondToAddModList-r18    CondExecutionCondToAddModList-r18                 OPTIONAL,   -- Need N

...

}

CondExecutionCondToAddModList-r18 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondExecutionCondToAddMod-r18

CondExecutionCondToAddMod-r18 ::=    SEQUENCE {

subsequentCondReconfigId-r18         CondReconfigId-r16,

subsequentCondExecutionCond-r18      SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Need M

subsequentCondExecutionCondSCG-r18   OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL,    -- Need M

...

}

CondExecutionCondToReleaseList-r18 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigId-r16

-- TAG-CONDRECONFIGTOADDMODLIST-STOP

A UE may be configured with conditional reconfiguration configuration without execution conditions. In R18, a UE considers whether a neighbour cell is a CPAC candidate cell based on whether condRRCReconfig is configured or not. With subsequent CPAC, it is possible for the network to release the execution conditions and keep condRRCReconfig-r16. Unless the execution conditions are configured, UE does not evaluate those cells for CPAC. Therefore, the UE needs to report to the network a cell as a candidate cell, only when the execution conditions are also available, since CPAC is possible only to those cells

Proposal 1-2: For MRO of subsequent CPAC, UE considers a neighbour cell as a candidate PSCell only if the candidate PSCell has the execution conditions configured.

Another aspect that needs to be discussed is reporting of the timeSinceCPAC-Reconfig, the time elapsed between the initiation of the execution of conditional reconfiguration for the target PSCell and the reception of the last conditionalReconfiguration for the SCG including the condRRCReconfig of the target PSCell. This is included to identify the case where the near failure occurred because conditionalReconfiguration is received late. Obviously, for subsequent CPAC,the scenario is not applicable. So UE can avoid this reporting. The absence of the field can also help the network to identify if the CPAC is subsequent CPAC.

Proposal 1-3: When the PSCell Addition or PSCellChange is due to subsequent CPAC, the UE doesn’t include the field timeSinceCPAC-Reconfig in SPR.

Proposal 1-4: Absence of timeSinceCPAC-Reconfig  helps network to identify CPAC as subsequent CPAC. No additional field is required for this purpose.

## MRO for LTM

For MRO for LTM, RAN2 decided to consider the following in RAN2#125bis [2].

RAN2 considers SHR, RA report and RLF for MCG LTM SON.

RAN2 will start work on MCG LTM.

RAN2 also identified the scenarios and subcases to be considered for LTM.

In RAN2#126/RAN2#127 [3], RAN2 discussed further on the contents of RLF report. In this contribution, we discuss on some of the remaining issues based on the previous agreements.

SHR contents

In RAN2#126, RAN2 had the below agreement:

If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

In our understanding this information is useful for SHR for LTM. It can be useful for the SHR for L3 handover also, as some cells may be configured for LTM mobility and some cells may be configured for L3 mobility.

Proposal 2-1: Agreement for UE logging the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report is applicable for SHR also.

In RAN2#127, there is an FFS

We aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit.

Once we log the L1 measurements for LTM, network can be implicitly identify that a cell is LTM candidate cell. There is no need for any explicit mechanism for this.

Proposal 2-2: Whether a neighbour	 cell is an LTM candidate or not is implicitly derived from the presence of L1 measurements for LTM in SHR.

Random Access Information for LTM

We think that it is useful for logging the RA purpose for early synchronisation and LTM cell switch separately.

Proposal 2-3: A new RA purpose is used for LTM cell switch and early synchronisation for TA.

We understand that the random access optimisation for the early TA needs to be done by both the source cell and candidate cell. For e.g. the thresholds at which the early TA is triggered is decided by source cell, while the RA parameters are decided by the candidate cell. The configuration of thresholds will make an impact on whether early TA is successful or not. So UE may include both source cell and candidate cell identifier in the RA report.

Proposal 2-4: UE includes the source and candidate cell identifiers in the RA information for early synchronisation for TA.

It is also important to know whether the random access for early TA is successful and how many attempts are made for early TA.

Proposal 2-5: If the UE has received TA value in LTM cell switch command, UE reports the TA is successful in the RA information.

Proposal 2-6: UE reports the number of PDCCH ordered RACH performed for early TA a candidate cell for a cell switch in the RA information.

Finally, it is important to log the RA report in all the cases-RA report, RLF report and SHR.

Proposal 2-7: RA information for early TA is included in RA report, RLF report and SHR.

## MRO for CHO with Candidate SCGs

In RAN2#127 [2], RAN2 agreed to the following regarding CHO with candidate SCG(s).

UE includes following information in RLF report:

- b.	Time information regarding condition fulfilment for CHO with candidate SCGs. Details are FFS. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled.

- c.	Measurement results of PCells and PSCells.

Details of the time information regarding condition fulfilment for CHO with candidate SCGs was FFS in the last meeting.

For CHO with candidate SCG(s) UE may be configured with two triggering events for CHO and two triggering events for associated candidate SCG(s). It is possible to configure multiple candidate PSCells associated with the same candidate PCell by adding multiple CondReconfig. Similarly, it is possible that a candidate PSCell is associated with multiple candidate PCells.

Observation 1: A candidate PCell may be associated to multiple candidate PSCells and a candidate PSCell may be associated to multiple candidate PCells.

Since each of the PCell condition and PSCell condition can be associated with two triggering conditions, and possible many-to-many association between PCells and PSCells, the number of combinations of fulfilled events can be very large.

For e.g. Let us consider the case where a candidate PCell, PCell1 is associated to 4 candidate PSCells PSCell1, PSCell2, PSCell3, PSCell4. Let us also assume that the first triggering condition for PCell1 is fulfilled and first triggering condition for all the associated candidate PSCells (PSCells1-4) are fulfilled.

If the timing information for all the cases are included seperately, we will need to include 4 different fields. We also may need to add a flag for each of these cells which indicates whether PCell event or PSCell event has occurred first. i.e. To get the timing information, the difference between first triggered event of PCell1 and the first triggered event of each of PSCell1-4 can be included. However this means that either a negative value of time also need to be included or some information to identify whether PCell1 of PSCell1 event has been fulfilled need to be reported. Moreover, if the same PSCells are associated to a different PCell2, the same information needs to be repeated for PCell2.

Observation 2: It will need a large number of field to report the timing relation between individual fulfilled events for PCells and PSCells.

A simpler way is to include the timing information between the fulfilled of an event and a reference point. A common reference point can be the time of occurrence of the connection failure (RLF/HOF).

Proposal 3-1: To identify the timing between fulfilled events of PCell and associated PSCells in CHO with candidate SCG(s), UE logs the time difference between first fulfilled event for PCell/PSCell and the time of occurrence of connection failure.

In addition, for the PSCell, the first fulfilled event and the time difference between first fulfilled event and the time difference between first fulfilled event and the second event, if it is fulfilled can be included. This is similar to the reporting for PCell in CHO in RLF report

Proposal 3-2: For each PSCell included in the RLF report, UE logs the following

a. First fulfilled event.

b. Time difference between first fulfilled event and the second event, if it is fulfilled.

In RLF report, the list of CHO candidates are included or a flag in the measurement results is used to identify if a neighbour cell is a candidate cell. For CHO with candidate SCG(s), the list of associated PSCells also can also be included.

Proposal 3-3: For each PCell, UE logs the list of associated PSCells in RLF report.

# Conclusion

Based on the discussion, we have the following proposals and observations.

MRO for subsequent CPAC

Proposal 1-1: R18 MRO for SCGFailureInformation/SPR is the baseline for MRO for subsequent CPAC.

Proposal 1-2: For MRO of subsequent CPAC, UE reports a neighbour cell as a candidate PSCell only if the candidate PSCell has the execution conditions configured.

Proposal 1-3: When the PSCell Addition or PSCellChange is due to subsequent CPAC, the UE doesn’t include the field timeSinceCPAC-Reconfig in SPR.

Proposal 1-4:Absence of timeSinceCPAC-Reconfig  helps network to identify CPAC as subsequent CPAC.No additional field is required for this purpose.

MRO for LTM

Proposal 2-1: Agreement for UE logging the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report is applicable for SHR also.

Proposal 2-2: Whether a neighbour	 cell is an LTM candidate or not is implicitly derived from the presence of L1 measurements for LTM in SHR.

Proposal 2-3: A new RA purpose is used for LTM cell switch and early synchronisation for TA.

Proposal 2-4: UE includes the source and candidate cell identifiers in the RA information for early synchronisation for TA.

Proposal 2-5: If the UE has received TA value in LTM cell switch command, UE reports that TA is successful in the RA information.

Proposal 2-6: UE reports the number of PDCCH ordered RACH performed for early TA in a candidate cell for a cell switch in the RA information.

Proposal 2-7: RA information for early TA is included in RA report, RLF report and SHR.

MRO for CHO with SCGs

Observation 1: A candidate PCell may be associated to multiple candidate PSCells and a candidate PSCell may be associated to multiple candidate PCells.

Observation 2: It will need a large number of fields to report the timing relation between individual fulfilled events for PCells and PSCells.

Proposal 3-1: To identify the timing between fulfilled events of PCell and associated PSCells in CHO with candidate SCG(s), UE logs the time difference between first fulfilled event for each candidate PCell/PSCell and the time of occurrence of connection failure in RLF report.

Proposal 3-2: For each PSCell included in the RLF report, UE logs the following

a. First fulfilled event.

b. Time difference between first fulfilled event and the second event, if it is fulfilled.

Proposal 3-3: For each PCell, UE logs the list of associated PSCells in RLF report.

# References

[1] RP-234038: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4

[2] R2-2406202: RAN2#126 Meeting Report

[3] R2-2404102: RAN2#125bis Meeting Report

[4] RAN2#127 Chair notes