3GPP TSG RAN WG2 Meeting #127-bis			R2-2408442

Hefei, CN, Oct. 14th – 18th, 2024

Agenda item:	8.3.2

Title:	Discussion on cluster based RRM measurement prediction for high speed railway communications

Source:	Beijing Jiaotong University (BJTU)

Document for:	Discussion

1. Introduction

In RAN 102 meeting, a new study item on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR was approved [1]. In the study item scope, study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects:

-	AI/ML based RRM measurement and event prediction,

- Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

- Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

- HO failure/RLF prediction (UE sided model) [RAN2]

- Measurement events prediction (UE sided model) [RAN2]

-	Study the need/benefits of any other UE assistance information for the network side model [RAN2]

-	The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

- NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

-	Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

- NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID

-	Potential specification impacts of AI/ML aided mobility [RAN2]

-	Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

In the 3GPP TSG-RAN WG2 #126 meeting, [2] proposed cluster approach for RRM measurement prediction and introduced its relationship between model input and output in temporal and spatial prediction. According to the summary of the 126th meeting [3], there are two main discussion points for cluster approach.

- whether we should also report some information relevant to cluster approach. Apart from two company (Huawei, Apple), other companies are fine with proposal from NTT docomo. Since RAN2 agreed at last meeting that “To define cluster approach at least based on the number of input cells, number of output cells and their relationship in temporal domain, spatial domain and frequency domain”. So rapporteur recommend to let company report as part of the “model input” and “model output”, if any.

- Huawei and ZTE raise the point that some of the parameters hidden in the Note 4/5/6 under table 1 should be listed in the template explicitly to align among company. Rapporteur recommend some of them on the reflector, where apart from Apple there is no negative comment is received.

In the 3GPP TSG-RAN WG2 #127 meeting, it was determined that cluster-based approach evaluation was optional and it was up to companies to select the number of cells for input and output (companies should clarify what they are using for cluster-based approach). In this paper, we propose a cluster-based RRM measurement prediction method for high speed railway (HSR) communications.

2. Motivation

HSR provides people with safe, fast, comfortable and economical modes of transportation, and has been widely developed around the world. Nevertheless, mobile communication systems are pivotal for ensuring rail safety, operations, maintenance, and delivering passenger information services. Emerging applications, such as railway multimedia dispatching, high-definition video monitoring on board and trackside, have posed stricter requirements on the reliability, data rate, and spectrum efficiency of the next-generation, dedicated mobile communication systems for HSR. However, the time-varying nature of wireless channels in HSR communications results in frequent cell handovers, leading to numerous data interruptions. This can significantly compromise both the safety of train communications and the overall user experience.

The management of mobility at cell level in 4G is based on the use of periodic channel state information reference signals and synchronization signal blocks. Unfortunately, the resulting large latency and high overhead are prohibitive for HSR communications, and, hence, innovative approaches are urgently needed to address these limitations. In the recent years, deep neural networks (DNNs) have found extensive applications in areas, such as computer vision and natural language processing, owing to their exceptional ability to extract features from intricate and high-dimensional data and DNNs have also proven their effectiveness in capturing the complex and nonlinear characteristics of wireless channels to facilitate beam-level mobility management.

Currently, there are two main AI-based RRM measurement prediction methods for network triggered L3-based handover. One is cell-specific/per-cell approach. The AI-based cell handover approach is to train a model for each cell to predict the future RSRP of that cell, and then deploy multiple trained models to the network side. This AI-based cell handover prediction method has a simple idea, but its shortcomings lie in the high deployment and management overhead of AI models, and it is difficult to inherit the trained models for different terminal locations, so it will encounter great obstacles in practical application. Another approach which we call cluster-based approach is to use an AI model to predict the future RSRP of multiple adjacent cells based on cell-level/cluster-level RSRP measurement information. This approach overcomes deployment overhead and time overhead which are good for HSR communications. Therefore, we propose a cluster-based temporal prediction approach for AI-assisted L3 handover in HSR communications, and introduce model’s input, output, network structure and the simulation results of this algorithm.

3. High-speed railway scenarios description

Fig. 1 The scenario of HSR communications

Without loss of generality, we consider an HSR FR2 mmWave wireless communication scenario with a user equipment (UE) and seven base stations (BS) shown in Fig.1. Each BS is equipped with three half-wavelength spaced uniform plane arrays (UPAs) of 32 antennas with sector orientation of 30, 150 and -90 degrees, respectively, and we consider a downlink communication system with analog beamforming architecture for each UPA. Meanwhile, the UE is equipped with two half-wavelength spaced antennas without beamforming and moves according to the street model. The distance between base stations is set to 200m, and the channel model is referenced to 3GPP TR 38.901 UMa for LOS propagation. Detailed simulation parameters are shown in Table 1.

Table 1 Simulation setting

| Parameters | Values |
| --- | --- |
| Channel | UMa |
| Bandwidth | 80MHz |
| Subcarrier bandwidth | 120KHz |
| Number of subcarriers | 64 |
| Center frequency | 30GHz |
| Antenna setup of BS | （M, N, P, Mg, Ng）=（4, 8, 1, 1, 1） |
| Height of BS | 25m |
| Antenna setup of UE | （M, N, P, Mg, Ng）=（2, 1, 1, 1, 1） |
| Height of UE | 1.5m |
| UE distribution | 100%outdoor |
| UE speed | 60km/h、120km/h、350km/h、500km/h |
| UE motion trajectory | City street |
| Transmitting power of BS | 40dbm |
| Number of sites | 7 |
| Number of sectors in a site | 3 |
| Number of UE | 1 |
| Sampling interval | 40ms |
| Distance between BSs | 200m |

Proposal #1: Consider high-speed railway as one of the scenarios for cluster based RRM measurement prediction methods. Study the implementation and design of cluster-based AI/ML models in various railroad track scenarios.

4 The cluster-based temporal prediction approach for HSR communications

At the RAN2 125 meeting, the following three cases are agreed for cell level measurement prediction:

- Case 1: To predict beam level results, then generate cell level results based on the predicted beam results.

- Case 2: To directly predict cell level results based on cell level results.

- Case 3: To directly predict cell level results based on beam level results

Different cases have different AI model’s input and output, and have different advantages and disadvantages. For case 1, using L1-RSRP as both the input and the output of the AI models. The AI model output is then filtered to derive the L3-RSRP, which is used to execute cell handovers. For this option, the output L1-RSRP of AI model can be applied to both beam-level and cell-level mobility management. Nevertheless, L1-RSRP encompasses extensive information that exceeds the requirements for handover, while the correlation between the input and output L1-RSRP is intricate, thereby amplifying the learning challenge for the AI models. For case 2, the input for AI models is the L3-RSRP of each cell at historical instances, with the output being either the L3-RSRP of each cell or the L3-RSRP of the top K cells at future TTT instances. The dimension of L3-RSRP information is lower than that of L1-RSRP. However, acquiring L3-RSRP, which requires averaging data over multiple time slots, introduces extra latency which is not desired in HSR communications. For case 3, leveraging L1-RSRP to forecast L3-RSRP via AI can be an advantageous strategy. This method reaps the benefits of the above two approaches, making use of the abundant data in L1-RSRP to enhance the AI model's prediction accuracy, while also streamlining the mapping between the model's input and output, which is good for HSR scenarios.

Model input/output: Here gives a detailed description of the AI model’s input and output of case 3. In literature, the input into AI models is the RSRP of beam pairs of all cells, leading to significant measurement overhead and computational complexity. To this end, we consider two approaches to reduce the measurement overhead for cluster based RRM measurement prediction. For the first approach, all beams from partially selected cells are used to predict the L3-RSRP for all cells. For the second approach, partially selected beams of each cell are used to predict the L3-RSRP for all cells. We utilize the partial L1-RSRP from the past six instances to predict the L3-RSRP in the next four instance.

Network structure: We employ CNN-based and LSTM-based models as shown in Fig.2. The LSTM-based model comprises four LSTM layers and each layer has 168 hidden units. The features extracted by the LSTM network undergo further processing via an FC layer and a sigmoid function for feature mapping. To enhance the performance of the LSTM network, we adopt four identical networks to predict L3-RSRP for four future instances separately. For the CNN-based model, a set of regular 2D convolution layers with 12, 24 and 32 kernels are employed before feeding it into two FC layers. To avoid overfitting during the training process, max pooling layers are incorporated after each CNN layer.

Fig. 2 The architecture of cluster based RRM prediction.

Model training: In the end, we will introduce the training methods of AI models for the above method. To enhance the prediction performance of the LSTM-based model, the output of this neural network is the predicted RSRP of all cells at a future moment, thus 4 networks need to be trained separately to predict 4 moments in the future. For CNN-based model, only one network needs to be trained to output RSRP of all cells in upcoming 4 moments.

Proposal #2: For cluster-based temporal prediction approach, consider using case 3 (To directly predict cell level results based on beam level results) as AI models’ input and output for HSR communications.

5 Simulation and results

The channel used in our experiments is generated by QuaDRiGa. We generate four datasets corresponding to various UE speeds of 60 km/h, 120 km/h, 350 km/h, and 500 km/h. Each dataset comprises 50,000 samples, which are split into training, validation, and test sets in a 3:1:1 ratio. For the proposed two approaches with equidistant sampling, L1-RSRP of 50% beams in each cell or all beams in 50% cells are selected as input to the AI model. The batch size is 256 and the maximum epoch is 800. The learning rate dynamically changes from10-3 to 10-8, utilizing the adaptive moment estimation optimizer. The RSRP difference is one evaluation metric of prediction accuracy for cell-level mobility management, which reflects the disparity between the predicted L3-RSRP and the actual L3-RSRP of all cells. In addition, we also consider the RLF rates as the system-level evaluation metric to illustrate the overall handover performance of the communication system.

The RSRP difference between the AI predictions and the actual measurements is shown in Fig. 3. The experimental results show that both CNN-based and LSTM-based models show excellent cluster-level temporal prediction performance. Table 2 shows the significantly reduced RLF rates in AI-assisted cell handover processes. Remarkably, the proposed two approaches to reduce the measurement overhead for HSR communications labeled as Part_Cell and Part_Beam, achieve comparable RLF performance with the traditional approach using all beam measurements of all cells, labeled as All_Beam_Cell, while we can save 50% beam measurement overhead for HSR communications. More detailed network architecture and simulation experiment results are introduced in [4]

Table 2 Comparison of the RLF rates and complexity of AI-based and Non-AI cell handover methods

Fig. 3 RSRP difference of AI models

Proposal #3: Consider using all beams from partially selected cells or partially selected beams of each cell as AI models’ input for cluster-based temporal prediction approach in HSR communications.

6 Conclusion

In this paper, we introduce a cluster-based temporal prediction approaches for L3 handover in HSR communications, focusing on the input, output, network structure and training methods of AI/ML models. Based on the discussion above, we propose the following proposals:

Proposal #1: Consider high-speed railway as one of the scenarios for cluster based RRM measurement prediction methods. Study the implementation and design of cluster-based AI/ML models in various railroad track scenarios.

Proposal #2: For cluster-based temporal prediction approach, consider using case 3 (To directly predict cell level results based on beam level results) as AI models’ input and output for HSR communications.

Proposal #3: Consider using all beams from partially selected cells or partially selected beams of each cell as AI models’ input for cluster-based temporal prediction approach in HSR communications.

7. References

[1] RP-234055, ‘Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR’.

[2] R2-2404700, ‘Discussion on RRM measurement prediction’.

[3] R2-2406085, ‘Summary of [POST126][031][AIMob] Simulations (OPPO)’.

[4] W. Li, W. Chen, S. Wang, Y. Zhang, M. Matthaiou, and B. Ai, “Ai-based beam-level and cell-level mobility management for high speed railway communications,” 2024. [Online]. Available: https: //arxiv.org/abs/2407.04336