3GPP TSG-RAN WG2 Meeting #127bis						R2-2408446

HEFEI, CHINA 14.10.2024-18.10.2024

Source: Vodafone, Qualcomm, Nokia, Interdigital, Xiaomi, Samsung, Deutsche Telekom, ZTE

Title: 	Energy Status report

Document for: Decision

Agenda Item: 8.2.2 - Functionality aspects

## Introduction

## During RAN WG2#127 agreed:

Agreements

-	RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages.   FFS details on signalling

One point of discussion in RAN WG2 was the possible overlapping with RAN WG1 discussions. The related discussion in RAN1 is captured in R1-2407532 “Final FL summary on frame structure and timing aspects for Rel-19 Ambient IoT”.

According to this report, RAN WG1 concluded during last meeting #118 that :

## Discussion

Overlapping between RAN WG2 and RAN WG1 discussion:

RAN1 is discussing (For the study of the potential impact of device unavailability due to charging by energy harvesting) that devices can be in on/off and possibly in sleep state and potential impact of device unavailability due to charging by energy harvesting.

There are some assumptions regarding further analyses like:

  - ON state supports at least: transmission, reception for communication

  - OFF state does not support at least: transmission, reception for communication

  - OFF state supports at least: energy harvesting

  - SLEEP state supports at least:

    - maintaining a memory content from ON state

    - Maintaining a timer (RAN1 to discuss purpose(s) of timer)

  - SLEEP state does not support at least: transmission

No additional physical layer signals/channels specific to support of SLEEP are introduced

Given that the RAN WG1 discussion is aimed to identify possible impacts of the device unavailability due to charging by energy harvesting we do not see an overlap to the definition of an energy status reporting from the device towards the reader.

Observation 1: The current stage of RAN WG1 study does not have a direct impact to the discussion on sending of energy status from the device to the reader. There is no overlap between RAN1 and RAN2 discussions.

Considering the current RAN WG1 discussion, there is no awareness from the reader about the states of the devices, which can potentially increase the overall impact of the device’s unavailability when charging (regardless of the energy source the tag is using).

Without the awareness of energy status of the device, it will be needed to assume that all devices will have similar energy storage capacity, and subsequent charging of the devices would have to be done in a more conservative manner, e.g. there would need to be more frequent charging periods considering the device with smallest energy storage to reduce the overall unavailability of the devices. We believe that having such energy status reporting would reduce/mitigate any possible impact identified in RAN WG1, as it would allow the reader to be aware of the energy status of the devices and avoid a conservative charging considering the smallest energy storage capability.

Observation 2: It does not seem that any of the states On/OFF/SLEEP is known to the reader.

Observation 3: It is currently not clear how the reader can provide the devices the information that they have to become available in a particular time in the future. If this information is provided together with paging, it might be too late for the involved UEs and this information would also need to consider the charging capabilities of the devices/device types.

Proposal 1: Taking into account the current progress of RAN1 work, it is proposed to progress the discussion on energy status transmission independent of RAN1 discussion to the UE states.

Paging repetitions:

During the last meeting RAN WG2 agreed, that the reader can start mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN

Agreements:

Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices

This is a helpful functionality, especially if the number of devices is big and there are business needs requiring the inventory of e.g. 80% of the devices within a group. On the other side, the continuous paging’s would require all UEs to read the paging’s and even duplications is avoided, all the UEs need to process the paging message.

Observation 4: A mechanism to avoid unnecessary paging repetitions contributes to the energy savings for all devices under consideration.

As discussed in R2-2406590, sending of energy status is a good solution to reduce the energy consumption for all devices within the paging area and can be used to avoid unnecessary paging’s

R2-2406590 further discuss that, the UE should have a possibility to transmit the energy status as part of MSG1,MSG3 and the command response to the reader in order to allow the reader and the Core Network to take it into account for subsequent actions according to the business need and therefore it is needed that the device has a possibility to transmit their status at any time, it sends uplink messages.

Proposal 2: It is proposed to include energy status into the normative work of a possible A-IoT WI in Rel 19 and capture it in the TR 38.769

Proposal 3: It is proposed to enable the UE to include energy status in D2R messages (e.g. MSG1,MSG3 and Command Response message).

Information to be included into the energy status bit:

During the last RAN WG2#127 companies discussed the ways the energy status can be re-presented and delivered from the device to the reader and the co-signing companies would like to summarize possible options:

- Indicating that the device does not have sufficient energy for subsequent D2R transmissions. E.g. If this bit is included in MSG1, it would indicate that the device does not have sufficient energy to send MSG3

- a number of bits or bytes that can be transmitted by the UE

- remaining energy capacity or charge (or percentage of such) of the UE

- a time required to gain energy to respond to future interrogations; and/or

- a time required to fully charge the UE

- an estimated time for the UE to run out of charge.

The information above can be provided together with the request to increase/decrease the bandwidth or the power level of the carrier wave to increase/decrease the speed of charging. Alternatively, the request to increase the bandwidth or the power level of the carrier wave can be interpreted as no remaining energy for the subsequent transmissions.

Note: the energy status report from the devices would not mandate carrier wave node behaviour, it would be up to implementation how to perform an adequate charging procedure.

It is proposed to leave the choice of exact energy status interpretation to the Work Item Phase, but capture possible alternatives in the TR 38.769, noting that other alternatives are not precluded.

Proposal 4: It is proposed to leave the exact choice and definition to the work item phase and discuss if any of the examples above could be captured in the 38.769 already.

## Conclusions

Observation 1: The current stage of RAN WG1 study does not have a direct impact to the discussion on sending of energy status from the device to the reader

Observation 2: It does not seem that any of the states On/OFF/SLEEP is known to the reader

Observation 3: It is currently not clear how the reader can provide the devices the information that they have to become available in time. If this information is provided together with paging, it might be too late for the involved UEs.

Proposal 1: Taking into account the current progress of RAN1 work, it is proposed to progress the discussion on energy status transmission independent of RAN1 discussion to the UE states.

Proposal 2: It is proposed to include energy status into the normative work of a possible A-IoT WI in Rel 19 and capture it in the TR 38.769

Proposal 3: It is proposed to enable the UE to include energy status in D2R messages (e.g. MSG1,MSG3 and Command Response message).

Proposal 4: It is proposed to leave the exact choice and definition to the work item phase and discuss if any of the examples above could be captured in the 38.769 already.

## References:

R2-2405233: Nokia “On AIoT Functionality Aspects”

R2-2406590: Vodafone, Qualcomm, Nokia, Interdigital, Xiaomi, Samsung, Deutsche Telekom „Energy Status Report“