3GPP TSG-RAN WG2 #127bis	R2-2408450

Hefei, China, August 2024

Agenda Item:	8.4.2

Source:	OPPO

Title:	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

Document for:	Discussion, Agreement

Introduction

Several aspects are discussed in this paper regarding the procedures and configuration of LP-WUS in RRC_IDLE/Inactive, with some of the related agreements in RAN1/RAN2 in the annex:

- LP-WUS configuration, e.g., common or dedicated signalling;

- LP-WUS subgrouping;

Discussion

Issue-1: LP-WUS configuration

There are several aspects to be discussed regarding the LP-WUS configuration from our perspective.

Firstly, in last meeting, we have made a baseline agreement that the network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE.  With this baseline, we understand that UE does not need to inform the network whether it triggers to monitor LP-WUS or not. However, we think it’s beneficial to enable the network to control whether the UE can monitor LP-WUS or not when the entry condition for LP-WUS monitoring is fulfilled. The reason is that, for some the traffic the UE is served, QoS requirement is the highest priority instead of power saving gain, e.g., service with stringent delay requirement. In this case, UE can be configured by the network to disable the LP-WUS monitoring even though it meets the condition to monitor the LP-WUS. In that case, network can release the UE with an indication on whether UE is allowed to monitor L-WUS when it’s in RRC Idle/Inactive state.

UE can be configured to monitor LP-WUS or not even though it meets the condition of monitoring LP-WUS, e.g., via RRC Release message.

Secondly, regarding the entry/exit thresholds configured separately for OFDM-based and OOK-based WUR, we had an FFS to further study the signalling details.

Agreement: Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS.

Whether the configurations of entry/exit thresholds are mandatory or optional, we think it would be good that network can always provide the configurations. The reasons are, for OOK-based WUR, the expected coverage is lower than the MR, thus a proper thresholds considering the coverage difference would be needed. For OFDM-based WUR, even though some companies argued that the coverage would be the same as the MR, however, the measurement conditions are different, i.e., the LP-SS and the SSB, thus the measured quality on the UE side is not the same. Considering these, we think it’s beneficial the network can always provide the configurations of the thresholds for the entry/exit conditions for both LP-WUR types. Otherwise, it’s up to UE implementation on whether to enter or exit the LP-WUS monitoring by using LR.

UE expects the network always configure the entry/exit conditions, otherwise it’s up to UE implementation on whether to enter/exit the LP-WUS monitoring by using LR.

Issue-2: LP-WUS subgrouping

By using the PEI, UEs in a PO can be further divided into one or several subgroups so that the UE shall monitor pagingonly if the subgroup to which the UE belongs is indicated as wake-up via PEI . With PEI, the paging false alarm can be reduced thus it can save the UE power. There are two ways to enable the subgrouping for PEI, one is the CN controlled subgrouping and the other is the UE ID based subgrouping. For CN controlled subgrouping, it’s the AMF to allocate the subgroup ID to the UE by NAS signaling, so that the UE can be indicated by the PEI whether the allocated subgroup the UE belongs to should monitor the associated paging or not. For UE ID based subgrouping, the gNB firstly assign the number of subgroup for the UE ID based subgrouping, then the UE derives the subgroup it belong to based on its UE ID.

For the UE supporting both LP-WUS and PEI, what is the UE behavior when both LP-WUS and PEI are configured simultaneously? There are following two cases:

- Case1: in the case when LP-WUS indicating UE wake-up to monitor paging, whether UE needs to monitor PEI before it’s monitoring paging? As agreed in RAN1: it agreed that it’s up to UE implementation. From our perspective, we think UE does not need to monitor PEI in the case when LP-WUS indicates UE should monitor the associate paging. Two reasons: one is that the extra monitoring PEI requires further power consumption, then the second one is that based on RAN1 agreements in RAN1#118, the maximum supported number of subgroup by LP-WUS would be no less  than PEI, so in practice the finer granularity can be provided by LP-WUS than PEI does.

Agreement

For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

- Case2: in the case when LP-WUS indicating UE NOT to wake-up to monitor paging, whether UE needs to monitor PEI? We think in this case, there is no need for the UE to monitor PEI either. First reason is the power consumption as the first case. The second reason is that, when LP-WUS does not wake up UE, it means all the UEs in the same subgroup does not need to monitor paging. Then why the UE needs to double confirm by PEI on this indication? Or is there any case that PEI indicates the UE to monitor paging instead while LP-WUS does not?

Based on the above, we propose:

UE does not need to monitor PEI, when both LP-WUS and PEI are supported by the UE, and these two features are configured simultaneously.

- Conclusion

We have the following proposals:

Proposal 1	UE can be configured to monitor LP-WUS or not even though it meets the condition of monitoring LP-WUS, e.g., via RRC Release message.

Proposal 2	UE expects the network always configure the entry/exit conditions, otherwise it’s up to UE implementation on whether to enter/exit the LP-WUS monitoring by using LR.

Proposal 3	UE does not need to monitor PEI, when both LP-WUS and PEI are supported by the UE, and these two features are configured simultaneously.

Annex

Following is the progress made in RAN1#116 and copied here for information.

R1-2401629 Summary #1 on LP-WUS operation in IDLE/INACTIVE mode Moderator (Apple) Agreement Multi-beam operations are supported for LP-WUS and LP-SS for idle mode R1-2401631 Summary #3 on LP-WUS operation in IDLE/INACTIVE mode Moderator (Apple) Agreement LP-WUS occasions (LOs) are defined for LP-WUS monitoring. - Each LO has one or more LP-WUS monitoring occasions (MOs), where UE can monitor for LP-WUS transmission in each of the LP-WUS MOs. - Different LP-WUS MOs may correspond to different beams in multi-beam operation - FFS whether or not each LO is defined as a time window that covers the corresponding LP-WUS MOs - FFS details - It is at least supported that a UE monitors LOs with a configured periodicity. - FFS eDRX, if supported Agreement For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not. Agreement It is supported that the UE monitors the legacy PO after receiving LP-WUS indicating wake-up. - FFS: support of UE monitoring dynamic PO Conclusion For idle/inactive mode, how to map a UE to a subgroup ID for LP-WUS is left to RAN2 to decide.

Following is the agreement made in RAN1#116bis.

Agreement For multi-beam operation of LP-WUS, UE assumes the same LP-WUS information payload is repeated in all transmitted beams corresponding to LP-WUS - the selection of the beam(s) for the reception of the LP-WUS is up to UE implementation Agreement Each LO consists of N * K LP-WUS MOs, where N is the number of beams corresponding to LP-WUS, and K is the number of LP-WUS MOs for each beam. - Option 1: K = 1 - Option 2: K can be larger than or equal to 1 - FFS if more than 1 LP-WUS is transmitted from the same beam, whether the information in these multiple LP-WUS is always the same or can be different Agreement From RAN1 perspective, at least the following metrics can be supported for RRM serving cell measurement performed by OOK-based receiver based on LP-SS: - LP-RSRP - LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols. - FFS: How to determine the received power of LP-SS in OOK ON symbols - LP-RSRQ - LP-RSRQ = LP-RSRP/LP-RSSI - For the definition of LP-RSSI for determination of LP-RSRQ, further consider the following options: - Option 1: LP-RSSI is the linear average of total received power in all LP-SS OOK symbols. - Option 2: LP-RSSI is the linear average of total received power in LP-SS OOK OFF symbols. - Option 3: LP-RSSI is the linear average of total received power in LP-SS OOK ON symbols. - FFS: LP-SINR Note: The exact metrics for OOK-based receiver to be used and defined in the specifications depend on the outcome of [RAN1]/RAN2/RAN4 discussions. Working Assumption From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - FFS the serving cell measurement metrics - The entry/exit thresholds can be configured separately for different types of LR - It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB. - Note: This may be revisited based on the RAN2/RAN4 discussion. Conclusion LP-SINR is not considered further as a metric for RRM serving cell measurement.

Following is the agreement made in RAN1#118.

Agreement For RRC idle/inactive state, support the following option for at least indicating subgroup information using LP-WUS: - Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs - UE monitoring one or more MOs (up to X MOs) for the same beam within an LO is supported - Value of X is larger than 1. FFS on additional details of X. Agreement [H][FL1] Proposal 3.1-1: Confirm the Working Assumption that OOK-4 with M=4 for 15KHz SCS is supported for LP-WUS. Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) - FFS M=1 for OOK-4. Agreement Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs (for non-CA case), select at least one from the following - Option 1: A bitmap with each bit corresponding to [one or more] UEs - Option 3: A codepoint value corresponding to [one or more] UEs - FFS details for extension of option 1 and/or 3 when UE is configured with CA Agreement For overlaid OFDM sequences for LP-WUS, support option 1-1 for OOK-4 M>1. - Option 1-1: overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing Agreement For overlaid OFDM sequences for LP-WUS, further down-selection between following two options for OOK-1 and OOK-4 with M=1(if supported) - Option 1-1: Overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing - Option 2: Overlaid sequence(s) are the sequence(s) of an OFDM symbol before IFFT processing - Note: Different options for OOK-1 and OOK-4 with M=1 (if supported) is not precluded – in which case, it should be deemed necessary Agreement The number of binary LP-SS sequences is 4. Agreement To determine the binary sequences for LP-SS, the evaluation assumes the following: - Sync accuracy: timing estimation error smaller than T us for P=90 % of the time for at least SNR=-3dB and SNR=-6dB (lower priority), - T=2us(optional), 5us for OOK-1 - T=1us(optional), 2us for OOK-4 with M=2 - T=0.5us(optional), 1us for OOK-4 with M=4 - Other values of SNR, T, M are up to company report. - Additionally, companies can submit results on SINR with details on how the interference was modelled. - Assume one-shot estimation - The time error is based on 20ppm maximum frequency error for the detection of the first LP-SS - Note: Companies can assume other values inside of 20ppm maximum as long as the values are justified - RRM measurement accuracy: measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s for at least SNR=-3dB and SNR=-6dB (lower priority), X, Y, and Z is up to company report, other values of SNR are up to company report. - As a starting point, the time error is based on 5-10ppm residual frequency error. - Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance - Note: Companies can assume other values outside of 5-10ppm as long as the values are justified - The frequency error: up to company report - Sampling rate: 3.84MHz (optional) or 7.68MHz assumed for 30kHz SCS. - Channel: AWGN and TDL-C 300ns is assumed. - Consider cross-correlation for 4 binary sequences under time and frequency error - Companies are encouraged to provide details on other additional simulation assumptions, high level description of their receiver algorithm, and assessment on power consumption Agreement For OOK-based LP-WUR, the LP-WUS design assumes the following: - As starting point, the time error is based on the 5-10ppm residual frequency error after frequency error correction without considering impact of drift. - The frequency error: up to company report. - Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance - Note: Companies can assume other values outside of 5-10ppm as long as the values are justified Agreement As a starting point, for both time error and frequency error, the overlaid OFDM sequence design of LP-WUS assumes that the residual frequency error is 0.1-5ppm for OFDM-based LP-WUR after frequency error correction without considering impact of drift. - Note: Companies can use any value within the above range with justification on the value (including impact on power consumption) Agreement Support Manchester coding for LP-WUS FFS other coding schemes Agreement The definitions of LP-RSRP and LP-RSSI for LP-RSRQ are updated as follows: LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols over the frequency resources defined by the number of REs that carry LP-SS. LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols over the frequency resources defined by the number of REs that carry LP-SS. Agreement At least support 1:1 association between LP-WUS MO(s)/LP-SS transmissions and SSB beams. Agreement For the wake-up delay, consider the following options: Option 2: UE reports one value from X candidate values for the wake-up delay via UE capability reporting. - FFS: X is 2, 3, 4 Above applies for IDLE/INACTIVE mode. Definition of wake-up delay: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring Agreement For the maximum number of subgroups per PO (X), down-select from the following options in RAN1#118bis: - Option 1: X = 8 - Option 2: X = 16 - Option 3: X = 32 - Option 4: X = 64 - Option 5: X = 128 - Option 6: X = 256 For decision in RAN1#118bis, companies are encouraged to provide the maximum number of information bits per LP-WUS (Z), the number of OFDM symbols occupied by LP-WUS per MO, the number of MOs for their preferred option. Conclusion There is no consensus in RAN1 on the support of dynamic PO. Agreement On the LO configuration for iDRX, offset value(s) between a LO and a reference PO/PF is/are configured. - FFS: one or multiple offset values, and if multiple offset values are supported, how a UE decides which value to use - FFS: the exact definition of the reference PF/PO and the detailed procedure for UE to determine the LO(s) corresponding to Option 1, 2, or 3 (if supported) - (Working Assumption) For each UE, the periodicity of LO is the same as its iDRX cycle. - If a UE receives a wake-up indication in a LP-WUS, consider the following alternatives - Alt 1: it monitors the PO associated with the offset. - Alt 2: it monitors the first PO after its reported wake-up delay. - Other alternatives are not precluded Note: The PO mentioned above refers to legacy PO configured for the UE. Agreement Send an LS to RAN2 and RAN4 to convey the following In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. Request RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs. Final LS in R1-2407559. Conclusion RAN1 will not initiate work on entry/exit conditions based on RRM measurement and RRM measurement offloading/relaxation conditions unless triggered by RAN2 and RAN4 Agreement For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - UE is configured with legacy C-DRX configurations as Rel-18 - UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration) - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration - LP-WUS triggers the start of a timer during which UE monitors PDCCH - FFS the timer is existing timer and/or new timer - UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL - No impact on RRM/RLM/BFD measurement requirements is assumed - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1) - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not - UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions. Agreement Select one of the following alternatives in RAN1#118bis Alt 1: For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported value Alt2: For RRC CONNECTED mode, UE reports multiple values for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported value - Different minimum time gaps correspond to different sleep states - Companies are encouraged to details on how the reported values are to be used by the network Agreement LP-WUS is at least supported for the case where a UE is configured with CA in RRC CONNECTED mode - FFS: DC Agreement For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS - FFS applicable QCL type(s)

Following was agreed in RAN2#125bis.

The LP-WUS related configuration for IDLE/INACTIVE state is provided via system information. FFS if dedicated configuration is needed. Working assumption: the LP-WUS configuration in SIB at least includes the following information: - - LP-SS configuration - - LP-WUS configuration - - FFS on Entry/exit condition for LP-WUS monitoring The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups.

RAN2#126 agreements:

RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions. The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - - LP-SS configuration - - LP-WUS configuration - - Entry/exit condition for LP-WUS monitoring (FFS if it always configured) Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE. RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8. From RAN2 perspective, no new procedure is introduced for SI reception/updates.

RAN2#127 agreements:

Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.