3GPP TSG-RAN WG2 Meeting #127bis 	R2-2408455

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.6.3

Source:	Panasonic

Title:	L1 measurement event configuration and reporting

Document for:	Discussion

# Introduction

In last meeting, RAN2 achieved the following agreements on LTM measurement event evaluation and reporting.

Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

Agreements on L1 measurement reporting

- Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

- Event triggered L1 measurement should be designed for the following LTM purposes:

a. Select the candidate beam/cell to trigger early synchronization

b. Select the target beam/cell and trigger LTM cell switch procedure

3.  For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In this paper, we focus on addressing some remaining issues. In particular, we discuss design details on resource configuration, report configuration and how to ensure the same RS type for both serving and candidate cells. Further, we discuss few aspects on event trigger conditions when configured for both early synchronization and cell switch procedures.

# Discussion

## Measurement resource configuration and report configuration

As agreed in previous meeting, the baseline of configuration signalling is:

- LTM measurement resource configuration is provided in LTM-config

- Event triggered report config is provided in serving cell config

In Rel-18, the LTM measurement resource configuration is provided via LTM-config  LTM-CSI-ResourceConfig which supports only SSB as reference signal (See below).

-- ASN1START

-- TAG-LTM-CSI-RESOURCECONFIG-START

LTM-CSI-ResourceConfig-r18 ::=      SEQUENCE {

ltm-CSI-ResourceConfigId-r18        LTM-CSI-ResourceConfigId-r18,

ltm-CSI-SSB-ResourceSet-r18         LTM-CSI-SSB-ResourceSet-r18,

...

}

LTM-CSI-SSB-ResourceSet-r18 ::=     SEQUENCE {

ltm-CSI-SSB-ResourceList-r18        SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF SSB-Index,

ltm-CandidateIdList-r18             SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

-- TAG-LTM-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

For Rel-19 event triggered L1 measurement, the CSI-RS needs to be supported. One straightforward approach is to extend the IE LTM-CSI-ResourceConfig to include CSI-RS resource set in addition to the SSB resource set. There are two options to do this:

- Option 1: One resource configuration configures both SSB resource set and CSI-RS resource set

- Option 2: Different reference signal types belong to different resource configurations

The option 1 has the benefit of being flexible and consumes less number of resource configuration ID space. On the other hand, option 2 does not affect the legacy Rel-18 resource configuration because the newly supported CSI-RS in Rel-19 can be configured in a separate resource configuration. Note that the above discussion on measurement resource configuration applies to candidate cell only, because we have agreed that for serving cell the current serving beam (beam corresponding to the indicated TCI state) is used for event evaluation. As will become clear later in our discussion, whether option 1 or 2 is adopted would impact how to ensure the same RS type is used for evaluation between the serving cell and candidate cells.

Proposal 1. For LTM event triggered measurement resource configuration (for candidate cells), RAN2 to further discuss the following two options to support CSI-RS:

- Option 1: One resource configuration configures both SSB resource set and CSI-RS resource set

- Option 2: Different reference signal types (SSB vs. CSI-RS) belong to different resource configurations

For the report configuration, it has been agreed to be provided in serving cell config. However, we are still missing an agreement on how to link the resource configuration with the report configuration. In Rel-18, this is done by including a resource configuration ID in the report configuration. We don’t see any issue to reuse this mechanism for Rel-19 event triggered report configuration.

Proposal 2. A resource configuration ID is included in the report configuration to associate a LTM event reporting with the measurement resources (for candidate cells).

## How to ensure the same RS type

As mentioned in section 1, the beam for event evaluation would be decided differently for serving cell and candidate cell as follows:

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

First, we suggest to confirm the working assumption i.e. same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5, because otherwise, the measurements are not comparable between serving and candidate cells.

To ensure the same RS type, in case that both RS types are configured in the resource configuration for candidate cells (see option 1 of proposal 1), the straightforward method is the RS type follows the serving cell. In more details, for serving cell, UE measures the QCL RS of the indicated TCI state, and for candidate cells, UE measures the configured RS(s) with the same RS type as the QCL RS of the indicated TCI state of the serving cell.

On the other hand, if the resource configuration for candidate cells contains only one RS type (see option 2 of proposal 1) and this type happens to be different than the QCL RS of the indicated TCI state of the serving cell, following serving cell might not be working. For example, the QCL RS of the indicate TCI state of serving cell is CSI-RS but the configured RS set for candidate cells only include SSB. We think it is feasible to use SSB as RS type as CSI-RS can be sourced to SSB eventually via QCL chain. This applies to candidate cell CSI-RS as well.

There is another scenario where SSB needs to be used for evaluation is in case that no TCI state is indicated in the serving cell. This can happen after initial access and before any TCI state is indicated to the UE. UE just keeps using the SSB beam selected from the random access procedure in the serving cell. Then for candidate cell evaluation, SSB should also be used.

To summarize, we have the following proposals.

Proposal 3. The RS type for event evaluation should follow the QCL RS associated with the indicated TCI state in the serving cell; or if this is not possible (e.g. resource configuration for candidate cell contains only SSB but serving cell indicated TCI state has CSI-RS as QCL RS, or no TCI state is indicated in the serving cell), RS type is SSB.

Proposal 4. If SSB is decided as RS type for event evaluation but the indicated TCI state has CSI-RS (for serving cell) or configured resource set has only CSI-RS (for candidate cell), UE measures the SSB which is QCL source of the indicated or configured CSI-RS.

## Cell or beam level measurement

In previous meeting, it has been agreed that

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case

However, we think it still needs to be clarified whether TTT is applied per beam or per cell. For example, beam A satisfies the event and TTT starts, during TTT, another beam B instead of beam A satisfies the event. Should TTT be re-started or just continued? From our perspective, it makes sense that in this case TTT is continued (i.e. not reset if candidate beam changes), because otherwise, the triggering of the event can be delayed too much. Furthermore, this is more aligned with the agreement “Any beam in candidate RS configuration can be used for LTM event evaluation.”

Proposal 5. TTT is per candidate cell rather than per beam, meaning that TTT is not reset if candidate beam satisfying the same event changes within the same candidate cell.

## Event triggered measurement reporting using other reporting types

Early DL/UL synchronization in LTM can be triggered based on the event measurement reporting from UE, and since the TA value is obtained/ maintained by the source gNB/gNB-DU at the end of the early UL synchronization operation, the following two failures scenarios in TA acquisition can occur during LTM procedure and degrade the overall handover performance at UE.

One scenario, when early synchronization is triggered before the cell switch operation with the target cell and the TA value as expected from the target cell (or target gNB/gNB-DU) is not available on time for the cell switch execution, then the cell switch operation is most likely to be continued as a RACH-based execution. Here the reason for early TA acquisition failure (to obtain on time) could be due to early synchronization procedure triggered too late, especially in a dynamic channel condition during UE mobility.

Second scenario, where the early UL synchronization is triggered much earlier with one or more candidate cells prior to LTM cell switch decision phase, then the TA acquisition can be ensured to be successful/ available for RACH-less cell switch. But consequentially, the computed TA value of each (one or more) candidate cells can become invalid by the time when necessary for LTM cell switch operation towards one target cell, because the early TA acquisition procedure was triggered too early.

Therefore, it is necessary to perform regular channel condition monitoring of candidate beams/ cells also during early synchronization procedure to ensure that the DL/UL synchronization is not triggered as ‘too early’ or ‘too late’ condition for the intended LTM cell switch operation.

Figure 1: LTM procedure (source – TS38.300v18.1; 9.2.3.5 L1/L2 triggered mobility)

To regularly monitor channel conditions during event triggered measurement reporting of candidate beams and to trigger early synchronization procedure in LTM (or cell switch procedure) accordingly, LTMx event trigger conditions must be configurable to report using other reporting types.

Observation 1: Regular reporting of L1 measurements is necessary during event evaluation in LTM, to avoid the situation of ‘too early’ or ‘too late’ triggering of early synchronization procedure.

Accordingly, the entering condition of LTMx events must be configurable to trigger as one or multiple options among periodic triggering, semi-persistent period triggering or by aperiodic requests sent from the network.

Proposal 6: LTMx event configuration to include other reporting type parameters to report measurements of LTM candidate cells/beams regularly, based on one or more of the following reporting types.

- Periodic triggering: LTMx event conditions are triggered to generate and report measurement results based on the configured periodicity.

- Semi-persistent triggering: LTMx event conditions are triggered to generate and report measurement results based on the configured periodicity, and if the semi-persistent activation is sent by the network.

- Aperiodic triggering: LTMx event conditions are triggered and reported once. While new measurement evaluations are reported when aperiodic requests are sent by the network.

## LTMx event trigger condition for early synchronization reporting.

In RAN2#126 meeting it was agreed to include evaluation conditions such as ‘entering condition’ and ‘leaving condition’ for LTMx events to report L1 measurements. And another agreement captured that the L1 measurement should be designed to trigger early synchronization and cell switch procedures. As it is understood from the legacy event configuration terminology, few of the event configuration methods are analysed when event trigger conditions are configured for early synchronization and further depicted some of the drawbacks when the same is implemented.

Method 1: Single LTMx event configuration for early synchronization and cell switch.

Entering condition for LTMx events can be shifted/ adjusted to fulfil early synchronization criteria as per LTM procedure requirements, such that UE starts reporting earlier for each of the evaluated beams as required for channel monitoring during early synchronization phase. Further, when LTMx event is configured to trigger periodically, then measurement results are continued to be reported for the cell switch procedure requirements as well. Such assumption is illustrated in figure 2, here event LTM4 (candidate beam becomes better than absolute threshold) is configured with threshold value shifted lower to fulfil early synchronization criteria along with periodicity configurations (like ‘report amount’ and ‘report interval’) to trigger measurement reporting for early synchronization first and then continued to be reported periodically during cell switch criteria.

Figure 2: Event LTM4 configured for early sync and cell switch measurement reporting

Considering the above assumption to configure entering condition commonly for early synchronization and cell switch, and since the measurement reports to trigger TA acquisition should not encounter situations like ‘too late’ scenario, the entering condition must be configured to report much earlier such that the measurement reports are available for regular channel monitoring during early synchronization at the gNB. This results in longer reporting amounts (in periodicity) when configured for measurement reporting first for early synchronization scenario and continued to be reported for cell switch scenario.

Observation 2: LTMx event configuration with single entering condition configured commonly for early synchronization and cell switch requirements, results in larger reporting amounts (in periodicity) especially when configured for channel monitoring as to overcome ‘too late’ scenarios for TA acquisition.

Also, when the threshold value is shifted to satisfy early synchronization for measurement reporting, it is likely that larger number of beams are triggered during evaluation of entering condition and since such measurements are continued to be reported for cell switch procedure (using the same periodicity) this method appears to occupy UL resources unnecessarily. As seen in figure 3.

Figure 3: Illustration of shifting threshold to fulfil early synchronization criteria

Observation 3: Shifting entering condition if configured for early synchronization, triggers larger number of beams. And when same (commonly configured) entering condition is used to evaluate measurements to report for cell switch procedure, appears to occupy more UL resources during reporting than expected.

Method 2: Combination of LTMx event configurations for early synchronization and cell switch.

Further to avoid triggering of larger number of beams during cell switch procedure (as seen is method-1), multiple LTMx event configuration could be defined for individual LTM procedures, with one (or more) LTMx event configured separately to trigger for early synchronization reporting (entering condition shifted to fulfil early synchronization criteria), while the other (one or more) LTMx event is configured for cell switch reporting. Now the LTMx event(s) which are dedicated to early synchronization reporting need not be configured with larger reporting amounts (in periodicity), which enables optimization of UL resource utilization during LTM procedure (occupying less resources for measurement reporting).

Observation 4: LTMx event configuration requires (or mandates) to always include multiple LTMx events within LTM event configuration to optimize UL resource utilization, during measurement reporting of corresponding LTM related procedure. For example, to configure with lower reporting amounts (in periodicity) to save UL bandwidth during early synchronization reporting.

Therefore, in view of the above drawbacks, when LTMx event condition (like one entering condition) is configured to report for both early synchronization and cell switch procedure in LTM, the following aspects are important to be addressed.

- LTMx event configuration to consider efficient UL resource utilization during reporting procedures in LTM.

- To trigger optimum number of beams for the respective LTM procedure. Ideally, cell switch reporting condition needs to trigger a smaller number of beams than early sync reporting condition.

- LTMx event configuration should be flexible to configure a single LTMx event or a combination of LTMx events based on different UE capabilities and mobility scenarios.

If a single LTMx event to be supported within LTM event configuration (i.e. not as a combination of multiple LTMx events being configured) and reported as per the requirements derived from the above bullet points, the entering condition must be enhanced according to the requirements of early synchronization criteria and cell switch criteria.

Proposal 7: LTMx event configuration to include separate event trigger conditions (like the entering condition), to be evaluated and reported each for early synchronization criteria and cell switch reporting criteria accordingly.

# Conclusion

In this paper, we discuss some remaining issues for L1 measurement event. The observations and proposals are as follows.

RS configuration and report configuration

Proposal 1. For LTM event triggered measurement resource configuration (for candidate cells), RAN2 to further discuss the following two options to support CSI-RS:

- Option 1: One resource configuration configures both SSB resource set and CSI-RS resource set

- Option 2: Different reference signal types (SSB vs. CSI-RS) belong to different resource configurations

Proposal 2. A resource configuration ID is included in the report configuration to associate a LTM event reporting with the measurement resources (for candidate cells).

Proposal 3. The RS type for event evaluation should follow the QCL RS associated with the indicated TCI state in the serving cell; or if this is not possible (e.g. resource configuration for candidate cell contains only SSB but serving cell indicated TCI state has CSI-RS as QCL RS, or no TCI state is indicated in the serving cell), RS type is SSB.

Proposal 4. If SSB is decided as RS type for event evaluation but the indicated TCI state has CSI-RS (for serving cell) or configured resource set has only CSI-RS (for candidate cell), UE measures the SSB which is QCL source of the indicated or configured CSI-RS.

Cell or beam level measurement

Proposal 5. TTT is per candidate cell rather than per beam, meaning that TTT is not reset if candidate beam satisfying the same event changes within the same candidate cell.

Combination of event triggered reporting and other reporting types

Observation 1: Regular reporting of L1 measurements is necessary during event evaluation in LTM, to avoid the situation of ‘too early’ or ‘too late’ triggering of early synchronization procedure.

Proposal 6: LTMx event configuration to include other reporting type parameters to report measurements of LTM candidate cells/beams regularly, based on one or more of the following reporting types.

- Periodic triggering: LTMx event conditions are triggered to generate and report measurement results based on the configured periodicity.

- Semi-persistent triggering: LTMx event conditions are triggered to generate and report measurement results based on the configured periodicity, and if the semi-persistent activation is sent by the network.

- Aperiodic triggering: LTMx event conditions are triggered and reported once. While new measurement evaluations are reported when aperiodic requests are sent by the network.

Triggering conditions for early sync and cell switch

Observation 2: LTMx event configuration with single entering condition configured commonly for early synchronization and cell switch requirements, results in larger reporting amounts (in periodicity) especially when configured for channel monitoring as to overcome ‘too late’ scenarios for TA acquisition.

Observation 3: Shifting entering condition if configured for early synchronization, triggers larger number of beams. And when same (commonly configured) entering condition is used to evaluate measurements to report for cell switch procedure, appears to occupy more UL resources during reporting than expected.

Observation 4: LTMx event configuration requires (or mandates) to always include multiple LTMx events within LTM event configuration to optimize UL resource utilization, during measurement reporting of corresponding LTM related procedure. For example, to configure with lower reporting amounts (in periodicity) to save UL bandwidth during early synchronization reporting.

Proposal 7: LTMx event configuration to include separate event trigger conditions (like the entering condition), to be evaluated and reported each for early synchronization criteria and cell switch reporting criteria accordingly.