3GPP TSG-RAN WG2 Meeting#127bis                                	R2-2408460

Hefei, China, October 14th – 18th, 2024

Agenda item:	8.9.2

Source: 	Google

Title: 	Access control and the S&F indication

Document for:	Discussion and Decision

# Introduction

In Rel-19, the benefits of enabling the Store & Forward operation for IoT-NTN are identified and the following scopes are included in the WID of Rel-19 IoT-NTN [1].

Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]. - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required.

The Rel-19 IoT-NTN discussion was kicked off in the RAN2#125bits meeting, and some progress was made in RAN2#126 and RAN2#127 meeting, with regard to the S&F indication provided by the network [2][3].

RAN2 Agreements in RAN2#126: - S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed - RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN. RAN2 Agreements in RAN2#127: - RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options) - RAN2 will consider both single satellite pass and multiple satellite pass scenarios - RAN2 will consider both MO and MT data within scope - UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature) - RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected - RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

In this paper, we discuss several possible designs regarding the S&F indication based on the current progress, and discuss the interaction between the RRC and NAS layer for supporting the S&F operation. Based on the discussion, several way forwards are proposed in this paper.

# Discussion

## Access control based on the S&F indication

It was agreed in the RAN2#127 meeting that the UE shall be informed whether its serving satellite is currently operating in S&F via system Information. Since the serving satellite would only operate in the S&F mode when the feeder link is not available, such an S&F indication also reflects the feeder link status. As it was also agreed in RAN2#127 meeting that there will be a way to bar legacy UEs and still allow Rel-19 S&F capable UEs, we think this S&F indication should also serve for this purpose. That is, when the S&F indication is present in the system information, the Rel-19 UEs shall ignore the legacy cell barring bits and check the S&F indication to determine whether to access the cell or not. Note that the S&F indication is only needed for the split-MME payload option, as in the full CN payload option, there should be no impact to the UE AS and UE NAS.

When the S&F indication is present in the system information, the Rel-19 UEs ignore the legacy cell barring bits and check the S&F indication to determine whether to access the cell or not.

On the other hand, if the S&F indication is not present and both legacy cellBarred and cellBarredNTN are set to barred in a cell, then the Rel-19 UEs (regardless of their S&F capability) are not allowed to access the cell. For the Rel-19 UEs capable of the S&F operation, if the S&F indication is present, these UEs are allowed to access the cell no matter which value is indicated in the S&F indication. Here the UE capable of the S&F operation means the UE that is able to deal with the long break occurring in a transaction due to the temporary unavailability of the feeder link.

If the S&F indication is not present and both legacy cellBarred and cellBarredNTN are set to barred, the cell is considered as barred for NTN access.

If the S&F indication is present, the UE capable of the S&F operation is allowed to access the cell.

The content of the S&F indication has no impact to the Rel-19 UEs capable of the S&F, but does have impact to the Rel-19 UEs not capable of the S&F. If the S&F indication indicates the serving cell is currently not operating in S&F (i.e., the feeder link is still available), Rel-19 UEs not capable of the S&F operation are allowed to access the cell. If the S&F indication can further indicate the remaining time of the feeder-link when the feeder-link is available, the access control for the Rel-19 UE not capable of S&F can further depend on the remaining time of the feeder-link. On the other hand, if the S&F indication indicates the serving cell is currently operating in S&F mode (i.e., the feeder-link is not available), we think the Rel-19 UE not capable of the S&F operation can still access the cell in certain circumstances (e.g., EDT transmission or being paged for DL data reception) but should refrain from accessing the cell for other circumstances.

If the S&F indication is present and indicates the serving cell is not operating in S&F, the Rel-19 UE not capable of the S&F operation is allowed to access the cell.

If the S&F indication is present and indicates the serving cell is operating in S&F, the Rel-19 UE not capable of the S&F operation may be allowed to access the cell, depending on the cause.

As mentioned earlier, to facilitate better access control for the Rel-19 UEs not capable of S&F, the S&F indication can indicate the remaining time of the feeder-link when the feeder-link is currently available, so that these UEs can determine whether to access the cell or not, based on how long the feeder-link will remain available.

The S&F indication can contain the information regarding the remaining time of the feeder-link.

For the ease of understanding, we use Table 1 below to summarize the access control details proposed in the paper.

Table 1. The access control base on the legacy cell barring bits and the S&F indication.

|  |  | Legacy UE | Rel-19 UE capable of S&F | Rel-19 UE not capable of S&F |
| --- | --- | --- | --- | --- |
| Legacy cell barring bits (cellBarred and cellBarredNTN) = barred | S&F is absent | Not allowed | Not allowed | Not allowed |
|  | S&F = true | Not allowed | Allowed | Maybe (depends on the cause) |
|  | S&F = false | Not allowed | Allowed | Allowed |
| Legacy cell barring bits (cellBarred and cellBarredNTN) = not barred | S&F is absent | Allowed | Allowed | Allowed |
|  | S&F = true | Allowed | Allowed | Maybe (depends on the cause) |
|  | S&F = false | Allowed | Allowed | Allowed |

Alternatively, we may also consider other options such as the option to simplify the S&F indication design and leave the access control to the network. That is, the S&F indication simply controls whether the Rel-19 UE (regardless of the S&F capability) can access the cell or not, and then the network determines whether to grant the access for each UE individually based on their S&F capabilities reported during the access attempt. For instance, the network can reject the access attempt from the UE not capable of the S&F operation if the feeder-link will not last for a long time. To facilitate such network decisions, we can make Rel-19 UEs inform the eNB of their S&F capabilities at the earliest timing during the initial access (i.e., via MSG3).

A Rel-19 UE can inform eNB of its S&F capability via MSG3.

## The suspension of UE’s RRC and NAS in split-MME payload option

While connecting to a cell operating in the satellite S&F mode without having a feeder link, the UE may first establish its RRC connection with the cell and then initiate a NAS procedure by transmitting a NAS message. For the split-MME payload option or for the scenario when only the full eNB is onboard, it may take quite some time for the UE to receive the response to the NAS message, most likely after the UE has become out of coverage of the connected satellite and returned to the coverage again of the satellite carrying the NAS response. Therefore, during the lifetime of a NAS transaction, the UE may have moved between different RRC states multiple times. As these transitions between different RRC states may clear/reset the NAS progress especially if the UE has moved to RRC_IDLE once, we should avoid UE moving to the idle state (RRC_IDLE as well as ECM_IDLE) whenever the feeder link or the service link becomes unavailable if there is an on-going NAS procedure. For this purpose, when the feeder is or becomes not available, eNB can suspend UE’s RRC connection by sending an RRC Connection Release message to the UE supporting the S&F satellite operation, where the RRC Connection Release message may include an rrc-InactiveConfig IE or set the releaseCause as ‘rrc-Suspend’. Upon receiving the RRC Connection Release message, the UE RRC indicates the suspension of the RRC to the NAS layer, which may result in the suspension to the NAS layer as well.

Since the existing RRC_INACTIVE state and RRC_IDLE state with RRC suspension is only applicable to the UE that has already attached/registered to a network, it can be further discussed if a new RRC suspension state/mode is needed (i.e., a new release cause is needed) to support the initial ATTACH scenario.

When the feeder-link is not available, eNB can suspend UE’s RRC and NAS by transmitting an RRC Connection Release message to the UE capable of the S&F satellite operation. FFS whether a new release cause is needed.

# Conclusion

In this paper, we discuss the detailed access control based on the new S&F indication, and respectfully ask RAN2 to discuss and consider the following proposals.

Proposal 1	When the S&F indication is present in the system information, the Rel-19 UEs ignore the legacy cell barring bits and check the S&F indication to determine whether to access the cell or not.

Proposal 2	If the S&F indication is not present and both legacy cellBarred and cellBarredNTN are set to barred, the cell is considered as barred for NTN access.

Proposal 3	If the S&F indication is present, the UE capable of the S&F operation is allowed to access the cell.

Proposal 4	If the S&F indication is present and indicates the serving cell is not operating in S&F, the Rel-19 UE not capable of the S&F operation is allowed to access the cell.

Proposal 5	If the S&F indication is present and indicates the serving cell is operating in S&F, the Rel-19 UE not capable of the S&F operation may be allowed to access the cell, depending on the cause.

Proposal 6	The S&F indication can contain the information regarding the remaining time of the feeder-link.

Proposal 7	A Rel-19 UE can inform eNB of its S&F capability via MSG3.

Proposal 8	When the feeder-link is not available, eNB can suspend UE’s RRC and NAS by transmitting an RRC Connection Release message to the UE capable of the S&F satellite operation. FFS whether a new release cause is needed.

# Reference

RP-234007, New WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3.

R2-2406202, Report of 3GPP TSG RAN WG2 meeting #126, ETSI MCC.

Draft report of 3GPP TSG RAN WG2 meeting #127, ETSI MCC.