3GPP TSG-RAN WG2 Meeting #127bis 	R2-2408463

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.2.3

Source:	Panasonic

Title:	Discussion on A-IoT paging

Document for:	Discussion

# Introduction

In last meeting, RAN2 achieved the following agreements on A-IoT paging.

Agreements 1 Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices.

In this paper, we further discuss multiple/subsequent paging and related issues.

# Discussion

## Multiple/subsequent paging

As mentioned in section 1, RAN2 has agreed to support mechanisms for reader to be able to trigger multiple/subsequent paging messages. The motivation is due to the facts that reader typically cannot reach all targeted devices in a coverage area by single one paging round. The reasons could be, e.g. device is not sufficiently charged yet and therefore cannot respond to the first (several) rounds of paging, or there is contention for UL resources when multiple devices try to respond and therefore some devices fail the access attempt. In such cases, the device who has already completed the access procedure triggered by the previous round of paging should not respond to the subsequent paging again.

To realize it, one way is to narrow down the paging targets by adjusting, e.g. the group ID, in the subsequent paging. However, this could introduce further latency as reader needs to check which devices have not replied in the first round, and it might not always feasible to adapt the group ID to the desired target devices.

Alternatively, a simpler way could be to just repeat the same paging message with same target group, and let the device to decide whether to respond again. In this case, as proposed in several papers (e.g. see [1][2]), a paging transaction ID can be included in the paging for device to know whether this is the same paging or a new one. We acknowledge that this method is useful. For this method to work, device needs to keep the transaction ID in the memory after it has successfully completed the access procedure triggered by this transaction ID.

Observation 1. A paging transaction ID can be useful for device to know whether it is a new paging or a subsequent paging. Device needs to keep the transaction ID in the memory after the access procedure triggered by this transaction has been completely successfully.

Yet another alternative is to use a timer to prevent duplicated responses. There are several options to start the timer. For example, timer can be started after the end of access occasion, or after completion of the whole access procedure triggered by this transaction (e.g. receiving ACK after Msg3). Device does not need to respond to any paging as long as timer is running. The timer value can be set to cover a typical number of subsequent paging messages. Comparing to different timer starting options mentioned above, if timer is already started after end of access occasion (i.e. after sending Msg1), it effective means that device would assume the access procedure would be completed.

Observation 2. A timer can be used to prevent device from responding to subsequent paging.

When comparing timer solution and paging transaction ID solution, timer would block any paging without differentiating a new one or repeated one. On the other hand, transaction ID solution enables device to respond a new transaction even in a middle of previous round of transaction. For example, the reader is sending 4 paging repetitions with the same transaction ID 1. After 2nd repetition, there is an urgent service request from CN to page the device. Then reader can insert a new paging with transaction ID 2 without need to wait for the end of 4 pagings of transaction ID 1. The device needs to respond to the new paging because it has a new transaction ID. In this regard, transaction ID solution is more flexible.

Proposal 1. Support paging transaction ID to avoid duplicated responses from device in case of subsequent paging.

## Multi-reader area

In some scenarios, a device can be located in a multi-reader overlapping area. See Fig.1. as an example.

Fig.1. Multi-reader overlapping area

In this case, device may receive paging messages from different readers with the same or different transaction IDs. Assuming no coordination among readers, it may happen that the paging transactions from different readers are interleaved, as illustrated in the following Fig.2.

Fig.2. Paging from multiple readers

In the scenario shown in Fig.2, even with transaction ID as proposed in proposal 1, the device may still need to respond to repeated paging messages because the device sees every paging as a new transaction compared to what has been kept in the device’s memory (assuming device only stores one transaction ID due to power/complexity constraint). Since A-IoT does not have a serving cell concept, it seems that some rule needs to be define for device to know the device-reader association. With such association, device will respond only to the corresponding reader until such association is changed.

Observation 3. In a multi-reader overlapping area, using transaction ID alone may not avoid duplicated responses from the device.

Proposal 2. RAN2 to further discuss the need to introduce device-reader association and whether/how such association is known to the device.

# Conclusion

In this paper, we have discussed multiple/subsequent paging and related issues. We have the following observations and proposals.

Observation 1. A paging transaction ID can be useful for device to know whether it is a new paging or a subsequent paging. Device needs to keep the transaction ID in the memory after the access procedure triggered by this transaction has been completely successfully.

Observation 2. A timer can be used to prevent device from responding to subsequent paging.

Proposal 1. Support paging transaction ID to avoid duplicated responses from device in case of subsequent paging.

Observation 3. In a multi-reader overlapping area, using transaction ID alone may not avoid duplicated responses from the device.

Proposal 2. RAN2 to further discuss the need to introduce device-reader association and whether/how such association is known to the device.

# Reference

[1] R2-2406681, Discussion on Ambient IoT paging, Apple, RAN2#127

[2] R2-2407244, Discussion on DL messages for Ambient IoT UEs, Ericsson, RAN2#127