3GPP TSG-RAN WG2 Meeting #127bis	R2-2408467

Hefei, China, Oct 14th – 18th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1937 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on HARQ process |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Ericsson, Nokia, LG Electronics Inc., Qualcomm Incorporated, Apple, ZTE, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core, NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Issue 1: HARQ FB determination Clause 5.3.2.2 specifies different conditions for determining whether to or not to instruct the physical layer to generate acknowledgement(s) of the data in this TB. It is noted that the configuration for HARQ process(es) with disabled HARQ feedback is separate/distinct from the configuration of HARQ feedback for a G-RNTI or a G-CS-RNTI or a configured downlink assignment for MBS multicast. Further, NTN service data reception can be through MBS multicast in a serving cell that is part of NTN. Therefore, condition for determining about HARQ feedback is particularly related to the transmission with which the concerned HARQ process is associated with. Previously, for this scenario of MBS multicast in NR NTN, RAN2#126 made an agreement as below. - MBS Multicast HARQ configuration takes precedence Also to be noted that earlier during MBS WI, RAN2 discussed and agreed for no segregation or no dedicated HARQ process(es) for MBS multicast. This is particularly because any such segregation limits the network operation and flexibility. Moreover, MBS multicast HARQ FB configuration can be dynamically changed (e.g. enabled/disabled through DCI), so it is even more complicated to do any such assignment of HARQ processes. As a result, the existing condition of not providing HARQ feedback “if the HARQ process is configured with disabled HARQ feedback” should be replaced with “if the HARQ process is configured with disabled HARQ feedback and is not associated with a transmission indicated with a G-RNTI or a G-CS-RNTI or a configured downlink assignment for MBS multicast” Issue 2: Spec misalignment between Rel-17 and Rel-18 Rel-17 and Rel-18 specifications are not consistent as the last OR condition in Rel-17 is specified in the ELSE section in Rel-18. Rel-17 1> if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted, is stopped or expired and if the cg-SDT-TimeAlignmentTimer, if configured, is not running; or 1> if the HARQ process is configured with disabled HARQ feedback: 2> if harq-FeedbackEnablingforSPSactive is configured with value true and the transmission is the first transmission on the configured downlink assignment after activation of the configured downlink assignment: 3> instruct the physical layer to generate acknowledgement(s) of the data in this TB. 2> else: 3> not instruct the physical layer to generate acknowledgement(s) of the data in this TB.. Rel-18 1> if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted, is stopped or expired, and the Serving Cell is not configured with two TAGs, and if the cg-SDT-TimeAlignmentTimer, if configured, is not running; or 1> if the Serving Cell on which the HARQ feedback is to be transmitted is configured with two TAGs and if the timeAlignmentTimer of the TAG, associated with the TCI state(s) used for transmitting the HARQ feedback, is stopped or expired: 2> not instruct the physical layer to generate acknowledgement(s) of the data in this TB. 1> else if the HARQ process is configured with disabled HARQ feedback: 2> if harq-FeedbackEnablingforSPSactive is configured with value true and the transmission is the first transmission on the configured downlink assignment after activation of the configured downlink assignment: 3> instruct the physical layer to generate acknowledgement(s) of the data in this TB. 2> else: 3> not instruct the physical layer to generate acknowledgement(s) of the data in this TB.. This happened as, CR 1560 to v17.3.0 on NR-NTN (R2-2301983) added an additional condition related to ‘harq-FeedbackEnablingforSPSactive’ for NTN, but before that, the UE behavior for all the ‘or’ conditions was simply to ‘not instruct the physical layer to generate acknowledgement(s) of the data in this TB’ only. Then, Rel-18 CR (R2-2313849) made changes that the changes in the CR 1560 apply only to the NTN feature (i.e. condition of ‘1> if the HARQ process is configured with disabled HARQ feedback’ for NTN). However, same is not reflected to Rel-17 spec. As a consequence, there is disparity across Rel-17 and Rel-18 specifications, which is highly undesired from spec consistency and maintainence perspectives. Also, it is not accurate in Rel-17 spec as the changes in the CR 1560 should apply only to the NTN feature. It is therefore required to align the Rel-17 spec to that of Rel-18. |
| Summary of change: |  | - In clause 5.3.2.2, the condition is further clarified as - if the HARQ process is configured with disabled HARQ feedback and is not associated with a transmission indicated with a G-RNTI or a G-CS-RNTI or a configured downlink assignment for MBS multicast” - In clause 5.3.2.2, the conditions in Rel-17 spec are made aligned with Rel-18 spec. 1> if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted, is stopped or expired and if the cg-SDT-TimeAlignmentTimer, if configured, is not running; or: 2> not instruct the physical layer to generate acknowledgement(s) of the data in this TB. - else if the HARQ process is configured with disabled HARQ feedback: Impact analysis Impacted functionality: HARQ for MBS in NTNs Impacted 5G architecture options: NR SA Inter-operability: - If the network is implemented according to the CR and the UE is not, the HARQ feedback may not be provided by UE. - If the UE is implemented according to the CR and the network is not, the UE may provide the HARQ feedback but network may not expect the feedback. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There may be inaccurate operation for HARQ feedback. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### <Start of Change>

#### 5.3.2.2	HARQ process

When a transmission takes place for the HARQ process, one or two (in case of downlink spatial multiplexing) TBs and the associated HARQ information are received from the HARQ entity.

For each received TB and associated HARQ information, the HARQ process shall:

1>	if the NDI, when provided, has been toggled compared to the value of the previous received transmission corresponding to this TB; or

1>	if the HARQ process is equal to the broadcast process, and this is the first received transmission for the TB according to the system information schedule indicated by RRC; or

1>	if the HARQ process is associated with a transmission indicated with a MCCH-RNTI for MBS broadcast, and this is the first received transmission for the TB according to the MCCH schedule indicated by RRC; or

1>	if the HARQ process is associated with a transmission indicated with a G-RNTI for MBS broadcast, and this is the first received transmission for the TB according to the MTCH schedule indicated by RRC or according to the scheduling indicated by DCI as specified in TS 38.214 [7]; or

1>	if this is the very first received transmission for this TB (i.e. there is no previous NDI for this TB):

2>	consider this transmission to be a new transmission.

1>	else:

2>	consider this transmission to be a retransmission.

The MAC entity then shall:

1>	if this is a new transmission:

2>	attempt to decode the received data.

1>	else if this is a retransmission:

2>	if the data for this TB has not yet been successfully decoded:

3>	instruct the physical layer to combine the received data with the data currently in the soft buffer for this TB and attempt to decode the combined data.

1>	if the data which the MAC entity attempted to decode was successfully decoded for this TB; or

1>	if the data for this TB was successfully decoded before:

2>	if the HARQ process is equal to the broadcast process:

3>	deliver the decoded MAC PDU to upper layers.

2>	else if this is the first successful decoding of the data for this TB:

3>	deliver the decoded MAC PDU to the disassembly and demultiplexing entity.

1>	else:

2>	instruct the physical layer to replace the data in the soft buffer for this TB with the data which the MAC entity attempted to decode.

1>	if the HARQ process is associated with a transmission indicated with a Temporary C-RNTI and the Contention Resolution is not yet successful (see clause 5.1.5); or

1>	if the HARQ process is associated with a transmission indicated with a MSGB-RNTI and the Random Access procedure is not yet successfully completed (see clause 5.1.4a); or

1>	if the HARQ process is equal to the broadcast process; or

1>	if the HARQ process is associated with a transmission indicated with a MCCH-RNTI or a G-RNTI for MBS broadcast; or

1>	if the HARQ process is associated with a transmission indicated with a G-RNTI or a G-CS-RNTI or a configured downlink assignment for MBS multicast and HARQ feedback is disabled for this G-RNTI or G-CS-RNTI or the corresponding G-CS-RNTI, as specified in clause 18 of TS 38.213 [6]; or

1>	if the HARQ process is associated with a transmission indicated with a G-RNTI or a G-CS-RNTI or a configured downlink assignment for MBS multicast and NACK only HARQ feedback is used for this G-RNTI or G-CS-RNTI or the corresponding G-CS-RNTI and the data for this TB is successfully decoded and the transmission is not the first transmission of PDSCH where the configured downlink assignment was (re-)initialised; or

1>	if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted, is stopped or expired and if the cg-SDT-TimeAlignmentTimer, if configured, is not running:

2>	not instruct the physical layer to generate acknowledgement(s) of the data in this TB.

1>	else if the HARQ process is configured with disabled HARQ feedback and is not associated with a transmission indicated with a G-RNTI or a G-CS-RNTI or a configured downlink assignment for MBS multicast:

2>	if harq-FeedbackEnablingforSPSactive is configured with value true and the transmission is the first transmission on the configured downlink assignment after activation of the configured downlink assignment:

3>	instruct the physical layer to generate acknowledgement(s) of the data in this TB.

2>	else:

3>	not instruct the physical layer to generate acknowledgement(s) of the data in this TB.

1>	else:

2>	instruct the physical layer to generate acknowledgement(s) of the data in this TB.

The MAC entity shall ignore NDI received in all downlink assignments on PDCCH for its Temporary C-RNTI when determining if NDI on PDCCH for its C-RNTI has been toggled compared to the value in the previous transmission.

NOTE:	If the MAC entity receives a retransmission with a TB size different from the last TB size signalled for this TB, the UE behavior is left up to UE implementation.

#### <End of Change>