3GPP TSG-RAN WG2 Meeting #127bis	R2-2408484

Hefei, China, Oct 14th~ Oct 18th, 2024

Agenda Item:	8.1.2.2

Source:	Mediatek Inc.

Title:	Further Discussion on Functionality-based LCM for UE-side Model

Document for:	Discussion, Decision

# Introduction

During the RAN2#127 meeting, significant progress was made concerning the functionality-based LCM for the UE-side model, specifically for the use case of beam management. Additionally, one LS[1] was sent to RAN1, containing questions whose answers are essential for facilitating further progress in RAN2.

In this contribution, we discuss the issue of ensuring consistency between training and inference and required signaling/procedure for UE-side model training based on the progress having already made in RAN1.

# Discussion

## Review of RAN1’s Discussion and Conclusion

The introduction of the associated ID in RAN1#116bis and subsequent meetings serves to ensure consistency in the network-side additional conditions during both the training and inference phases for UE-side models in beam management cases (BM-Case 1 and BM-Case 2).

Agreement From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) - A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation with NW-sided additional conditions - B: UE(s) collects the data corresponding to the associated ID(s) - C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). - D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., - Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed - Alt.4: Model ID is determined by pre-defined rule(s) in the specification - FFS: how to report - Note: D is to facilitate AI/ML model inference - Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification. Agreement Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B: - Opt1: Based on associated ID (Referring to AI 9.1.3.3) - FFS on what can be assumed by UE with the same associated ID across training and inference - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework - Opt 2: Performance monitoring based - FFS details - Other options are not precluded. RAN1#117 Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell. - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study). RAN1#118 Confirm the following Working assumption. Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study) Agreement For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID

The introduction of the associated ID serves the following key purposes:

- Ensures Consistency between model training and inference: It guarantees uniformity in network conditions across both training and inference stages for UE-side models, facilitating accurate and reliable model performance.

- Protects Proprietary network implementation Information: The associated ID shields sensitive and proprietary network implementation details by encapsulating them within an identifier, safeguarding critical information from exposure.

- Establishes Direct Linkage between UE-side models and the network condition: The associated ID directly connects UE-side models to specific network conditions, enabling models to adapt to and reflect real-time network environments accurately.

- Replacement for Model Identification Procedure:  Instead of relying on a potentially cumbersome and inefficient model identification procedure, the associated ID streamlines the process of model selection and deployment, enhancing operational efficiency.

Observation 1: The introduction of the associated ID aims to ensure the consistency between the model training and inference phases providing a direct linkage to relevant network conditions, shielding the proprietary network implementation information and streamlining the model identification process.

The use of associated IDs, as discussed from the RAN1 perspective, facilitates an approach for developing UE-side AI/ML models.

## LCM for UE-Side Model

The example procedure (noted as AI-Example1) of MI-Option1 according to RAN1#116bis meeting is illustarated in Figure 1 with steps of A, B, C and D.

Figure 1 Example Procedure of MI-Option1 according to RAN1 agreements

Initially, the network signals data collection configurations along with their associated IDs in step A, tailored for specific sub-use cases and additional network-side conditions. Subsequently, the UE collects data corresponding to these associated IDs in step B, which then serves as the foundation for the development (training and updating) of AI/ML models at the UE side in step C. Finally, the UE reports back to the network about its AI/ML models in step D, linking them to the original associated IDs. This process ensures the consistency in model training and inference processes for UE-side model while safeguarding the network side's specific implementation and configuration details from disclosure. By associating network-side conditions and specific configurations with an ID, signaling overhead can be minimized.

Data Collection

From the RAN2 perspective, the associated ID is considered a type of network-side condition. In step A, it is signaled from the network to the UE along with the data collection configuration. The data collection configuration includes detailed parameters specifying the RAN configuration (e.g., configuration of set A/B) and how the data collection should be performed, serving a different role from the associated ID. The associated ID and the data collection configuration are complementary pieces of information that together enable the UE to uniquely identify the network conditions and RAN configurations under which data is collected and to perform data collection accurately. In step B, the UE performs data collection according to the information received in step A.

Proposal 1: RAN2 assumes that the associated ID is signaled along with the data collection related configurations, which provides configuration for data collection for UE-side model training.

Proposal 2: RAN2 assumes that the associated ID represents the network additional condition and the data collection related configuration is not part of it.

Model Training

In step C, the AI/ML models are developed. Under the assumption that the UE-side model is trained by a server, such as an OTT server, datasets containing data collected from a large number of UEs are accessed and utilized for model training. These datasets are categorized into different groups according to the RAN configuration and the associated ID.

When developing AI/ML models, various approaches can be employed to balance generalization performance and model accuracy:

- Single Associated ID and Single Configuration: The AI/ML model is trained with a dataset characterized by one associated ID and a single RAN configuration. This approach focuses on a very specific scenario, ensuring high accuracy for that particular setup.

- Single Associated ID and Multiple Configurations: The AI/ML model is trained with a dataset characterized by one associated ID but multiple RAN configurations. This approach allows the model to generalize better across different configurations while maintaining the same network condition.

- Multiple Associated IDs and Single Configuration: The AI/ML model is trained with a dataset characterized by multiple associated IDs but a single RAN configuration. This approach helps the model to adapt to various network conditions within a specific RAN setup.

- Multiple Associated IDs and Multiple Configurations: The AI/ML model is trained with a dataset characterized by multiple associated IDs and multiple RAN configurations. This approach aims to create a highly generalized model that can perform well across a wide range of scenarios and conditions.

The AI/ML model should be applicable to specific scenarios/configurations as defined by the RAN configurations and network conditions identified by the associated IDs. The relationship between the AI/ML models and RAN configuration as well as the network additional condition identified by the associated ID is illustrated in Figure 2.

In functionality-based LCM, the models are not visible to the network, although each physical model may be identified by a model ID. The developed models with different model IDs are managed by the UE side. The functionality can be defined as a sub-use case or a sub-use case combined with the RAN configuration. This depends on the content of the RAN configuration, how dynamically it changes, and the number of different RAN configurations that need to be considered, which should be decided by RAN1.

The processing of various datasets collected under different RAN configurations and network conditions, as well as the development of the UE-side models, are implementation issues and do not need to be standardized. Although the linkage between the the funcitonalities with AI/ML models, RAN configuration and the associated ID is implementation-specific and maintained by the server responsible for UE-side model training, such linkage should be known by the UE to determine the applicability of the functionality.

Observation 2: The UE-side models are developed using datasets collected from various UEs. These datasets are typically categorized based on RAN configurations and associated IDs. The AI/ML models are then trained and developed according to these categorized datasets, which ultimately determines the applicability of the functionalities that incorporate the resulting AI/ML models.

Proposal 3: RAN2 assumes that for each functionality, the AI/ML models are trained and developed using datasets characterized by either a single associated ID and single configuration, a single associated ID and multiple configurations, multiple associated IDs and a single configuration, or multiple associated IDs and multiple configurations.

Proposal 4: The linkage between the funcitonalities with AI/ML models, RAN configuration and the associated ID is known by the UE to determine the applicability of the functionality, which is maintained by the server for UE-side model training.

## Availability Reporting

During the RAN1#116 and RAN1#116bis meetings, RAN1 concluded that while model transfer/delivery cases z5, z2, and z3 are deprioritized, cases z1 and z4 remain under consideration. According to proposal 4, indicating the availability of AI/ML models for UE-side functionality is not necessary for applicability reporting. However, in the context of model transfer discussions, supporting cases z1 and z4—which involve transferring models from the network to the UE—may necessitate AI/ML model availability reporting for UE-side functionality.

As depicted in Figure 2, the network-controlled model transfer procedure is the baseline approach. Here, the UE must signal its AI/ML model availability for UE-side functionality. The network then uses this information, along with the UE capability report, to decide whether to transfer the AI/ML model to the UE. If the network considers the applicability of the AI/ML model during the transfer, such as in case z4 where only the model's parameters are updated, it may not be necessary for the UE to send a separate applicability report to the network. Essentially, the network could transfer only those AI/ML models that are applicable for inference to the UE.

Figure 3 Exampel of Model Transfer Z1

While AI/ML functionality availability reporting and applicability reporting serve different LCM purposes—model inference and model transfer, respectively—the baseline method, UAI, can be utilized for AI/ML functionality availability reporting.

Proposal 5: AI/ML functionality availability reporting and applicability reporting serve distinct LCM purposes, with the former facilitating model transfer and the latter supporting model inference processes.

Proposal 6: The UAI framework can be used for AI/ML functionality availability reporting.

# Conclusion

Review of RAN1 discussion and conclusion:

Observation 1: The introduction of the associated ID aims to ensure the consistency between the model training and inference phases providing a direct linkage to relevant network conditions, shielding the proprietary network implementation information and streamlining the model identification process.

Data Collection

Proposal 1: RAN2 assumes that the associated ID is signaled along with the data collection related configurations, which provides configuration for data collection for UE-side model training.

Proposal 2: RAN2 assumes that the associated ID represents the network additional condition and the data collection related configuration is not part of it.

Model Training

Observation 2: The UE-side models are developed using datasets collected from various UEs. These datasets are typically categorized based on RAN configurations and associated IDs. The AI/ML models are then trained and developed according to these categorized datasets, which ultimately determines the applicability of the functionalities that incorporate the resulting AI/ML models.

Proposal 3: RAN2 assumes that for each functionality, the AI/ML models are trained and developed using datasets characterized by either a single associated ID and single configuration, a single associated ID and multiple configurations, multiple associated IDs and a single configuration, or multiple associated IDs and multiple configurations.

Proposal 4: The linkage between the funcitonalities with AI/ML models, RAN configuration and the associated ID is known by the UE to determine the applicability of the functionality, which is maintained by the server for UE-side model training.

Availability Reporting

Proposal 5: AI/ML functionality availability reporting and applicability reporting serve distinct LCM purposes, with the former facilitating model transfer and the latter supporting model inference processes.

Proposal 6: The UAI framework can be used for AI/ML functionality availability reporting.

# Reference

[1] R2-2407848, LS on applicable functionality reporting for beam management UE-sided model