3GPP TSG-RAN WG2 Meeting #127bis                                                             R2-2408485

Hefei, China, Oct 14th~ Oct 18th, 2024

Agenda Item:	8.1.3

Source:	Mediatek Inc.

Title:	Enhanced MDT for Data collection for Network Side Model Training

Document for:	Discussion, Decision

# Introduction

In the RAN2#126 meeting, following agreements were reached specifically for network-side training data collection pertaining to the use case of AI BM.

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature. FFS how to handle case when single RRC message is not sufficient. FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting. FFS whether and what event-based reporting is supported and FFS on network request reporting

Subsequently, in the RAN2#127 meeting, further agreements were made regarding the data collection of network-side model training.

Agreements - As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support. - UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. This is across all use cases - When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging. - Measurements for data collection purposes and logging based can be controlled based on power state of the UE. It is up to UE implementation how the UE determines power state. FFS whether the UE stops autonomously or if it reports to the network . - FFS whether AS buffer event based reporting is supported. FFS if we send availability indication or full report if it is supported - FFS on event based data collection/logging - On-demand request from the network is supported. FFS details on signalling Agreements - The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements) - Data collection report will not be transmitted over SRB1. FFS which SRB is used.

This paper delves into the specifics of data logging and reporting in the RRC_CONNECTED state, aiming to address several FFS issues identified in previous RAN2 meetings.

# Discussion

## Data Logging/Reporting in the same RRC_CONNECTED State

During the RAN2#126 meeting, it was agreed that Immediate MDT would serve as the baseline framework for OAM-centric data collection for training a network-sided model. In the subsequent RAN2#127 meeting, a series of agreements were made to support logging on top of the Immediate MDT mechanism.  The following question is whether to restrict data logging/reporting to the same RRC_CONNECTED state, i.e., without RRC state transition, or to allow data logging/reporting to be performed in different RRC_CONNECTED states, i.e., with RRC state transitions.

- Option 1: Data logging and reporting within the same RRC_CONNECTED state without RRC state transition.

- Option 2: Data logging and reporting cross different RRC_CONNECTED states with RRC state transition.

Option 1

Under Option 1, the UE collects and logs data, storing it for future reporting in the same RRC_CONNECTED state. When the UE receives the RRCRelease message, which moves the UE to the RRC_INACTIVE or RRC_IDLE model,  the measurement configuration for data logging and reporting is released and any stored data/logs are subsequently discarded.

Figure 1. Option 1- Data logging and reporting within the same RRC_CONNECTED state

Option 2

Under Option 2, the UE is capable of performing data collection/logging across different RRC_CONNECTED states with RRC state transition. The UE start/resumes data collecting/logging when in RRC_CONNECTE state and stops/suspends data collecting/logging when in RRC_INACTIVE/IDLE state. The UE continuously stores the collected data/logs, irrespective of RRC state transitions for future reporting. Similar to logged MDT, the collected data/logs as well as the mesurement configuration for data logging/reporting are preserved after RRC connection release.

Figure 2. Option 2-Data logging/reporting across different RRC_CONNECTED states

Compared with Option 1, Option 2 offers greater flexibility for both the UE and the network in terms of data reporting. Specifically, the UE is not required to report data within the same RRC_CONNECTED state. This flexibility provides several benefits:

- Resource efficiency and reliable report: Option 2 enables the UE to report data when it experiences favorable channel conditions. This is especially beneficial in situations where network coverage is poor or the UE is out of coverage. The UE can report the data once it returns to normal coverage.

- Network load management: Option 2 gives network more flexibility to determine when to retrieve data stored on the UE, such as during off-peak hours, thereby managing network load more effectively.

- Power consumption and battery saving: Continuous reporting can drain the UE's battery faster. By logging data and reporting it later, the UE can conserve battery life.

However, in order to get those benefits, option 2 is more complex than option1. Considering the time limitation of Rel-19, option 1 is considered as the baseline solution.

Proposal 1: In Rel-19, data logging and reporting for data collection for model training is performed within the same RRC_CONNECTED state.

Proposal 2: The UE releases the measurement configuration for data logging and reporting upon receiving the RRCRelease message.

In logged MDT, under normal operation, the network should retrieve all stored data and logs before transmitting an RRCRelease message to the UE. Given that on-demand requests from the network are supported for data collection for model training, the network should adhere to the same requirement, ensuring no residual data or logs are left behind when transitioning the UE to the RRC_INACTIVE or RRC_IDLE state. It is the network's responsibility to collect any stored data/logs before transmitting the RRCRelease message. In abnormal case that there is residual data or logs left when receiving the RRCRelease message, the UE discards the stored data or logs.

Proposal 3: In normal operation, the network should retrieve all stored data and logs before transmitting an RRCRelease message to the UE.

Propossal 4: The UE discards the stored logs/data upon receiving the RRCRelease message.

## Data Logging/Reporting Mechanism

In logged MDT, geographical scope of measurement logging may be configured to restrict where the defined set of measurements needs to be collected. A UE will log measurements as long as it is within the configured logging area. For data collection for model training, area-specific data collection is also required. This is because different scenarios, configurations, and sites often have unique characteristics that a single model cannot generalize well to. This specificity allows for the creation of models tailored to particular scenairos/conditions/sites, thereby providing significant performance benefits.

Additionally, UEs may have limitations in storing all related models, making it impractical to maintain a comprehensive set of models on the device. Area-specific data logging supports the development of models that can be delivered or transferred to the UE as needed at the cost of the associated overhead and latency. This approach ensures that models are optimized for the specific conditions/scenarios/sites they will encounter, leading to more accurate and efficient performance.

Proposal 5: Area-specific data logging for data collection for model training is supported and configured by the network. The UE performs data logging as long as it is within the configured logging area.

In logged MDT, the UE includes a relative timestamp for each logged measurement. For data logging intended for model training, timestamps are also essential. They provide the temporal context necessary to understand the sequence and timing of events, which is crucial for analyzing patterns and trends over time, especially for temporal domain prediction. Timestamps facilitate event correlation, helping to link logged data with specific network events. Additionally, they help validate the integrity and consistency of the collected data, allowing for the identification and correction of anomalies during the data collection process. The need for timestamps and their format for different use cases should be confirmed by RAN1.

Proposal 6: RAN2 assumes that the UE includes a timestamp for each logged measurement. The necessity of the timestamp and its format for each use case should be confirmed by RAN1.

In RAN2#127 meeting, it was agreed that as the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. Therefore, when to start data logging is controlled by the network.

Measurements for data collection and logging can be controlled based on the power state of the UE. It is up to the UE implementation to determine the power state, which implies that the UE can stop data logging when it is in a low power state. According to Proposal 1, there is no need for the UE to report the stop of data logging to the network, as the network knows when the UE starts data logging within the same RRC_CONNECTED state and will retrieve all stored data and logs before transmitting an RRCRelease message to the UE. Furthermore, based on the timestamps in the report, the network can infer that the UE has autonomously stopped data logging.

The same principle applies when the UE reaches its buffer limitation. In this case, the UE stops measurement for data collection and logging autonomously without triggering a report. The network already has comprehensive control and awareness over the data logging process. It knows when the UE starts data logging and is aware of the AS buffer size, allowing it to proactively retrieve stored data before the buffer reaches its maximum capacity. This preemptive data retrieval ensures that the buffer does not overflow, eliminating the need for an AS buffer event-based reporting. Even if the data is not retrieved by the network when releasing RRC connection, the UE can discard the stored data/logs.  Additionally, the network can rely on data collected from multiple UEs to compensate for any potential data loss from a single UE, ensuring robust and comprehensive data collection. This redundancy, combined with the network's proactive management, makes AS buffer-based reporting redundant and unnecessary.

Proposal 7: The UE stops data logging autonomously when it reaches its buffer limitation or is in a low power state. The UE does not report the stop of data logging to the network.

Proposal 8: AS buffer event-based reporting is not supported, i.e., a measurement report is not triggered when the UE reaches its buffer limitation.

A data availability report is not necessarily required to trigger the network to retrieve the data/logs stored on the UE side. However, similar to logged MDT, with each request, the network may only receive a portion of the total UE log. To indicate that the reported data is just a segment, the UE should include a data availability indicator to convey that further measurement information is available.

Proposal 9: In response to the network's request to retrieve data, the UE sends a data availability indication along with the data if there is any additional residual data remaining.

# Conclusion

Proposal 1: In Rel-19, data logging and reporting for data collection for model training is performed within the same RRC_CONNECTED state.

Proposal 2: The UE releases the measurement configuration for data logging and reporting upon receiving the RRCRelease message.

Proposal 3: In normal operation, the network should retrieve all stored data and logs before transmitting an RRCRelease message to the UE.

Propossal 4: The UE discards the stored logs/data upon receiving the RRCRelease message.

Proposal 5: Area-specific data logging for data collection for model training is supported and configured by the network. The UE performs data logging as long as it is within the configured logging area.

Proposal 6: RAN2 assumes that the UE includes a timestamp for each logged measurement. The necessity of the timestamp and its format for each use case should be confirmed by RAN1.

Proposal 7: The UE stops data logging autonomously when it reaches its buffer limitation or is in a low power state. The UE does not report the stop of data logging to the network.

Proposal 8: AS buffer event-based reporting is not supported, i.e., a measurement report is not triggered when the UE reaches its buffer limitation.

Proposal 9: In response to the network's request to retrieve data, the UE sends a data availability indication along with the data if there is any additional residual data remaining.

# Reference

[1] RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm

[2] 3GPP TR 38.843 V18.0.0