3GPP TSG-RAN WG2 Meeting #127bis	R2-2408486

Hefei, China, Oct 14th~ Oct 18th, 2024

Agenda Item:	8.1.5

Source:	Mediatek Inc.

Title:	Further Analysis on Model Transfer/Delivery Solutions

Document for:	Discussion, Decision

# Introduction

In the RAN#105 meeting, the study objectives for model transfer/delivery were revised as follows in the updated WID[1]:

- Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

During the Release 18 study, RAN2 identified the following solutions to facilitate AI/ML model transfer/delivery and captured in the TP[2]:

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling.

- Solution 2a: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling.

- Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling.

- Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data.

- Solution 2b: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via User Plane (UP) data.

- Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data.

- Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP).

- Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE.

Regarding Solution 4b, RAN2 discussed two methods: CP signaling and an UP tunnel (e.g., IP tunnel). However, the feasibility of these methods was not studied or analyzed.

In this contribution, we analyse the impact on different model transfer/delivery solutions, taking into account the conclusion reached by RAN1 on specific model transfer/delivery cases. In light of the SA's decision to exclude model transfer/delivery from the scope of the study on Core Network Enhanced Support for AI/ML, we suggest prioritizing solutions that require minimal engagement from SA WGs. This approach will streamline RAN2's efforts towards developing more self-contained solutions within its purview for Rel-19.

# Discussion

## Review of RAN1’s Discussion and Conclusion

During Rel-18 study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN1 introduced different options for model delivery/transfer to UE. These options consider different combinations of model storage location, training locations and model formats for both UE-side models and the UE part of two-sided models [2]:

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top. | Outside 3GPP Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format. | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format. | 3GPP Network | NW-side |
| z3 | model transfer in open format. | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support. | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE, i.e., any other model structure not covered in z4, including any model structure that is only partially known. | 3GPP Network | NW-side |
| Note: The definition of various Cases is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization. |  |  |  |

In the study phase of Rel-19 WI, RAN1 had already made the prioritization for different model transfer/delivery cases:

Conclusion in RAN1#116 From RAN1 perspective, the model transfer/delivery Case z5 is deprioritized for Rel-19. Conclusion in RAN1#116bis From RAN1 perspective, the model transfer/delivery Case z2 is deprioritized at least for UE-sided model in Rel-19 due to the following reasons: - Risk of proprietary design disclosure - Burden of offline cross-vendor collaboration Conclusion in RAN1#116bis From RAN1 perspective, the model transfer/delivery Case z3 is deprioritized for Rel-19 due to the following reasons (compared to Case y): - No much benefit compared to Case y - Risk of proprietary design disclosure - Large burden of offline cross-vendor collaboration - Additional burden on model storage within in 3GPP network Agreement in RAN1#117 From RAN1 perspective, for model delivery/transfer Case z4, further study the following alternatives (including the necessity/feasibility/benefits): - Alt. A - Step A-1: UE reports the supported known model structure(s) to network - Step A-2: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step A-1 - FFS: whether some additional step(s), and/or whether other information is needed - Alt. B - Step B-0: UE reports to NW its support of model transfer/delivery case z4 - Note: Step B-0 may be before or after Step B-1, or not necessary - Step B-1: NW indicates to UE the candidate known model structure(s) - Step B-2: UE reports to NW which model structure(s) out of the candidate known model structure(s) indicated in Step B-1 is supported - Step B-3: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step B-2 - FFS: whether some additional step(s), and/or whether other information is needed - Note: Other alternative(s) is not precluded - Note: Other method(s) of parameter exchange from NW to UE side is a separate discussion. Agreement in RAN1#118 From RAN1 perspective, the “known model structure(s)” of the model transfer/delivery Case z4 at least include known information on the following aspects - Model type/backbone (e.g., Transformer, CNN and so on) - In case model type is a neural network - Number of layers - Layer types/structure (e.g., full connected, activation layer and so on) - Layer size (e.g., the number of parameters of a layer) - Connection between different layers - model input/output related information Conclusion in RAN1#118 From RAN1 perspective, model transfer is needed at least for some (e.g., Option 3b) of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported).

Observation 1: Model transfer/delivery case z2, z3 and z4 are deprioritized by RAN1 in Rel-19 and model transfer/delivery case y, z1 and z4 are being studied.

Observation 2: The RAN1 study on case z4, including its necessity, feasibility, and benefits, has not yet been concluded.

Proposal 1: RAN2 prioritizes cases y and z1 as the primary scenarios for analyzing various model transfer/delivery solutions.

## Model Transfer/Delivery Case y

According to the TR and RAN1 discussion, case y is characterized by the need for a model delivery mechanism that operates transparently to 3GPP signalling. This implies that the delivery of AI/ML models should not interfere with, nor rely on, the existing 3GPP network signalling infrastructure. The solution intends to control and manage the model transfer/delivery process without direct involvement from the RAN.

Among the various solutions explored by RAN2, Solution 4a is identified as the one that meets the requirements of case y. Solution 4a utilizes OTT mechanisms that function externally to the 3GPP network frameworks. This approach ensures that the model delivery process is independent of 3GPP signalling, thereby fulfilling the primary requirement of case y.

The management and control of the AI/ML model transfer/delivery in Solution 4a are handled by an external OTT server. This is in line with the characteristic of case y, which stipulates that the RAN should not be directly involved in the control of the model transfer/delivery process.

A significant benefit of Solution 4a lies in its unrestricted capacity for the size and parameters of AI/ML models. This characteristic harmonizes with the essential advantage of case y, offering the flexibility to accommodate models of varying sizes without affecting RAN specifications.

Observation 3: Solution 4a fulfills the requirements of case y, through its use of an OTT mechanism for model transfer/delivery. This approach is transparent to 3GPP signaling and eliminates the need for RAN direct involvement in controlling the model transfer/delivery process.

Based on the analysis presented, it is proposed that RAN2 should conclude that Solution 4a is the only suitable option for case y.

Proposal 2: RAN2 conclude that Solution 4a is the only suitable option for model transfer/delivery case y.

## Model Transfer/Delivery Case z1

Case z1 delineates a model transfer scenario where the AI/ML model is developed on the UE-side server, subsequently stored within the 3GPP network infrastructure, and transferred to the UE using a proprietary format over the air interface.

During RAN2 discussions, the focus regarding model transfer/delivery has been on the implications for the air interface, specifically the choice of protocol layer and whether the CP/UP tunnel is utilized for transferring the AI/ML model from the 3GPP network to the UE. However, RAN2 has not addressed the process by which the model is initially transferred from the UE-side server to the designated storage entity within the 3GPP network.

Therefore, the method of delivering the AI/ML model from the UE-side server to the network's model storage entity falls outside the purview of 3GPP standardization effort and does not need to be standardized.

Observation 4: RAN2 has not addressed the process by which the model is initially transferred from the UE-side server to the designated storage entity within the 3GPP network, which is out of 3GPP scope.

Proposal 3: The delivery mechanism of the AI/ML model from the UE-side server to the model storage entity within the 3GPP network should be regarded as outside the scope of 3GPP and does not require standardization.

During the RAN2 discussions, it was observed that Solution 2 and Solution 3 share several similarities, notably that both solutions involve storing the AI/ML models within the CN domain and executing model transfer between the UE and a specific entity within the CN domain. The primary distinction between these two solutions lies in the specific CN domain entity they involve: Solution 2 engages the LMF for Positioning accuracy enhancements use case, whereas Solution 3 involves a different CN entity.

Given the specific requirements of use cases such as beam management and CSI feedback enhancement, there is a clear preference for enabling network control and management over model transfer/delivery at the gNB. In light of these considerations, the advantages and necessity of supporting Solution 2 have not been convincingly justified.

Observation 5: Solution 2 and Solution 3 share several similarities. The advantages and necessity of supporting Solution 2 for the use case of beam management and CSI feedback enhancement have not been justified.

During the SA#105 meeting, updates were made to the SID on Core Network Enhanced Support for AI/ML, and WT1.2 (Model Transfer / Delivery) was removed from the scope of the Rel-19 study in SA2. This decision implies that any model transfer/delivery solutions falling under the purview of SA2 and requiring its evaluation should likewise be considered a lower priority in RAN2 discussions. Solution 2a and 2b require SA2’s involvement. Specifically, for the use case of enhancing positioning accuracy, Solution 3b is being deprioritized because it relies on a UP tunnel for transferring models from the LMF to the UE, a process that necessitates further study and assessment by SA2.

Observation 6: WT1.2 (Model Transfer / Delivery) was removed from the scope of the Rel-19 study in SA2.

Proposal 4: From RAN2 perspective, Solution 2a and 2b for model transfer are deprioritized in Rel-19.

Proposal 5: From RAN2 perspective, Solution 3b for model transfer are deprioritized in Rel-19.

For solution 4b, one possible option is that OAM can transfer/deliver AI/ML models to the UE, e.g., via IP tunnel. Since there is no IP tunnel between OAM and the UE in current system architecture, this solution is out of RAN2 scope and requires SA5’s evaluation in the feasibility, this option can be deprioritized in Rel-19.

Observation 7: There is no IP tunnel between OAM and the UE in current system architecture. Introduction of IP tunnel between the UE and OAM requires huge standardization and implementation effort and requires SA5’s evalution.

Proposal 6: From RAN2 perspective, solution 4b with UP tunnel between OAM and the UE is deprioritized in Rel-19.

For model transfer/delivery case z1, following model transfer/delivery solutions can be futher studied in RAN2:

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling.

- Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling.

- Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data.

- Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE (OAM can transfer/deliver AI/ML models to UE via OAM→RAN→UE, where Control Plane (CP) signalling is used for RAN→UE.)

From the UE's standpoint, Solutions 1a and 4b are functionally equivalent with respect to their mechanisms over the air interface, as both execute model transfers from the gNB to the UE using RRC signaling. Therefore, for the purposes of evaluation, Solutions 1a and 4b, which both employ RRC signaling for transferring models AI/ML from the gNB to the UE, can be merged into a single analysis. In this approach, Solution 1a can serve as the baseline for assessment, with the understanding that conclusions drawn will also be applicable to Solution 4b unless otherwise specified.

Proposal 7: RAN2 focus on the analysis on model transfer/delivery Solution 1a, 1b, Solution 3a and solution 4b (OAM transfers/delivers AI/ML models to UE via gNB where RRC signaling is used between gNB and UE).

## Evaluation on solution 1a, 1b, 3a and 4b for case z1

The following areas are considered to evaluate the different model transfer/delivery solutions:

-	A1: Large, no upper limit model/model parameter size,

-	A2: Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs),

-	A3: Network controllability on model transfer/delivery (e.g., management decision at gNB),

-	A4: Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB).

In our understanding, A1 and A3 are fundamental aspects to consider initially. A2 and A4 can be addressed later. A2 primarily depends on the generalization performance of the AI models and the frequency of model transfer/delivery required. Meanwhile, the analysis of A4 is contingent upon A1, particularly in terms of the required support for model size.

A3: Network controllability on model transfer/delivery (e.g., management decision at gNB)

For case z1, the AI/ML models are stored in the network and network controllability on model transfer is beneficial in case z1 for the following reasons:

- Optimized AI Performance with Scenario/configuration specific models: Network controllability allows for the transfer of AI/ML models that are specifically optimized for different scenarios, configurations, or network conditions. This approach is crucial for achieving optimal performance, especially in cases where a single, generalized model may not adequately address the diverse configurations and conditions of the network environments.

- Resource and Operational Efficiency: Network control over model transfers ensures the judicious use of radio resources and streamlines network operations. This includes intelligent scheduling of model transfer to minimize impact on network traffic based on current network load and user demand, thereby optimizing both resource usage and operational processes.

- Security and Integrity: By maintaining network control, the network can better safeguard the transfer of AI/ML models, preventing unauthorized access and ensuring the integrity of the models during transit.

- Effective Network Storage Management: With AI/ML models stored within the network, controllability over model transfers is important for managing storage constraints at the network side. The network takes the responsibility to decide when to transfer the model, taking into account the available storage and the necessity for model updates, thereby ensuring that the network always stores with the most effective and up-to-date models.

Observation 8: In case z1, network controllability of AI/ML model transfer is key to delivering optimized performance for for the scenario/configuration-specific models, ensuring efficient use of resources and streamlined operations, maintaining the security and integrity of AI/ML models and facilitating effective network storage management.

Proposal 8: For case z1, network should have full control on model transfer from network to the UE over the air interface including the initiation, termination and management of model transfer.

A1: Large, no upper limit model/model parameter size

The evaluation of model size for the beam management use case is documented in the TR[2]. For other use cases, model complexity was evaluated based on the number of parameters rather than model size, despite the positive correlation between model size and the number of parameters.

|  | Model complexity in number of model parameters | Model complexity in number of model size | Computational complexity (FLOPs) |
| --- | --- | --- | --- |
| BM-Case 1 DL Tx beam | More than 1k to 4.9M majority reported less than 1M or about 1M | 50Kbytes to 20Mbytes majority reported less than 0.1Mbytes ~ 0.6Mbytes | ~2.7K to 222M majority reported less than 1M or 10s M |
| BM-Case 1 DL beam pair | 72k to 4.9M majority reported less than 0.1s M ~ 1M | 0.17Mbytes to 21Mbytes majority reported less than 1Mbytes ~ 4Mbytes | 15K to 224M majority reported less than 1M ~ 4 M |
| BM-Case 2 DL Tx beam | 35k to 11M majority reported less than 0.1s M ~ 1M | 0.5Mbytes to 15Mbytes majority reported about 1s Mbytes | ~90K to 54M majority reported less than 0.1s M or 1s M |
| BM-Case 2 DL beam pair | 20k to 13M majority reported about 0.1M~1M | 0.08M to 15M majority reported about 1Mbytes | ~90K to 443M majority reported less than 0.4 M or 1s M |

Table: AI/ML model complexity/computation complexity used in the evaluations for AI/ML in beam management [2, Table 6.3.2-1]

For different BM sub-use cases, the model size varies. The majority reported is about 1Mbytes. Considering the maximum capacity of RRC message size is 9Kbytpes, which is even not sufficient for the minimized reported model size 0.1Mbytes, which is nearly 12 times of a RRC message size. For the practicle implementation of the AI/ML model, the AI/ML model with flexible size should be considered, implying that the AI/ML models can be even larger.

Observation 9: The size of the AI model will be in the order of megabytes, and currently, the size of the RRC message is insufficient to carry the AI model.

If proposal 7 is agreeable, RAN2 need to consider how to carry the AI/ML model with ~Mbytes from the gNB to UE for the use case of BM and CSI feedback enhancement and from LMF to UE for the use case of positioning accuracy enhancement.

Taking the use case of BM for example, we can either increase the number of segments supported for RRC message for solution 1a or requires PDU session termination at gNB for solution 1b.

Increasing the number of segments supported for RRC message for solution 1a may have following drawbacks:

- Increased Complexity: Supporting more RRC message segments introduces additional complexity in the RRC protocol. This complexity arises from the need to manage and ensure the integrity of multiple segments, handle errors, and implement mechanisms for retransmission in case of lost or corrupted segments.

- Higher Latency: The transmission of larger models through increased RRC message segments can lead to higher latency. This is due to the time required to transmit multiple segments over the air interface, coupled with the processing time needed at the receiver to reassemble the segments into the original message.

- Compatibility Issues: Extending the number of segments may raise compatibility issues with existing hardware and software, requiring updates or modifications to ensure interoperability. This could lead to significant implementation challenges and costs.

- Obstruct other traffic and RRC procedures: using normal SRBs (SRB1, SRB2) for large model transfers could delay or interrupt the transmission of user data and slow down RRC procedures, potentially impacting the efficiency of network operations leading to increased latency and reduced network responsiveness.

Terminating a DRB at the gNB instead of the UPF in solution 1b represents a significant architectural shift from the current system where the PDU session is typically terminated at the UPF. Deviating from this design could lead to interoperability issues, challenges with standard compliance, and potential difficulties in multi-vendor environments.

Observation 10: Increasing the number of segments for RRC messages introduces challenges such as added complexity, higher latency, compatibility issues, and potential obstruction of other traffic and RRC procedures. Meanwhile, terminating a DRB at the gNB instead of the UPF marks a significant architectural shift, potentially resulting in interoperability issues, compliance challenges, and difficulties in environments with multiple vendors.

The requirements for solutions 1a/1b/4b and solution 3a are consistent, focusing on supporting the transfer of large-sized AI/ML models, regardless of whether it's via SRB or DRB. Specifically, these solutions involve terminating the designated radio bearer for model transfer at the gNB (solutions 1a/1b/4b) or at the LMF (solution 3a). RAN2 can discuss whether to introduce a specific radio bearer dedicated to model transfer, which would be capable of handling large AI/ML models and would terminate between the UE and either the gNB or LMF.

Following issues need to be addressed if the specific radio bearer is introduced.

- Scalability Concerns: gNBs are deployed in larger numbers and are more geographically distributed than UPFs. Handling the large amount of AI/ML models with large size at the gNB level could lead to scalability issues, as each gNB would need to be equipped to manage a significant amount of AI/ML models, which could be cost-prohibitive.

- Mobility Management: Mobility management could become more complex if the specific radio bearer are terminated at the gNB. The UPF plays a crucial role in maintaining session continuity during user mobility. Without UPF termination, additional mechanisms would be required at the gNB to ensure seamless handovers and session persistence.

Proposal 9: RAN2 discuss whether to introduce a specific radio bearer dedicated to model transfer, which would be capable of handling large AI/ML models and would terminate between the UE and either the gNB or LMF.

# Conclusion

Review of RAN1 discussion and conclusion:

Observation 1: Model transfer/delivery case z2, z3 and z4 are deprioritized by RAN1 in Rel-19 and model transfer/delivery case y, z1 and z4 are being studied.

Observation 2: The RAN1 study on case z4, including its necessity, feasibility, and benefits, has not yet been concluded.

Proposal 1: RAN2 prioritizes cases y and z1 as the primary scenarios for analyzing various model transfer/delivery solutions.

Model Transfer/Delivery Case y

Observation 3: Solution 4a fulfills the requirements of case y, through its use of an OTT mechanism for model transfer/delivery. This approach is transparent to 3GPP signaling and eliminates the need for RAN direct involvement in controlling the model transfer/delivery process.

Proposal 2: RAN2 conclude that Solution 4a is the only suitable option for model transfer/delivery case y.

Model Transfer/Delivery Case z1

Observation 4: RAN2 has not addressed the process by which the model is initially transferred from the UE-side server to the designated storage entity within the 3GPP network, which is out of 3GPP scope.

Observation 5: Solution 2 and Solution 3 share several similarities. The advantages and necessity of supporting Solution 2 for the use case of beam management and CSI feedback enhancement have not been justified.

Observation 6: WT1.2 (Model Transfer / Delivery) was removed from the scope of the Rel-19 study in SA2.

Observation 7: There is no IP tunnel between OAM and the UE in current system architecture. Introduction of IP tunnel between the UE and OAM requires huge standardization and implementation effort and requires SA5’s evalution.

Proposal 3: The delivery mechanism of the AI/ML model from the UE-side server to the model storage entity within the 3GPP network should be regarded as outside the scope of 3GPP and does not require standardization.

Proposal 4: From RAN2 perspective, Solution 2a and 2b for model transfer are deprioritized in Rel-19.

Proposal 5: From RAN2 perspective, Solution 3b for model transfer are deprioritized in Rel-19.

Proposal 6: From RAN2 perspective, solution 4b with UP tunnel between OAM and the UE is deprioritized in Rel-19.

Proposal 7: RAN2 focus on the analysis on model transfer/delivery Solution 1a, 1b, Solution 3a and solution 4b (OAM transfers/delivers AI/ML models to UE via gNB where RRC signaling is used between gNB and UE).

Evaluation on solution 1a, 1b, 3a and 4b for case z1

Observation 8: In case z1, network controllability of AI/ML model transfer is key to delivering optimized performance for for the scenario/configuration-specific models, ensuring efficient use of resources and streamlined operations, maintaining the security and integrity of AI/ML models and facilitating effective network storage management.

Observation 9: The size of the AI model will be in the order of megabytes, and currently, the size of the RRC message is insufficient to carry the AI model.

Observation 10: Increasing the number of segments for RRC messages introduces challenges such as added complexity, higher latency, compatibility issues, and potential obstruction of other traffic and RRC procedures. Meanwhile, terminating a DRB at the gNB instead of the UPF marks a significant architectural shift, potentially resulting in interoperability issues, compliance challenges, and difficulties in environments with multiple vendors.

Proposal 8: For case z1, network should have full control on model transfer from network to the UE over the air interface including the initiation, termination and management of model transfer.

Proposal 9: RAN2 discuss whether to introduce a specific radio bearer dedicated to model transfer, which would be capable of handling large AI/ML models and would terminate between the UE and either the gNB or LMF.

# Reference

[1] RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm

[2] 3GPP TR 38.843 V18.0.0

[3] SP-241383, New SID on Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML), vivo(rapporteur), MediaTek Inc.(rapporteur)