))3GPP TSG-RAN WG2 Meeting #127-bis	R2-2408488

Hefei, China, October 14-18, 2024 	revision of R2-2406606

Agenda Item:	8.8.4

Source:	THALES

Title:	Discussion on MBS Broadcast service area signaling

Document for:	Decision

# 1	Introduction

The approved work item justifications and objectives for NR NTN Evolution in Rel-19 [1] emphasizes the following:

3) MBS feature provides an important add-value for NR NTN system, leveraging the large coverage of the NTN compared to TN. Terrestrial MBS features are equally available for NR NTN in the 5G specifications, but for some cases the intended service area is expected to be smaller than the coverage of a Uu cell, some enhancements need to be done to notify the service area of a Broadcast service.

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]** - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

In this paper, we discuss how to encode and signal the MBS broadcast service area in a NTN cell.

# 2	Discussion

### MBS Broadcast session intended service area

From the system architecture point of view, the MBS broadcast session can already be linked to a specific area as defined in [2]-§6.2.3 “Location dependent MBS service”. The MBS session creation is described in [2]-§7.1.1.2. When the location dependent MBS Session (e.g. broadcast) is announced (step 7. in Figure 7.1.1.2-1), the MBS service area information associated to MBS Session ID e.g. TMGI (Temporary Mobile Group Identifier) is announced by the AF (Application Function) to the UEs. The MBS service area information can be Cell ID list, TAI list, geographical are information or civic address. It was already agreed in RAN2#127 that the intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

Figure 1 - MBS Broadcast location-dependent sessions

In Figure 1 is depicted to the typical architecture for MBS broadcast location-dependent. The MBS session area is defined by the AF, transmitted by the MB-SMF (associated with MBS Session ID) to UE through the NG-RAN. What is new for NTN, is the MBS broadcast service area broadcasting to the UE (illustrated by the colors green, purple, and orange) in the physical cell (blue). The MBS session#1 shows that an intended service area can be larger than a physical cell. The MBS session#2 shows that an intended service area can be smaller than a cell and in this example, it could corresponds to one physical satellite beam belonging to the physical cell. The MBS session#3 is positioned to cover a specific geographical region (e.g. a country) and covers multiple cells.

Observation 1	The intended geographical service area for MBS broadcast could be larger than physical cell, smaller or astride multiple cells

Observation 2	The intended service area could correspond to a satellite physical beam in a cell and therefore best represented by a circle

Observation 3	The intended service area could correspond to a geographical region with borders and therefore best represented by a polygon

### Signaling of the intended service area

The standard already support the broadcast in a geographical area [5]-§16.10.1 [2]-§6.2.3, it is location dependent MBS service. The MBS Session ID is used but a different Area Session ID is used for each MBS service area. However, this mechanism is related to the content distribution while UEs are only aware of the MBS Session ID. The MBS Service Area is associated to a list of Cell ID or TAI. It is also defined the Local MBS service [2]-§6.2.2 where only UEs within the MBS service area may receive content data while UEs outside the MBS service area are not allowed to receive location specific content. For local MBS service, the Service Announcement may include the MBS service area. The MBS service area used by AF can be Cell ID list, TAI list, geographical area information or civic address information. The information is provided to the MBSF Client, but for the WID objective, it is no sufficient, the UE shall be informed that a MBS broadcast session is available in the area without proceeding to a reception attempt.

Observation 4	Location dependent MBS service area is used to distribute content to NG-RAN nodes based on list of Cell ID or TAI. FFS if geographical area defined in Rel-19 should be added to Service Area ID used for location dependent MBS service.

Observation 5	The local MBS service procedure is able to restrict reception of a content based on geographical area information, but at the MBSF client level.

Figure 2 - MBS broadcast session acquisition

The UE acquires necessary configurations for MCCH for MBS broadcast reception from SIB20 and SIB21.  After that, UE acquires configuration for MTCH from MCCH. RAN2 (RAN2#126 agreement) considers to include the service area information in SIB20, SIB21 or MBSBroadcastConfiguration.

As illustrated in Figure 5, when a UE enters a cell, SIB20 contains the information required to acquire the MCCH/MTCH configuration for MBS broadcast. MCCH contains the MBSBroadcastConfiguration, including the MBS-SessionInfoList. The IE provides the configuration of each MBS session provided by MBS broadcast in the current cell.

Observation 6	The intended service area information can be signaled either on SIB20 or MCCH (i.e. MBSBroadcastConfiguration)

The system can already use a cell-based content distribution based on existing location depend MBS procedures. The problem to solve comes when the cell is large, covering multiple area, e.g. in GEO. The SIB20 is expected to be broadcasted in the cell, so the UE cannot discriminate directly if it is an area where a MBS broadcast service is available, contrary to TN where SIB20 is broadcasted only when a MBS service is available. This information should be beneficial to the UE in terms of energy consumption, since, it does not have to decode MCCH to decide if a MBS Broadcast service is available in the area (different to the cell-level). At the SIB20 level, the MBS session ID associated to an intended service area is not necessary. Therefore, the intended service area signaled in SIB20 could be an addition of all MBS broadcast service intended area in the cell.

Proposal 1		Prioritize to signal MBS broadcast services intended service area in SIB20. The intended service area corresponds to the addition of all intended service areas of all MBS Broadcast services in the cell, not associated to a MBS Session ID.

Observation 7	By default, if the SIB20 does not contain an intended service area, then all MBS Broadcast services are available in the whole cell.

After, there is information related to a specific MBS broadcast session, containing in MBSBroadcastConfiguration. The intended service area can be associated to a MBS session ID in the MBS-SessionInfo IE. It would allow the UE to discriminate the MBS services in the area before informing the upper layer i.e. MBSF client.

Proposal 2		The MBS broadcast intended service area associated to a MBS Session ID is optionally signaled in the MBSBroadcastConfiguration. FFS: how to align the intended service area signaled in SIB20 and MBSBroadcastConfiguration according to the different periodicity of SIB20 and MBSBroadcastConfiguration

### Geographical Area coding

CMAS (Commercial Mobile Alert System) is used as Public Warning System (PWS) to deliver multiple, concurrent warning text messages. This emergency alerts broadcast can be either within the whole network or within certain geographical areas, down to the size of a single cell. In SIB8 for CMAS information broadcast, the geographical area can be defined to define where the CMAS message is valid:

SIB8 ::= SEQUENCE { messageIdentifier BIT STRING (SIZE (16)), serialNumber BIT STRING (SIZE (16)), warningMessageSegmentType ENUMERATED {notLastSegment, lastSegment}, warningMessageSegmentNumber INTEGER (0..63), warningMessageSegment OCTET STRING, dataCodingScheme OCTET STRING (SIZE (1)) OPTIONAL, -- Cond Segment1 warningAreaCoordinatesSegment OCTET STRING OPTIONAL, -- Need R lateNonCriticalExtension OCTET STRING OPTIONAL, ... }

Especially, the warningAreaCoordinatesSegment IE defines the geographical coordinate. This information element conveys the description of Warning Area Geometries specified by the Alert Originator to the mobile device to perform geo-fencing on the alert [4]. The use case is very similar to the one of the intended service area for MBS Broadcast where the network wants to indicate to the UE geographical zones associated to a MBS Session. The [3]-§9.3.63 defines the Warning Area Coordinates IE as follow:

The Warning Area Coordinates IE indicates alert area coordinates of a warning message. The eNB and NG-RAN node pass this information to the UE. The UE uses this information to conditionally display (see clause 9.1.3.4.2 and clause 9.1.3.5.2) the warning message contents.

The Warning coordinates are coded as a Tag-Length-Value finite bit string as specified in [4]-§5.2:

Figure 3 - Warning Area Coordinates

Where:

    - Tag (4 bits): Indicates the type of parameter. 0010 for polygon and 0011 for circle.

    - Length (10 bits): Indicates the length of the entire WAC TLV in octets.

    - Value (variable number of octets, up to 1022): The value of the parameter.

This coding offers the possibility to indicate coordinates using polygons or circles to define intended service area. The Warning Area Coordinate for polygon (tag 0010) is given below:

Figure 4 - Warning Area Coordinates polygon

Each vertex adds 24 bits (3 bytes) to the IE size to be added to the 2 bytes of the header. Since the maximum IE’s value size is 1020 bytes, 340 vertices can define a geographical area. A square defined geographical coordinates size is 5 (the first and the last point should match) x 3 bytes + 2 bytes equal 17 bytes.

The coordinates for circles (tag 0011) is given below:

Figure 5 - Warning Area Coordinates circle

The size of a coordinates with circles is 10 bytes.

The others tags are not expected to be used (geo-fencing maximum wait time and CMSP defined use).

Observation 8	SIB8 for CMAS signaling uses the Warning Area Coordinated IE to encode geographical region

In Rel-18 was defined CoverageAreaInfo for TN cell broadcasting in NTN via SIB 25 as follow:

SIB25-r18 ::= SEQUENCE { coverageAreaInfoList-r18 CoverageAreaInfoList-r18 OPTIONAL, -- Need R lateNonCriticalExtension OCTET STRING OPTIONAL, ... } CoverageAreaInfoList-r18 ::= SEQUENCE (SIZE (1..maxTN-AreaInfo-r18)) OF CoverageAreaInfo-r18 CoverageAreaInfo-r18 ::= SEQUENCE { tn-AreaId-r18 TN-AreaId-r18, tn-ReferenceLocation-r18 ReferenceLocation-r17, tn-DistanceRadius-r18 INTEGER(0..65535) }

It looks very similar to the WAC for circle. The circle can be sufficient to meet the WID objectives since we can define area smaller or larger than a beam footprint.

The distance radius is coded on 16 bits instead of 20 bits for WAC. The argument to choose CoverageAreaInfo instead of WAC for intended service area coding could not be determined only on the impact of the SIB size whilst the size difference for a circle-type encoding with the two methods are similar.

The main argument to re-use the Warning Area Coordinates coding is the possibility to define either circle or polygon for the intended service area, according to what is provided by the MBS broadcast AF for the location at the MB-SMF. The same coding to the content provider to the UE through the SI message is used for indicating the intended service area, as it is already in use for PWS [3].

Proposal 3		Re-use the Warning Area Coordinates coding for MBS Broadcast service intended area

We propose the following for the intended service area signaling to the SIB20:

-- ASN1START -- TAG-SIB20-START SIB20-r17 ::= SEQUENCE { mcch-Config-r17 MCCH-Config-r17, cfr-ConfigMCCH-MTCH-r17 CFR-ConfigMCCH-MTCH-r17 OPTIONAL, -- Need S lateNonCriticalExtension OCTET STRING OPTIONAL, ..., [[ cfr-ConfigMCCH-MTCH-RedCap-r18 CFR-ConfigMCCH-MTCH-r17 OPTIONAL, -- Need S mcch-ConfigRedCap-r18 MCCH-Config-r17 OPTIONAL -- Cond CFR-RedCap ]], [[ mbs-Broadcast-ServiceArea-r19 MBS-Broadcast-ServiceArea-r19 OPTIONAL ]] } MBS-Broadcast-ServiceArea-r19 ::= SEQUENCE { warningAreaCoordinatesSegment OCTET STRING, geofencing-Preference-r19 ENUMERATED {intended, unintended} } -- TAG-SIB20-STOP -- ASN1STOP

Table 1 - SIB20 with proposed serviceAreaList

Where:

    - MBS-Broadcast-ServiceArea-R19 is the area in the cell where MBS Broadcast service is present

    - warningAreaCoordinatesSegment as defined for SIB8, coding defined in [4]

    - geofencing-Preference-r19 enables to tell that the service intended area is inside the coordinates or outside the coordinate

Proposal 4		Consider the SIB20 update presented in Table 1 to support MBS broadcast service intended area for Rel-19 NTN. FFS the maximum number of intended service area that can be broadcasted in the SIB

# 3	Conclusion

In the previous sections we made the following observations and proposals:

< MBS Broadcast session intended service area>

Observation 1	The intended geographical service area for MBS broadcast could be larger than physical cell, smaller or astride multiple cells

Observation 2	The intended service area could correspond to a satellite physical beam in a cell and therefore best represented by a circle

Observation 3	The intended service area could correspond to a geographical region with borders and therefore best represented by a polygon

- < Signaling of the intended service area>

Observation 4	Location dependent MBS service area is used to distribute content to NG-RAN nodes based on list of Cell ID or TAI. FFS if geographical area defined in Rel-19 should be added to Service Area ID used for location dependent MBS service.

Observation 5	The local MBS service procedure is able to restrict reception of a content based on geographical area information, but at the MBSF client level.

Observation 6	The intended service area information can be signaled either on SIB20 or MCCH (i.e. MBSBroadcastConfiguration)

Proposal 1		Prioritize to signal MBS broadcast services intended service area in SIB20. The intended service area corresponds to the addition of all intended service areas of all MBS Broadcast services in the cell, not associated to a MBS Session ID.

Observation 7	By default, if the SIB20 does not contain an intended service area, then all MBS Broadcast services are available in the whole cell.

Proposal 2		The MBS broadcast intended service area associated to a MBS Session ID is optionally signaled in the MBSBroadcastConfiguration. FFS: how to align the intended service area signaled in SIB20 and MBSBroadcastConfiguration according to the different periodicity of SIB20 and MBSBroadcastConfiguration

- < Geographical Area coding>

Observation 8	SIB8 for CMAS signaling uses the Warning Area Coordinated IE to encode geographical region

Proposal 3		Re-use the Warning Area Coordinates coding for MBS Broadcast service intended area

Proposal 4		Consider the SIB20 update presented in Table 1 to support MBS broadcast service intended area for Rel-19 NTN. FFS the maximum number of intended service area that can be broadcasted in the SIB

# References

RP-241789, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN#105, 2024-09, Melbourne, AU.

3GPP TS 23.247, “Architectural enhancements for 5G multicast-broadcast services”

3GPP TS 23.041, “Technical realization of Cell Broadcast Service (CBS)”

ATIS 0700041, “WEA 3.0: Device-Based Geo-Fencing”

3GPP TS 38.300, “NR; NR and NG-RAN Overall Description; Stage 2”, Rel-18.

3GPP TS 26.502, “5G multicast-broadcast services; User service architecture”, Rel-18.

SIB20 contains the information required to acquire the MCCH/MTCH configuration for MBS broadcast

SIB21 contains the mapping between the current and/or neighbouring carrier frequencies and MBS Frequency Selection Area Identities (FSAI)

MBSBroadcastConfiguration: message contains the control information applicable for MBS broadcast services transmitted via broadcast MRB