3GPP TSG-RAN2#127bis	R2-2408491

Hefei, China, Oct 14-18, 2024

Agenda item:		8.7.6

Source:		Lenovo

Title:		 XR Rate Control

Document for:		Discussion and Decision

- Introduction

For XR traffic, it would be beneficial to enable the application to adapt the codec rate based on the network conditions to avoid dropping of packets over the air interface. Such discussion has been ongoing in the RAN Plenary with the following being agreed as the way forward in the last meeting, RAN #105 [1].

- Specify in MAC layer XR rate control signaling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

This contribution looks at how the XR rate control information may be signaled in the downlink to support source rate adaption to uplink congestion.

- Discussion

Currently, there already exists a mechanism in NR for bitrate control on a per LCH basis. The Recommended Bitrate (RBR) MAC CE can be used by the gNB to indicate a new bitrate to the UE to combat for uplink congestion or other changes in network conditions that may impact the data transmission. This MAC CE is a 2-byte one with 6-bits for Logical Channel ID (LCID) and 6-bits for the indication of the recommended bitrate. The bitrate value is indicated by means of an index and a bitRateMultiplier component [2]. The index points to a bitrate value ranging from 0 to 8000kbps and the bitRateMultiplier can take values 1, 40, 70, 100 and 200 making the total bitrate value ranging from 0 to 1.6Gbps.

- The Recommended Bitrate MAC CE can indicate bitrate values ranging from 0 to 1.6Gbps.

For XR applications that have very strict requirements in terms of data rate and latencies, it is beneficial for the application to be able to adapt the source rates based on the congestion and network conditions on the uplink. During the plenary meetings, many companies have been discussing whether the Recommended Bitrate MAC CE can be reused for XR applications. In our view, the existing Recommended Bitrate MAC CE may pose certain inefficiencies with respect to XR:

- Granularity:

Currently the Recommended Bitrate MAC CE is signalled on a per LCH basis, i.e., on a per DRB basis. In the case of XR, multiple QoS flows may be mapped to the same DRB. Upon receiving a RBR MAC CE, the UE cannot differentiate the bitrate for each QoS flow which can impact the upper layer throughput.

- Bitrate Range:

Although the RBR MAC CE can cover up to 1.6Gbps, the granularity of the values may not be sufficient for XR applications. With only 56 indices and a single multiplier, the recommended bitrate value may not accurately match the bitrate required by the application which can degrade the user-experience.

- The UE cannot differentiate the bitrate for each QoS flow of a DRB from the RBR MAC CE.

- The range of bitrate values of the legacy RBR MAC CE may not be sufficient for XR applications.

In order to avoid major changes/ additions in the specs during the final release of 5G, it may make sense to reuse the existing RBR MAC CE. But based on the above discussion, such a solution may only be feasible if the mapping between a QoS flow to DRB is one to one. Hence, in order to throttle the throughput accurately and prevent disrupting user-experience, we think it would only be useful to reuse the existing RBR MAC CE if no more than one QoS flow is mapped to a single DRB.

RAN2 to discuss whether legacy RBR MAC CE is reused for UL rate control for a UE when no more than one QoS flow is mapped to a single DRB.

- Conclusion

Observation 1: The Recommended Bitrate MAC CE can indicate bitrate values ranging from 0 to 1.6Gbps.

Observation 2: The UE cannot differentiate the bitrate for each QoS flow of a DRB from the RBR MAC CE.

Observation 3: The range of bitrate values of the legacy RBR MAC CE may not be sufficient for XR applications.

Proposal 1: RAN2 to discuss whether legacy RBR MAC CE is reused for UL rate control for a UE when no more than one QoS flow is mapped to a single DRB.

- References

[1] RP-242390: Way Forward on WID Scope Revision, Nokia

[2] 3GPP TS 38.321, Medium Access Control (MAC) protocol specification.