3GPP TSG-RAN WG2 Meeting #127Bis	R2-2408496

Heifei, CN, 21 – 25 October, 2024

Agenda Item:	8.7.4.2

Source: 	Canon Research Centre France

Title:	Discussion on Delay status report

Document for:	Discussion and decision

# Introduction

During RAN2#127 in Maastricht the following agreements related to DSR were recorded in [2]:

Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)

# Discussion

During last meeting it was agreed to enhance delay status reporting with multiple pairs of data volume and remaining time.

In this paper we discuss two FFS:

- FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds => section 2.1

- FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR => 2.2

## Trigger and reporting thresholds

As agreed during last meeting, there is a single triggering threshold and multiple reporting thresholds.

Three profiles can be considered regarding the relationship between the triggering threshold and the reporting thresholds:

- The triggering threshold is smaller than, or equal to, the smaller reporting threshold (e.g., a reporting threshold which is greater than at least one other reporting threshold, e.g., the greatest reporting threshold). => the status report is configured with a first pair which reports critical data (e.g., the most critical data), and the other pairs are arranged to report the details of less critical data (e.g., data which is less critical than the data reported by the first pair)

- All reporting thresholds may be lower than, or equal to, the triggering threshold. => each of the pairs of the status report may be configured to report critical data. Critical data may be refined through multiple pairs, which thereby provides more accurate (i.e., detailed) reporting of the critical data

- The triggering threshold is between two (different) reporting thresholds (e.g., the greatest reporting threshold and the smallest reporting threshold). For example, the triggering threshold may be configured to be equal to the mean value of the two reporting thresholds. => the status report includes three pairs, the first two pairs are for reporting critical data (e.g., T1, N1a and T0.5, N1b, respectively) and the third pair is for reporting less critical data (e.g., T2, N2)

Figure 2-1 Three profiles of trigger and report thresholds

As illustrated in Figure 2-1, the content and the semantic of the DSR may be different depending on the considered profile. According to the kind of information sought by the gNB, a different configuration option of the thresholds can be submitted to the UE.

In release 18, the triggering threshold is configured by RRC as “remainingTimeThreshold”. Also in release 18, the reporting threshold is represented by “delay-critical” data definition referring to remainingTimeThreshold:

Delay-critical PDCP SDU: if pdu-SetDiscard is not configured, a PDCP SDU for which the remaining time till discardTimer expiry is less than the remainingTimeThreshold. If pdu-SetDiscard is configured, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThreshold.

Observation 1: In release 18, triggering threshold and reporting threshold have the same value.

For release 19, depending on the relationship between the triggering threshold and the reporting thresholds, the report will have different semantics, leading to different advantages and drawbacks.

| Profile name | Relation between triggers | Pro | Cons |
| --- | --- | --- | --- |
| Fine grained delay critical | Triggering threshold >= higher report threshold | Delay critical data are accurately reported in multiple pairs | Potential future delay critical data are not reported |
| Mixed | Lower report threshold < Triggering threshold < higher report threshold | Balance between delay critical and potential future delay critical |  |
| Potential future delay critical | Triggering threshold <= Lower report threshold | Potential future delay critical data are reported fine grained in multiple pairs | Delay critical data are reported in a single pair |

Table 2-1 Comparison between different threshold settings

Observation 2: Each setting of reporting and triggering thresholds may be advantageous to certain scenario

Proposal 1: Let the network configure the triggering and reporting thresholds without constraints.

Communication environment may change multiple times during an XR session, and a gNB might need different type of DSR information depending on changing conditions. It can be advantageous for the network (gNB) to be able to change dynamically the relationship between the triggering threshold and the different reporting thresholds.

For example (option a), a plurality of different profiles (e.g., such as those defined in Table 2-1) can be statically configured by the network at the UE. A first profile may be set as the default profile, the network being able to switch between the plurality of profiles dynamically (e.g., by MAC CE).

Another possibility (option b) the UE may be configured to autonomously choose between a set of profiles configured by the network.

Finally (option c), there is also the possibility to let the network configure dynamically both the triggering and the reporting thresholds.

Proposal 2: Study the possibility to dynamically configure different triggering and reporting thresholds among three options:

- Option a: static profiles, dynamic switch between profiles controlled by the network

- Option b: static profiles, autonomous UE switching between configured profiles

- Option c: dynamic thresholds setting by the network

## Non delay critical data

Both delay critical and non-delay critical data coexist in one LCH, and it can happen that non-delay critical data are present ahead of delay critical data in the TX buffer (Head Of Line non-delay critical data). In previous meeting, it has been discussed whether to include or not the HOL non-delay critical data into the count of delay critical data ([3]).

As explained in previous section, non-delay critical data can be taken into account when using an appropriate profile of the relationship between thresholds (triggering, reporting) i.e. options 1 or 3. For example, Figure 2-1 shows that N2 data volume which is included in the TX buffer, ahead of other data volumes, can be reported with its actual remaining time T2 and it does not have to be counted as  delay critical data because the T1 remaining time is reported together with the N1 data volume. Further, since it is possible to dynamically change the profiles, we do not think that we need further enhancements other than threshold profile management described earlier to handle the reporting of HOL non-delay critical data.

Observation 3: Non-delay critical data, including data ahead of critical data in the buffer, can be taken into account in the DSR when using the appropriate profile of the relationship between the triggering and the reporting thresholds.

.

However, the potential presence of non-delay critical data ahead in the buffer of an LCH may cause issues with the triggering of DSR as illustrated in next figure:

Figure 2-2 Triggering issue

First, a TX buffer is represented containing four different data volumes (N1 to N4). N2 is ahead in the TX buffer, followed by N1, N3 then N4. The remaining times associated to each data quantities are noted from smallest to largest T1 to T4. T1 corresponds also to the triggering threshold.

When N1 volume reached T1 remaining time, the DSR is triggered and the following pairs are reported: (T1, N1), (T2, N2) and (T3, N3).

After a while, as illustrated in the lower half of the figure, N2 volume is entirely sent and N1 volume is partially sent with a remaining data volume equal to N1’. At that point, non-delay critical data N2, has been served first because it was at the head of the TX buffer. A consequence is that delay-critical data N1’ are still present in the TX buffer and the network has no knowledge of that situation. The network will not get any new DSR information until a new data volume for reporting reaches the triggering threshold T1. Indeed, release 18 specifications [1] states that DSR can only be triggered by data that have not been previously reported in a DSR. In this example, new DSR will not be triggered until the N4 volume, currently at the T3 remaining time, reaches the T1 remaining time.

Observation 4: The network may have no knowledge of non-delay critical data being transmitted before delay-critical data

During the past meeting, some LCP related papers ([6],[7]) suggested that delay-critical data could be prioritized when sending data from one TX buffer containing a mix of delay-critical and non-delay-critical data. That would have solved this issue but it was also dismissed in other papers as being too complex to implement ([4],[5]).

One simple solution would be to allow the delay critical data which have already been reported in a previous DSR to trigger a new DSR under certain conditions. For example, triggering of a new DSR by delay critical data that have been already reported, would be allowed if non-delay-critical data were ahead in the TX buffer and delay critical data have not been sent over a given amount of time. In the example of Figure 2-2, the UE would know that N1’ delay-critical data remain in its TX buffer, and the UE knows that new DSR will be triggered not earlier than T3-T1 when N4 data volume will become data-critical, then it can decide that if N1’ is not sent during a waiting period and that T3-T1 is too long, then a new DSR is triggered.

Observation 5: The proposed enhancement of trigger conditions allows to accurately inform the network about remaining time-critical data with no extra complexity in buffer management.

Proposal 3: Enhance release 18 DSR trigger conditions to allow delay-critical data already reported in a DSR to trigger a new DSR under specific conditions e.g. non-delay-critical data were present ahead in the TX buffer

If the new trigger is adopted then it can be useful to indicate in the DSR which data volume has already been reported but is still pending for transmission and which data volume is new.

Proposal 4: In the DSR each reported pair has an indication to discriminate newly reported and already reported data volume

A third solution would consist in sorting the pairs in the DSR message according to their respective position in the TX buffer. In this case, the position of the pair in the DSR message would reflect the position of the data volume in the TX buffer and the network would at least be aware that non-delay-critical data are ahead in the Tx buffer. But it may be complex for implementation to keep track or to have knowledge of each data volume’s position in the transmit buffer. Also, this would not work if the data volumes are not contiguous in the TX buffer but interleave with each other.

Observation 6: Enhancing the triggering condition is the only solution that has no impact on TX buffer implementation.

# Conclusion

According to the analysis in section 2, it is proposed:

Observation 1: In release 18, triggering threshold and reporting threshold have the same value.

Observation 2: Each setting of reporting and triggering thresholds may be advantageous to certain scenario

Proposal 1: Let the network configure the triggering and reporting thresholds without constraints.

Proposal 2: Study the possibility to dynamically configure different triggering and reporting thresholds among three options:

- Option a: static profiles, dynamic switch between profiles controlled by the network

- Option b: static profiles, autonomous UE switching between configured profiles

- Option c: dynamic thresholds setting by the network

Observation 3: Non-delay critical data, including data ahead of critical data in the buffer, can be taken into account in the DSR when using the appropriate profile of the relationship between the triggering and the reporting thresholds.

Observation 4: The network may have no knowledge of non-delay critical data being transmitted before delay-critical data

Observation 5: The proposed enhancement of trigger conditions allows to accurately inform the network about remaining time-critical data with no extra complexity in buffer management.

Proposal 3: Enhance release 18 DSR trigger conditions to allow delay-critical data already reported in a DSR to trigger a new DSR under specific conditions e.g. non-delay-critical data were present ahead in the TX buffer

Proposal 4: In the DSR each reported pair has an indication to discriminate newly reported and already reported data volume

Observation 6: Enhancing the triggering condition is the only solution that has no impact on TX buffer implementation.

# References

- NR; Medium Access Control (MAC) protocol specification, (Release 18)

- R2-2407575	Report from session on R18 MBS, R18 QoE and R19 XR	Session chair (Huawei)

- R2-2407047	Discussion on scheduling enhancements	Ericsson

- R2-2406741	Discussion on XR scheduling enhancements	China Telecom

- R2-2406784	Scheduling enhancements for Rel-19 XR	Samsung

- R2-2406989	Further discussion on scheduling enhancement on XR	CMCC

- R2-2406371	Discussion on delay-aware LCP enhancement	TCL